3GPP TSG-RAN WG5 Meeting #98	R5-230335

Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0170 | rev | - | Current version: | 16.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of BDS B2a and B3I signal test contents in TS 37.571-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduce the global B2a and B3I signals in the network-assisted BDS System, as part of A-GNSS positioning methods in LTE and NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating GNSS Auxiliary Information in Table 5.4.1.1-1. - Adding pc_BDS_B2a and pc_BDS_B3I in MultiFreqAGNSS condition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test spec will not applicable to BDS B2a and B3I signal testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 7.3.4.2.3.3, 7.3.4.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

5.4.1	Default assistance data information elements

5.4.1.1	GNSS Assistance Data Elements

Table 5.4.1.1-1 defines the GNSS assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message. The GNSS assistance data provided depends on the mode being used in the test case, the assistance data supported by the UE and the GNSSs supported by the UE. GNSS assistance data IEs not supported by the UE shall not be sent. GNSS assistance data IEs supported by the UE but not listed in Table 5.4.1.1-1 shall not be sent. The content of the assistance data elements is defined in 37.571-5 [12] clause 7.

Table 5.4.1.1-1: GNSS assistance data to be provided to the UE

| GNSS Assistance Data IE supported by UE | Mode used in test case |  |  |
| --- | --- | --- | --- |
|  | UE-based | UE-assisted, GNSS-AcquisitionAssistance supported by UE | UE-assisted, GNSS-AcquisitionAssistance not supported by UE |
| GNSS-Reference Time | Yes | Yes | Yes |
| GNSS-ReferenceLocation | Yes | No | Yes |
| GNSS-IonosphericModel | Yes | No | No |
| GNSS-TimeModelList | Yes(1) | No | Yes(1) |
| GNSS-NavigationModel | Yes | No | Yes |
| GNSS-AcquisitionAssistance | No | Yes | No |
| GNSS-Almanac | No | No | Yes |
| GNSS-UTC-Model | Yes(3) | Yes(3) | Yes(3) |
| GNSS-AuxiliaryInformation | Yes(2) | Yes(2) | Yes(2) |
| NOTE 1: Only if more than one GNSS supported by the UE. NOTE 2: Only if GLONASS supported by the UE, and/or if the UE supports multiple signals per GNSS, and/or if BDS B1C/B2a signal type supported by the UE. NOTE 3: Only if GLONASS and at least one other GNSS supported by the UE. |  |  |  |

< Unchanged sections omitted >

# 7.3.4.2.3.3	Specific message contents

Table 7.3.4.2.3.3-1: RESET UE POSITIONING STORED INFORMATION (step 1, Table 7.3.4.2.3.2-1)

| Derivation Path: 36.509 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | Sub-tests 7, 15: 0 0 0 0 0 0 0 0 Sub-test 16: 0 0 0 0 0 0 1 0 Sub-test 17: 0 0 0 0 0 0 1 1 Sub-test 18: 0 0 0 0 0 1 0 1 | Sub-tests 7, 15: GNSS Sub-test 16: MBS Sub-test 17: WLAN Sub-test 18: Sensor |  |

Table 7.3.4.2.3.3-2: RESET UE POSITIONING STORED INFORMATION (step 1a, Table 7.3.4.2.3.2-1)

| Derivation Path: 36.509 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | 0 0 0 0 0 0 0 1 | OTDOA |  |

Table 7.3.4.2.3.3-3: DLInformationTransfer (steps 1b, 1d, 2, 3, 4a2, 4b2 and 4b4, Table 7.3.4.2.3.2-1)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| dlInformationTransfer-r8 SEQUENCE { |  |  |  |  |
| dedicatedInfoType CHOICE { |  |  |  |  |
| dedicatedInfoNAS OCTET STRING |  | Set according to Table 7.3.4.2.3.3-4 | DOWNLINK GENERIC NAS TRANSPORT |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.4.2.3.3-4: DOWNLINK GENERIC NAS TRANSPORT (steps 1b, 1d, 2, 3, 4a2, 4b2 and 4b4, Table 7.3.4.2.3.2-1)

| Derivation Path: 24.301 Table 8.2.31.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 0111 | EPS mobility management messages |  |
| Security header type |  | 0000 | Plain NAS message |  |
| Downlink generic NAS transport message identity |  | 01101000 | Downlink generic NAS transport |  |
| Generic message container type |  | 00000001 | LTE Positioning Protocol (LPP) message container |  |
| Generic message container |  | Step 1b: Set according to Table 7.3.4.2.3.3-4a | LPP Request Capabilities. |  |
|  |  | Step 2: Set according to Table 7.3.4.2.3.3-5 | LPP Provide Assistance Data |  |
|  |  | Step 3: Set according to Table 7.3.4.2.3.3-6 | LPP Request Location Information |  |
|  |  | Steps 1d, 4a2, 4b2 and 4b4: Set according to Table 7.3.4.2.3.3-10 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/ Correlation ID |  |

Table 7.3.4.2.3.3-4a: LPP Request Capabilities (step 1b, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 5.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 5.4-1 |  |  |  |  |

Table 7.3.4.2.3.3-5: LPP Provide Assistance data (step 2, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 5.4-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 5.4-2 with the following exceptions: |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) |  |  |
| } |  |  |  |  |

Table 7.3.4.2.3.3-6: LPP Request Location Information (step 3, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 5.4-3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| As defined in Table 5.4-3 with the following exceptions: |  |  |  |  |  |  |  |
| locationInformationType |  | locationMeasurementsRequired |  |  |  |  |  |
| qos SEQUENCE { |  |  |  |  |  |  |  |
| horizontalAccuracy |  | Not present |  |  |  |  |  |
| verticalCoordinateRequest |  | FALSE |  |  |  |  |  |
| verticalAccuracy |  | Not present |  |  |  |  |  |
| responseTime SEQUENCE { |  |  |  |  |  |  |  |
| time |  | 32 |  |  |  |  |  |
| responseTimeEarlyFix-r12 |  | Sub-tests 5, 6 FDD, 6 TDD, 11, 12, 13, 14, 15, 16, 17, 18: not present Sub-test 7: 10 |  | Rel-12 onwards |  |  |  |
| } |  |  |  |  |  |  |  |
| velocityRequest |  | FALSE |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| a-gnss-RequestLocationInformation |  | Set according to Table 7.3.4.2.3.3-11 |  |  |  |  |  |

Table 7.3.4.2.3.3-7: ULInformationTransfer (steps 1c, 4 a1, 4b1 and 4b3, Table 7.3.4.2.3.2-1)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ULInformationTransfer ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ulInformationTransfer-r8 SEQUENCE { |  |  |  |  |
| dedicatedInfoType CHOICE { |  |  |  |  |
| dedicatedInfoNAS OCTET STRING |  | Set according to Table 7.3.4.2.3.3-8 | UPLINK GENERIC NAS TRANSPORT |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.4.2.3.3-8: UPLINK GENERIC NAS TRANSPORT (steps 1c, 4 a1, 4b1 and 4b3, Table 7.3.4.2.3.2-1)

| Derivation Path: 24.301 Table 8.2.32.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | 0111 | EPS mobility management messages |  |
| Security header type |  | 0000 | Plain NAS message |  |
| Uplink generic NAS transport message identity |  | 01101001 | Uplink generic NAS transport |  |
| Generic message container type |  | 00000001 | LTE Positioning Protocol (LPP) message container |  |
| Generic message container |  | Step 1c: Set according to Table 7.3.4.2.3.3-8a | LPP Provide Capabilities |  |
|  |  | Steps 4 a1, 4b1 and 4b3: Set according to Table 7.3.4.2.3.3-9 | LPP Provide Location Information |  |
| Additional information |  | Present | The UE includes the Routing Identifier received in the Additional Information IE of the DOWNLINK GENERIC NAS TRANSPORT message (step 1b or 3 Table 7.3.4.2.3.2-1) |  |

Table 7.3.4.2.3.3-8a: LPP Provide Capabilities. (step 1c, Table 7.3.4.2.3.2-1)

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID SEQUENCE { |  |  |  |  |
| initiator |  | locationServer |  |  |
| transactionNumber |  | (0..255) | Contains the same value as the corresponding field in the LPP Request Capabilities message in step 1b, Table 7.3.4.2.3.2-1 |  |
| } |  |  |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | (0..255) |  |  |
| acknowledgement SEQUENCE { |  | Present, or not present |  |  |
| ackRequested |  | TRUE |  |  |
| ackIndicator |  | Not present |  |  |
| } |  |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| provideCapabilities-r9 SEQUENCE { |  |  |  |  |
| commonIEsProvideCapabilities |  | Dependent on UE capabilities | Rel-14 onwards |  |
| a-gnss-ProvideCapabilities |  | Dependent on UE capabilities |  |  |
| otdoa-ProvideCapabilities |  | Dependent on UE capabilities |  |  |
| ecid-ProvideCapabilities SEQUENCE{ |  | Dependent on UE capabilities |  |  |
| ueRxTxSupTDD-r13 |  | Present (TRUE) for sub-test 6 TDD | Rel-13 onwards |  |
| } |  |  |  |  |
| epdu-ProvideCapabilities |  | Not present |  |  |
| sensor-ProvideCapabilities-r13 |  | Dependent on UE capabilities | Rel-13 onwards |  |
| tbs-ProvideCapabilities-r13 |  | Dependent on UE capabilities | Rel-13 onwards |  |
| wlan-ProvideCapabilities-r13 |  | Dependent on UE capabilities | Rel-13 onwards |  |
| bt-ProvideCapabilities-r13 |  | Dependent on UE capabilities | Rel-13 onwards |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.4.2.3.3-9: LPP Provide Location Information (steps 4 a1, 4b1 and 4b3, Table 7.3.4.2.3.2-1)

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { |  |  |  |
| initiator | locationServer |  |  |
| transactionNumber | (0..255) | Contains the same value as the corresponding field in LPP Request Location Information message in step 3, Table 7.3.4.2.3.1-1 |  |
| } |  |  |  |
| endTransaction | Step 4a1, 4b3: TRUE |  |  |
|  | Step 4b1: FALSE |  |  |
| sequenceNumber | (0..255) |  |  |
| acknowledgement SEQUENCE { | Present, or not present |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation-r9 SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation SEQUENCE { | Step 4a1, 4b3: May be present |  |  |
|  | Step 4b1: Present |  |  |
| locationEstimate | Not present |  |  |
| velocityEstimate | Not present |  |  |
| locationError | Not present |  |  |
| earlyFixReport-r12 | Step 4a1, 4b3: Not present | Rel-12 onwards |  |
|  | Step 4b1: Any value acceptable |  |  |
| } |  |  |  |
| a-gnss-ProvideLocationInformation SEQUENCE { | Step 4a1: Present for sub-tests 7, 15 |  |  |
|  | Step 4b1, 4b3: May be present | One of a-gnss-ProvideLocationInformation or otdoa-ProvideLocationInformation shall be present |  |
| gnss-SignalMeasurementInformation SEQUENCE { | Present |  |  |
| measurementReferenceTime | Present. Any value acceptable |  |  |
| gnss-MeasurementList SEQUENCE (SIZE(1..n)) OF SEQUENCE { | Present. SIZE n is the number of GNSSs supported by the UE, one instance for each GNSS supported by the UE |  |  |
| gnss-ID | Present |  |  |
| gnss-SgnMeasList | Present, one instance for each frequency within the GNSS supported by the UE. Any value acceptable |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| gnss-LocationInformation | Not present |  |  |
| gnss-Error | Not present |  |  |
| } |  |  |  |
| otdoa-ProvideLocationInformation SEQUENCE { | Step 4a1: Present for sub-tests 5, 7 |  |  |
|  | Step 4b1, 4b3: May be present | One of a-gnss-ProvideLocationInformation or otdoa-ProvideLocationInformation shall be present |  |
| otdoa-SignalMeasurementInformation | Present. Any value acceptable |  |  |
| otdoa-Error | May be present |  |  |
| } |  |  |  |
| ecid-ProvideLocationInformation SEQUENCE { | Present for sub-test 6 FDD, 6 TDD. |  |  |
| ecid-SignalMeasurementInformation | Present. Any value acceptable |  |  |
| ecid-Error | Not present |  |  |
| } |  |  |  |
| epdu-ProvideLocationInformation | Not present |  |  |
| sensor-ProvideLocationInformation-r13 SEQUENCE { | Present for sub-test 14, 18 | Rel-13 onwards |  |
| sensor-MeasurementInformation-r13 | Present. Any value acceptable |  |  |
| sensor-Error-r13 | May be present |  |  |
| } |  |  |  |
| tbs-ProvideLocationInformation-r13 SEQUENCE { | Present for sub-tests 12, 16 | Rel-13 onwards |  |
| tbs-MeasurementInformation-r13 SEQUENCE { | Present |  |  |
| measurementReferenceTime-r13 | Any value acceptable |  |  |
| mbs-SgnMeasList-r13 | Any value acceptable |  |  |
| } |  |  |  |
| tbs-Error-r13 | Not present |  |  |
| } |  |  |  |
| wlan-ProvideLocationInformation-r13 SEQUENCE { | Present for sub-test 11, 17 | Rel-13 onwards |  |
| wlan-MeasurementInformation-r13 | Present. Any value acceptable |  |  |
| wlan-Error-r13 | May be present |  |  |
| } |  |  |  |
| bt-ProvideLocationInformation-r13 SEQUENCE { | Present for sub-test 13 | Rel-13 onwards |  |
| bt-MeasurementInformation-r13 | Present. Any value acceptable |  |  |
| bt-Error-r13 | May be present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.4.2.3.3-10: LPP Acknowledgement (steps 1d, 4a2, 4b2 and 4b4, Table 7.3.4.2.3.2-1)

| Derivation Path: 36.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |  |
| transactionID |  | Not present |  |  |
| endTransaction |  | TRUE |  |  |
| sequenceNumber |  | Not present |  |  |
| acknowledgement SEQUENCE { |  |  |  |  |
| ackRequested |  | FALSE |  |  |
| ackIndicator |  | (0..255) | Contains the same value of the sequenceNumber field in step 1c or 4 a1 or 4b1 or 4b3, Table 7.3.4.2.3.2-1 |  |
| } |  |  |  |  |
| lpp-MessageBody |  | Not present. |  |  |
| } |  |  |  |  |

Table 7.3.4.2.3.3-11: A-GNSS Request Location Information (step 3, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 5.4-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 5.4-4 with the following exceptions: |  |  |  |  |
| multiFreqMeasReq |  | TRUE, if UE supports multi frequency GNSS |  | MultiFreqAGNSS |

| Condition | Description |
| --- | --- |
| MultiFreqAGNSS | The UE supports at least one of pc_A_GPS_L2C, pc_A_GPS_L5, pc_QZSS_QZS_L1C, pc_QZSS_QZS_L2C, pc_QZSS_QZS_L5, pc_GLONASS_G2, pc_GLONASS_G3, pc_GALILEO_E5a, pc_GALILEO_E5b, pc_GALILEO_E6, pc_GALILEO_E5aE5b, pc_BDS_B1C, pc_BDS_B2a or pc_BDS_B3I |

< Unchanged sections omitted >

# 7.3.4.4.3.3	Specific message contents

Table 7.3.4.4.3.3-1: RESET UE POSITIONING STORED INFORMATION (step 1, Table 7.3.4.4.3.2-1)

| Derivation Path: 36.509 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | Sub-tests 7, 15: 0 0 0 0 0 0 0 0 Sub-test 16: 0 0 0 0 0 0 1 0 Sub-test 17: 0 0 0 0 0 0 1 1 Sub-test 18: 0 0 0 0 0 1 0 1 | Sub-tests 7, 15: GNSS Sub-test 16: MBS Sub-test 17: WLAN Sub-test 18: Sensor |  |

Table 7.3.4.4.3.3-2: RESET UE POSITIONING STORED INFORMATION (step 1a, Table 7.3.4.4.3.2-1)

| Derivation Path: 36.509 clause 6.9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE Positioning Technology |  | 0 0 0 0 0 0 0 1 | OTDOA |  |

Table 7.3.4.4.3.3-3: DLInformationTransfer (steps 1b, 1d, 2, 4, 4b, 5a2, 5b2 and 5b4, Table 7.3.4.4.3.2-1)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| DLInformationTransfer ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| dlInformationTransfer-r8 SEQUENCE { |  |  |  |  |
| dedicatedInfoType CHOICE { |  |  |  |  |
| dedicatedInfoNAS OCTET STRING |  | Set according to Table 7.3.4.4.3.3-4 | DOWNLINK GENERIC NAS TRANSPORT |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.4.4.3.3-4: DOWNLINK GENERIC NAS TRANSPORT (steps 1b, 1d, 2, 4, 4b, 5a2, 5b2 and 5b4, Table 7.3.4.4.3.2-1)

| Derivation Path: 24.301 Table 8.2.31.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  |  | EPS mobility management messages |  |
| Security header type |  | 0000 | Plain NAS message |  |
| Downlink generic NAS transport message identity |  | 01101000 | Downlink generic NAS transport |  |
| Generic message container type |  | 00000001 | LTE Positioning Protocol (LPP) message container |  |
| Generic message container |  | Step 1b: Set according to Table 7.3.4.4.3.3-4a | LPP Request Capabilities |  |
|  |  | Step 2: Set according to Table 7.3.4.4.3.3-5 | LPP Request Location Information |  |
|  |  | Steps 4 and 4b: Set according to Table 7.3.4.4.3.3-10 | LPP Provide Assistance Data |  |
|  |  | Steps 1d, 5a2, 5b2 and 5b4: Set according to Table 7.3.4.4.3.3-12 | LPP Acknowledgement |  |
| Additional information |  | Present | Routing Identifier/ Correlation ID |  |

Table 7.3.4.4.3.3-4a: LPP Request Capabilities (step 1b, Table 7.3.4.4.3.2-1)

| Derivation Path: Table 5.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 5.4-1 |  |  |  |  |

Table 7.3.4.4.3.3-5: LPP Request Location Information (step 2, Table 7.3.4.4.3.2-1)

| Derivation Path: Table 5.4-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 5.4-3 with the following exceptions: |  |  |  |  |
| locationInformationType |  | locationMeasurementsRequired |  |  |
| a-gnss-RequestLocationInformation |  | Set according to Table 7.3.4.4.3.3-6 |  | Sub-tests 7, 15 |
| otdoa-RequestLocationInformation |  | Set according to Table 7.3.4.4.3.3-6a |  | Sub-test 5, 7 |
| qos SEQUENCE { |  |  |  |  |
| horizontalAccuracy |  | Not present |  |  |
| verticalCoordinateRequest |  | FALSE |  |  |
| verticalAccuracy |  | Not present |  |  |
| responseTime SEQUENCE { |  |  |  |  |
| time |  | 32 |  |  |
| responseTimeEarlyFix-r12 |  | Sub-tests 5, 15, 16: not present Sub-test 7: 10 | Rel-12 onwards |  |
| } |  |  |  |  |
| velocityRequest |  | FALSE |  |  |
| } |  |  |  |  |
| sensor-RequestLocationInformation-r14 |  | Set according to Table 7.3.4.4.3.3-6c | Rel-14 onwards | Sub-test 18 |
| tbs-RequestLocationInformation-r13 |  | Set according to Table 7.3.4.4.3.3-6b | Rel-13 onwards | Sub-test 16 |
| wlan-RequestLocationInformation-r14 |  | Set according to Table 7.3.4.4.3.3-6d | Rel-14 onwards | Sub-test 17 |

Table 7.3.4.4.3.3-6: A-GNSS Request Location Information (step 2, Table 7.3.4.4.3.2-1)

| Derivation Path: Table 5.4-4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| As defined in Table 5.4-4 with the following exceptions: |  |  |  |  |  |  |  |
| assistanceAvailability |  | TRUE |  |  |  |  |  |
| multiFreqMeasReq |  | TRUE, if UE supports multi frequency GNSS |  |  |  | MultiFreqAGNSS |  |

| Condition | Description |
| --- | --- |
| MultiFreqAGNSS | The UE supports at least one of pc_A_GPS_L2C, pc_A_GPS_L5, pc_QZSS_QZS_L1C, pc_QZSS_QZS_L2C, pc_QZSS_QZS_L5, pc_GLONASS_G2, pc_GLONASS_G3, pc_GALILEO_E5a, pc_GALILEO_E5b, pc_GALILEO_E6, pc_GALILEO_E5aE5b, pc_BDS_B1C, pc_BDS_B2a or pc_BDS_B3I |