3GPP TSG-RAN5 Meeting #98	R5-230615

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0316 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add applicabilities for new eNS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New eNS test cases are introduced to RAN5#98 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added applicabilities of new eNS test cases 9.1.13.2, 9.3.1.4, 10.1.8.4, and 10.1.8.5. And added new test conditions CXXX for 9.3.1.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicabilities of new eNS test cases will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table 4.1-4a: Applicability of Protocol conformance Mobility and Session management test cases, ref. TS 38.523-1 [2]

| Clause |  | TC Title |  | Release |  | Applicability |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Condition |  | Comment |  |
| 9 |  | Mobility management |  |  |  |  |  |  |  |
| 9.1 |  | 5GS mobility management |  |  |  |  |  |  |  |
| 9.1.1 |  | Primary authentication and key agreement |  |  |  |  |  |  |  |
| 9.1.1.1 |  | EAP based primary authentication and key agreement / EAP-AKA' related procedures |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.1.2 |  | EAP based primary authentication and key agreement / Reject |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.1.3 |  | EAP based primary authentication and key agreement / EAP message transport / Abnormal |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.1.4 |  | 5G AKA based primary authentication and key agreement / 5G-AKA related procedures |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.1.5 |  | 5G AKA based primary authentication and key agreement / Reject |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.1.6 |  | 5G AKA based primary authentication and key agreement / Abnormal |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.2 |  | Security mode control |  |  |  |  |  |  |  |
| 9.1.2.1 |  | NAS security mode command |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.2.2 |  | Protection of initial NAS signalling messages |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.2.3 |  | Integrity protection / Correct functionality of 5G NAS integrity algorithm / SNOW3G |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.2.4 |  | Integrity protection / Correct functionality of 5G NAS integrity algorithm / AES |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.2.5 |  | Integrity protection / Correct functionality of 5G NAS integrity algorithm / ZUC |  | Rel-15 |  | C84 |  | UEs supporting 5G Core and ZUC algorithm |  |
| 9.1.2.6 |  | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / SNOW3G |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.2.7 |  | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / AES |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.2.8 |  | Ciphering and deciphering / Correct functionality of 5G NAS encryption algorithm / ZUC |  | Rel-15 |  | C84 |  | UEs supporting 5G Core and ZUC algorithm |  |
| 9.1.3 |  | Identification |  |  |  |  |  |  |  |
| 9.1.3.1 |  | Identification procedure |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.4 |  | Generic UE configuration update |  |  |  |  |  |  |  |
| 9.1.4.1 |  | Generic UE configuration update / New 5G-GUTI, NITZ, registration requested, network slicing indication, new allowed NSSAI / Acknowledgement from the UE |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5 |  | Registration |  |  |  |  |  |  |  |
| 9.1.5.1 |  | Initial registration |  |  |  |  |  |  |  |
| 9.1.5.1.1 |  | Initial registration / Success / 5G-GUTI reallocation, last visited TAI |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.2 |  | Initial registration / 5GS services / Equivalent PLMN list handling |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.3 |  | Initial registration / 5GS services / NSSAI handling |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.3a |  | Initial registration / 5GS services / NSSAI handling / NSSAI storage |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.4 |  | Initial registration / 5GS services / MICO mode / TAI list handling |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.5 |  | Initial registration / Abnormal / Failure after 5 attempts |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.6 |  | Initial registration / Rejected / Illegal UE |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.7 |  | Void |  |  |  |  |  |  |  |
| 9.1.5.1.8 |  | Initial registration / Rejected / Serving network not authorized |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.9 |  | Initial registration / Abnormal / Change of cell into a new tracking area |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.10 |  | Initial registration / Rejected / PLMN not allowed |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.11 |  | Initial registration / Rejected / Tracking area not allowed |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.12 |  | Initial registration / Rejected / Roaming not allowed in this tracking area |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.13 |  | Initial registration / Rejected / No suitable cells in tracking area |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.14 |  | Initial registration / Rejected / Congestion / Abnormal / T3346 |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.15 |  | Initial registration / Success / Extended and spare fields in CAG information list |  | Rel-15 only |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.1.16 |  | Initial Registration / Success / MUSIM |  | Rel-17 |  | C219 |  | UEs supporting 5G Core and Multi-SIM features |  |
| 9.1.5.2 |  | Mobility and periodic registration update |  |  |  |  |  |  |  |
| 9.1.5.2.1 |  | Mobility registration update / TAI list handling |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.2.2 |  | Periodic registration update / Accepted |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.2.4 |  | Mobility registration update / The lower layer requests NAS signalling connection recovery |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.2.5 |  | Void |  |  |  |  |  |  |  |
| 9.1.5.2.7 |  | Mobility and periodic registration update / Rejected / UE identity cannot be derived by the network |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.2.8 |  | Mobility and periodic registration update / Rejected / Implicitly de-registered |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.5.2.9 |  | Void |  |  |  |  |  |  |  |
| 9.1.6 |  | De-registration |  |  |  |  |  |  |  |
| 9.1.6.1 |  | UE-initiated de-registration |  |  |  |  |  |  |  |
| 9.1.6.1.1 |  | UE-initiated de-registration / Switch off / Abnormal / De-registration and 5GMM common procedure collision |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.6.1.2 |  | UE-initiated de-registration / Normal de-registration / Abnormal / Transmission failure without TAI change from lower layers, de-registration and 5GMM common procedure collision, T3521 timeout |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.6.1.3 |  | UE-initiated de-registration / Abnormal / Change of cell into a new tracking area |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.6.1.4 |  | Void |  |  |  |  |  |  |  |
| 9.1.6.2 |  | Network-initiated de-registration |  |  |  |  |  |  |  |
| 9.1.6.2.1 |  | Network-initiated de-registration / De-registration for 3GPP access / Re-registration required |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.6.2.2 |  | Network-initiated de-registration / De-registration for 3GPP access / Re-registration not required |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.7 |  | Service request |  |  |  |  |  |  |  |
| 9.1.7.1 |  | Service request / Idle mode uplink user data transport / Rejected / Restricted service area, abnormal / T3517, T3525 |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.7.2 |  | Service request / Connected mode user data transport / Abnormal / T3517 |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.7.4 |  | Service request / MUSIM / Rejection of paging |  | Rel-17 |  | C220 |  | UEs supporting 5G Core and Multi-SIM Reject paging request |  |
| 9.1.8 |  | SMS over NAS |  |  |  |  |  |  |  |
| 9.1.8.1 |  | SMS over NAS / MO and MT SMS over NAS / Idle mode |  | Rel-15 |  | C33 |  | UEs supporting 5G Core and SMS over NAS and UE configured to not use SMSoIP |  |
| 9.1.8.2 |  | SMS over NAS / Multiple MO and MT SMS over NAS / Connected mode |  | Rel-15 |  | C33 |  | UEs supporting 5G Core and SMS over NAS and UE configured to not use SMSoIP |  |
| 9.1.9 |  | RACS |  |  |  |  |  |  |  |
| 9.1.9.1 |  | RACS / Network assigned UE radio capability ID |  | Rel-16 |  | C108 |  | UEs supporting 5G Core and RACS |  |
| 9.1.9.2 |  | RACS / UE configuration update / UE radio capability ID |  | Rel-16 |  | C108 |  | UEs supporting 5G Core and RACS |  |
| 9.1.9.3 |  | RACS / PLMN change within registration area / From NW assigned to Manufacturer assigned UE Radio Capability ID |  | Rel-16 |  | C177 |  | UEs supporting 5G Core and RACS and Manufacturer assigned Radio Capability ID |  |
| 9.1.9.4 |  | RACS / USIM change / Handling of URCID |  | Rel-16 |  | C108 |  | UEs supporting 5G Core and RACS |  |
| 9.1.9.5 |  | RACS / Handling of delete indication for NW assigned UE radio capability ID |  | Rel-16 |  | C108 |  | UEs supporting 5G Core and RACS |  |
| 9.1.9.6 |  | RACS / Change in radio capability / NW assigned URCID |  | Rel-16 |  | C108 |  | UEs supporting 5G Core and RACS |  |
| 9.1.9.7 |  | RACS / Inter-system mobility registration update / Handling of UE radio capability ID |  | Rel-16 |  | C178 |  | UEs supporting 5G Core and E-UTRA and RACS |  |
| 9.1.10 |  | Network slice-specific authentication and authorization |  |  |  |  |  |  |  |
| 9.1.10.1 |  | NSSAA / EAP message transport / Success |  | Rel-16 |  | C147 |  | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |  |
| 9.1.10.2 |  | Network slice-specific authentication and authorization / EAP message transport / Abnormal |  | Rel-16 |  | C147 |  | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |  |
| 9.1.10.3 |  | NSSAA / Initial registration / Rejected NSSAI, pending NSSAI |  | Rel-16 |  | C147 |  | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |  |
| 9.1.10.4 |  | NSSAA / Initial registration / Reject |  | Rel-16 |  | C147 |  | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |  |
| 9.1.10.6 |  | NSSAA / UE configuration update / Rejected NSSAI |  | Rel-16 |  | C147 |  | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |  |
| 9.1.11 |  | SNPN / Mobility management aspects |  |  |  |  |  |  |  |
| 9.1.11.1 |  | SNPN / Initial registration / Rejected / Temporarily not authorized for this SNPN |  | Rel-16 |  | C131 |  | UEs supporting 5G Core and SNPN |  |
| 9.1.11.2 |  | SNPN / Initial registration / Rejected / Permanently not authorized for this SNPN |  | Rel-16 |  | C131 |  | UEs supporting 5G Core and SNPN |  |
| 9.1.11.3 |  | SNPN / EAP based primary authentication and key agreement / EAP-AKA' related procedures |  | Rel-16 |  | C131 |  | UEs supporting 5G Core and SNPN |  |
| 9.1.12 |  | NSAC / Mobility management aspects |  |  |  |  |  |  |  |
| 9.1.12.1 |  | NSAC / Initial registration / Back-off timer |  | Rel-17 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.12.2 |  | NSAC / Initial registration / Back-off timer is not provided or zero |  | Rel-17 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.12.3 |  | NSAC / Initial registration / Rejected / equivalent PLMNs |  | Rel-17 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.12.4 |  | NSAC / Generic UE configuration update / Rejected NSSAI |  | Rel-17 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.12.5 |  | NSAC / De-registration / 5GMM cause value #62 and rejected NSSAI |  | Rel-17 |  | C21 |  | UEs supporting 5G Core |  |
| 9.1.13 |  | NSSRG / Mobility management aspects |  |  |  |  |  |  |  |
| 9.1.13.1 |  | NSSRG / Initial registration |  | Rel-17 |  | C230 |  | UEs supporting 5G Core and NSSRG |  |
| 9.1.13.2 |  | NSSRG / Generic UE configuration update |  | Rel-17 |  | C230 |  | UEs supporting 5G Core and NSSRG |  |
| 9.2 |  | 5GS Non-3GPP Access Mobility Management |  |  |  |  |  |  |  |
| 9.2.1 |  | Primary authentication and key agreement procedure |  |  |  |  |  |  |  |
| 9.2.1.1 |  | EAP based primary authentication and key agreement |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.1.2 |  | 5G AKA based primary authentication and key agreement |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.2 |  | Security Mode Control |  |  |  |  |  |  |  |
| 9.2.2.1 |  | NAS security mode command |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.2.2 |  | Protection of initial NAS signalling messages |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.3 |  | Void |  |  |  |  |  |  |  |
| 9.2.4 |  | Generic UE configuration |  |  |  |  |  |  |  |
| 9.2.4.1 |  | Generic UE configuration update |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.5 |  | Registration |  |  |  |  |  |  |  |
| 9.2.5.1 |  | Initial Registration |  |  |  |  |  |  |  |
| 9.2.5.1.1 |  | Initial registration / Success / 5G-GUTI reallocation, Last visited TAI |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.5.1.2 |  | Initial registration / 5GS services / NSSAI handling |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.5.1.3 |  | Void |  |  |  |  |  |  |  |
| 9.2.5.1.4 |  | Initial registration / Rejected / Congestion / Abnormal cases / T3346 |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.5.2 |  | Mobility Registration |  |  |  |  |  |  |  |
| 9.2.5.2.1 |  | Void |  |  |  |  |  |  |  |
| 9.2.5.2.2 |  | Mobility registration update/Change of SMS over NAS capability |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.6 |  | De-registration |  |  |  |  |  |  |  |
| 9.2.6.1 |  | UE-initiated de-registration |  |  |  |  |  |  |  |
| 9.2.6.1.1 |  | UE-initiated de-registration / switch off |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.6.2 |  | Network-initiated de-registration |  |  |  |  |  |  |  |
| 9.2.6.2.1 |  | Network-initiated de-registration / De-registration for Non-3GPP access / Re-registration required |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.6.2.2 |  | Network-initiated de-registration / De-registration for Non 3GPP access / Re-registration not required |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.7 |  | Service request |  |  |  |  |  |  |  |
| 9.2.7.1 |  | Service request / IDLE mode uplink user data transport / Rejected / Restricted service area, Abnormal / T3517 |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 9.2.7.2 |  | Service request / CMM CONNECTED mode/uplink user data transport / Abnormal / T3517 |  | Rel-15 |  | C58 |  | UEs supporting 5G core over non-3GPP Access Network, WLAN and (ICMP or ICMP IPv6) |  |
| 9.2.8 |  | SMS over NAS |  |  |  |  |  |  |  |
| 9.2.8.1 |  | SMS over NAS / MO SMS over NAS - 5GMM-Idle mode |  | Rel-15 |  | C30 |  | UEs supporting 5G core over non-3GPP Access Network and SMS over NAS and WLAN |  |
| 9.3 |  | Inter-system mobility |  |  |  |  |  |  |  |
| 9.3.1 |  | 5GS-EPC Inter-system mobility |  |  |  |  |  |  |  |
| 9.3.1.1 |  | Inter-system mobility registration update / Single-registration mode with N26 / 5GMM-IDLE / 5GC to EPC |  | Rel-15 |  | C26 |  | UEs supporting 5GS and E-UTRA |  |
| 9.3.1.2 |  | Inter-system mobility registration update / Single-registration mode with N26 / 5GMM-IDLE / EPC to 5GC |  | Rel-15 |  | C26 |  | UEs supporting 5GS and E-UTRA |  |
| 9.3.1.3 |  | Inter-system mobility and periodic registration update / Rejected / Single-registration mode with N26 / Handling of EPC relevant parameters |  | Rel-15 |  | C26 |  | UEs supporting 5GS and E-UTRA |  |
| 9.3.1.4 |  | NSAC / interworking with EPC |  | Rel-17 |  | CXXX |  | UEs supporting 5GS and E-UTRA and NSSRG |  |
| 10 |  | Session management |  |  |  |  |  |  |  |
| 10.1 |  | 5GS session management |  |  |  |  |  |  |  |
| 10.1.1 |  | PDU session authentication and authorization |  |  |  |  |  |  |  |
| 10.1.1.1 |  | PDU session authentication and authorization / During the UE-requested PDU session procedure |  | Rel-15 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.1.2 |  | PDU session authentication and authorization / After the UE-requested PDU session procedure |  | Rel-15 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.2 |  | Network-requested PDU session modification |  |  |  |  |  |  |  |
| 10.1.2.1 |  | Network-requested PDU session modification / Accepted |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 10.1.2.2 |  | Network-requested PDU session modification / Abnormal / PDU session in state PDU SESSION INACTIVE |  | Rel-15 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.3 |  | Network-requested PDU session release |  |  |  |  |  |  |  |
| 10.1.3.1 |  | Void |  |  |  |  |  |  |  |
| 10.1.3.2 |  | Network-requested PDU session release / Insufficient resources, insufficient resources for specific slice and DNN, abnormal / Invalid PDU session identity |  | Rel-15 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.4 |  | UE-requested PDU session establishment |  |  |  |  |  |  |  |
| 10.1.4.1 |  | UE-requested PDU session establishment / Abnormal / T3580 |  | Rel-15 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.5 |  | UE-requested PDU session modification |  |  |  |  |  |  |  |
| 10.1.5.1 |  | UE-requested PDU session modification |  | Rel-15 |  | C63 |  | UEs supporting 5G Core and UE requested PDU session modification procedure |  |
| 10.1.6 |  | UE-requested PDU session release |  |  |  |  |  |  |  |
| 10.1.6.1 |  | UE-requested PDU session release / Abnormal / Collision with network-requested PDU session modification procedure |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 10.1.6.2 |  | UE-requested PDU session release / Abnormal / Collision with network-requested PDU session release procedure |  | Rel-15 |  | C21 |  | UEs supporting 5G Core |  |
| 10.1.7 |  | Network-requested PDU session release |  |  |  |  |  |  |  |
| 10.1.7.1 |  | SNPN / Network-requested PDU session release / Accepted / Insufficient resources / T3396, Accepted / Insufficient resources for specific slice and DNN / T3584 |  | Rel-16 |  | C131 |  | UEs supporting 5G Core and SNPN |  |
| 10.1.8 |  | NSAC / Session management aspects |  |  |  |  |  |  |  |
| 10.1.8.1 |  | NASC / PDU session establishment reject / Maximum number of PDU sessions reached / Back-off timer is neither zero nor deactivated |  | Rel-17 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.8.2 |  | NASC / PDU session establishment reject / Maximum number of PDU sessions reached / Back-off timer is deactivated |  | Rel-17 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.8.3 |  | NASC / PDU session establishment reject / Maximum number of PDU sessions reached / Back-off timer is zero or not included |  | Rel-17 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.8.4 |  | NASC / 5GSM message not forwarded / Back-off timer |  | Rel-17 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.1.8.5 |  | NASC / Maximum number of PDU sessions reached / Emergency service |  | Rel-17 |  | C39 |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| 10.2 |  | EN-DC session management |  |  |  |  |  |  |  |
| 10.2.1 |  | Network initiated procedures |  |  |  |  |  |  |  |
| 10.2.1.1 |  | Default EPS bearer context activation |  | Rel-15 |  | C01 |  | UEs supporting EN-DC |  |
| 10.2.1.2 |  | Dedicated EPS bearer context activation |  | Rel-15 |  | C01 |  | UEs supporting EN-DC |  |
| 10.2.2 |  | UE initiated procedures |  |  |  |  |  |  |  |
| 10.2.2.1 |  | EPS bearer resource allocation / modification |  | Rel-15 |  | C16 |  | UEs supporting EN-DC and UE requested bearer resource allocation and modification procedures |  |
| 10.3 |  | 5GS Non-3GPP Access Session Management |  |  |  |  |  |  |  |
| 10.3.1 |  | PDU session authentication and authorization |  |  |  |  |  |  |  |
| 10.3.1.1 |  | PDU session authentication and authorization / during the UE-requested PDU session procedure |  | Rel-15 |  | C159 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment |  |
| 10.3.2 |  | Network-requested PDU session modification |  |  |  |  |  |  |  |
| 10.3.2.1 |  | Network-requested PDU session modification /Accepted/Rejected |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 10.3.3 |  | Network-requested PDU session Release |  |  |  |  |  |  |  |
| 10.3.3.1 |  | Network-requested PDU session release / accepted/ with and without reactivation |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 10.3.4 |  | UE-requested PDU session establishment |  |  |  |  |  |  |  |
| 10.3.4.1 |  | UE-requested PDU session establishment / Abnormal / T3580 |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 10.3.5 |  | UE-requested PDU session modification |  |  |  |  |  |  |  |
| 10.3.5.1 |  | UE-requested PDU session modification/Success |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| 10.3.6 |  | UE-requested PDU session release |  |  |  |  |  |  |  |
| 10.3.6.1 |  | UE-requested PDU session release / Abnormal / Collision with network-requested PDU session modification procedure |  | Rel-15 |  | C29 |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |

< Skipped part >

Table 4.2-1: Applicability of Protocol conformance test cases Conditions

| Condition |  | Test case Selection Expression |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| C01 |  | IF A.4.1-3/2 THEN R ELSE N/A |  | UEs supporting EN-DC |  |
| C02 |  | IF (A.4.3.4-1/2 OR A.4.3.4-1/3) THEN R ELSE N/A |  | UEs supporting 5GS and RLC UM Mode |  |
| C03 |  | IF A.4.3.5-1/1 THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle |  |
| C04 |  | IF A.4.3.5-1/2 THEN R ELSE N/A |  | UEs supporting 5GS and short DRX cycle |  |
| C05 |  | IF A.4.3.4-1/3 THEN R ELSE N/A |  | UEs supporting 5GS and RLC UM with 6-bit length of RLC sequence number |  |
| C06 |  | IF A.4.3.4-1/2 THEN R ELSE N/A |  | UEs supporting 5GS and RLC UM with 12-bit length of RLC sequence number |  |
| C07 |  | IF A.4.3.4-1/1 THEN R ELSE N/A |  | UEs supporting 5GS and RLC AM with 12-bit length of RLC sequence number |  |
| C07A |  | IF A.4.3.4-1/1A THEN R ELSE N/A |  | UEs supporting 5GS and RLC AM with 18-bit length of RLC sequence number |  |
| C08 |  | IF A.4.3.3-1/1 THEN R ELSE N/A |  | UEs supporting 5GS and 12-bit length of PDCP sequence number |  |
| C08A |  | IF A.4.3.3-1/1A THEN R ELSE N/A |  | UEs supporting 5GS and 18-bit length of PDCP sequence number |  |
| C09 |  | IF [10] A.4.4-1/99 THEN R ELSE N/A |  | UEs supporting 5GS and ZUC Algorithm |  |
| C10 |  | IF A.4.1-3/2 AND A.4.3.7-1/2 THEN R ELSE N/A |  | UEs supporting EN-DC and UL transmission via both MCG path and SCG path for the split DRB |  |
| C11 |  | IF (A.4.3.2-1/2 OR A.4.3.2-1/3) THEN R ELSE N/A |  | UEs supporting 5GS and 256QAM for PDSCH for FR1/FR2 |  |
| C12 |  | IF (A.4.3.2-1/4) THEN R ELSE N/A |  | UEs supporting 5GS and 256QAM for PUSCH |  |
| C13 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting |  |
| C14 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) |  |
| C15 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.3.6-1/4 OR A.4.3.6-1/40) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |  |
| C16 |  | IF A.4.1-3/2 AND [10] A.4.4-1/18 AND [10] A.4.4-1/19 THEN R ELSE N/A |  | UEs supporting EN-DC and UE requested bearer resource allocation and modification procedures |  |
| C17 |  | IF A.4.3.2-1/1 THEN R ELSE N/A |  | UEs supporting 5GS and PDSCH reception based on semi-persistent scheduling |  |
| C18 |  | IF A.4.3.2-1/10 THEN R ELSE N/A |  | UEs supporting 5GS and Type 1 PUSCH transmissions with configured grant |  |
| C19 |  | IF A.4.3.2-1/11 THEN R ELSE N/A |  | UEs supporting 5GS and Type 2 PUSCH transmissions with configured grant |  |
| C20 |  | IF A.4.3.2-1/12 THEN R ELSE N/A |  | UEs supporting 5GS and PDSCH aggregation |  |
| C21 |  | IF A.4.1-5/1 THEN R ELSE N/A |  | UEs supporting 5G Core |  |
| C21A |  | IF A.4.1-5/1 AND A.4.3.7-1/4 THEN R ELSE N/A |  | UEs supporting 5G Core and reflective QoS |  |
| C22 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 |  |
| C23 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND A.4.3.7-1/1 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |  |
| C24 |  | IF A.4.1-3/2 AND A.4.3.6-1/3 AND A.4.3.6-1/2 AND A.4.1-4/3 THEN R ELSE N/A |  | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC within FR1 |  |
| C25 |  | IF A.4.1-3/2 AND A.4.3.6-1/3 AND A.4.3.6-1/2 AND A.4.1-4/4 THEN R ELSE N/A |  | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC including FR2 |  |
| C26 |  | IF ([10] A.4.1-1/1 OR [10] A.4.1-1/2) THEN R ELSE N/A |  | UEs supporting 5GS and E-UTRA |  |
| C27 |  | IF A.4.1-5/1 AND A.4.3.6-1/1 THEN R ELSE N/A |  | UEs supporting 5G Core and NR measurements and Event A triggered reporting |  |
| C28 |  | IF A.4.3.2-1/13 THEN R ELSE N/A |  | UEs supporting 5GS and supplemental uplink with dynamic switch |  |
| C29 |  | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A |  | UEs supporting 5G core over non-3GPP Access Network and WLAN |  |
| C30 |  | IF A.4.1-5/2 AND A.4.3.7-1/6 AND [10] A.4.1-1/5 THEN R ELSE N/A |  | UEs supporting 5G core over non-3GPP Access Network and SMS over NAS and WLAN |  |
| C31 |  | IF A.4.1-5/1 AND A.4.3.6-1/5 THEN R ELSE N/A |  | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting |  |
| C32 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA |  |
| C33 |  | IF A.4.1-5/1 AND A.4.3.7-1/6 AND NOT [10] A.4.4-2/32 THEN R ELSE N/A |  | UEs supporting 5G Core and SMS over NAS and UE configured to not use SMSoIP |  |
| C34 |  | IF A.4.1-5/1 AND [10] A.4.4-1/84 THEN R ELSE N/A |  | UEs supporting 5G Core and MinimumPeriodicSearchTimer |  |
| C35 |  | IF A.4.1-5/1 AND (A.4.3.7-1/8 OR A.4.3.7-1/7) THEN R ELSE N/A |  | UEs supporting 5G Core and (ETWS reception or CMAS reception) |  |
| C36 |  | IF A.4.1-5/1 AND [10] A.4.4-1/69 THEN R ELSE N/A |  | UEs supporting 5G Core and user initiated PLMN reselection in automatic mode on NR |  |
| C37 |  | IF A.4.1-5/1 AND (A.4.1-2/1 OR A.4.1-2/2) THEN R ELSE N/A |  | UEs supporting 5G Core and more than 1 FDD or TDD NR band |  |
| C38 |  | IF A.4.1-5/1 AND A.4.1-1/1 AND A.4.1-1/2 THEN R ELSE N/A |  | UEs supporting 5G Core and NR FDD and NR TDD |  |
| C39 |  | IF A.4.1-5/1 AND A.4.3.7-1/9 THEN R ELSE N/A |  | UEs supporting 5G Core and additional UE-requested PDU establishment |  |
| C40 |  | IF A.4.1-5/1 AND A.4.3.6-1/6 THEN R ELSE N/A |  | UEs supporting 5G Core and SS-SINR measurements |  |
| C41 |  | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band contiguous CA |  |
| C42 |  | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting 5G Core and inter-band CA |  |
| C43 |  | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band non-contiguous CA |  |
| C44 |  | IF (A.4.1-4A/1 OR A.4.1.4A/3) THEN R ELSE N/A |  | UEs supporting 5GS and intra-band contiguous CA |  |
| C45 |  | IF (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting 5GS and inter-band CA |  |
| C46 |  | IF (A.4.1-4A/2 OR A.4.1.4A/4) THEN R ELSE N/A |  | UEs supporting 5GS and intra-band non-contiguous CA |  |
| C47 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/11 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |  |
| C48 |  | Void |  |  |  |
| C49 |  | IF A.4.1-5/1 AND A.4.3.6-1/2 THEN R ELSE N/A |  | UE supporting 5G Core and two independent measurement gap configurations for FR1 and FR2 |  |
| C50 |  | IF A.4.1-5/1 AND A.4.3.6-1/5 AND A.4.3.6-1/42 THEN R ELSE N/A |  | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting and E-UTRA RS-SINR measurements |  |
| C51 |  | IF A.4.3.2-1/21 THEN R ELSE N/A |  | UEs supporting 5GS and PUSCH aggregation |  |
| C52 |  | IF A.4.1-5/1 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.3.6-1/4 OR A.4.3.6-1/40) THEN R ELSE N/A |  | UEs supporting 5G Core and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |  |
| C53 |  | IF A.4.3.5-1/4 THEN R ELSE N/A |  | UEs supporting 5GS and Logical Channel SR-Delay Timer |  |
| C54 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |  |
| C55 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band contiguous CA |  |
| C56 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and inter-band CA |  |
| C57 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band non-contiguous CA |  |
| C58 |  | IF A.4.1-5/2 AND [10] A.4.1-1/5.AND A.4.4-1/1 |  | UEs supporting 5G core over non-3GPP Access Network, WLAN and (ICMP or ICMP IPv6) |  |
| C59 |  | IF A.4.1-5/1 AND A.4.3.6-1/8 THEN R ELSE N/A |  | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured |  |
| C60 |  | IF A.4.1-5/1 AND A.4.3.6-1/7 THEN R ELSE N/A |  | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured |  |
| C61 |  | IF A.4.1-3/2 AND A.4.3.3-1/6 THEN R ELSE N/A |  | UEs supporting EN-DC and PDCP duplication over split SRB1/2 |  |
| C62 |  | IF A.4.1-3/2 AND A.4.3.3-1/4 THEN R ELSE N/A |  | UEs supporting EN-DC and PDCP duplication over split DRB |  |
| C63 |  | IF A.4.1-5/1 AND A.4.3.7-1/13 THEN R ELSE N/A |  | UEs supporting 5G Core and UE requested PDU session modification procedure |  |
| C64 |  | IF A.4.3.2-1/23 THEN R ELSE N/A |  | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |  |
| C65 |  | IF A.4.3.2-1/23 AND (A.4.3.2-1/4) THEN R ELSE N/A |  | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |  |
| C66 |  | IF (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND (A.4.3.2-1/42 OR A.4.3.2-1/43) THEN R ELSE N/A |  | UEs supporting 5GS and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation up to 2 or up to 4) |  |
| C67 |  | IF A.4.1-3/2 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting EN-DC and Intra-Band Contiguous CA |  |
| C68 |  | IF A.4.1-3/2 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting EN-DC and Intra-Band Non-Contiguous CA |  |
| C69 |  | IF A.4.1-3/2 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting EN-DC and Inter-Band CA |  |
| C70 |  | IF A.4.3.5-1/1 AND A.4.3.5-1/2 THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle and Short DRX Cycle |  |
| C71 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND A.4.3.6-1/3 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |  |
| C72 |  | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C73 |  | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C74 |  | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C75 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |  |
| C76 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |  |
| C77 |  | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and EN-DC with 2 NR UL carriers |  |
| C78 |  | IF A.4.1-5/1 AND [9] A.3A/50 AND [9] A.4/2B AND [9] A.15/1 AND [9] A.3A/61 THEN R ELSE N/A |  | UEs supporting 5G Core and Initiating session and MTSI speech and SMS over IP |  |
| C79 |  | IF A.4.1-5/1 AND [9] A.3A/50 AND [9] A.4/2B AND [9] A.15/3 THEN R ELSE N/A |  | UEs supporting 5G Core and Initiating session and MTSI video |  |
| C80 |  | IF A.4.1-4/6 THEN R ELSE N/A |  | UEs supporting NR-DC |  |
| C81 |  | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1.4A/3) AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band contiguous CA and UL NR CA with 2 carriers |  |
| C81A |  | IF A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and intra-band contiguous CA and EN-DC with 2 NR UL carriers |  |
| C82 |  | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and inter-band CA and UL NR CA with 2 carriers |  |
| C82A |  | IF A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/4 OR A.4.1-4/5) AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and inter-band CA and EN-DC with 2 NR UL carriers |  |
| C83 |  | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1.4A/4) AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band non-contiguous CA and UL NR CA with 2 carriers |  |
| C83A |  | IF A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A |  | UEs supporting EN-DC and intra-band non-contiguous CA and EN-DC with 2 NR UL carriers |  |
| C84 |  | IF A.4.1-5/1 AND [10] A.4.4-1/99 THEN R ELSE N/A |  | UEs supporting 5G Core and ZUC algorithm |  |
| C85 |  | Void |  |  |  |
| C85A |  | IF (A.4.1-5/1 AND A.4.4-2/9) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/14 THEN R ELSE N/A |  | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |  |
| C85B |  | IF (A.4.1-5/1 AND A.4.4-2/8) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/32 THEN R ELSE N/A |  | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and IMS voice over NR |  |
| C86 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 |  |
| C87 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND A.4.3.6-1/3 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |  |
| C88 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C89 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C90 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |  |
| C91 |  | IF A.4.1-5/1 AND [10] A.4.4-1/98 THEN R ELSE N/A |  | UEs supporting 5G Core and ManualModeNetworkSelectionException |  |
| C92 |  | IF A.4.1-5/1 AND A.4.3.7-1/14 THEN R ELSE N/A |  | UEs supporting 5G Core and emergency services in NR connected to 5GCN |  |
| C93 |  | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A |  | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands |  |
| C94 |  | IF A.4.1-5/1 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A |  | UEs supporting 5G Core and multiple NR bands |  |
| C95 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 AND A.4.3.7-1/15 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback and voiceFallbackIndication |  |
| C96 |  | IF A.4.1-5/1 AND A.4.1-3/2 AND A.4.3.8-1/10 THEN R ELSE N/A |  | UEs supporting 5G Core and EN-DC and inter-RAT Handover from NR to EN-DC |  |
| C97 |  | IF A.4.1-4/6 AND A.4.3.7-1/2 THEN R ELSE N/A |  | UEs supporting NR-DC and UL transmission via both MCG path and SCG path for the split DRB |  |
| C98 |  | IF A.4.1-4/6 AND A.4.3.3-1/4 THEN R ELSE N/A |  | UEs supporting NR-DC and PDCP duplication over split DRB |  |
| C99 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND (A.4.3.8-1/6 OR A.4.3.8-1/7 OR A.4.3.8-1/8)THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and (inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC or inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC or inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC) |  |
| C100 |  | IF A.4.1-5/1 AND [9] A.15/1 AND A.4.3.5-1/9 THEN R ELSE N/A |  | UEs supporting 5G Core and MTSI speech and bit rate recommendation query message |  |
| C101 |  | IF A.4.1-5/1 AND A.4.3.8-1/9 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-frequency DAPS handover |  |
| C102 |  | IF A.4.3.2-1/30 THEN R ELSE N/A |  | UEs supporting 5GS and cross slot scheduling |  |
| C103 |  | IF A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle and DRX adaptation |  |
| C104 |  | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-1/2 AND A.4.3.2A.1-2/2 AND A.4.3.3-1/5 THEN R ELSE N/A |  | UEs supporting 5GC and Intra-band contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |  |
| C105 |  | IF (A.4.3.4-1/2 OR A.4.3.4-1/3) AND A.4.3.3-1/7 THEN R ELSE N/A |  | UEs supporting 5GS and RLC UM mode and PDCP ethernet header compression |  |
| C106 |  | IF A.4.1-5/1 AND A.4.3.10-1/1 THEN R ELSE N/A |  | UE supporting 5G core and NR sidelink mode 1 transmission |  |
| C107 |  | IF A.4.3.2-1/32 THEN R ELSE N/A |  | UE’s supporting multi-DCI based multi-TRP |  |
| C108 |  | IF A.4.1-5/1 AND A.4.3.7-1/17 THEN R ELSE N/A |  | UEs supporting 5G Core and RACS |  |
| C109 |  | IF A.4.1-5/1 AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G Core and RRC_INACTIVE |  |
| C109A |  | IF A.4.1-5/1 AND A.4.3.7-1/19 AND ([10] A.4.4-2/5 OR [10] A.4.4-2/8) THEN R ELSE N/A |  | UEs supporting 5G Core and RRC_INACTIVE and (Support of CS/PS mode 2 or Support of PS mode 2) |  |
| C110 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and RRC_INACTIVE |  |
| C111 |  | IF A.4.1-5/1 AND (A.4.3.7-1/8 OR A.4.3.7-1/7) AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G Core and (ETWS reception or CMAS reception) and RRC_INACTIVE |  |
| C112 |  | Void |  |  |  |
| C113 |  | IF A.4.1-5/1 AND A.4.3.2/1 THEN R ELSE N/A |  | UEs 5GS and PDSCH reception based on multiple semi-persistent scheduling |  |
| C114 |  | IF A.4.1-5/1 AND A.4.3.5-1/6 THEN R ELSE N/A |  | UEs supporting 5GS and LCH-based UL grant prioritization |  |
| C115 |  | IF A.4.1-5/1 AND A.4.3.8-1/11 THEN R ELSE N/A |  | UEs supporting 5G Core and conditional handover |  |
| C116 |  | IF A.4.1-5/1 AND A.4.3.8-1/11 AND A.4.3.8-1/13 THEN R ELSE N/A |  | UEs supporting 5G Core and conditional handover and supporting 2 trigger events for same execution condition |  |
| C117 |  | IF A.4.1-5/1 AND A.4.3.8-1/11 AND A.4.3.8-1/12 THEN R ELSE N/A |  | UEs supporting 5G Core and conditional handover and conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover |  |
| C118 |  | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND A.4.3.2A.1-2/1 AND A.4.3.2-1/83 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band contiguous CA and UL NR CA with 2 carriers and two PUCCH group in CA with a same numerology |  |
| C119 |  | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND A.4.3.2A.1-2/1 AND A.4.3.2-1/83 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band non-contiguous CA and UL NR CA with 2 carriers and two PUCCH group in CA with a same numerology |  |
| C120 |  | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND A.4.3.2A.1-2/1 AND A.4.3.2-1/83 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and inter-band CA and UL NR CA with 2 carriers and two PUCCH group in CA with a same numerology |  |
| C121 |  | Void |  |  |  |
| C122 |  | IF A.4.1-5/1 AND A.4.4-1/5 THEN R ELSE N/A |  | UEs supporting 5G Core and UL PDCP Packet Delay per DRB |  |
| C123 |  | IF A.4.1-5/1 AND A.4.4-1/6 THEN R ELSE N/A |  | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE |  |
| C124 |  | IF A.4.1-5/1 AND A.4.4-1/4 AND A.4.4-1/6 THEN R ELSE N/A |  | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE and equipped with a GNSS receiver to provide detailed location information |  |
| C125 |  | IF A.4.1-5/1 AND A.4.4-1/6 AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G core and RRC_INACTIVE and logged measurements in RRC_IDLE and RRC_INACTIVE. |  |
| C126 |  | IF A.4.1-5/1 AND A.4.4-1/4 THEN R ELSE N/A |  | UEs supporting 5G Core and equipped with a GNSS or A-GNSS receiver to provide detailed location information. |  |
| C127 |  | IF A.4.1-5/1 AND [10] A.4.1-1/6 AND A.4.3.8-1/11 THEN R ELSE N/A |  | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |  |
| C128 |  | IF A.4.1-5/1 AND A.4.1-1/3 THEN R ELSE N/A |  | UE supporting 5G core and NR sidelink |  |
| C129 |  | IF A.4.1-5/1 AND A.4.3.7-1/18 THEN R ELSE N/A |  | UEs supporting 5G Core and RRC message Segmentation in the UL |  |
| C130 |  | IF A.4.1-5/1 AND A.4.3.8-1/15 THEN R ELSE N/A |  | UEs supporting 5G Core and inter-frequency DAPS handover |  |
| C131 |  | IF A.4.1-5/1 AND A.4.3.7-1/24 THEN R ELSE N/A |  | UEs supporting 5G Core and SNPN |  |
| C132 |  | IF A.4.1-5/1 AND A.4.3.7-1/23 THEN R ELSE N/A |  | UEs supporting 5G Core and CAG |  |
| C133 |  | IF A.4.1-5/1 AND A.4.3.7-1/21 THEN R ELSE N/A |  | UEs supporting 5G Core and RRC connection release with Deprioritisation |  |
| C134 |  | IF A.4.3.2-1/45 THEN R ELSE N/A |  | UEs supporting PUSCH repetition type B |  |
| C135 |  | IF A.4.3.2-1/46 THEN R ELSE N/A |  | UEs supporting 2-Step RACH |  |
| C136 |  | IF A.4.1-5/1 AND A.4.4-1/3 THEN R ELSE N/A |  | UEs supporting 5G Core and delivery of rachReport upon request from the network. |  |
| C137 |  | IF A.4.1-5/1 AND A.4.4-1/12 THEN R ELSE N/A |  | UEs supporting 5G core and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |  |
| C138 |  | IF A.4.1-5/1 AND A.4.4-1/13 THEN R ELSE N/A |  | UEs supporting 5G core and WLAN measurements in RRC_IDLE and RRC_INACTIVE state |  |
| C139 |  | IF A.4.1-5/1 AND (A.4.4-1/7 OR A.4.4-1/8 OR A.4.4-1/9) THEN R ELSE N/A |  | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355 |  |
| C140 |  | IF A.4.1-5/1 AND A.4.4-1/10 THEN R ELSE N/A |  | UEs supporting 5G core and Bluetooth Measurement Collection in Immediate MDT |  |
| C141 |  | IF A.4.1-5/1 AND A.4.4-1/11 THEN R ELSE N/A |  | UEs supporting 5G core and WLAN Measurement Collection in Immediate MDT |  |
| C142 |  | IF A.4.1-5/1 AND A.4.3.5-1/10 THEN R ELSE N/A |  | UEs supporting 5GS and PUSCH transmissions on multiple configured uplink grants |  |
| C143 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/4 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and standalone GNSS receiver to provide detailed location information |  |
| C144 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/6 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and logged measurements in RRC_IDLE and RRC_INACTIVE |  |
| C145 |  | IF A.4.1-5/1 AND A.4.3.7-1/29 THEN R ELSE N/A |  | UEs supporting 5G Core and release preference assistance information |  |
| C146 |  | IF A.4.3.2-1/52 THEN R ELSE N/A |  | UEs supporting monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |  |
| C146a |  | Void |  | C |  |
| C147 |  | IF A.4.1-5/1 AND A.4.3.7-1/26 AND A.4.3.7-1/27 THEN R ELSE N/A |  | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |  |
| C148 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/21 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and RRC connection release with Deprioritisation |  |
| C149 |  | IF A.4.1-4/6 AND A.4.3.6-1/2 THEN R ELSE N/A |  | UEs supporting NR-DC and two independent measurement gap configurations for FR1 and FR2 |  |
| C150 |  | IF A.4.1-5/1 AND (A.4.3.6-1/48 OR A.4.3.6-1/49) THEN R ELSE N/A |  | UEs supporting 5G Core and SFTD measurements between NR PCell and NR neighbour cell |  |
| C151 |  | IF A.4.1-3/2 AND (A.4.3.6-1/43 OR A.4.3.6-1/44) AND (A.4.3.6-1/46 OR A.4.3.6-1/47) THEN R ELSE N/A |  | UEs supporting EN-DC and SFTD measurement between E-UTRA PCell and an NR neighbour cell, and SFTD measurement between E-UTRA PCell and NR PSCell |  |
| C152 |  | IF A.4.1-4/6 AND (A.4.3.6-1/48 OR A.4.3.6-1/49) AND (A.4.3.6-1/50 OR A.4.3.6-1/51) THEN R ELSE N/A |  | UEs supporting NR-DC and SFTD measurement between NR PCell and an NR neighbour cell, and SFTD measurement between NR PCell and NR PSCell |  |
| C153 |  | IF A.4.1-3/2 AND A.4.3.8-1/19 THEN R ELSE N/A |  | UEs supporting EN-DC and conditional PSCell change |  |
| C154 |  | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.7-1/19 AND A.4.3.5-1/14 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |  |
| C155 |  | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.7-1/19 AND A.4.3.5-1/14 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR MCG SCell activation |  |
| C156 |  | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.7-1/19 AND A.4.3.5-1/14 THEN R ELSE N/A |  | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE- and direct NR MCG SCell activation |  |
| C157 |  | IF A.4.1-4/6 AND A.4.3.7-1/3 AND A.4.3.7-1/1 THEN R ELSE N/A |  | UEs supporting NR-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |  |
| C158 |  | IF A.4.1-5/1 AND A.4.1-4/6 AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G Core and NR-DC and RRC_INACTIVE |  |
| C159 |  | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A |  | UEs supporting 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment |  |
| C160 |  | IF A.4.1-3/3 THEN R ELSE N/A |  | UEs supporting NE-DC |  |
| C161 |  | IF A.4.1-5/1 AND A.4.3.7-1/21 AND [10] A.4.4-1/98 THEN R ELSE N/A |  | UEs supporting 5G Core and RRC connection release with Deprioritisation and ManualModeNetworkSelectionException |  |
| C162 |  | IF A.4.1-5/1 AND [9] A.22/8 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.15/1 THEN R ELSE N/A |  | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and MTSI speech |  |
| C162A |  | IF A.4.1-5/1 AND [9] A.22/8 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.3A/61 THEN R ELSE N/A |  | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and SMS over IP |  |
| C163 |  | IF A.4.1-5/1 AND A.4.1-1/3 AND A.4.3.10-1/3 THEN R ELSE N/A |  | UE supporting 5G core and NR sidelink and Sidelink CSI report |  |
| C164 |  | IF A.4.1-5/1 AND A.4.3.10-1/1 AND A.4.3.10-1/3 THEN R ELSE N/A |  | UE supporting 5G core and NR sidelink mode 1 transmission and Sidelink CSI report |  |
| C165 |  | IF A.4.1-5/1 AND A.4.3.7-1/33 THEN R ELSE N/A |  | UE supporting 5G Core and V2X communication |  |
| C166 |  | IF A.4.1-5/1 AND A.4.3.7-1/34 THEN R ELSE N/A |  | UE supporting 5G Core and V2X communication over NR-PC5 |  |
| C167 |  | IF A.4.1-5/1 AND A.4.3.7-1/24 AND A.4.3.7-1/30 THEN R ELSE N/A |  | UEs supporting 5G Core and SNPN and user initiated SNPN reselection in automatic mode on NR |  |
| C168 |  | IF A.4.1-5/1 AND A.4.3.7-1/23 AND A.4.3.7-1/31 THEN R ELSE N/A |  | UEs supporting 5G Core and CAG and Autonomous search function on NR |  |
| C169 |  | IF A.4.1-5/1 AND A.4.3.7-1/23 AND A.4.3.7-1/52 THEN R ELSE N/A |  | UEs supporting 5G Core and CAG and acquisition of CGI information from neighbour NR NPN cell |  |
| C170 |  | IF A.4.1-5/1 AND [10](A.4.1-1/1 OR A.4.1-1/2) AND [9]A.12/64 AND [11]A.10/16 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |  |
| C171 |  | IF A.4.1-5/1 AND [10](A.4.1-1/1 OR A.4.1-1/2) AND [9]A.12/64 AND [11]A.10/16 AND [11]A.10/19 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |  |
| C172 |  | IF A.4.1-5/1 AND [9] A.22/9 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.15/3 THEN R ELSE N/A |  | UEs supporting 5G Core and NG.114 v2.0 default configuration video exempt and 3GPP PS data off and Initiating session and MTSI video |  |
| C173 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [9] A.21/2 |  | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |  |
| C174 |  | IF A.4.1-5/1 AND [9]A.12/64 AND [11]A.10/16 AND [11]A.10/19 THEN R ELSE N/A |  | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |  |
| C175 |  | IF A.4.3.5-1/11 THEN R ELSE N/A |  | UEs supporting 5GS and selection of logical channels for each UL grant based on RRC configured restriction |  |
| C176 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |  |
| C177 |  | IF A.4.1-5/1 AND A.4.3.7-1/17 AND A.4.3.7-1/35 THEN R ELSE N/A |  | UEs supporting 5G Core and RACS and Manufacturer assigned Radio Capability ID |  |
| C178 |  | IF A.4.1-5/1 AND [10](A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.7-1/17 AND [10]A.4.4-1/215THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and RACS |  |
| C179 |  | IF A.4.3.2-1/80 THEN R ELSE N/A |  | UEs supporting DCI DL Priority Indicator |  |
| C180 |  | IF A.4.3.2-1/81 AND A.4.3.2-1/82 THEN R ELSE N/A |  | UEs supporting DCI UL Priority Indicator and LCH grant prioritisation |  |
| C181 |  | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-1/2 AND A.4.3.2A.1-2/2 AND A.4.3.3-1/5 THEN R ELSE N/A |  | UEs supporting 5GC and Intra-band non-contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |  |
| C182 |  | IF A.4.1-3/3 AND A.4.3.6-1/1 AND A.4.3.6-1/3 THEN R ELSE N/A |  | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting). |  |
| C183 |  | IF A.4.1-3/3 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A |  | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands |  |
| C184 |  | IF A.4.1-5/1 AND [9]A.12/64 AND [11]A.10/17 THEN R ELSE N/A |  | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |  |
| C185 |  | IF A.4.1-5/1 AND [9]A.12/63 AND [11]A.10/17 AND A.4.3.7-1/14 THEN R ELSE N/A |  | UEs supporting 5G Core and IMS eCall type of emergency services over 5GS and Automatic type of eCall initiation and emergency services in NR connected to 5GCN |  |
| C186 |  | IF A.4.1-5/1 AND [10]A.4.1-1/6 AND [9]A.12/64 AND [11]A.10/16 AND A.4.3.8-1/14 THEN R ELSE N/A |  | UEs supporting 5G Core and UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and NR to UTRA-FDD CELL_DCH CS handover |  |
| C187 |  | IF A.4.1-5/1 AND ([10]A.4.1-1/6 OR [10]A.4.1-1/7) AND [9]A.12/64 AND [11]A.10/16 THEN R ELSE N/A |  | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |  |
| C188 |  | IF A.4.1-5/1 AND ([10]A.4.1-1/6 OR [10]A.4.1-1/7) AND [9]A.12/64 AND [11]A.10/17 THEN R ELSE N/A |  | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |  |
| C189 |  | IF A.4.1-5/1 AND ([10]A.4.1-1/6 OR [10]A.4.1-1/7) AND [9]A.12/63 AND [11]A.10/16 THEN R ELSE N/A |  | UEs supporting 5G Core and (UTRA OR GERAN) and eCall type of emergency services over 5GS and Manual type of eCall initiation |  |
| C190 |  | IF A.4.1-5/1 AND A.4.3.6-1/54 THEN R ELSE N/A |  | UEs supporting 5G Core and Idle/Inactive Measurements |  |
| C191 |  | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.6-1/55 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and Idle/Inactive Measurements |  |
| C192 |  | IF A.4.1-5/1 AND A.4.3.7-1/19 AND A.4.3.6-1/54 THEN R ELSE N/A |  | UEs supporting 5G Core and RRC_INACTIVE and Idle/Inactive Measurements |  |
| C193 |  | IF A.4.1-5/1 AND A.4.3.7-1/19 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.6-1/55 THEN R ELSE N/A |  | UEs supporting 5G Core and RRC_INACTIVE and E-UTRA and Idle/Inactive Measurements |  |
| C194 |  | IF A.4.1-3/3 AND A.4.3.7-1/2 THEN R ELSE N/A |  | UEs supporting NE-DC and UL transmission via both MCG path and SCG path for the split DRB |  |
| C195 |  | IF A.4.1-4/6 AND A.4.3.3-1/6 THEN R ELSE N/A |  | UEs supporting NR-DC and PDCP duplication over split SRB1/2 |  |
| C196 |  | IF A.4.1-3/3 AND A.4.3.3-1/6 THEN R ELSE N/A |  | UEs supporting NE-DC and PDCP duplication over split SRB1/2 |  |
| C197 |  | IF A.4.1-5/1 AND [10](A.4.1-1/1 OR A.4.1-1/2) AND [9]A.12/64 AND [11]A.10/17 THEN R ELSE N/A |  | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |  |
| C198 |  | IF A.4.1-5/1 AND [9] A.6a/2 THEN R ELSE N/A |  | UEs supporting 5G Core and IMS security |  |
| C199 |  | IF A.4.1-3/2 AND A.4.3.5-1/12 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting EN-DC, direct NR SCG SCell activation and Intra-Band Contiguous CA |  |
| C200 |  | IF A.4.1-3/2 AND A.4.3.5-1/12 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting EN-DC, direct NR SCG SCell activation and Intra-Band Non-Contiguous CA |  |
| C201 |  | IF A.4.1-3/2 AND A.4.3.5-1/12 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting EN-DC, direct NR SCG SCell activation and Inter-Band CA |  |
| C202 |  | IF A.4.1-4/6 AND A.4.3.5-1/12 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting NR-DC, direct NR SCG SCell activation and intra-band contiguous CA |  |
| C203 |  | IF A.4.1-4/6 AND A.4.3.5-1/12 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting NR-DC, direct NR SCG SCell activation and intra-band non-contiguous CA |  |
| C204 |  | IF A.4.1-4/6 AND A.4.3.5-1/12 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7)THEN R ELSE N/A |  | UEs supporting NR-DC, direct NR SCG SCell activation and inter-band CA |  |
| C205 |  | Void |  |  |  |
| C206 |  | IF A.4.1-3/3 AND A.4.3.6-1/5 THEN R ELSE N/A |  | UEs supporting NE-DC and Inter-RAT E-UTRA measurements and Event B triggered reporting |  |
| C207 |  | IF A.4.1-5/1 AND A.4.3.7-1/39 THEN R ELSE N/A |  | UEs supporting 5G core and reception of segmented DL RRC messages. |  |
| C208 |  | IF A.4.1-5/1 AND A.4.4-1/2 AND A.4.3.8-1/20 AND [10] A.4.1-1/5 AND [10] A.4.4-1/117 THEN R ELSE N/A |  | UEs supporting 5G Core and IMS and handover from 5G Core to EPC over non-3GPP Access Network and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and WLAN. |  |
| C209 |  | IF A.4.1-5/1 AND A.4.3.12-1/2 AND A.4.3.12-1/6 THEN R ELSE N/A |  | UEs supporting 5G Core and RedCap and relaxed RRM measurements in RRC_CONNECTED |  |
| C210 |  | IF A.4.1-5/1 AND A.4.3.7-1/43THEN R ELSE N/A |  | UEs supporting 5G Core and eDRX |  |
| C211 |  | IF A.4.3.2-1/85 THEN R ELSE N/A |  | UEs supporting repetition of Message 3 PUSCH |  |
| C212 |  | IF A.4.1-5/1 AND A.4.3.12-1/2 THEN R ELSE N/A |  | UEs supporting 5G Core and RedCap |  |
| C212X |  | IF A.4.1-5/1 AND A.4.3.12-1/2 AND A.4.3.7-1/19 THEN R ELSE N/A |  | UEs supporting 5G Core and RedCap and RRC_INACTIVE |  |
| C213 |  | IF A.4.1-5/1 AND A.4.3.8-1/1 THEN R ELSE N/A |  | UE supporting 5G Core and broadcast reception |  |
| C214 |  | IF A.4.1-5/1 AND A.4.3.8-1/2 THEN R ELSE N/A |  | UE supporting 5G Core and dynamic scheduling for multicast for PCell |  |
| C215 |  | IF A.4.1-5/1 AND A.4.3.8-1/2 AND A.4.3.8-1/3 THEN R ELSE N/A |  | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast |  |
| C216 |  | IF A.4.1-5/1 AND A.4.3.8-1/2 AND A.4.3.8-1/3 AND A.4.3.8-1/4 THEN R ELSE N/A |  | UE supporting 5G Core and dynamic scheduling for multicast for PCell and ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast and PTP retransmission for multicast on the same cell as multicast initial transmission |  |
| C217 |  | IF A.4.1-5/1 AND A.4.3.2-1/49 THEN R ELSE N/A |  | UEs supporting 5G Core and NR standalone shared spectrum channel access |  |
| C218 |  | IF A.4.1-5/1 AND A.4.3.2-1/49 AND A.4.4-1/93 THEN R ELSE N/A |  | UEs supporting 5G Core and NR standalone shared spectrum channel access and RSSI measurements and channel occupancy reporting |  |
| C219 |  | IF A.4.1-5/1 AND A.4.3.13-1/1 THEN R ELSE N/A |  | UEs supporting 5G Core and Multi-SIM features |  |
| C220 |  | IF A.4.1-5/1 AND A.4.3.13-1/4 THEN R ELSE N/A |  | UEs supporting 5G Core and Multi-SIM Reject paging request |  |
| C221 |  | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.7-1/19 AND A.4.3.5-1/15 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation |  |
| C222 |  | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.7-1/19 AND A.4.3.5-1/15 THEN R ELSE N/A |  | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE and direct NR SCG SCell activation |  |
| C223 |  | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.7-1/19 AND A.4.3.5-1/15 THEN R ELSE N/A |  | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE and direct NR SCG SCell activation |  |
| C224 |  | IF A.4.1-5/1 AND A.4.3.7-1/42 THEN R ELSE N/A |  | UEs supporting 5G Core and PEI |  |
| C225 |  | IF A.4.1-3/2 AND (A.4.3.6-1/61 OR A.4.3.6-1/62) THEN R ELSE N/A |  | UEs supporting EN-DC and Idle/Inactive Measurements |  |
| C226 |  | IF A.4.1-5/1 AND A.4.3.5-1/13 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A |  | UEs supporting 5G Core and direct NR MCG SCell activation and intra-band contiguous CA |  |
| C227 |  | IF A.4.1-5/1 AND A.4.3.5-1/13 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A |  | UEs supporting 5G Core and direct NR MCG SCell activation and intra-band non-contiguous CA |  |
| C228 |  | IF A.4.1-5/1 AND A.4.3.5-1/13 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A |  | UEs supporting 5G Core and direct NR MCG SCell activation and inter-band CA |  |
| C229 |  | IF A.4.1-5/1 AND A.4.1-4/6 AND A.4.3.7-1/19 AND A.4.3.7-1/44 THEN R ELSE N/A |  | UEs supporting 5G Core and NR-DC and RRC_INACTIVE and (re-)configuration of an SCG during the resume procedure. |  |
| C230 |  | IF A.4.1-5/1 AND A.4.3.7-1/37 THEN R ELSE N/A |  | UEs supporting 5G Core and NSSRG |  |
| C231 |  | IF A.4.1-5/1 AND A.4.3.7-1/24 AND A.4.3.7-1/40 THEN R ELSE N/A |  | UEs supporting 5G Core and SNPN and configuration of access identities in the list of subscriber data |  |
| C232 |  | IF A.4.3.2-1/46 AND A.4.4-1/14 THEN R ELSE N/A |  | UEs Supporting 2-Step RACH and Random access SDT |  |
| C233 |  | IF A.4.4-1/14 THEN R ELSE N/A |  | UEs Supporting Random access SDT |  |
| C234 |  | IF A.18/5 AND [93] A.4.3.7-1/32 AND A.15/1 AND A.4/16 AND A.21/1 AND C44 AND A.22/11 THEN R ELSE N/A |  | NR and IMS voice over NR and MTSI and MTSI speechand preconditions and NG.114 v1.0 and NG.114 v1.0 default configuration EVS/Br and NG.114 v1.0 default configuration EVS/Bw |  |
| C235 |  | IF A.4.3.3-1/8 THEN R ELSE N/A |  | UEs supporting 5GS and uplink data compression operation |  |
| C236 |  | IF A.4.3.3-1/8 and A.4.3.3-1/9 THEN R ELSE N/A |  | UEs supporting 5GS and uplink data compression operation and UL data compression with SIP static dictionary |  |
| C237 |  | IF [10] A.4.4-1/117 AND [10] A.4.1-1/5 AND A.4.3.8-1/21 AND [5] A.4.4-1/2 AND A.4.1-5/1 THEN R ELSE N/A |  | UEs supporting WLAN and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and handover from EPC over non-3GPP Access Network to 5G Core and IMS and 5G Core.. |  |
| CXXX |  | IF ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/37 THEN R ELSE N/A |  | UEs supporting 5GS and E-UTRA and NSSRG. |  |