3GPP TSG-RAN5 Meeting #98	R5-230643

# Athens, 27th Feb 2023- 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3530 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of RedCap TC 7.1.1.1.17-UE identification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - SS change the SIB1 in Step 12 but the message content for Table 7.1.1.1.17.3.3-5 and Table 7.1.1.1.17.3.3-2 indicated Step 13. - Paging is missing in the Message Sequence of Step 4 and Step 15. - It is not clear that the different configuration between Step1 and Step 12 in SIB1. In Step1, RedCap specific random access resource is included in the initialUplinkBWP and initialUplinkBWP-RedCap-r17 is not configured. In Step 12, initialUplinkBWP-RedCap-r17 is configured and RedCap specific random access resource is only included in the initialUplinkBWP-RedCap-r17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Correct the condition in message content for Table 7.1.1.1.17.3.3-5 and Table 7.1.1.1.17.3.3-2 from Step 13 to Step 12. 2. Add paging message in the Message Sequence of Step 4 and Step 15 3. Clarify the configuration in Table 7.1.1.1.17.3.3-2/4/4A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.1.17	Random access procedure / RedCap UE identification

7.1.1.1.17.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { UE starts Random Access procedure  }

then { the UE uses the RedCap specific random access resource in Msg1}

}

(2)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { UE starts Random Access procedure  }

then { the UE uses the RedCap specific LCID in Msg3 }

}

7.1.1.1.17.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.321, clauses 5.1.1, 5.1.1b, 5.1.1c ,5.1.1d and 6.2.1, TS38.300, clauses 16.13.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.1.1]

…

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources:

…

-	featurePriorities: priorities for features, such as REDCAP, Slice group(s), etc. (see clause 5.1.1d);

…

-	startPreambleForThisPartition: the first preamble associated with the set of Random Access Resources applicable to the Random Access procedure;

…

-	numberOfPreamblesForThisPartition: the number of consequtive preambles associated with the set of Random Access Resources applicable to the Random Access procedure;

…

When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall:

…

1>	perform the BWP operation as specified in clause 5.15;

1>	select the set of Random Access resources applicable to the current Random Access procedure according to clause 5.1.1b;

…

[TS 38.321, clause 5.1.1b]

The MAC entity shall:

…

1>	if contention-free Random Access Resources have not been provided for this Random Access procedure and one or more of the features including REDCAP and/or a specific slice group(s) and/or SDT and/or MSG3 repetition is applicable for this Random Access procedure:

Editor's Note: FFS if some clarification is needed on how feature applicability is known (e.g. from RRC etc)

2>	if none of the sets of Random Access resources are available for the current Random Access procedure (as specified in clause 5.1.1c):

3>	select the set of Random Access resources that are not associated with any feature indication (as specified in clause 5.1.1c) for this Random Access procedure.

2>	else if there are one or more set(s) of Random Access resources available (as specified in clause 5.1.1c) and one of these set(s) of Random Access resources can be used for indicating all features triggering this Random Access procedure:

3>	select the available set of Random Access resources for this Random Access procedure.

…

[TS 38.321, clause 5.1.1c]

The MAC entity shall for each set of configured Random Access resources for 4-step RA type and for each set of configured Random Access resources for 2-step RA type:

1>	if REDCAP indication is configured for a set of Random Access resources:

2>	consider the set of Random Access resources as not available for a RACH procedure for which REDCAP indication is not applicable.

…

[TS 38.321, clause 5.1.1d]

The MAC entity shall:

1>	among the available sets of Random Access resources, identify those configured with a feature which has the highest priority assigned in featurePriorities among all the features applicable to this Random Access procedure as specified in TS 38.331 [5].

1>	if a single set of Random Access resources is identified:

2>	select this set of Random Access resources.

…

[TS 38.300, clause 16.13.3]

A RedCap UE can be identified by the network during Random Access procedure via MSG3/MSGA from a RedCap specific LCID(s) and optionally via MSG1/MSGA (PRACH occasion or PRACH preamble). For RedCap UE identification via MSG1/MSGA, RedCap specific Random Access configuration may be configured by the network. For MSG3/MSGA, a RedCap UE is identified by the dedicated LCID(s) indicated for CCCH identification (CCCH or CCCH1) regardless whether RedCap specific Random Access configuration is configured by the network.

[TS 38.321, clause 6.2.1]

Table 6.2.1-2 Values of LCID for UL-SCH

| Codepoint/Index |  | LCID values |
| --- | --- | --- |
| 0 |  | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]), except for a RedCap UE |
| 1–32 |  | Identity of the logical channel of DCCH and DTCH |
| 33 |  | Extended logical channel ID field (two-octet eLCID field) |
| 34 |  | Extended logical channel ID field (one-octet eLCID field) |
| 35 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) for a RedCap UE |  |
| 36 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) for a RedCap UE |  |
| 37–42 |  | Reserved |
| 43 |  | Truncated Enhanced BFR (one octet Ci) |
| 44 |  | Timing Advance Report |
| 45 |  | Truncated Sidelink BSR |
| 46 |  | Sidelink BSR |
| 47 |  | Reserved |
| 48 |  | LBT failure (four octets) |
| 49 |  | LBT failure (one octet) |
| 50 |  | BFR (one octet Ci) |
| 51 |  | Truncated BFR (one octet Ci) |
| 52 |  | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]), except for a RedCap UE |
| 53 |  | Recommended bit rate query |
| 54 |  | Multiple Entry PHR (four octets Ci) |
| 55 |  | Configured Grant Confirmation |
| 56 |  | Multiple Entry PHR (one octet Ci) |
| 57 |  | Single Entry PHR |
| 58 |  | C-RNTI |
| 59 |  | Short Truncated BSR |
| 60 |  | Long Truncated BSR |
| 61 |  | Short BSR |
| 62 |  | Long BSR |
| 63 |  | Padding |

7.1.1.1.17.3	Test description

7.1.1.1.17.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

UE:

-	None

Preamble:

-	The UE is in 1N-A state on NR Cell 1 using generic procedure parameter Connectivity (NR) and Test loop function(Off) according to TS 38.508-1 [4].

7.1.1.1.17.3.2	Test procedure sequence

Table 7.1.1.1.17.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the SIB1 of NR Cell 1 to configure the specific PRACH resource for RedCap in initialUplinkBWP. | - | - | - | - |
| 2 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 3 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 4 | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |
| 5 | Check: Does the UE transmits Preamble on PRACH with ra-PreambleIndex range from 8 to 11 on NR Cell 1? | --> | PRACH Preamble | 1 | P |
| 6 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 5. | <-- | Random Access Response | - |  |
| 7 | Check: Does the UE transmits a MAC PDU containing an RRCSetupRequest message by setting LCID to 35? | --> | MAC PDU (RRCSetupRequest) | 2 | P |
| 8 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing an RRCSetup message and ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (RRCSetup and UE Contention Resolution Identity MAC CE) | - | - |
| 9 | UE transmit a MAC PDU containing an RRCSetupComplete message indicating acceptance of RRCSetup message | --> | MAC PDU (RRCSetupComplete) | - | - |
| 10 | The test steps 5 to 8 of generic test procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 11 | The SS transmits RRCRelease. | <-- | RRCRelease | - |  |
| 12 | The SS changes the SIB1 of NR Cell 1 to configure the specific PRACH resource for RedCap in initialUplinkBWP-RedCap. | - | - | - | - |
| 13 | The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. | <-- | PDCCH (DCI 1_0): Short Message | - | - |
| 14 | Wait for 2.1* modification period second for the UE to receive new system information. (Note 1) | - | - | - | - |
| 15 | The SS transmits a Paging message including a matched identity. | <-- | Paging | - | - |
| 16 | Check: Does the UE transmit Preamble on PRACH with ra-PreambleIndex range from 12 to 15 on NR Cell 1? | --> | PRACH Preamble | 1 | P |
| 17 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 16. | <-- | Random Access Response | - |  |
| 18 | Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message by setting LCID to 35? | --> | MAC PDU (RRCSetupRequest) | 2 | P |
| 19 | The SS schedules PDCCH transmission addressed to TC-RNTI to transmit a valid MAC PDU containing an RRCSetup message and ‘UE Contention Resolution Identity’ MAC control element with matched ‘Contention Resolution Identity’. | <-- | MAC PDU (RRCSetup and UE Contention Resolution Identity MAC CE) | - | - |
| 20 | UE transmit a MAC PDU containing an RRCSetupComplete message indicating acceptance of RRCSetup message | --> | MAC PDU (RRCSetupComplete) | - | - |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

7.1.1.1.17.3.3	Specific message contents

Table 7.1.1.1.17.3.3-1: SIB1 (step 1 and step12, Table 7.1.1.1.17.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 7.1.1.1.17.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| featurePriorities-r17 SEQUENCE { |  |  |  |  |
| redCapPriority-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.1.17.3.3-2: ServingCellConfigCommonSIB (Table 7.1.1.1.17.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | Table 7.1.1.1.17.3.3-3 | Step1 |
| uplinkConfigCommon | UplinkConfigCommonSIB | TS 38.508-1 [4] Table 4.6.3-202 | Step 12 |
| uplinkConfigCommon-v1700 | Not present |  | Step 1 |
| uplinkConfigCommon-v1700 SEQUENCE { |  |  | Step12 |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon-RedCap | Table 7.1.1.1.17.3.3-4A |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.17.3.3-3: UplinkConfigCommonSIB (Table 7.1.1.1.17.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-202 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon | Table 7.1.1.1.17.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.1.17.3.3-4: BWP-UplinkCommon (Table 7.1.1.1.17.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon | Table 7.1.1.1.17.3.3-5 | Step 1 |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.17.3.3-4A: BWP-UplinkCommon-RedCap (Table 7.1.1.1.17.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon | Table 7.1.1.1.17.3.3-5 | Step 12 |
| } |  |  |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon-RedCap | Table 7.1.1.1.17.3.3-6 | Step 12 |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.17.3.3-5: RACH-ConfigCommon (Table 7.1.1.1.17.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| featureCombinationPreamblesList-r17 SEQUENCE (SIZE(1..maxFeatureCombPreamblesPerRACHResource-r17)) OF FeatureCombinationPreambles-r17 { | 1 entry |  |  |
| FeatureCombinationPreambles-r17[1] SEQUENCE { |  | entry 1 |  |
| featureCombination-r17 SEQUENCE { |  |  |  |
| redCap-r17 | True |  |  |
| } |  |  |  |
| startPreambleForThisPartition-r17 | 8 |  | Step 1 |
|  | 12 |  | Step 12 |
| numberOfPreamblesPerSSB-ForThisPartition-r17 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.17.3.3-6: PUCCH-ConfigCommon-RedCap (Table 7.1.1.1.17.3.3-4A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not present |  |  |
| pucch-ResourceCommonRedCap-r17 | 0 |  |  |
| } |  |  |  |

<End of modified section 1>