3GPP TSG-RAN5 Meeting #98	R5-230704

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3546 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of TC 12.1.7.1 - PC5-only operation / Sidelink UE capability transfer via PC5 RRC / One-way and two-way transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The pre-test condition for the system simulator need to mention that GNSS should be used as synchronization reference source. Also the pre-test condition for the UE should mention that UE should be equipped with default USIM. Lastly, the editors note in cl 12.1.7.1.3.2 needs to be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the pre-test conditions, and removed the editors note from cl 12.1.7.1.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose not in-line with other V2X testcases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 12.1.7.1	PC5-only operation / Sidelink UE capability transfer via PC5 RRC / One-way and two-way transfer

12.1.7.1.1	Test Purpose (TP)

(1)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE receives a UECapabilityEnquirySidelink message from peer UE }

then { UE sends a UECapabilityInformationSidelink message to peer UE }

}

(2)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE is configured by upper layer to initiate capability transfer procedure }

then { UE sends a UECapabilityEnquirySidelink message to peer UE }

}

(3)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE is configured by upper layer to initiate capability transfer procedure and to provide UE radio access capabilities }

then { UE sends a UECapabilityEnquirySidelink message with ue-CapabilityInformationSidelink to peer UE }

}

12.1.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.9.2.1, 5.8.9.2.2, 5.8.9.2.3 and 5.8.9.2.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, subclause 5.8.9.2.1]

This clause describes how the UE compiles and transfers its sidelink UE capability information for unicast to the initiating UE.

Figure 5.8.9.2.1-1: Sidelink UE capability transfer

[TS 38.331, subclause 5.8.9.2.2]

The UE may initiate the sidelink UE capability transfer procedure upon indication from upper layer when it needs (additional) UE radio access capability information.

[TS 38.331, subclause 5.8.9.2.3]

The initiating UE shall set the contents of UECapabilityEnquirySidelink message as follows:

1>	include in UE radio access capabilities for sidelink within ue-CapabilityInformationSidelink, if needed;

NOTE 1:	It is up to initiating UE to decide whether ue-CapabilityInformationSidelink should be included.

1>	set frequencyBandListFilterSidelink to include frequency bands for which the peer UE is requested to provide supported bands and band combinations;

NOTE 2:	The initiating UE is not allowed to send the UECapabilityEnquirySidelink message without including the field frequencyBandListFilterSidelink.

1>	submit the UECapabilityEnquirySidelink message to lower layers for transmission.

[TS 38.331, subclause 5.8.9.2.4]

The peer UE shall set the contents of UECapabilityInformationSidelink message as follows:

1>	include UE radio access capabilities for sidelink within ue-CapabilityInformationSidelink;

1>	compile a list of "candidate band combinations" only consisting of bands included in frequencyBandListFilterSidelink, and prioritized in the order of frequencyBandListFilterSidelink (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on).

1>	include into supportedBandCombinationListSidelinkNR as many band combinations as possible from the list of "candidate band combinations", starting from the first entry;

1>	include the received frequencyBandListFilterSidelink in the field appliedFreqBandListFilter of the requested UE capability;

1>	submit the UECapabilityInformationSidelink message to lower layers for transmission.

NOTE:	If the UE cannot include all band combinations due to message size or list size constraints, it is up to UE implementation which band combinations it prioritizes.

12.1.7.1.3	Test description

12.1.7.1.3.1	Pre-test conditions

System Simulator:

-	SS-UE

-	Operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	SS UE uses GNSS as the synchronization reference source.

-	GNSS simulator

UE:

- UE is authorised to perform NR sidelink communication.

- UE is equipped with USIM containing default value (as per TS38.508-1[4] clause 4.8.3.3.3).

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On).

12.1.7.1.3.2	Test procedure sequence

Table 12.1.7.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE receives a UECapabilityEnquirySidelink message from SS-UE on SL-SRB3. | <-- | PC5 RRC: UECapabilityEnquirySidelink | - | - |
| 2 | Check: Does the UE send a UECapabilityInformationSidelink message? |  | PC5 RRC: UECapabilityInformationSidelink | 1 | P |
| 3 | The UE is configured by upper layers to initiate capability transfer procedure and to include only the single frequency band and band combination which is currently used in this TC for communication over PC5 interface. Note: This step is triggered by MMI or AT command. | - | - | - | - |
| 4 | Check: Does the UE send on SL-SRB3 a UECapabilityEnquirySidelink message? | --> | PC5 RRC: UECapabilityEnquirySidelink | 2 | P |
| 5 | The SS-UE sends a UECapbilityInformationSidelink message. | <-- | PC5 RRC: UECapabilityInformationSidelink | - | - |
| 6 | The UE is configured by upper layers to initiate capability transfer procedure, to include only the single frequency band and band combination which is currently used in this TC for communication over PC5 interface and to provide UE radio access capabilities. Note: This step is triggered by MMI or AT command. | - |  |  |  |
| 7 | Check: Does the UE send on SL-SRB3 a UECapabilityInformationSidelink message with ue-CapabilityInformationSidelink-r16 IE? | --> | PC5 RRC: UECapabilityEnquirySidelink | 3 | P |
| 8 | The SS-UE sends a UECapabilityInformationSidelink message. | <-- | PC5 RRC: UECapabilityInformationSidelink | - | - |

12.1.7.1.3.3	Specific message contents

Table 12.1.7.1.3.3-1: UECapabilityEnquirySidelink (step 1, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquirySidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquirySidelink-r16 SEQUENCE { |  |  |  |
| frequencyBandListFilterSidelink-r16 | FreqBandList with condition NR_SL_PC5 | Includes only the single frequency band and band combination which is currently used in this TC for communication over the PC5 interface | NR_SL_PC5 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.7.2.3.3-2: UECapabilityInformationSidelink (step 2, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquirySidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformationSidelink-r16 SEQUENCE { |  |  |  |
| pdcp-ParametersSidelink-r16 SEQUENCE { |  |  |  |
| outOfOrderDeliverySidelink-r16 | Checked |  | pc_outOfOrderDeliverySidelink_r16 |
| } |  |  |  |
| rlc-ParametersSidelink-r16 SEQUENCE { |  |  |  |
| am-WithLongSN-Sidelink-r16 | Checked |  | pc_amWithLongSN_Sidelink_r16 |
| um-WithLongSN-Sidelink-r16 | Checked |  | pc_umWithLongSN_Sidelink_r16 |
| } |  |  |  |
| supportedBandCombinationListSidelinkNR-r16 | Checked | Includes all band combinations which the UE (= UE Under Test) supports for NR Sidelink acc. to the declared UE capabilities |  |
| supportedBandListSidelink-r16 | Checked | Includes all frequency bands which the UE (= UE Under Test) supports for NR Sidelink acc. to the declared UE capabilities |  |
| appliedFreqBandListFilter-r16 | Checked | Includes only the single frequency band and band combination which the SS-UE requested to include in the IE frequencyBandListFilterSidelink-r16 of UECapabilityEnquirySidelink message acc. to Table 12.1.7.2.3.3-1. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.7.1.3.3-3: UECapabilityEnquirySidelink (step 4, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquirySidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquirySidelink-r16 SEQUENCE { |  |  |  |
| frequencyBandListFilterSidelink-r16 | FreqBandList with condition NR_SL_PC5 | Includes only the single frequency band and band combination which is currently used in this TC for communication over the PC5 interface | NR_SL_PC5 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.7.1.3.3-4: UECapabilityInformationSidelink (steps 5 and 8, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.331 [6], clause 6.6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformationSidelink ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier-r16 | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformationSidelink-r16 SEQUENCE { |  |  |  |
| accessStratumReleaseSidelink-r16 | rel16 |  |  |
| supportedBandCombinationListSidelinkNR-r16 |  | Includes only the single band combination which is currently used in this TC for communication over the PC5 interface. |  |
| supportedBandListSidelink-r16 |  | Includes only the single frequency band which is currently used in this TC for communication over the PC5 interface |  |
| appliedFreqBandListFilter-r16 |  | Includes only the single frequency band and band combination which the UE requested to include in the IE frequencyBandListFilterSidelink-r16 of UECapabilityEnquirySidelink message acc. to Table 12.1.7.1.3.3-1. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.7.1.3.3-5: UECapabilityEnquirySidelink (step 7, Table 12.1.7.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-6 with condition TWO_WAY_ENQUIRY |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquirySidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityEnquirySidelink-r16 SEQUENCE { |  |  |  |
| frequencyBandListFilterSidelink-r16 | FreqBandList with condition NR_SL_PC5 | Includes only the single frequency band and band combination which is currently used in this TC for communication over the PC5 interface | NR_SL_PC5 |
| ue-CapabilityInformationSidelink-r16 | Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |