3GPP TSG-RAN5 Meeting #98	R5-231079

# Athens,Greece, 27th Feb – 3rd Mar. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2735 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating test frequencies for n79 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to annex.C, the valid kSSB should meet condition: kSSB MOD N =0, where N= SCSSSB / 15 kHz for FR1. And for band n79 when 10MHz CBW @ 15kHz SCS applies, CORESET#0 is 48RBs and NRB is 52RBs. The Offset Carrier CORESET#0 should be no larger than 4RBs. The test frequencies for n79 10MHz CBW is incorrect and need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Editor’s note in 4.3.1; correcting test frequencies for n79 10MHz CBW. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test frequency for n79 is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.3.1	Test frequencies

Editor's Note: n7 (Mid: 25MHz, High: 50MHz), n8 (High: 35MHz), n25 (High: 45MHz), n40 (High: 100MHz), n65 (High: 50MHz), n66 (High: 45MHz), n71 (High: 35MHz), n75(High: 50MHz), and n80 (High: 40MHz) are currently not yet included in the test channel bandwidth tables due to test frequency have not been defined yet. Further analysis is needed for the test coverage of these test channel bandwidths and bands.

4.3.1.0	General

The test frequencies are based on operating bands defined in TS 38.101-1 [7], TS 38.101-2 [8] and TS 38.101-3 [9].

<Unchanged Text Skipped>

4.3.1.1	Test frequencies for NR operating bands in FR1

4.3.1.1.1	NR operating bands in FR1

<Unchanged Text Skipped>

4.3.1.1.1.79	Reference test frequencies for NR operating band n79

Table 4.3.1.1.1.79-1: Test frequencies for NR operating band n79, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 103 | 52 | Downlink | Low | 4405.02 | 693668 | 4400.34 | 693356 | 0 | 30 | 8475 | 693696 | 4 | 4 | 0 (4) | 8 |
|  |  | & | Mid | 4700.1 | 713340 | 4677.06 | 711804 | 102 |  | 8680 | 713376 | 12 | 4 | 0 (4) | 110 |
|  |  | Uplink | High | 4994.64 | 732976 | 4899.24 | 726616 | 504 |  | 8884 | 732960 | 8 | 0 | 0 (4) | 508 |
| 20 | 106 | Downlink | Low | 4410 | 694000 | 4400.46 | 693364 | 0 | 30 | 8476 | 693792 | 8 | 11 | 0 (4) | 15 |
|  |  | & | Mid | 4699.995 | 713333 | 4672.095 | 711473 | 102 |  | 8676 | 712992 | 7 | 0 | 0 (4) | 106 |
|  |  | Uplink | High | 4990.005 | 732667 | 4889.745 | 725983 | 504 |  | 8878 | 732384 | 5 | 5 | 0 (4) | 513 |
| 40 | 216 | Downlink | Low | 4420.02 | 694668 | 4400.58 | 693372 | 0 | 30 | 8480 | 694176 | 0 | 43 | 0 (4) | 47 |
|  |  | & | Mid | 4699.98 | 713332 | 4662.18 | 710812 | 102 |  | 8672 | 712608 | 8 | 23 | 0 (4) | 129 |
|  |  | Uplink | High | 4980 | 732000 | 4869.84 | 724656 | 504 |  | 8864 | 731040 | 0 | 4 | 0 (4) | 512 |
| 50 | 270 | Downlink | Low | 4425 | 695000 | 4400.7 | 693380 | 0 | 30 | 8480 | 694176 | 4 | 42 | 0 (4) | 46 |
|  |  | & | Mid | 4699.98 | 713332 | 4657.32 | 710488 | 102 |  | 8672 | 712608 | 8 | 50 | 0 (4) | 156 |
|  |  | Uplink | High | 4974.99 | 731666 | 4859.97 | 723998 | 504 |  | 8864 | 731040 | 10 | 58 | 0 (4) | 566 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The High range frequency can’t cover the two RBs at the right edge of band due to the limit of GSCN range in TS 38.101-1 table 5.4.3.3-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-2: Test frequencies for NR operating band n79, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 4405.02 | 693668 | 4400.7 | 693380 | 0 | 30 | 8475 | 693696 | 4 | 3 | 0 (0) | 6 |
|  |  | & | Mid | 4700.01 | 713334 | 4658.97 | 710598 | 102 |  | 8680 | 713376 | 18 | 3 | 0 (0) | 210 |
|  |  | Uplink | High | 4995 | 733000 | 4809.24 | 720616 | 504 |  | 8884 | 732960 | 8 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 4410 | 694000 | 4400.82 | 693388 | 0 | 30 | 8476 | 693792 | 20 | 2 | 1 (4) | 12 |
|  |  | & | Mid | 4700.01 | 713334 | 4654.11 | 710274 | 102 |  | 8676 | 712992 | 6 | 1 | 0 (0) | 206 |
|  |  | Uplink | High | 4990.02 | 732668 | 4799.4 | 719960 | 504 |  | 8878 | 732384 | 16 | 3 | 0 (0) | 1014 |
| 40 | 106 | Downlink | Low | 4420.02 | 694668 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4644.21 | 709614 | 102 |  | 8672 | 712608 | 18 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 4980 | 732000 | 4779.48 | 718632 | 504 |  | 8864 | 731040 | 0 | 3 | 0 (0) | 1014 |
| 50 | 133 | Downlink | Low | 4425 | 695000 | 4401.06 | 693404 | 0 | 30 | 8480 | 694176 | 4 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4639.35 | 709290 | 102 |  | 8672 | 712608 | 6 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 4974.99 | 731666 | 4769.61 | 717974 | 504 |  | 8864 | 731040 | 10 | 26 | 1 (4) | 1068 |
| 60 | 162 | Downlink | Low | 4430.01 | 695334 | 4400.85 | 693390 | 0 | 30 | 8480 | 694176 | 18 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4634.13 | 708942 | 102 |  | 8672 | 712608 | 18 | 36 | 1 (4) | 284 |
|  |  | Uplink | High | 4969.98 | 731332 | 4759.38 | 717292 | 504 |  | 8864 | 731040 | 20 | 54 | 1 (4) | 1124 |
| 80 | 217 | Downlink | Low | 4440 | 696000 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4624.23 | 708282 | 102 |  | 8656 | 711072 | 6 | 0 | 1 (4) | 212 |
|  |  | Uplink | High | 4959.99 | 730666 | 4739.49 | 715966 | 504 |  | 8848 | 729504 | 2 | 46 | 1 (4) | 1108 |
| 100 | 273 | Downlink | Low | 4450.02 | 696668 | 4400.88 | 693392 | 0 | 30 | 8480 | 694176 | 16 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4614.15 | 707610 | 102 |  | 8656 | 711072 | 6 | 28 | 1 (4) | 268 |
|  |  | Uplink | High | 4950 | 730000 | 4719.42 | 714628 | 504 |  | 8832 | 727968 | 20 | 37 | 1 (4) | 1090 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-6 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-3: Test frequencies for NR operating band n79, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0.

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 4405.02 | 693668 | 4401.06 | 693404 | 0 | 30 | - | 693644 |
|  |  | & | Mid | 4700.01 | 713334 | 4622.61 | 708174 | 102 |  | - | 713310 |
|  |  | Uplink | High | 4995 | 733000 | 4628.16 | 708544 | 504 |  | - | 732976 |
| 20 | 24 | Downlink | Low | 4410 | 694000 | 4401.36 | 693424 | 0 | 30 | - | 693664 |
|  |  | & | Mid | 4700.01 | 713334 | 4617.93 | 707862 | 102 |  | - | 712998 |
|  |  | Uplink | High | 4990.02 | 732668 | 4618.5 | 707900 | 504 |  | - | 732332 |
| 40 | 51 | Downlink | Low | 4420.02 | 694668 | 4401.66 | 693444 | 0 | 30 | - | 693684 |
|  |  | & | Mid | 4700.01 | 713334 | 4608.21 | 707214 | 102 |  | - | 712350 |
|  |  | Uplink | High | 4980 | 732000 | 4598.76 | 706584 | 504 |  | - | 731016 |
| 50 | 65 | Downlink | Low | 4425 | 695000 | 4401.6 | 693440 | 0 | 30 | - | 693680 |
|  |  | & | Mid | 4700.01 | 713334 | 4603.17 | 706878 | 102 |  | - | 712014 |
|  |  | Uplink | High | 4974.99 | 731666 | 4588.71 | 705914 | 504 |  | - | 730346 |
| 60 | 79 | Downlink | Low | 4430.01 | 695334 | 4401.57 | 693438 | 0 | 30 | - | 693678 |
|  |  | & | Mid | 4700.01 | 713334 | 4598.13 | 706542 | 102 |  | - | 711678 |
|  |  | Uplink | High | 4969.98 | 731332 | 4578.66 | 705244 | 504 |  | - | 729676 |
| 80 | 107 | Downlink | Low | 4440 | 696000 | 4401.48 | 693432 | 0 | 30 | - | 693672 |
|  |  | & | Mid | 4700.01 | 713334 | 4588.05 | 705870 | 102 |  | - | 711006 |
|  |  | Uplink | High | 4959.99 | 730666 | 4558.59 | 703906 | 504 |  | - | 728338 |
| 100 | 135 | Downlink | Low | 4450.02 | 696668 | 4401.42 | 693428 | 0 | 30 | - | 693668 |
|  |  | & | Mid | 4700.01 | 713334 | 4577.97 | 705198 | 102 |  | - | 710334 |
|  |  | Uplink | High | 4950 | 730000 | 4538.52 | 702568 | 504 |  | - | 727000 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

<End of Changes>