3GPP TSG-RAN5 Meeting #98	R5-231176

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

# Source:	Qualcomm Incorporated, Rohde & Schwarz

# Title:	Impact of DNS IP address inclusion in PDU Session Establishment Accept message

# Document for:	Discussion

## 1.	Introduction

Prose CR [5] introduced inclusion of DNS IPV4 and IPV6 addresses in PDU Session Establishment Accept message when UE requests for it in PDU Session Establishment Request message. This causes an issue while running NR protocol test cases from [1], [2].

## 2.	Discussion

In NR, when mobile data is enabled, UE requested PDU session is established automatically after completion of Registration procedure on same RRC connection.

Device under test may be configured to trigger multiple PDU sessions (IMS and Internet). In a common scenario execution with multi PDU configuration, IMS DNN would automatically be brought up after the registration procedure and additional PDU session (for internet DNN) may be brought up automatically by UE when mobile data is enabled on the UE.

Prose CR [5] introduced inclusion of DNS server IP address in PDU Session Establishment Accept message if the UE requested for it in the PDU Session Establishment Request message. However, when Mobile data is ON, UE may start sending multiple DNS queries and thereby initiate unexpected service request procedures. This causes failure of NR protocol test cases in [1], [2].

The changes in [5] (associated with TTCN CR [6]) were introduced to handle the issue observed while running XCAP test cases (a XCAP enabled UE may resolve the IP address of the XCAP server by sending DNS query).

For non-XCAP test cases, it is not necessary to have DNS IP address in PDU Session Establishment Accept message. Hence to address the afore mentioned execution issue, it is recommended to not include DNS IP address in PDU Session Establishment Accept message.

For XCAP test cases, as specified in clause A.21 of [2], UE shall be registered to IMS on an IMS PDU session and at least have one PDU session for Internet active. It is recommended to have DNS IP address in PDU Session Establishment Accept messages for all PDU sessions. However, the XCAP testcases may still fail as the UE may start sending multiple DNS queries and thereby initiate unexpected service request procedures if Mobile data is ON.

## 3.	Proposal

- Proposal #1:	Update default message content of PDU SESSION ESTABLISHMENT ACCEPT in Table 4.7.2-2 of [3] to have DNS address assigned only for XCAP test cases and

- Proposal #2:	Update XCAP testcase related procedures in [2] about the impact of Mobile data, px_MobileDataOn (defined in [4]).

## 4.	References

[1]	TS 38.523-1	LTE; 5G; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	TS 34.229-5	5G; Internet Protocol (IP) multimedia call control protocol based on Session Initiation         Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)

[3]	TS 38.508-1	LTE; 5G; 5GS; User Equipment (UE) conformance specification; Part 1: Common test environment

[4]	TS 36.523-3	LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test suites

[5]	R5-224166	Correction to Table 4.7.2-2 PDU Session Establishment Accept

[6]	R5s220841	Correction to NR XCAP test cases