3GPP TSG-RAN5 Meeting #98	R5-231778
Athens, Greece, 27th February – 3rd March 2023

Agenda item:	5.4.17

Source:	Keysight Technologies

Title:	Discussion on FR2 PC1 MU

Document for:	Discussion and Endorsement

# Introduction

In order to meet Japanese regulation requirements [1], during RAN5#95-e meeting was requested to accelerate the MU and TT definition for PC1 devices during this year. Significant progress was made during previous two meetings as captured in discussions [2] to [7] and associated CRs.

The aim of this document is to consolidate the agreements from previous meeting and to progress in discussions to define MU and TT definition for the pending test cases.

# Discussion

## Consolidation of previous meeting agreements

During previous meeting, MU and TT were defined for FR2a and NTC for ACLR, General Spurious and Rx Spurious test cases. However, final values were left in [ ] just to have some extra margin to further review the proposed values.

After further check, for ACLR test it has been observed that TT formula was not correctly applied and value should be 5.23dB instead of 5.31dB. Apart from that minor issue, the rest of the values for ACLR, General Spurious and Rx Spurious test cases seem correct and then can be consolidated by removing the [ ] in order to get them completed for PC1 for FR2a and NTC.

Proposal 1. Remove [ ] for MU and TTs defined for PC1 for FR2a and NTC for ACLR, General Spurious and Rx Spurious test cases.

## OBW test

After further checking, we conclude no special treatment is required for PC1 and hence values from PC3 can be reused.

Proposal 2. For OBW test, reuse MU and TT from PC3 to PC1.

## Influence of Noise

Based on empirical analysis of our test system, this is the influence of noise proposed for the following test cases.

| Test case | Frequency range | Relaxation [dB] | Influence of noise [dB] |  |
| --- | --- | --- | --- | --- |
| MOP-EIRP | FR2a | 0 (agreed in [8]) | 0.13 (agreed in [8]) |  |
|  | FR2b | 0 | 0.2 |  |
| MOP-TRP | FR2a | 0 (agreed in [8]) | 0.13 (agreed in [8]) |  |
|  | FR2b | 0 | 0.2 |  |
| MOP-Spherical | FR2a | 0 | 0.2 |  |
|  | FR2b | 0 | 0.35 |  |
| MPR | FR2a | 0 | 0.1 |  |
|  | FR2b | 0 | 0.3 |  |
| Minimum output power | FR2a | 0 | 0.3 |  |
|  | FR2b | 0 | 0.6 |  |
| ACLR (ACP) | FR2a | 0 | 0.95 (agreed in [4]) |  |
|  | FR2b | 0 | 0.95 |  |
| SEM | FR2a | 0 (agreed in [4]) | 1.81 (agreed in [4]) |  |
|  | FR2b | 0 | 2.08 (NOTE 1, 3) |  |
| Tx spurious Co-existence | n260 (Aggressor band : n257, n261) | 5 (Same as PC3) | 1.69 (NOTE 1, 4) |  |
|  | n257, n261 (Aggressor band : n260) | 3.3 (Same as PC3) | 1.69 (NOTE 1, 4) |  |
|  | 23.6 GHz ≤ f ≤ 24.0GHz | 0.3 (Same as PC3) (agreed in [4]) | 2.34 (NOTE 1, 2) (agreed in [4]) |  |
|  | 57 GHz ≤ f ≤ 66GHz | Same as defined for PC3 in Table B.2.2.27-1 (agreed in [4]) |  |  |
| Additional spurious emission | NS_202 (7.25GHz <=f <=12.75GHz) | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
|  | NS_202 (12.75GHz <=f <=23.45GHz) | Same as defined for PC3 in Table B.2.2.27-1 |  |  |
|  | NS_202, NS_203 (23.6GHz <=f <=24.0GHz) | 0.3 (Same as PC3) (agreed in [4]) |  | 2.34 (NOTE 1, 2) (agreed in [4]) |
|  | NS_202 (23.45GHz <=f <=40.8GHz) | 13 (Same as PC3) |  | 1.32 (NOTE 1, 5) |
|  | NS_202 (40.8GHz <=f <=66GHz) | 13 (Same as PC3) |  | 1.32 (NOTE 1, 5) |
| NOTE 1: values assuming up to 6% of grid points with EIRP > 43dBm, as agreed in [3]. NOTE 2: values assuming SNR = -9dB for points with EIRP > 43dBm / SNR = 6dB otherwise and considering the proposed relaxation. NOTE 3: values assuming SNR = -8dB for points with EIRP > 43dBm / SNR = 6dB otherwise. NOTE 4: values assuming SNR = -6dB for points with EIRP > 43dBm / SNR = 6dB otherwise and considering the proposed relaxation. NOTE 5: values assuming SNR = -3dB for points with EIRP > 43dBm / SNR = 6dB otherwise and considering the proposed relaxation. |  |  |  |  |

- Table 1. Proposed influence of noise for PC1 devices for MOP tests.

Regarding ACLR test case, as previously commented, current value agreed for FR2a is based on the worst MPR case. However, such an MPR case does not apply to any of the ACLR test points whose worst case is about 10dB higher. Despite we can respect the agreement for FR2a, in case of FR2b, we consider that approach cannot be followed, to assume worst MPR case even if it does not apply, because in that case it cannot be justified later which ACLR test points need to be relaxed because there is no one indeed. As indicated during discussion in previous meeting, 0.5dB for FR2a and 0.95dB for FR2b are acceptable influence of noise for current ACLR test points for PC1. Even less is acceptable from our side (0.15dB for FR2a and 0.3dB in FR2b as per our original proposal made in [16]).

Proposal 3. For PC1 devices, for the test cases listed in table 1, agree influence of noise and relaxation proposed in it.

Given that no relaxation would be required for Minimum output power and that influence of noise would be lower than 1dB, the TT formula used for PC3 should be updated for PC1 as follows:

TT = ΔSNRmr + 0.65 x (MTSUIFF – ΔSNRmr))

Proposal 4. For PC1 devices, in Minimum output power test case, define TT = ΔSNRmr + 0.65 x (MTSUIFF - ΔSNRmr).

## PC1 Testing on FR2b

Japanese regulation requirements in [1] are focused on FR2a, in particular n257. In MU definition, frequency range is mainly affecting to the influence of noise contribution.

Despite FR2b is of lower priority, whenever possible, it seems convenient to define MU for FR2b at the same time to leverage corresponding discussions for FR2a.

Proposal 5. Not to delay unnecessarily the MU and TT definition for PC1 devices in FR2b.

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1. Remove [ ] for MU and TTs defined for PC1 for FR2a and NTC for ACLR, General Spurious and Rx Spurious test cases.

Proposal 2. For OBW test, reuse MU and TT from PC3 to PC1.

Proposal 3. For PC1 devices, for the test cases listed in table 1, agree influence of noise and relaxation proposed in it.

Proposal 4. For PC1 devices, in Minimum output power test case, define TT = ΔSNRmr + 0.65 x (MTSUIFF - ΔSNRmr).

Proposal 5. Not to delay unnecessarily the MU and TT definition for PC1 devices in FR2b.

# References

- R5-223621, “Handling of FR2 Power Class 1 in RAN5”, NTT DOCOMO, INC., RAN5#95-e

- R5-227984, “Discussion on FR2 PC1 MU”, Keysight, RAN5#97

- R5-226243, “On AP96.32 Number of grid points exceeding 43dBm”, Keysight, RAN5#97

- R5-227973, “PC1 MU - General Update in 38.903 section B.2.2”, Keysight, RAN5#97

- R5-227983, “MU discussion on FR2 PC1”, Anritsu, NTT DOCOMO INC., RAN5#97

- R5-227988, “Handling of FR2 PC1 in RAN5”, NTT DOCOMO INC., Anritsu, Keysight, RAN5#97

- R5-227987, “On the MU for FR2 PC1 TRx test cases”, ROHDE & SCHWARZ, RAN5#97

- R5-225639, “MU discussion on FR2 PC1”, Anritsu, NTT DOCOMO INC., RAN5#96-e

- R5-225221, “On PC1 measurement uncertainties for other test cases”, Keysight, RAN5#96-e

- R5-225643, “Handling of FR2 PC1 in RAN5”, NTT DOCOMO INC., Anritsu, Keysight, RAN5#96-e

- R5-225641, “On the MU for FR2 PC1 TRx test cases”, ROHDE & SCHWARZ, RAN5#96-e

- TS 38.521-2, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”, v17.1.0 (2022-12)

- TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, v17.0.0 (2022-12)

- R5-226243, “On AP96.32 Number of grid points exceeding 43dBm”, Keysight Technologies, 3GPP TSG-RAN5 Meeting #97, November 2022.

- R5-226244, “On TRP MU for Spurious Emissions”, Keysight Technologies, 3GPP TSG-RAN5 Meeting #97, November 2022.

- R5-226109, “Discussion on FR2 PC1 MU”, Keysight Technologies, 3GPP TSG-RAN5 Meeting #97, November 2022.