3GPP TSG-RAN5 Meeting #99	R5-232856

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0678 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Demodulation performance testcases for PDSCH CRS interference mitigation under NR-LTE coexistense |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Europe Inc. Sweden |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of Demodulation performance testcases for PDSCH CRS interference mitigation under NR-LTE coexistense |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of test cases 5.2.2.1.18, 5.2.3.1.17, 5.2.2.2.19, 5.2.3.2.18 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

##### 5.2.2.1.17	2Rx FDD FR1 PDSCH performance for RedCap

<< Sections skipped >>

5.2.2.1.17.5	Test Requirement

Tables 5.2.2.1.17.3-3 and 5.2.2.1.17.3-4 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.17.5-1 and Table 5.2.2.1.17.5-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.17.5-1: Test Requirements for Rank 1

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD R.PDSCH. 1-1.1 HD-FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | 0.1 |
| 1-2 | R.PDSCH.1-2.1 FDD R.PDSCH. 1-1.2 HD-FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 70 | [9.0] |
| 1-3 | R.PDSCH.1-4.1 FDD R.PDSCH. 1-1.5 HD-FDD | 10 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | 25.6 |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

Table 5.2.2.1.17.5-2: Test Requirements for Rank 2

| Test num. | Reference channel (Note 1) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD R.PDSCH. 1-2.1 HD-FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 70 | 20.4 |
| Note 1: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |  |  |  |  |

##### 5.2.2.1.18	2Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

5.2.2.1.18.0             Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.1.18.0-4, with the addition of test parameters in Tables 5.2.2.1.18.0-2 and 5.2.2.1.18.0-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.18.0-1.

Table 5.2.2.1.18.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 2 receive antenna conditions with CRS rate matching configured for the serving cell | 1-1 |

Table 5.2.2.1.18.0-2: Test parameters for the serving cell

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDCCH configuration | Symbols with PDCCH |  | Symbol# 2 |  |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DM-RS for downlink |  | 3 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 2 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier. |  |  |  |

Table 5.2.2.1.18.0-3: Test parameters for the LTE interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Propagation conditions and MIMO configuration (Note 1) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| INR (Note 2) |  | dB | 10.45 | 4.6 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| Carrier centre subcarrier location |  |  | Same as the serving carrier centre subcarrier location | Same as the serving carrier centre subcarrier location |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 4 | 4 |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Probability of occurrence of PDSCH data |  | % | 20 | 20 |
| Probability of occurrence of transmission rank | Rank 1 | % | 80 | 80 |
|  | Rank 2 | % | 20 | 20 |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 | -3 |
|  |  | dB | 0 | 0 |
| Precoding granularity |  | PRB | 6 | 6 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| MBSFN |  |  | Not configured | Not configured |
| Network-based CRS interference mitigation |  |  | Disabled | Disabled |
| Note 1: The channel for the LTE interference cells and the serving cell are independent. Note 2: Defined in B.6.1. |  |  |  |  |

Table 5.2.2.1.18.0-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.3 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.9 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.1.18.

5.2.2.1.18_1                2Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x2 MIMO for both              NSA and SA

5.2.2.1.18_1.1             Test purpose

5.2.2.1.18_1.2             Test applicability

5.2.2.1.18_1.3             Test description

5.2.2.1.18_1.3.1          Initial conditions

5.2.2.1.18_1.3.2          Test procedure

5.2.2.1.18_1.3.3          Message contents

5.2.2.1.18_1.3.3_1      Message exceptions for SA

5.2.2.1.18_1.3.3_2      Message exceptions for NSA

5.2.2.1.18_1.4             Test requirement

Table 5.2.2.1.18_1.4-1: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.3 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | 11.9+TT |

<< Sections skipped >>

##### 5.2.2.2.15	2Rx TDD FR1 PDSCH mapping type A performance on band with shared spectrum access

<< Sections skipped >>

5.2.2.2.15.3.4	Test requirement

Tables 5.2.2.2.15.0-2 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 for each throughput test shall meet or exceed the specified value in Table 5.2.2.2.15.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.2.15.3.4-1: Test requirements for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-18.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 13.8 + TT |
| 1-2 | R.PDSCH.2-18.2 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.1 + TT |
| 1-3 | R.PDSCH.2-18.3 TDD | 60 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.2 + TT |
| 1-4 | R.PDSCH.2-18.4 TDD | 80 / 30 | 16QAM, 0.48 | FR1.30-7 | TDLA30-10 | 2x2, ULA Low | 70 | 14.5 + TT |

##### 5.2.2.2.19	2Rx TDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario

5.2.2.2.19.0             Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.19.0-4, with the addition of test parameters in Tables 5.2.2.2.19.0-2 and 5.2.2.2.19.0-3 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.19.0-1.

Table 5.2.2.2.19.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify PDSCH CRS interference mitigation performance under 2 receive antenna conditions with CRS rate matching configured for the serving cell | 1-1 |

Table 5.2.2.2.19.0-2: Tests parameters for serving cell PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate Matchin (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | Hz | 20 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: No MBSFN is configured on LTE carrier. Note 2: Network-based CRS interference mitigation is disabled on LTE carrier |  |  |  |

Table 5.2.2.2.19.0-3: Tests parameter for interference cells

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | TDD | TDD |
| TDD UL-DL pattern |  |  | DSUDDDSUDD S = 10D + 2G + 2U | DSUDDDSUDD S = 10D + 2G + 2U |
| INR (Note 1) |  | dB | 10.45 | 4.6 |
| LTE Bandwidth |  | MHz | 20 | 20 |
| Carrier centre subcarrier location |  |  | Same as the NR serving carrier centre subcarrier location | Same as the NR serving carrier centre subcarrier location |
| Cyclic Prefix |  |  | Normal | Normal |
| Physical cell ID |  |  | 1 | 2 |
| CRS pattern | Number of antenna ports |  | 4 | 4 |
|  | v-shift |  | 1 | 2 |
| Downlink power allocation |  | dB | -6 | -6 |
|  |  | dB | -6 | -6 |
|  |  | dB | 0 | 0 |
| PDSCH transmission mode |  |  | TM4 | TM4 |
| PDSCH loading level |  | % | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. | 20% probability of occurrence of LTE data transmission in time domain, and full bandwidth allocation in frequency domain. |
| Transmission rank |  | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |
| Interference model |  |  | As specified in clause B.7 | As specified in clause B.7 |
| Time offset to the serving cell |  | us | 3 | -1 |
| Frequency offset to the serving cell |  | Hz | 300 | -100 |
| Propagation conditions and MIMO configuration (Note 2) |  |  | TDLA30-10 ULA Low | TDLA30-10 ULA Low |
| Precoding granularity |  | PRB | 8 | 8 |
| Note 1: Defined in B.6.1 Note 2: The channel for the LTE interference cells and the serving cell are independent. |  |  |  |  |

Table 5.2.2.2.19.0-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.3 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 12.5 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.2.19.

5.2.2.2.19_1                2Rx TDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x2 MIMO for both              NSA and SA

5.2.2.2.19_1.1             Test purpose

5.2.2.2.19_1.2             Test applicability

5.2.2.2.19_1.3             Test description

5.2.2.2.19_1.3.1          Initial conditions

5.2.2.2.19_1.3.2          Test procedure

5.2.2.2.19_1.3.3          Message contents

5.2.2.2.19_1.3.3_1      Message exceptions for SA

5.2.2.2.19_1.3.3_2      Message exceptions for NSA

5.2.2.2.19_1.4             Test requirement

Table 5.2.2.2.19_1.4-1: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.3 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 12.5+TT |

<< End of Changes >>