3GPP TSG-RAN5 Meeting #99	R5-233721

# Korea, Incheon, May 22nd - 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2264 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update for PC2 PC3 n39 A-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_PC2_n39-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A-MPR for n39 needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A-MPR for n39 has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR conformance test for n39 can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_07, NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, and NS_35 to all types of NR UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 2 UE not supporting txDiversity-r16 release 15 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 UE not supporting txDiversity-r16 release 15 and forward.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5, 10 |  |  |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n83 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.3.3.3.12 | n30 | 5,10 |  | Table 6.5.3.3.3.12-1 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.3.17-1 | Table 6.2.3.3.17-2 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.26-1, Table 6.2.3.3.26-3 | Table 6.2.3.3.26-2, Table 6.2.3.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.27-1 | Table 6.2.3.3.27-1 |
| NS_50 | 6.5.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz,10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

6.2.3.3.2	A-MPR for NS_04

For NS_04, A-MPR is not added to MPR. Also, when NS_04 is signalled, MPR shall be set to zero in the PCMAX equations to avoid double-counting MPR.

Allowed maximum power reduction is defined as A-MPR=max(MPR, A-MPR'),

Note that A-MPR'=0 dB means only MPR is applied,

where A-MPR' is defined as

if RBstart ≤ fstart,max,IMD3 / (12SCS) and LCRB ≤ AWmax,IMD3 / (12SCS) and FC - BWChannel/2 < FUL_low + offsetIMD3,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A2 relative to 23 dBm for power class 3, PC2 A4 relative to 26 dBm for power class 2, and PC1.5_A6 relative to 29 dBm for power class 1.5,

else,

if RBstart ≤ LCRB/2 + start / (12SCS) and LCRB ≤ AWmax,regrowth / (12SCS) and FC - BWChannel/2 < FUL_low + offsetregrowth,
then

the A-MPR' is defined according to Table 6.2.3.3.2-2 PC3_A1 relative to 23 dBm for power class 3, PC2 A3 relative to 26 dBm for power class 2, and PC1.5_A5 relative to 29 dBm for power class 1.5,

else

A-MPR' = 0 dB and apply MPR.

With the parameters defined in Table 6.2.3.3.2-1.

Table 6.2.3.3.2-1: Parameters for region edges and frequency offsets

| Parameter | Symbol | Value |  |  | Related condition |
| --- | --- | --- | --- | --- | --- |
|  |  | CP-OFDM |  | DFT-s-OFDM |  |
| Max allocation start in IMD3 region | fstart,max,IMD3 | 0.33 BWChannel |  |  | RBstart ≤ fstart,max,IMD3 / (12SCS) |
| Max allocation BW in IMD3 region | AWmax,IMD3 | 4 MHz |  |  | LCRB ≤ AWmax,IMD3 / (12SCS) |
| Freq. offset required to avoid A-MPR in IMD3 region | offsetIMD3 | BWChannel – 6 MHz |  |  | FC - BWChannel/2 ≥ FUL_low + offsetIMD3 |
| Right edge of regrowth region | start | 0.08 BWChannel |  |  | RBstart ≤ LCRB/2 + start / (12SCS) |
| Max allocation BW in regrowth region | AWmax,regrowth | 100 MHz |  |  | LCRB ≤ Min(LCRB,Max, AWmax,regrowth / (12SCS)) |
| Freq. offset required to avoid A-MPR in regrowth region | offsetregrowth | Max (10 MHz, 0.25* BWChannel MHz) | Max (10 MHz, 0.45* BWChannel MHz) |  | FC - BWChannel/2 ≥ FUL_low + offsetregrowth |

Table 6.2.3.3.2-2: A-MPR' values Access

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16-QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64-QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256-QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64-QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256-QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.2.

6.2.3.3.3	A-MPR for NS_10

Table 6.2.3.3.3-1: A-MPR for NS_10

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB (RBs) | 1 –20 |
|  | A (dB) | ≤ 36 |
| 20 | RBstart | 0 – 15 |
|  | LCRB (RBs) | 1 – 20 |
|  | A (dB) | ≤ 66 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. NOTE 5: The A-MPR for DFT-s-OFDM is the total backoff and is obtained by taking the maximum value of MPR + A-MPR specified in Table 6.2.3.3.1-1 and Table 6.2.4-1 in TS 36.521-1 [21] and A value specified in Table 6.2.3.3.3-1. NOTE 6: The A-MPR for CP-OFDM is the total backoff and is obtained by adding the A value in Table 6.2.3.3.3-1 to the corresponding MPR specified in Table 6.2.2.3-1. |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.3.

<<UNCHANGED PARTS SKIPPED>>

6.2.3.3.19	A-MPR for NS_50

Table 6.2.3.3.19-1: A-MPR regions for NS_50 (Power Class 3)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 25 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 6.48 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 30 MHz | ≤ LCRB*12*SCS - 5 | > 5 | A7 |
|  | ≤ 8.64 | ≤ 1.44 | A8 |
|  |  | ≤ 3.6 | A9 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32, ≤ 10.44 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8 | A2 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A6 |
|  | > 31.68 | > 0 | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-2. |  |  |  |

Table 6.2.3.3.19-2: A-MPR for NS_50 (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 2 |  |
|  | QPSK | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2 |  |
|  | 16 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 | ≤ 2 | ≤ 5 | ≤ 2.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 7 | ≤ 3 | ≤ 5 |  | ≤ 5 |  |  |
|  | 256 QAM | ≤ 11 | ≤ 7 |  | ≤ 5 |  | ≤ 5 |  |  |
| CP-OFDM | QPSK | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 16 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 | ≤ 3.5 | ≤ 6.5 |  | ≤ 3.0 |
|  | 64 QAM | ≤ 12 | ≤ 8 | ≤ 4.5 | ≤ 5 |  | ≤ 6.5 |  |  |
|  | 256 QAM | ≤ 12 | ≤ 8 |  |  |  | ≤ 6.5 |  |  |

Table 6.2.3.3.19-3: A-MPR regions for NS_50 (Power Class 2)

| Channel Bandwidth (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10 MHz | ≤ 1.44 | < 1.44 | A5 |
|  | ≤ 1.8 | ≥ 2.7+2* RBstart*12*SCS | A4 |
|  | >1.8 | ≥ 8.1- RBstart*12*SCS | A4 |
| 15 MHz | ≤ 2.88 | < 2.7 | A5 |
|  | ≤ 3.24 | ≥ 2.7+2* RBstart*12*SCS | A3 |
|  | >3.24 | ≥ 12.42- RBstart*12*SCS | A4 |
| 20 MHz | ≤ 4.32 | < 3.6 | A5 |
|  | ≤ 4.5 | ≥ 3.6+2* RBstart*12*SCS | A3 |
|  | >4.5 | ≥ 17.1- RBstart*12*SCS | A4 |
| 25 MHz | ≤ LCRB*12*SCS – 5 | > 5 | A2 |
|  | ≤ 6.48 | ≤ 1.44 | A5 |
|  | > 8.28 | > max (21.6 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
|  | >1.8, ≤6.48 | > 1.44, ≤ 3.6 | A6 |
|  | > LCRB *12*SCS – 5, ≤ 1.8 | > 1.44 | A4 |
| 30 MHz | ≤ LCRB*12*SCS – 5 | >5 | A2 |
|  | ≤ 7.56 | ≤ 1.44 | A5 |
|  | >1.8, ≤7.56 | > 1.44, ≤ 3.6 | A6 |
|  | ≤ 1.8 | >1.44, <RBstart*12*SCS+5 | A4 |
|  | > 10.8 | > max (26.64 – RBstart*12*SCS, 0), <RBstart*12*SCS+5 | A4 |
| 40 MHz | ≤ 4.32 | > 0 | A1 |
|  | > 4.32 | > RBstart*12*SCS + 11.88 | A8 |
|  | > 4.32, ≤ 12.96 | ≤ 10.8 | A3 |
|  | > 4.32, ≤ 18 | > 10.8, <= RBstart*12*SCS + 11.88 | A7 |
|  | > 18, ≤ 31.68 | > max (31.68 – RBstart*12*SCS, 0) | A4 |
|  | > 31.68 | > 0 | A9 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.19-4. |  |  |  |

Table 6.2.3.3.19-4: A-MPR for NS_50 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) | A8 (dB) | A9 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 11.5 | ≤4.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | QPSK | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 16 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 | ≤ 2.5 | ≤ 4 | ≤ 1.5 | ≤ 3.5 | ≤ 10 | ≤ 8 |
|  | 64 QAM | ≤ 11.5 | ≤ 5.5 | ≤ 4 |  | ≤ 4 |  |  | ≤ 10 | ≤ 8 |
|  | 256 QAM | ≤ 11.5 | ≤ 5.5 |  |  |  |  |  | ≤ 10 | ≤ 8 |
| CP-OFDM | QPSK | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 | ≤ 2 | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 16 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 64 QAM | ≤ 12.5 | ≤ 7 | ≤ 5.5 | ≤ 4 | ≤ 4 |  | ≤ 5 | ≤ 11 | ≤ 8 |
|  | 256 QAM | ≤ 12.5 | ≤ 7 |  |  |  |  |  | ≤ 11 | ≤ 8 |

<<UNCHANGED PARTS SKIPPED>>

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

Table 6.2.3.4.1-1: Test Configuration table for NS_06 and NS_35

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_35 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 16 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 22 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 25 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void NOTE 4: This configuration is only applicable for NS_06 power class 1 UE. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |

| A-MPR test parameters for NS_04 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 57 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 58 | Low |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 60 | High |  |  |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |

Table 6.2.3.4.1-2a: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 |  |  | 2505 | 501000 | 2500.32 | 500064 | 0 | 15 | 6255 | 500430 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2512.5 | 502500 | 2505.39 | 501078 | 0 | 15 | 6270 | 501630 | 4 | 1 | 2 (4) | 5 |

zz

Table 6.2.3.4.1-2b: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2505 | 501000 | 2500.68 | 500136 | 0 | 30 | 6261 | 500910 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2512.5 | 502500 | 2505.66 | 501132 | 0 | 30 | 6273 | 501870 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2c: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2505 | 501000 | 2501.04 | 500208 | 0 | 15 | - | 500568 |
| 15 | 18 |  |  | 2512.5 | 502500 | 2506.02 | 501204 | 0 | 15 | - | 501564 |

Table 6.2.3.4.1-2d: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 |  |  | 2511 | 502200 | 2506.32 | 501264 | 0 | 15 | 6270 | 501630 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2513.505 | 502701 | 2506.395 | 501279 | 0 | 15 | 6273 | 501870 | 5 | 2 | 2 (4) | 6 |
| 20 | 106 |  |  | 2520 | 504000 | 2510.46 | 502092 | 0 | 15 | 6282 | 502590 | 10 | 1 | 1 (2) | 3 |
| 30 | 160 |  |  | 2535 | 507000 | 2520.6 | 504120 | 0 | 15 | 6306 | 504510 | 10 | 0 | 0 (0) | 0 |
| 40 | 216 |  |  | 2550 | 510000 | 2530.56 | 506112 | 0 | 15 | 6333 | 506670 | 6 | 1 | 2 (4) | 5 |
| 50 | 270 |  |  | 2565 | 513000 | 2540.7 | 508140 | 0 | 15 | 6357 | 508590 | 6 | 0 | 1 (2) | 2 |

Table 6.2.3.4.1-2e: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2511 | 502200 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2513.52 | 502704 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 20 | 51 |  |  | 2520 | 504000 | 2510.82 | 502164 | 0 | 30 | 6288 | 503070 | 14 | 0 | 2 (2) | 4 |
| 30 | 78 |  |  | 2535 | 507000 | 2520.96 | 504192 | 0 | 30 | 6312 | 504990 | 2 | 0 | 1 (1) | 2 |
| 40 | 106 |  |  | 2550 | 510000 | 2530.92 | 506184 | 0 | 30 | 6336 | 506910 | 2 | 0 | 0 (0) | 0 |
| 50 | 133 |  |  | 2565 | 513000 | 2541.06 | 508212 | 0 | 30 | 6363 | 509070 | 22 | 0 | 1 (1) | 2 |
| 60 | 162 |  |  | 2580 | 516000 | 2550.84 | 510168 | 0 | 30 | 6387 | 510990 | 10 | 0 | 1 (1) | 2 |
| 70 | 189 |  |  | 2595 | 519000 | 2560.98 | 512196 | 0 | 30 | 6414 | 513150 | 6 | 0 | 3 (3) | 6 |
| 80 | 217 |  |  | 2610 | 522000 | 2570.94 | 514188 | 0 | 30 | 6438 | 515070 | 6 | 0 | 2 (2) | 4 |
| 90 | 245 |  |  | 2625 | 525000 | 2580.9 | 516180 | 0 | 30 | 6462 | 516990 | 6 | 0 | 1 (1) | 2 |
| 100 | 273 |  |  | 2640 | 528000 | 2590.86 | 518172 | 0 | 30 | 6486 | 518910 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2f: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2511 | 502200 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 15 | 18 |  |  | 2513.52 | 502704 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 20 | 24 |  |  | 2520 | 504000 | 2511.36 | 502272 | 0 | 15 | - | 502632 |
| 30 | 38 |  |  | 2535 | 507000 | 2521.32 | 504264 | 0 | 15 | - | 504624 |
| 40 | 51 |  |  | 2550 | 510000 | 2531.64 | 506328 | 0 | 15 | - | 506688 |
| 50 | 65 |  |  | 2550 | 510000 | 2526.6 | 505320 | 0 | 15 | - | 505680 |
| 60 | 79 |  |  | 2580 | 516000 | 2551.56 | 510312 | 0 | 15 | - | 510672 |
| 70 | 93 |  |  | 2595 | 519000 | 2561.52 | 512304 | 0 | 15 | - | 512664 |
| 80 | 107 |  |  | 2610 | 522000 | 2571.48 | 514296 | 0 | 15 | - | 514656 |
| 90 | 121 |  |  | 2625 | 525000 | 2581.44 | 516288 | 0 | 15 | - | 516648 |
| 100 | 135 |  |  | 2640 | 528000 | 2591.4 | 518280 | 0 | 15 | - | 518640 |

<< UNCHANGED PARTS SKIPPED >>

Table 6.2.3.4.1-31: Test Configuration table for NS_50 (Power Class 3)

Editor’s note: “Additional test frequencies for NS_04” is FFS.

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column) |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 3) |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | 1897.5 | 25 | Default | N/A | DFT-s-OFDM | QPSK | Outer_Full (A7) |  |  |  |  |
| 2 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |  |  |
| 3 | 1897.5 | 25 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) |  | 5@0 (A9) |  |
| 4 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A7) |  |  |  |  |
| 5 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |  |  |
| 6 | 1900 | 30 | Default |  |  | QPSK | 20@0 (A9) | 10@0 (A9) |  | 5@0 (A9) |  |
| 7 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 8 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |  |
| 9 | 1900 | 40 | Default |  |  | QPSK | 180@25 (A2) | 90@13 (A2) |  | 40@7 (A2) |  |
| 10 | 1900 | 40 | Default |  |  | QPSK | 108@101 (A6) | 54@51 (A6) |  | 25@26 (A6) |  |
| 11 | 1900 | 40 | Default |  |  | QPSK | 36@177 (A5) | 16@89 (A5) |  | 6@45 (A5) |  |
| 12 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |  |  |
| 13 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |  |  |
| 14 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) |  | 5@0 (A9) |  |
| 15 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |  |  |
| 16 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |  |  |
| 17 | 1900 | 30 | Default |  |  | 256 QAM | 20@0 (A9) | 10@0 (A9) |  | 5@0 (A9) |  |
| 18 | 1900 | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 19 | 1900 | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |  |
| 20 | 1900 | 40 | Default |  |  | 256 QAM | 180@25 (A2) | 90@13 (A2) |  | 40@7 (A2) |  |
| 21 | 1900 | 40 | Default |  |  | 256 QAM | 108@101 (A6) | 54@51 (A6) |  | 25@26 (A6) |  |
| 22 | 1900 | 40 | Default |  |  | 256 QAM | 36@177 (A5) | 16@89 (A5) |  | 6@45 (A5) |  |
| 23 | 1897.5 | 25 | Default | N/A | CP-OFDM | QPSK | Outer_Full (A7) |  |  |  |  |
| 24 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |  |  |
| 25 | 1897.5 | 25 | Default |  |  | QPSK | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 26 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A7) |  |  |  |  |
| 27 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A8) |  |  |  |  |
| 28 | 1900 | 30 | Default |  |  | QPSK | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 29 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 30 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 31 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A2) |  | 93@13 (A2) |  | 44@7 (A2) |
| 32 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A6) |  | 55@51 (A6) |  | 25@26 (A6) |
| 33 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A5) |  | 17@89 (A5) |  | 6@45 (A5) |
| 34 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |  |  |
| 35 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |  |  |
| 36 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 37 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A7) |  |  |  |  |
| 38 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A8) |  |  |  |  |
| 39 | 1900 | 30 | Default |  |  | 256 QAM | 20@0 (A9) |  | 10@0 (A9) |  | 5@0 (A9) |
| 40 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 41 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 42 | Low | 40 | Default |  |  | 256 QAM | 191@25 (A2) |  | 93@13 (A2) |  | 44@7 (A2) |
| 43 | Low | 40 | Default |  |  | 256 QAM | 115@101 (A6) |  | 55@51 (A6) |  | 25@26 (A6) |
| 44 | Low | 40 | Default |  |  | 256 QAM | 39@177 (A5) |  | 17@89 (A5) |  | 6@45 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-32: Test Configuration table for NS_50 (Power Class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | (See Freq column) |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 10MHz, 15MHz, 20MHz, 25 MHz, 30MHz, 40MHz |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |  |
| A-MPR test parameters for NS_50 (Power Class 2) |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |
| 1 | 1890 | 10 | Default | N/A | DFT-s-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 2 | 1890 | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |  |
| 3 | 1890 | 10 | Default |  |  | QPSK | 40@11 (A4) | 18@6 (A4) |  | / |  |
| 4 | 1892.5 | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 5 | 1892.5 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 6 | 1892.5 | 15 | Default |  |  | QPSK | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |  |
| 7 | 1895 | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 8 | 1895 | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 9 | 1895 | 20 | Default |  |  | QPSK | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |  |
| 10 | 1897.5 | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 11 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 12 | 1897.5 | 25 | Default |  |  | QPSK | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |  |
| 13 | 1897.5 | 25 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 14 | 1897.5 | 25 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 15 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 16 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 17 | 1900 | 30 | Default |  |  | QPSK | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 18 | 1900 | 30 | Default |  |  | QPSK | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 19 | 1900 | 30 | Default |  |  | QPSK | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |  |
| 20 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 21 | 1900 | 40 | Default |  |  | QPSK | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |  |
| 22 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |  |
| 23 | 1900 | 40 | Default |  |  | QPSK | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |  |
| 24 | 1900 | 40 | Default |  |  | QPSK | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |  |
| 25 | 1900 | 40 | Default |  |  | QPSK | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |  |
| 26 | 1890 | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 27 | 1890 | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |  |
| 28 | 1890 | 10 | Default |  |  | 256 QAM | 40@11 (A4) | 18@6 (A4) |  | / |  |
| 29 | 1892.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 30 | 1892.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 31 | 1892.5 | 15 | Default |  |  | 256 QAM | 60@19 (A4) | 27@10 (A4) |  | 15@5 (A4) |  |
| 32 | 1895 | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 33 | 1895 | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 34 | 1895 | 20 | Default |  |  | 256 QAM | 80@26 (A4) | 36@13 (A4) |  | 18@7 (A4) |  |
| 35 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 36 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 37 | 1897.5 | 25 | Default |  |  | 256 QAM | 75@48 (A4) | 36@25 (A4) |  | 18@13 (A4) |  |
| 38 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 39 | 1897.5 | 25 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 40 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 41 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 42 | 1900 | 30 | Default |  |  | 256 QAM | 20@11 (A6) | 10@6 (A6) |  | 5@3 (A6) |  |
| 43 | 1900 | 30 | Default |  |  | 256 QAM | 27@0 (A4) | 12@0 (A4) |  | 6@0 (A4) |  |
| 44 | 1900 | 30 | Default |  |  | 256 QAM | 90@63 (A4) | 45@32 (A4) |  | 24@18 (A4) |  |
| 45 | Low | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 46 | Low | 40 | Default |  |  | 256 QAM | 180@25 (A8) | 90@13 (A8) |  | 40@7 (A8) |  |
| 47 | Low | 40 | Default |  |  | 256 QAM | 60@25 (A3) | 30@13 (A3) |  | 15@7 (A3) |  |
| 48 | Low | 40 | Default |  |  | 256 QAM | 90@25 (A7) | 45@13 (A7) |  | 20@7 (A7) |  |
| 49 | Low | 40 | Default |  |  | 256 QAM | 108@101 (A4) | 54@51 (A4) |  | 25@26 (A4) |  |
| 50 | Low | 40 | Default |  |  | 256 QAM | 36@177 (A9) | 16@89 (A9) |  | 6@45 (A9) |  |
| 51 | 1890 | 10 | Default | N/A | CP-OFDM | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 52 | 1890 | 10 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |  |
| 53 | 1890 | 10 | Default |  |  | QPSK | 41@11 (A4) |  | 18@6 (A4) |  | / |
| 54 | 1892.5 | 15 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 55 | 1892.5 | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 56 | 1892.5 | 15 | Default |  |  | QPSK | 60@19 (A4) |  | 28@10 (A4) |  | 13@5 (A4) |
| 57 | 1895 | 20 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 58 | 1895 | 20 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |  |
| 59 | 1895 | 20 | Default |  |  | QPSK | 80@26 (A4) |  | 38@13 (A4) |  | 17@7 (A4) |
| 60 | 1897.5 | 25 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 61 | 1897.5 | 25 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 62 | 1897.5 | 25 | Default |  |  | QPSK | 74@47 (A4) |  | 37@24 (A4) |  | 19@13 (A4) |
| 63 | 1897.5 | 25 | Default |  |  | QPSK | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 64 | 1897.5 | 25 | Default |  |  | QPSK | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 65 | 1900 | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |  |
| 66 | 1900 | 30 | Default |  |  | QPSK | Edge_1RB_left (A5) |  |  |  |  |
| 67 | 1900 | 30 | Default |  |  | QPSK | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 68 | 1900 | 30 | Default |  |  | QPSK | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 69 | 1900 | 30 | Default |  |  | QPSK | 88@61 (A4) |  | 44@31 (A4) |  | 22@16 (A4) |
| 70 | 1900 | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |  |
| 71 | 1900 | 40 | Default |  |  | QPSK | 191@25 (A8) |  | 93@13 (A8) |  | 44@7 (A8) |
| 72 | 1900 | 40 | Default |  |  | QPSK | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 73 | 1900 | 40 | Default |  |  | QPSK | 91@25 (A7) |  | 46@13 (A7) |  | 23@7 (A7) |
| 74 | 1900 | 40 | Default |  |  | QPSK | 115@101 (A4) |  | 55@51 (A4) |  | 25@26 (A4) |
| 75 | 1900 | 40 | Default |  |  | QPSK | 39@177 (A9) |  | 17@89 (A9) |  | 6@45 (A9) |
| 76 | 1890 | 10 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 77 | 1890 | 10 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |  |
| 78 | 1890 | 10 | Default |  |  | 256 QAM | 41@11 (A4) |  | 18@6 (A4) |  | / |
| 79 | 1892.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 80 | 1892.5 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 81 | 1892.5 | 15 | Default |  |  | 256 QAM | 60@19 (A4) |  | 28@10 (A4) |  | 13@5 (A4) |
| 82 | 1895 | 20 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 83 | 1895 | 20 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |  |
| 84 | 1895 | 20 | Default |  |  | 256 QAM | 80@26 (A4) |  | 38@13 (A4) |  | 17@7 (A4) |
| 85 | 1897.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 86 | 1897.5 | 25 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 87 | 1897.5 | 25 | Default |  |  | 256 QAM | 74@47 (A4) |  | 37@24 (A4) |  | 19@13 (A4) |
| 88 | 1897.5 | 25 | Default |  |  | 256 QAM | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 89 | 1897.5 | 25 | Default |  |  | 256 QAM | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 90 | 1900 | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |  |
| 91 | 1900 | 30 | Default |  |  | 256 QAM | Edge_1RB_left (A5) |  |  |  |  |
| 92 | 1900 | 30 | Default |  |  | 256 QAM | 20@11 (A6) |  | 10@6 (A6) |  | 5@3 (A6) |
| 93 | 1900 | 30 | Default |  |  | 256 QAM | 27@0 (A4) |  | 13@0 (A4) |  | 6@0 (A4) |
| 94 | 1900 | 30 | Default |  |  | 256 QAM | 88@61 (A4) |  | 44@31 (A4) |  | 22@16 (A4) |
| 95 | 1900 | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |  |
| 96 | 1900 | 40 | Default |  |  | 256 QAM | 191@25 (A8) |  | 93@13 (A8) |  | 44@7 (A8) |
| 97 | 1900 | 40 | Default |  |  | 256 QAM | 60@25 (A3) |  | 30@13 (A3) |  | 15@7 (A3) |
| 98 | 1900 | 40 | Default |  |  | 256 QAM | 91@25 (A7) |  | 46@13 (A7) |  | 23@7 (A7) |
| 99 | 1900 | 40 | Default |  |  | 256 QAM | 115@101 (A4) |  | 55@51 (A4) |  | 25@26 (A4) |
| 100 | 1900 | 40 | Default |  |  | 256 QAM | 39@177 (A9) |  | 17@89 (A9) |  | 6@45 (A9) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |  |

<< UNCHANGED PARTS SKIPPED >>

6.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1, with the following exceptions for each network signalling value.

For almost contiguous allocation testing, message contents are according to TS 38.508-1 [5] subclause 4.6.1 with the following exceptions:

Table 6.2.3.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  |  |
| } |  |  |  |

6.2.3.4.3.1	Message contents exceptions for network signalling value “NS_03”

1.	Information element additionalSpectrumEmission is set to NS_03. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n2, n25 and n66

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_03) |  |  |

Table 6.2.3.4.3.1-2: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_03" and NR band n70

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_03) |  |  |

<< UNCHANGED PARTS SKIPPED >>

6.2.3.4.3.34	Message contents exceptions for network signalled value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.34-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2.3.4.3.35	Message contents exceptions for network signalled value "NS_50"

1.	Information element additionalSpectrumEmission is set to NS_50. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.35-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_50"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_50) |  |  |

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2.3.5-1 to Table 6.2.3.5-38. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

<< UNCHANGED PARTS SKIPPED >>

Table 6.2.3.5-37: UE Power Class 3 test requirements (NS_50) for band n39 (Step 3)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0- TT |  |
| 2 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 3 | 23 | 0 | 1 | 0 | 0 |  | 22 |  | 2.0 |  | 2 | 25.0 + TT | 20.0 - TT |  |
| 4 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 5 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT4 |  |
| 6 | 23 | 0 | 1 | 0 | 0 |  | 23 |  | 2.0 |  | 2 | 25.0 + TT | 21.0 - TT |  |
| 7 | 23 | 0 | 1 | 11 | 0 |  | 12 |  | 6.0 |  | 2 | 25.0 + TT | 6.0 - TT |  |
| 8 | 23 | 0 | 1 | 3 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 9 | 23 | 0 | 1 | 7 | 0 |  | 16 |  | 5.0 |  | 2 | 25.0 + TT | 11.0 - TT4 |  |
| 10 | 23 | 0 | 1 | 2 | 0 |  | 21 |  | 2.0 |  | 2 | 25.0 + TT | 19.0 - TT |  |
| 11 | 23 | 0 | 1 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 12 | 23 | 0 | 4.5 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 13 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 14 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5- TT |  |
| 15 | 23 | 0 | 4.5 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 16 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 17 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 18 | 23 | 0 | 4.5 | 11 | 0 |  | 12 |  | 6.0 |  | 2 | 25.0 + TT | 6.0 - TT |  |
| 19 | 23 | 0 | 4.5 | 0 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |
| 21 | 23 | 0 | 3 | 7 | 0 |  | 16 |  | 5.0 |  | 2 | 25.0 + TT | 11.0 - TT |  |
| 22 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 23 | 23 | 0 | 3 | 5 | 0 |  | 18 |  | 4.0 |  | 2 | 25.0 + TT | 14.0 - TT |  |
| 24 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 – TT |  |
| 25 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 26 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 27 | 23 | 0 | 3 | 6.5 | 0 |  | 16.5 |  | 5.0 |  | 2 | 25.0 + TT | 11.5 - TT |  |
| 28 | 23 | 0 | 3 | 0 | 0 |  | 20 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 - TT |  |
| 29 | 23 | 0 | 3 | 0 | 0 |  | 20.0 |  | 2.5 |  | 2 | 25.0 + TT | 17.5 – TT |  |
| 30 | 23 | 0 | 3 | 12 | 0 |  | 11 |  | 6.0 |  | 2 | 25.0 + TT | 5.0 - TT |  |
| 31 | 23 | 0 | 3 | 4.5 | 0 |  | 18.5 |  | 4.0 |  | 2 | 25.0 + TT | 14.5 - TT |  |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 |  | 23 |  | 0 |  | 3 |  | 3.5 |  | 0 |  |  | 19.5 |  |  |  |  | 3.5 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 16 - TT |  |  |  |  |
| 33 |  | 23 |  | 0 |  | 3 |  | 5 |  | 0 |  |  | 18 |  |  |  |  | 4.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 14.0 - TT |  |  |  |  |
| 34 |  | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 - TT |  |  |  |  |
| 35 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 - TT |  |  |  |  |
| 36 |  | 23 |  | 0 |  | 6.5 |  | 6 |  | 0 |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 - TT |  |  |  |  |
| 37 |  | 23 |  | 0 |  | 6.5 |  | 6.5 |  | 0 |  |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 - TT |  |  |  |
| 38 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 - TT |  |  |  |
| 39 |  | 23 |  | 0 |  | 6.5 |  | 0 |  | 0 |  |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 - TT4 |  |  |  |
| 40 |  | 23 |  | 0 |  | 6.5 |  | 12 |  | 0 |  |  | 11 |  |  |  |  | 6.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 5.0 – TT |  |  |  |  |
| 41 |  | 23 |  | 0 |  | 6.5 |  | 4..5 |  | 0 |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 – TT |  |  |  |  |
| 42 |  | 23 |  | 0 |  | 6.5 |  | 8 |  | 0 |  |  | 15 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 10.0 – TT |  |  |  |  |
| 43 |  | 23 |  | 0 |  | 6.5 |  | 3.5 |  | 0 |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 – TT |  |  |  |  |
| 44 |  | 23 |  | 0 |  | 6.5 |  | 5 |  | 0 |  |  | 16.5 |  |  |  |  | 5.0 |  |  |  |  | 2 |  |  | 25.0 + TT |  |  | 11.5 - TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n25 n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: For bands n91, n92, n93 and n94 the maximum power reduction requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-38: UE Power Class 2 test requirements (NS_50) for band n39 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  |  | TL,c (dB) |  |  | Upper limit (dBm) |  | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 2 | 26 | 0 | 1 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 3 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 4 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 5 | 26 | 0 | 1 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 6 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 7 | 26 | 0 | 3.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 - TT |  |
| 8 | 26 | 0 | 1 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 9 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 10 | 26 | 0 | 1 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 11 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19.0 - TT |  |
| 12 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 – TT |  |
| 13 | 26 | 0 | 0 | 1 | 0 |  | 25 |  | 2.0 |  | 3 | 28.0 + TT | 22 - TT |  |
| 14 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 – TT |  |
| 15 | 26 | 0 | 1 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 16 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 17 | 26 | 0 | 0 | 1 | 0 |  | 25 |  | 2.0 |  | 3 | 28.0 + TT | 22 - TT |  |
| 18 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 19 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 20 | 26 | 0 | 1 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 21 | 26 | 0 | 0 | 10 | 0 |  | 16 |  | 2.0 |  | 3 | 28.0 + TT | 13 - TT |  |
| 22 | 26 | 0 | 0 | 4 | 0 |  | 22 |  | 5.0 |  | 3 | 28.0 + TT | 17.0 - TT |  |
| 23 | 26 | 0 | 0 | 3.5 | 0 |  | 22.5 |  | 2.0 |  | 3 | 28.0 + TT | 19.5 – TT |  |
| 24 | 26 | 0 | 0 | 2.5 | 0 |  | 23.5 |  | 2.0 |  | 3 | 28.0 + TT | 20.5 - TT |  |
| 25 | 26 | 0 | 0 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14.0 - TT |  |
| 26 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 27 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 28 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 29 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 30 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 31 | 26 | 0 | 4.5 | 2.5 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 32 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 33 | 26 | 0 | 4.5 | 4 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 34 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 35 | 26 | 0 | 4.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 36 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 37 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 38 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 39 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 40 | 26 | 0 | 4.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 41 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 42 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 43 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 44 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 45 | 26 | 0 | 4.5 | 11.5 | 0 |  | 14.5 |  | 5.0 |  | 3 | 28.0 + TT | 9.5 - TT |  |
| 46 | 26 | 0 | 4.5 | 10 | 0 |  | 16 |  | 5.0 |  | 3 | 28.0 + TT | 11 - TT |  |
| 47 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 48 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 49 | 26 | 0 | 4.5 | 0 | 0 |  | 21.5 |  | 2.0 |  | 3 | 28.0 + TT | 18.5 - TT |  |
| 50 | 26 | 0 | 4.5 | 8 | 0 |  | 18 |  | 5.0 |  | 3 | 28.0 + TT | 13 - TT |  |
| 51 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 52 | 26 | 0 | 3 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 53 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 54 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 55 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 56 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 57 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 58 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 59 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 60 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 61 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 62 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 63 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 64 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 65 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 66 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 67 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 68 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 69 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 70 | 26 | 0 | 3 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 71 | 26 | 0 | 1.5 | 11 | 0 |  | 15 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 72 | 26 | 0 | 1.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 73 | 26 | 0 | 1.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 - TT |  |
| 74 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 75 | 26 | 0 | 1.5 | 8 | 0 |  | 18 |  | 4 |  | 3 | 28.0 + TT | 14 - TT |  |
| 76 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 77 | 26 | 0 | 3 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 78 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 79 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 80 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 81 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 82 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 83 | 26 | 0 | 3 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 84 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 85 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 - TT |  |
| 86 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 87 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 88 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 89 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 90 | 26 | 0 | 3 | 7 | 0 |  | 19 |  | 3.5 |  | 3 | 28.0 + TT | 15.5 – TT |  |
| 91 | 26 | 0 | 3.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 92 | 26 | 0 | 1.5 | 2 | 0 |  | 24 |  | 2.0 |  | 3 | 28.0 + TT | 21 - TT |  |
| 93 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 - TT |  |
| 94 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 95 | 26 | 0 | 3 | 12.5 | 0 |  | 13.5 |  | 5.0 |  | 3 | 28.0 + TT | 8.5 - TT |  |
| 96 | 26 | 0 | 1.5 | 11 | 0 |  | 15 |  | 5.0 |  | 3 | 28.0 + TT | 10 - TT |  |
| 97 | 26 | 0 | 1.5 | 5.5 | 0 |  | 20.5 |  | 2.5 |  | 3 | 28.0 + TT | 17.5 - TT |  |
| 98 | 26 | 0 | 1.5 | 5 | 0 |  | 21 |  | 2.0 |  | 3 | 28.0 + TT | 18 – TT |  |
| 99 | 26 | 0 | 1.5 | 4 | 0 |  | 22 |  | 2.0 |  | 3 | 28.0 + TT | 19 – TT |  |
| 100 | 26 | 0 | 1.5 | 8 | 0 |  | 18 |  | 4.0 |  | 3 | 28.0 + TT | 14 - TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< END OF CHANGES >>