3GPP TSG-RAN5 Meeting #99	R5-23XXXX

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0XXX | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of generic Functional Alias Generic Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There were no procedures for MCData functional alias status determination and status changes and MCPTT generic procedures needed changes |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added generic procedures "UE initiated MCX functional alias status determination and subscription", and "UE initiated MCX functional alias status change"” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functional alias testing may not work properly and may produce false results |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.36, 5.3.37 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 5.3.36	UE initiated MCX functional alias status determination and subscription

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.36.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3.36.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.36.3	Procedure

Table 5.3.36.3-1: MCX functional alias status determination and subscription

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCX client) request to determine the current status of a functional alias and later notification of status changes of a functional alias. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 2a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the procedure described in clause 5.4.3 'MCX CO communication in E-UTRA' take place. | - | - | - | - |
| 3 | Check: Does the UE (MCX client) send a SIP SUBSCRIBE requesting the status of any existing functional aliases? | --> | SIP SUBSCRIBE | - | P |
| 4 | The SS (MCX server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | The SS (MCX server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 6 | Check: Does the UE (MCX client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 2) | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI NOTE 2: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.36.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3.36.4-1: SIP SUBSCRIBE (step 3, Table 5.3.36.3-1)

| Derivation Path: Table 5.5.2.14-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| value | "4294967295" | to receive the current status and later notification | TS 24.379 [9] clause 9A.2.1.3 TS 24.282 [87] clause 22.2.1.3 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  | MCPTT |
| MIME-part-body | MCData-Info as described in Table 5.3.36.4-2 |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| MIME body part |  | MCData Info |  | MCDATA |
| MIME-part-body | MCData-Info as described in Table 5.3.36.4-3 |  | TS 24.282 [87] clause 22.2.1.3 |  |

Table 5.3.36.4-2: MCPTT-Info in SIP PUBLISH (Table 5.3.36.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| request-type | "functional-alias-status-determination" |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.3 |  |

Table 5.3.36.4-3: MCData-Info in SIP SUBSCRIBE (Table 5.3.36.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdatainfo |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | "functional-alias-status-determination" |  | TS 24.282 [87] clause 22.2.1.3 |  |
| mcdata-request-uri | px_MCData_ID_User_A |  | TS 24.282 [87] clause 22.2.1.3 |  |

Table 5.3.36.4-4: SIP 200 (OK) (step 4, Table 5.3.36.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition SUBSCRIBE-RSP

Table 5.3.36.4-5: SIP NOTIFY (step 5, Table 5.3.36.3-1)

| Derivation Path: Table 5.5.2.8-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF for MCPTT as described in Table 5.5.3.5.2-1 (NOTE 1) |  | TS 24.379 [9] clause 9A.2.2.2.5 | MCPTT |
| MIME-part-body | PIDF for MCData as described in Table 5.5.3.5.2-3 (NOTE 1) |  | TS 24.282 [87] clause 22.2.2.2.5 | MCDATA |
| NOTE 1: PIDF document contains tuple with empty <status> element (i.e. there are no <functionalAlias> entries at all) and not containing a <p-id-fa> element |  |  |  |  |

### 5.3.37	UE initiated MCX functional alias status change

Within the context of this procedure, MCX refers to MCPTT, MCVideo or MCData.

5.3.37.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3.37.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.37.3	Procedure

Table 5.3.37.3-1: MCX functional alias status change

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCX client) request to change the status of a functional alias to 'activated'. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 2a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the procedure described in clause 5.4.3 'MCX CO communication in E-U’RA' take place. | - | - | - | - |
| 3 | Check: Does the UE (MCX client) send a SIP PUBLISH requesting the status change of a functional alias? | --> | SIP PUBLISH | - | P |
| 4 | The SS (MCX server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | The SS (MCX server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 6 | Check: Does the UE (MCX client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| 7 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 2) | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI NOTE 2: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.37.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3.37.4-1: SIP PUBLISH (step 3, Table 5.3.37.3-1)

| Derivation Path: Table 5.5.2.11-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info | TS 24.379 [9] clause 9A.2.1.2 | MCPTT |
| MIME-part-body | MCData-Info as described in Table 5.3.37.4-2 |  |  |  |
| MIME body part |  | MCData Info | TS 24.282 [87] clause 22.2.1.2 | MCDATA |
| MIME-part-body | MCData-Info as described in Table 5.3.37.4-3 |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF for MCPTT as described in Table 5.3.37.4-4 |  | TS 24.379 [9] clause 9A.2.1.2 | MCPTT |
| MIME-part-body | PIDF for MCData as described in Table 5.3.37.4-5 |  | TS 24.282 [87] clause 22.2.1.2 | MCDATA |

Table 5.3.37.4-2: MCPTT-Info in SIP PUBLISH (Table 5.3.37.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.2 |  |

Table 5.3.37.4-3: MCData-Info in SIP PUBLISH (Table 5.3.37.4-1)

| Derivation Path: Table 5.5.3.2.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-request-uri | px_MCData_ID_User_A |  | TS 24.282 [87] clause 22.2.1.2 |  |

Table 5.3.37.4-4: PIDF for MCPTT in SIP PUBLISH (Table 5.3.37.4-1)

Derivation Path: Table 5.5.3.5.1-1, condition FUNCTIONAL_ALIAS_STATUS_CHANGE

Table 5.3.37.4-5: PIDF for MCData in SIP PUBLISH (Table 5.3.37.4-1)

Derivation Path: Table 5.5.3.5.1-3, condition FUNCTIONAL_ALIAS_STATUS_CHANGE

Table 5.3.37.4-6: SIP 200 (OK) (step 4, Table 5.3.37.3-1)

Derivation Path: Table 5.5.2.17.1.2-1, condition PUBLISH-RSP

Table 5.3.37.4-7: SIP NOTIFY (step 5, Table 5.3.37.3-1)

| Derivation Path: Table 5.5.2.8-1, condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF |  |  |
| MIME-part-body | PIDF for MCPTT as described in Table 5.3.37.4-8 |  | TS 24.379 [9] clause 9A.2.2.2.5 | MCPTT |
| MIME-part-body | PIDF as described in Table 5.3.37.4-9 |  | TS 24.282 [87] clause 22.2.2.2.5 | MCDATA |

Table 5.3.37.4-8: PIDF for MCPTT in SIP NOTIFY (Table 5.3.37.4-7)

Derivation Path: Table 5.5.3.5.2-1, condition FUNCTIONAL_ALIAS_ACTIVATED, NOTIFY_FOR_PUBLISH

Table 5.3.37.4-9: PIDF for MCData in SIP NOTIFY (Table 5.3.37.4-7)

Derivation Path: Table 5.5.3.5.2-3, condition FUNCTIONAL_ALIAS_ACTIVATED, NOTIFY_FOR_PUBLISH

<END OF CHANGES>