3GPP RAN5 #2-5G-NR Adhoc  	R5-182021

# Taoyuan, Taiwan, 9 - 13, April 2018

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.521-1 | CR | N/A | rev | N/A | Current version: | 0.3.1 |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections of minimum conformance requirements and test requirement for 6.2.2 Maximum Power Reduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  |  | Date: |  | 2018-04-11 |
| Category: | N/A |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Minimum conformance requirements for MPR have been updated in 38.101-1(R4-1802403).Corresponding updates need to be provided to 38.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the minimum conformance requirements and table 6.2.2.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Minimum conformance requirements for MPR in TS38.521-1 are mismatched to the core specifications in TS38.101. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Revised from R5-181848r1. |  |  |  |  |  |  |  |  |

<Start of modified section>

6.2.2	Maximum Power Reduction (MPR)

Editor’s Note:

- Initial condition is not complete.

- Message contents is not complete.

- Test tolerance is not complete.

- How to deal with TDD slots with transient periods is FFS.

- Starting time position of 200ms in test procedure 2 is FFS.

- Combination of test frequency, Channel BW, SCS and Test ID is FFS.

- Test procedure to switch OFDM waveform is FFS.

- Table format in 6.2.2.5 need to follow what was done in A-MPR test in LTE instead of MPR test in LTE.

6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

The requirements of this test apply in test cases 6.5.2.2.1 Adjacent Channel Leakage power Ratio to all types of NR Power Class [2] and 3 UE release 15 and forward

6.2.2.3	Minimum conformance requirements

For UE Power Class [2] and 3, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.1.5-1 due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Table 6.2.2.3-1.

Table 6.2.2.3-1: Maximum Power Reduction (MPR) for Power Class [2] and 3

| Modulation | MPR (dB) |  |
| --- | --- | --- |
|  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM PI/2 BPSK | ≤ 0.5 | 0 |
| DFT-s-OFDM QPSK | ≤ 1 | 0 |
| DFT-s-OFDM 16 QAM | ≤ 2 | ≤ 1 |
| DFT-s-OFDM 64 QAM | ≤ 2.5 |  |
| DFT-s-OFDM 256 QAM | 4.5 |  |
| CP-OFDM QPSK | ≤ 3 | ≤ 1.5 |
| CP-OFDM 16 QAM | ≤ 3 | ≤ 2 |
| CP-OFDM 64 QAM | ≤ 3.5 |  |
| CP-OFDM 256 QAM | ≤ 6.5 |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

LCRB,Max is the maximum number of RB for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = LCRB/2 rounded down to next integer with floor at 1

RBStart,High = LCRB,Max – RBStart,Low – LCRB

Where Inner RB allocation range is specified as follows: Inner RB allocation are LCRB/2 away from each edge of the maximum RB allocation for all LCRB ≤ LCRB,Max/2 rounded up to the next integer.

RBStart,Inner : valid RBStart values for Inner RB allocations

For LCRB ≤ LCRB,Max/2 rounded up to the next integer, RBStart,Low ≤ RBStart,Inner ≤ RBStart,High

Where Outer RB allocation range is all allocations which are not Inner RB allocation

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.2.

6.2.2.4	Test description

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table [TBD]. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes [TBD]. Configurations of PDSCH and PDCCH before measurement are specified in Annex [TBD].

Table 6.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause [TBD] |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause [TBD] |  | TBD |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause [TBD] |  | FFS |  |
| Test SCS as specified in TS 38.508-1 [5] subclause [TBD] |  | FFS |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Reduction (MPR) test case | DFT-s-OFDM PI/2 BPSK | Outer Full |
| 2 |  | DFT-s-OFDM QPSK | Outer Full |
| 3 |  | DFT-s-OFDM 16 QAM | Inner Full |
| 4 |  | DFT-s-OFDM 16 QAM | Outer Full |
| 5 |  | DFT-s-OFDM 64 QAM | Outer Full |
| 6 |  | DFT-s-OFDM 256 QAM | Outer Full |
| 7 |  | CP-OFDM QPSK | Inner Full |
| 8 |  | CP-OFDM QPSK | Outer Full |
| 9 |  | CP-OFDM 16 QAM | Inner Full |
| 10 |  | CP-OFDM 16 QAM | Outer Full |
| 11 |  | CP-OFDM 64 QAM | Outer Full |
| 12 |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in [TBD].

2.	The parameter settings for the cell are set up according to [TBD].

3.	Downlink signals are initially set up according to [TBD], and uplink signals according to [TBD].

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1.

5.	Propagation conditions are set according to [TBD].

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1]  for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms).

6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause [TBD].

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 and Table 6.2.2.5-2.

Table 6.2.2.5-1: UE Power Class test requirements for Power Class 3

| NR Band |  | Class 3 (dBm) |  |  | Modulation |  |  | Outer RB allocation Tol. (dB) |  |  | Inner RB allocation Tol. (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n413 |  | 23 |  |  | DFT-s-OFDM PI/2 BPSK |  |  | + 2 + TT / - 2.5 - TT |  |  | N/A4 |  |
|  |  |  |  |  | DFT-s-OFDM QPSK |  |  | + 2 + TT / - 3 - TT |  |  | N/A4 |  |
|  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | + 2 + TT / - 4 - TT |  |  | + 2 + TT / - 3 - TT |  |
|  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | + 2 + TT / - 5 - TT |  |  |  |  |
|  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  |  |  | +2 + TT / - 8.5 -TT |  |  |  |  |  |  |
|  |  |  |  |  | CP-OFDM QPSK |  |  | + 2 + TT / - 5.5 - TT |  |  | + 2 + TT / - 3.5 - TT |  |
|  |  |  |  |  |  | CP-OFDM 16 QAM |  |  |  |  | + 2 + TT / - 5.5 - TT |  |  |  | + 2 + TT / - 4 - TT |  |  |
|  |  |  |  |  |  | CP-OFDM 64 QAM |  |  |  |  | + 2 + TT / - 7 - TT |  |  |  |  |  |  |
|  |  |  |  |  |  | CP-OFDM 256 QAM |  |  |  |  | + 2 + TT / - 11.5 - TT |  |  |  |  |  |  |
| n71 n78 n80 |  | 23 |  |  | DFT-s-OFDM PI/2 BPSK |  |  | + 2 + TT / - 3 - TT |  |  | + 2 + TT / - 2.5 - TT |  |
|  |  |  |  |  | DFT-s-OFDM QPSK |  |  | + 2 + TT / - 3.5 - TT |  |  | + 2 + TT / - 2.5 - TT |  |
|  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | + 2 + TT / - 4.5 - TT |  |  | + 2 + TT / - 3.5 - TT |  |
|  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | + 2 + TT / - 5 - TT |  |  |  |  |
|  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | + 2 + TT / - 8.5 - TT |  |  |  |  |
|  |  |  |  |  | CP-OFDM QPSK |  |  | + 2 + TT / - 5.5 - TT |  |  | + 2 + TT / - 4 - TT |  |
|  |  |  |  |  | CP-OFDM 16 QAM |  |  | + 2 + TT / - 5.5 - TT |  |  | + 2 + TT / - 4.5 - TT |  |
|  |  |  |  |  | CP-OFDM 64 QAM |  |  | + 2 + TT / - 7 - TT |  |  |  |  |
|  |  |  |  |  | CP-OFDM 256 QAM |  |  | + 2 + TT / - 11.5 - TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: The test of inner RB allocation of DFT-s-OFDM PI/2 BPSK and DFT-s-OFDM QPSK modulation is carried out in 6.2.1. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements for Power Class 2

| NR Band | Class 2 (dBm) | Modulation | Outer RB allocation Tol. (dB) | Inner RB allocation Tol. (dB) |
| --- | --- | --- | --- | --- |
| n413 | 26 | DFT-s-OFDM PI/2 BPSK | TBD | TBD |
|  |  | DFT-s-OFDM QPSK | TBD | TBD |
|  |  | DFT-s-OFDM 16 QAM | TBD | TBD |
|  |  | DFT-s-OFDM 64 QAM | TBD |  |
|  |  | DFT-s-OFDM 256 QAM | TBD |  |
|  |  | CP-OFDM QPSK | TBD | TBD |
|  |  | CP-OFDM 16 QAM | TBD | TBD |
|  |  | CP-OFDM 64 QAM | TBD |  |
|  |  | CP-OFDM 256 QAM | TBD |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths (Figure 5.3.3-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |

<End of modified section>