3GPP TSG5 RAN5 5G-NR Adhoc #2	R5-181933

Taoyuan, Taiwan, 9– 13th April, 2018

# Source:	Qualcomm Incorporated

# Title:	Test and Applicability Consideration for FR1 and FR2

# Document for:	Endorsement

Agenda Item:	5.1.19

## 1.	Introduction

The purpose of this paper is to discuss understanding of RAN5 Signalling group in terms of handling FR1 and FR2 bands for Test and Applicability.

## 2.	Discussion

5G NR brings in new bands for FR2 (mmW), while FR1 (Sub-6Ghz) bands are renamed from LTE bands as well new FR1 bands (above n77 till n257) are defined in TS38.101.

It has been observed that apart from band consideration there are some fundamental differences in RRC Messaging and PHY aspects for FR1 and FR2. There may also be procedural changes in between these bands as well.

For Test prose, below is RAN5 understanding

- Most Test Cases are common for FR1 and FR2 with no change in Test Prose. However, there will be underlying test model differences which will be handled in TTCN.

- For Test Cases where RRC message contents differ between FR1 and FR2, these will either be captured in TS 38.508-1 or in Test Prose of TS 38.523-1.

- For test where minor procedure\behavior is different between FR1 and FR2, such cases will also be resolved in TS38.523-1 (ex- exception, “if” statements).

- For a select few Test Cases, where the FR1 and FR2 behavior cannot be handled in same Test Case, we will proceed to define a band applicability.

## 3.	Proposal

1. Unless otherwise needed, RAN5 will not define Band applicability per Test Cases.

## 4.	References

[1] 		38.1010

[2]		38.508-1