3GPP TSG5 RAN5 5G-NR Adhoc #2	R5-182088

Taoyuan, Taiwan, 9– 13th April 2018

# Source:	Qualcomm Incorporated

# Title:	Handling of IMS PDN in 5G NR NSA mode

# Document for:	Endorsement

Agenda Item:	5.1.19

## 1.	Introduction

The purpose of this paper is to discuss handling of IMS PDN in signalling TC for 5G NR NSA mode.

## 2.	Discussion

UE is required to establish IMS PDN for voice and other IMS related services. As E-UTRA NW’s mature, it is evident that VoLTE is the choice for voice services and thus majority of NW’s now support VoLTE or will eventually support it. For 5G NSA Capable UE’s, VoLTE will be supported as the voice solution for day1 unlike the case of E-UTRA where CSFB fall-backs were also accepted initially.

Test Case prose, TTCN and other procedures might need to be revisited on how to handle the IMS PDN for NSA Phase of 5G NR. Further, as per the agreement [1] to allow IP data check on upper layer protocol Test cases, there will be a need to allow IMS and Internet PDN.

IMS PDN most likely be established on E-UTRA for 5G NR NSA (EN-DC).

However, as per 36.331-f10: below 2 capability bits have been added

| ims-VoiceOverNR-PDCP-MCG-Bearer Indicates whether the UE supports IMS voice over NR PDCP with only MCG RLC bearer. | Yes |
| --- | --- |
| ims-VoiceOverNR-PDCP-SCG-Bearer Indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer. | Yes |

As per latest RAN2 Agreement [7] based on discussion paper [8], below proposals were added and new IMS Voice\VoLTE related capability bits were added to ASN.1

Proposal 1.    If UE supports “volteOverNR-PDCP-15” the UE supports VoLTE over MCG RLC bearer only.

=> Agreed with the naming change “IMS-VoiceOverNR-PDCP-MCG-Bearer-15”

Proposal 2.    Introduce capability bit to indicate whether VoLTE DRB on SCG RLC bearer only is supported or not. Name as “volteOverSCG-Bearer-15”.

=> Agreed with naming change “IMS-VoiceOverNR-PDCP-SCG-Bearer-15”

Proposal 3.    VoLTE is supported for split bearer in MR-DC.

=> Noted.

Proposal 4.    Introduce capability bit to indicate whether VoLTE DRB on split bearer is supported or not. Name as “volteOverSplit-Bearer-15”.

=> Noted.

Proposal 5.    Agree the FDD/TDD diff for EN-DC related capabilities in table 1.

| High level IEs | capability | FDD/TDD diff | Remark |
| --- | --- | --- | --- |
| IRAT-ParametersNR | en-DC-r15 | No | The network use bandcombination to know which LTE bands can work together with NR bands for EN-DC; en-DC capability is general capability, do not need to distinguish FDD/TDD here. |
|  | supportedBandListNR-r15 | No | It is unrelated to FDD and TDD |
|  | measEventB1-NR-r15 | Yes | Same as LTE event B1 since IOT test opportunity may be different between FDD and TDD; |
|  | measPeriodicalNR-r15 | Yes | In LTE periodic measurement for inter RAT to UTRA was conditional, based on whether inter RAT measurement event is supported or not which indeed is FDD/TDD different. |
|  | tdm-Pattern-r15 | Yes | Tdm-Pattern support for FDD and TDD could be different. |
| BasebandParameters-r15 | basebandProcessingCombList-r15 | No | No new capabilities inside BPC; Base band processing capability should be independent of FDD and TDD; |
| PDCP-ParametersNR | rohc-Profiles-r15 | No | Same as LTE” supportedROHC-Profiles” |
|  | rohc-ContextMaxSessions-r15 | No | Same as LTE “maxNumberROHC-ContextSessions” |
|  | rohc-ProfilesUL-Only-r15 | No | Same as LTE “supportedUplinkOnlyROHC-Profiles” |
|  | rohc-ContextContinue-r15 | No | Same LTE “supportRohcContextContinue” |
|  | outOfOrderDelivery-r15 | No | Do not see the difference between FDD and TDD |
|  | sn-SizeLo-r15 | No | Do not see the difference between FDD and TDD |

=> Agreed with the above table.

Based on above agreements, it may be incorrect to assume that UE can allow SCG and Split DRB to be set-up on IMS PDN.

### 2.1 Summary of IMS handling in E-UTRA

Initially, E-UTRA Test Case were written and test models were designed to only handle single PDN. With the introduction and evolution of IMS enabled services in E-UTRA, IMS PDN was also required during attach (or on-demand) process. To minimize the prose and TTCN impact for introduction of this second PDN for existing test purposes, below steps were taken for impacted test cases -

- Deactivate Multi-PDN in the preamble.

- Deactivated Multi-PDN in the test case main body before verifying any test purpose.

- For 1 and 2 above, Internet PDN was deactivated and test proceeded with IMS PDN.

- For a few test cases, Multi-PDN is activated throughout the test case and all the test purpose verified with it Multi PDN allowed.

- Loopback modes were modified [4], [5],[6]

IMS PDN and Internet PDN can be used as default and secondary and vice-versa.

### 2.2 Handling of IMS PDN in 5G NR NSA

The test model for 5G NR NSA, especially EN-DC mode, is heavily leveraged from existing test model since E-UTRA is required as the control plane anchor. For multi-PDN capable UEs, existing test model takes the IMS bearer (multiPDN) for all protocol testing by supressing Internet bearer. However, based on the latest RAN2 agreement [7], [8] this approach\assumption may not be possible.

Also, as per the recent agreement to allow checking UE’s IP Data with ping operation [1], impacted RRC test cases would require Internet PDN to be allowed to keep UE behaviour close to real world, is those test cases IMS PDN should also be allowed.

Considering above limitation [7],[8] and desire to keep realistic UE’s behaviour, RAN5 may have to consider allowing 2 default bearers (Internet and IMS) as well as UE’s which only have single bearer (Internet).

For workarounds, it can be checked that if network requests establishment of a Dedicated EPS Bearer linked to the IMS PDN, configured over an EN-DC split AM RLC bearer, and not used for carrying IMS voice packets will be allowed

## 3.	Way Forward

1. Send an LS to RAN2 for further clarification of AM RLC bearer linked to dedicated EPS bearer of the IMS PDN. If so, there might be possibility to keep existing test model.

2. Explore the possibility of allowing 2 default PDN’s and their handling in various procedures and test case prose in EN-DC.

## 4.	References

[1] 		R5-181169 - Adding UE’s IP Stack Check for 5G NR”

[2]	R5-150645, “Test Case limitations on handling IMS signaling for Non-IMS E-UTRA Test Cases”. Source: Qualcomm Incorporated, Anritsu Ltd., Anite, Rohde & Schwarz.

[3]	R5-155779, “Test Case limitations on handling IMS signalling (Single-PDN/ Multi-PDN) for Non-IMS E-UTRA and Inter-RAT Test Cases”. Source: Qualcomm Incorporated.

[4]	R5-153969, “Testing of Loopback mode A and B activated test cases with IMS enabled UE”. Source: Qualcomm Incorporated, Anritsu Ltd., Anite.

[5]	R5-153970, “Update to Loopback data procedure with IMS signalling”. Source: Anritsu Ltd., Qualcomm Incorporated, Samsung, MCC TF160.

[6]	R5-162144, “Handling of multi-PDNs-configured UEs”. Source: MCC TF160.

[7]	R2-1804034 - Summary of ASN.1 review session on part 7 (UE capability), Intel

[8]	R2-1802941 - Open issues on EN-DC capabilities on LTE side (I044, I089), Intel