3GPP TSG-RAN WG5 Meeting #4-5G-NR Adhoc	R5-190056

# Singapore, Singapore, January 21th - 25th 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0055 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TT updates among different test setups |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During RAN5#79 (May-2018), following proposal that same TT should be applied among all test setups was endorsed. However, this endorsement was not captured into TS 38.521-2 correctly. Proposal :Define TT based on either MUIFF or MUDFF (How to define TT from MUIFF is TBD). #R5-183912 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define same TT value regardless of test setup (DUT size). For MOP and REFSENS, TT values based on MUIFF (DUT ≤ 30cm) are applied considering common UE size in the current market. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TT work will become larger and RAN5 work can’t be completed in time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1.5, 6.5.2.1.5, 6.5.2.3.5, 7.3.2.5, Annex F.3.2, F.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Updates in this document is based on endorsed discussion paper (R5-183912, RAN5#79). |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

6.2.1.1	UE maximum output power - EIRP and TRP

<Unchanged sections skipped>

6.2.1.1.5	Test requirement

The EIRP derived in step 4 and TRP derived in step 5 shall not exceed the values specified in Table 6.2.1.1.5-1 to Table 6.2.1.1.5-4.

Table 6.2.1.1.5-1: UE maximum output test requirements for power class 1

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 35+TT | 55 | 40.0-TT |
| n258 | 35+TT | 55 | 40.0-TT |
| n260 | 35+TT | 55 | 38.0-TT |
| n261 | 35+TT | 55 | 40.0-TT |

Table 6.2.1.1.5-2: UE maximum output test requirements for power class 2

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 29-TT |
| n258 | 23+TT | 43 | 29-TT |
| n260 |  |  |  |
| n261 | 23+TT | 43 | 29-TT |

Table 6.2.1.1.5-3: UE maximum output test requirements for power class 3

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 22.4-TT |
| n258 | 23+TT | 43 | 22.4-TT |
| n260 | 23+TT | 43 | 20.6-TT |
| n261 | 23+TT | 43 | 22.4-TT |

Table 6.2.1.1.5-3a: Test Tolerance (Max TRP for Power class 3)

| Test item | FR2a | FR2b |
| --- | --- | --- |
| Max TRP | [2.7] dB | [3.0] dB |
| Min peak EIRP | [2.8] dB | [3.0] dB |

Table 6.2.1.1.5-4: UE maximum output power test requirements for power class 4

| Operating band | Max TRP (dBm) | Max EIRP (dBm) | Min peak EIRP (dBm) |
| --- | --- | --- | --- |
| n257 | 23+TT | 43 | 34-TT |
| n258 | 23+TT | 43 | 34-TT |
| n260 | 23+TT | 43 | 31-TT |
| n261 | 23+TT | 43 | 34-TT |

6.2.1.2	UE maximum output power - Spherical coverage

<Unchanged sections skipped>

6.5.2.1	Spectrum Emission Mask

<Unchanged sections skipped>

6.5.2.1.5	Test requirement

The measured EIRP derived in step 4, shall fulfil requirements in Table [TBD-Min peak EIRP requirement] as appropriate, and the power (TRP) of any UE emission shall fulfil requirements in Table.6.5.2.1.5-1.

Table 6.5.2.1.5-1: General NR spectrum emission mask for Range 2

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz | Measurement bandwidth |
| 0-5 | -5 + TT | -5 + TT | -5 + TT | -5 + TT | 1 MHz |
| 5-10 | -13 + TT | -5 + TT | -5 + TT | -5 + TT | 1 MHz |
| 10-20 | -13 + TT | -13 + TT | -5 + TT | -5 + TT | 1 MHz |
| 20-40 | -13 + TT | -13 + TT | -13 + TT | -5 + TT | 1 MHz |
| 40-100 | -13 + TT | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 100-200 |  | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| 200-400 |  |  | -13 + TT | -13 +TT | 1 MHz |
| 400-800 |  |  |  | -13 + TT | 1 MHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5.2.1.5-1a NOTE 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. NOTE 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel. |  |  |  |  |  |

Table 6.5.2.1.5-1a: Test Tolerance (Spectrum emission mask)

| 23.45GHz ≤ f ≤ 30.3GHz | 30.3GHz < f ≤ 40.8GHz |
| --- | --- |
| [4.6] dB | [5.6] dB |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5.2.2	Additional spectrum emissions mask

FFS.

6.5.2.3	Adjacent channel leakage ratio

<Unchanged sections skipped>

6.5.2.3.5	Test requirement

If the measured adjacent channel power, derived in step 5, is greater than -35 dBm then the measured NR ACLR, derived in step 6, shall be higher than the limits in table 6.5.2.3.5-1.

Table 6.5.2.3.5-1: General requirements for NRACLR

|  | Channel bandwidth / NRACLR / Measurement bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| NRACLR for band n257, n258 | 17 + TT dB | 17 + TT dB | 17 + TT dB | 17 + TT dB |
| NRACLR for band n260 | 16 + TT dB | 16 + TT dB | 16 + TT dB | 16 + TT dB |
| NR channel Measurement bandwidth | 47.52 MHz | 95.04 MHz | 190.08 MHz | 380.16 MHz |
| Adjacent channel centre frequency offset [MHz] | +50 / -50 | +100.0 / -100.0 | +200 / -200 | +400 / -400 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5.2.3.5-1a |  |  |  |  |

Table 6.5.2.3.5-1a: Test Tolerance (Adjacent channel leakage ratio)

| 23.45GHz ≤ f ≤ 30.3GHz | 30.3GHz < f ≤ 40.8GHz |
| --- | --- |
| [4.6] dB | [5.0] dB |

6.5.3	Spurious emissions

<Unchanged sections skipped>

7.3.2	Reference sensitivity power level

<Unchanged sections skipped>

7.3.2.5	Test requirement

The maximum input level is defined as a directional requirement. The requirement is verified in beam locked mode in The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2 and A.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5) with peak reference sensitivity specified in Tables 7.3.2.5-1 to 7.3.2.5-4. The requirement is verified with the test metric of EIS (Link=Beam peak search grids, Meas=Link Angle).

Table 7.3.2.5-1: Reference sensitivity for power class 1

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97.5+TT | -94.5+TT | -91.5+TT | -88.5+TT |
| n258 | -97.5+TT | -94.5+TT | -91.5+TT | -88.5+TT |
| n260 | -94.5+TT | -91.5+TT | -88.5+TT | -85.5+TT |
| n261 | -97.5+TT | -94.5+TT | -91.5+TT | -88.5+TT |

Table 7.3.2.5-2: Reference sensitivity for power class 2

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -94.5+TT | -91.5+TT | -88.5+TT | -85.5+TT |
| n258 | -94.5+TT | -91.5+TT | -88.5+TT | -85.5+TT |
| n260 |  |  |  |  |
| n261 | -94.5+TT | -91.5+TT | -88.5+TT | -85.5+TT |

Table 7.3.2.5-3: Reference sensitivity for power class 3

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -88.3+TT | -85.3+TT | -82.3+TT | -79.3+TT |
| n258 | -88.3+TT | -85.3+TT | -82.3+TT | -79.3+TT |
| n260 | -85.7+TT | -82.7+TT | -79.7+TT | -76.7+TT |
| n261 | -88.3+TT | -85.3+TT | -82.3+TT | -79.3+TT |

Table 7.3.2.5-3a: Test Tolerance (Reference sensitivity for power class 3)

f ≤ 40.8 GHz

[3.2] dB

Table 7.3.2.5-4: Reference sensitivity for power class 4

| Operating band | REFSENS (dBm) / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n257 | -97+TT | -94+TT | -91+TT | -88+TT |
| n258 | -97+TT | -94+TT | -91+TT | -88+TT |
| n260 | -95+TT | -92+TT | -89+TT | -86+TT |
| n261 | -97+TT | -94+TT | -91+TT | -88+TT |

7.3A	Reference sensitivity for CA

<Unchanged sections skipped>

F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The TT values defined in this section are valid only until RAN5#85 (November 2019).

- RAN5 expects to have the following actions performed by RAN5#85 in efforts to improve the TT values

- For updated TT evaluation only, MU analysis taking into the account the data sheet from commercially available test systems or any additional experimental data to justify the values

- UE vendors’ have supplied further input on TT based on the performance of the existing mmWave UE against the RF test cases.

- At least one statistical analysis has been done on possibilities to improve test confidence level.

- Measurement results & evaluation of network performance / commercially available devices has been provided.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP and TRP) | PC1 Minimum peak EIRP TBD Max TRP TBD Max EIRP 0 dB PC2 Minimum peak EIRP TBD Max TRP TBD Max EIRP 0 dB PC3 Minimum peak EIRP [2.8] dB (FR2a) [3.0] dB (FR2b) Max TRP [2.7] dB (FR2a) [3.0] dB (FR2b) Max EIRP 0 dB PC4 Minimum peak EIRP TBD Max TRP TBD Max EIRP 0 dB |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 TBD PC2 TBD PC3 TBD PC4 TBD |  |
| 6.2.2 UE maximum output power for modulation / channel bandwidth | TBD |  |
| 6.2.3 UE maximum output power with additional requirements | TBD |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.3.1 Minimum output power | TBD |  |
| 6.3.2 Transmit OFF power | 0 dB |  |
| 6.3.3.2 General ON/OFF time mask | TBD |  |
| 6.3.3.3 Transmit power time mask for slot boundaries | TBD |  |
| 6.3.3.4 PRACH time mask | TBD |  |
| 6.3.3.5 PUCCH time mask | TBD |  |
| 6.3.3.5.1 Long PUCCH time mask | TBD |  |
| 6.3.3.5.2 Short PUCCH time mask | TBD |  |
| 6.3.3.7 PUSCH-PUCCH and PUSCH-SRS time masks | TBD |  |
| 6.4.1 Frequency error | [0.005] ppm |  |
| 6.4.2.1 Error vector magnitude | 0%, up to 64QAM | Minimum requirement + TT |
| 6.4.2.2 Carrier leakage | TBD |  |
| 6.4.2.3 In-band emissions | TBD |  |
| 6.4.2.4 EVM equalizer spectrum flatness | TBD |  |
| 6.4.2.5 EVM equalizer spectrum flatness for BPSK modulation | TBD |  |
| 6.5.1 Occupied bandwidth | 0 kHz | Minimum requirement + TT |
| 6.5.2.1 Spectrum Emission Mask | [4.6] dB (23.45GHz ≤ f ≤ 30.3GHz) [5.6] dB (30.3GHz ≤(1) f ≤ 40.8GHz) | Editor's note: ≤(1) will become < |
| 6.5.2.2 Additional Spectrum Emissions Mask | TBD |  |
| 6.5.2.3 Adjacent Channel Leakage Ratio | Absolute requirement 0 dB Relative requirement BW = 50MHz [4.6] dB (23.45GHz ≤ f ≤ 30.3GHz) [5.0] dB (30.3GHz ≤(1) f ≤ 40.8GHz) | Editor's note: ≤(1) will become < |
| 6.5.3.1 Transmitter Spurious emissions | 0 dB | Minimum requirement + TT |
| 6.5.3.2 Spurious emission band UE co-existence | 0 dB | Minimum requirement + TT |
| NOTE 1: FR2a: 23.45GHz ≤ f < 32.125GHz FR2b: 32.125GHz ≤ f ≤ 40.8GHz |  |  |

F.3.3	Measurement of receiver

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- The TT values defined in this section are valid only until RAN5#85 (November 2019).

- RAN5 expects to have the following actions performed by RAN5#85 in efforts to improve the TT values

- For updated TT evaluation only, MU analysis taking into the account the data sheet from commercially available test systems or any additional experimental data to justify the values

- UE vendors’ have supplied further input on TT based on the performance of the existing mmWave UE against the RF test cases.

- At least one statistical analysis has been done on possibilities to improve test confidence level.

- Measurement results & evaluation of network performance / commercially available devices has been provided.

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | [3.2] dB (FR2a, FR2b) |  |
| 7.4 Maximum input level | TBD |  |
| 7.5 Adjacent channel selectivity | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.6.2 In-band blocking | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.6.3 Out-of-band blocking | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.7 Spurious response | 0 dB | Wanted signal power + TT T-put limit unchanged |
| 7.9 Spurious emissions | 0 dB | Minimum requirement + TT T-put limit unchanged |
| NOTE 1: FR2a and FR2b are specified in Table F.3.2-1. |  |  |

Annex G (normative): Uplink Physical Channels

<Unchanged sections skipped>