3GPP TSG-RAN5 Meeting NR#4 AdHoc	R5-190057

# Singapore, 21th – 25th Jan. 2019

Title:							MU for FR1 UL-MIMO SA TRx Tests

Source:			Anritsu

Agenda Item:			5.1.15

Document for:			Endorsement

# 1.	Introduction

In this paper, MU for FR1 single carrier UL-MIMO SA TCs is proposed.

# 2.	Discussion

# For UL-MIMO Tx tests, some requirements are defined for the sum of powers from each antenna connector while others are defined for each antenna connector. For the former case, as already discussed in EN-DC MU in [1], the MU will become RSS of MU of each measurement. For the latter case, the same MU as for the corresponding non-UL MIMO TCs can be reused.

# Table 1 summarizes the proposed MU values for Tx test cases.

Table 1 FR1 SA MU for single carrier UL-MIMO Tx TCs

| # TC | # Requirement defined for | # Proposed MU value |
| --- | --- | --- |
| # 6.2D.1 UE maximum output power for UL-MIMO | # Sum of power of each antenna connector | # RSS of 2 non-UL MIMO MUs in corresponding non-UL MIMO TC |
| # 6.3D.1 Minimum output power for UL-MIMO | # Sum of power of each antenna connector | # RSS of 2 non-UL MIMO MUs in corresponding non-UL MIMO TC |
| # 6.3D.2 Transmit OFF power for UL-MIMO | Each UE antenna connector | # Same as the MU in corresponding non-UL MIMO TC |
| # 6.3D.3 Transmit ON/OFF time mask for UL-MIMO | Each UE antenna connector | # Same as the MU in corresponding non-UL MIMO TC |
| # 6.3D.4.1 Absolute power tolerance for UL-MIMO | # Sum of power of each antenna connector is measured | # RSS of 2 non-UL MIMO MUs in corresponding non-UL MIMO TC |
| # 6.3D.4.2 Relative power tolerance for UL-MIMO | # Sum of power of each antenna connector | # TBD |
| # 6.3D.4.3 : Aggregate power tolerance for UL-MIMO | # Sum of power of each antenna connector | # TBD |
| # 6.4D Transmit signal quality for UL-MIMO | # Each UE antenna connector | # Same as the MU in corresponding non-UL MIMO TC |
| # 6.5D.1 Occupied bandwidth for UL-MIMO | # Each UE antenna connector | Same as the MU in corresponding non-UL MIMO TC |
| # 6.5D.2 Out of band emission for UL-MIMO (ACLR, SEM) | # Each UE antenna connector | Same as the MU in corresponding non-UL MIMO TC |
| # 6.5D.3 Spurious emissions for UL-MIMO | # Each UE antenna connector | Same as the MU in corresponding non-UL MIMO TC |
| # 6.5D.4 Transmit intermodulation for UL-MIMO | # Each UE antenna connector | Same as the MU in corresponding non-UL MIMO TC |

# For UL-MIMO Rx tests, for DL, the way of DL signal injection is same as no-UL MIMO TCs, then the same MU as for non-UL MIMO TC can be used.  Some test cases require the confirmation of UL power configuration (as side condition of the Rx requirement) and the MU for that can be reused from Tx power TCs. Table 2 summarizes the MU proposal for SA UL-MIMO Rx TCs.

Table 2 SA FR1 MU for Rx TCs

| # TC | # Proposed MU value |
| --- | --- |
| # 7.3D Reference sensitivity for UL-MIMO | # DL power : Same as corresponding non-UL MIMO MU # UL power : Same as 6.2D.1 |
| # 7.4D Maximum input level for UL-MIMO | # DL power : Same as corresponding non-UL MIMO MU # UL power : Same as 6.2D.1 |
| # Other Cl.7 tests | # TBD |

# Proposal 1: RAN5 to adopt the values in Table 1 for single carrier FR1 UL-MIMO Tx tests

# Proposal 2: RAN5 to adopt the values in Table 2 for single carrier FR1 UL-MIMO Rx tests

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

# Proposal 1: RAN5 to adopt the values in Table 1 for single carrier FR1 UL-MIMO Tx tests

# Proposal 2: RAN5 to adopt the values in Table 2 for single carrier FR1 UL-MIMO Rx tests

# 4.	References

[1] R5-185641, “Discussion on TRx MU for EN-DC within FR1”, Anritsu, RAN5 NR#3 AdHoc, Jaipur, India, Oct 2018