3GPP TSG-RAN WG5 meeting#4 5G-NR Adhoc	R5-190255

# Singapore, Singapore, 21st – 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0376 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new 5G-NR Idle Mode TC 6.1.2.19 - Speed-dependent cell reselection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of 5GS Idle Mode TC 38.523-1 6.1.2.19 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new Test Case as part of Idle SA Option 2 WP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA Option 2 WP implementation will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.19 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR xxxx |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |

<Start of new section>

#### 6.1.2.19	Speed-dependent cell reselection

# 6.1.2.19.1	Test Purpose (TP)

(1)

with {UE in NR RRC_IDLE state and in high mobility state}

ensure that {

when {UE detects the cell ranked as the best cell taking into account scaling factor}

then {UE reselects the new cell}

}

# 6.1.2.19.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.304, clauses 5.2.4.3.0 and 5.2.4.3.1. Unless otherwise stated these are Rel-15 requirements.

[TS 38.304, clause 5.2.4.3.0]

The UE mobility state is determined if the parameters (TCRmax, NCR_H, NCR_M and TCRmaxHyst) are broadcasted in system information for the serving cell.

State detection criteria:

…

High-mobility state criteria:

-	If number of cell reselections during time period TCRmax is greater than NCR_H.

The UE shall not consider consecutive reselections where a cell is reselected again right after one reselection for mobility state detection criteria.

State transitions:

The UE shall:

-	if the criteria for High-mobility state is detected:

-	enter High-mobility state.

…

If the UE is in High- or Medium-mobility state, the UE shall apply the speed dependent scaling rules as defined in subclause 5.2.4.3.1.

[TS 38.304, clause 5.2.4.3.1]

UE shall apply the following scaling rules:

-	If neither Medium- nor High-mobility state is detected:

-	no scaling is applied.

-	If High-mobility state is detected:

-	Add the sf-High of "Speed dependent ScalingFactor for Qhyst" to Qhyst if broadcasted in system information;

-	For NR cells, multiply TreselectionNR by the sf-High of "Speed dependent ScalingFactor for TreselectionNR" if broadcasted in system information;

…

In case scaling is applied to any TreselectionRAT parameter, the UE shall round up the result after all scalings to the nearest second.

# 6.1.2.19.3	Test Description

6.1.2.19.3.1	Pre-test conditions

System Simulator:

-	 3 NR Cells: NR Cell 1, 2 and 4 as specified in TS 38.508-1[4] table 4.4.2-3.

UE:

-	None.

Preamble:

-	NR Cell 1 is set to "Serving Cell" and NR Cell 2 and NR Cell 4 are set to non-suitable "Off" as defined in TS 38.508-1[4] table 6.2.2.1-3.

-	Steps 2 to 20 of the registration procedure described in TS 38.508-1[4] table 4.5.2.2-2 are performed on NR Cell 1

# 6.1.2.19.3.2	Test procedure sequence

Table 6.1.2.19.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause.

# Table 6.1.2.19.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | -94 | NR Cell 2 becomes the highest ranked cell. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -94 | -88 | NR Cell 4 becomes the highest ranked cell. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | -94 | NR Cell 1 becomes the highest ranked cell. |

# Table 6.1.2.19.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level setting according to the row "T1" in table 6.1.2.19.3.2- 1. | - | - | - | - |
| 2 | Wait for 40s for the UE to perform cell reselection procedure. | - | - | - | - |
| 3 | The SS changes the power level setting according to the row "T2" in table 6.1.2.19.3.2- 1. | - | - | - | - |
| 4 | Wait for 14s for the UE to perform cell reselection procedure. | - | - | - | - |
| 5 | The SS changes the power level setting according to the row "T3" in table 6.1.2.19.3.2- 1. | - | - | - | - |
| 6 | Wait for 14s for the UE to perform cell reselection procedure. | - | - | - | - |
| 7 | The SS changes the power level setting according to the row "T1" in table 6.1.2.19.3.2- 1. | - | - | - | - |
| 8 | Wait for 14s for the UE to perform cell reselection procedure. | - | - | - | - |
| 9 | The SS changes the TAC of NR Cell 4 to 2. | - | - | - | - |
| 10 | The SS changes the power level setting according to the row "T2" in table 6.1.2.19.3.2- 1. | - | - | - | - |
| 11 | Check: Does the UE send an RRCSetupRequest on NR Cell 4 within 2s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 12 | Check: Does the UE send an RRCSetupRequest on NR Cell 4 within 5s after step 11? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 13-17 | Steps 2 to 6 of the mobility registration updating procedure described in TS 38.508-1[4] subclause 4.9.3 are performed on NR Cell 4. | - | - | - | - |

# 6.1.2.19.3.3	Specific message contents

# Table 6.1.2.19.3.3-1: SystemInformationBlockType2 for NR Cell 1, NR Cell 2, and NR Cell 4 (preamble and all steps, Table 6.1.2.19.3.2-2)

Derivation Path: 38.508-1 [FFS – SystemInformationBlockType1]

# Table 6.1.2.19.3.3-2: SystemInformationBlockType3 for NR Cell 1, NR Cell 2, and NR Cell 4 (preamble and all steps, Table 6.1.2.19.3.2-2)

Derivation Path: 38.508-1 [FFS – SystemInformationBlockType3]

# Table 6.1.2.19.3.3-3: SystemInformationBlockType1 for NR Cell 4 (step 9, Table 6.1.2.19.3.2-2)

Derivation Path: 38.508-1 [FFS – SystemInformationBlockType1]

<End of new section>