3GPP TSG-RAN5 Meeting NR#4 AdHoc	R5-190460

# Singapore, 21th – 25th Jan. 2019

Title:							SNR estimation update for FR2 priority 1 and 2 TRx TCs

Source:			Anritsu

Agenda Item:			5.1.15

Document for:			Endorsement

# 1.	Introduction

# SNR estimation was provided for each test cases in [1]-[3].

# In the discussion during email-agreement after RAN5#82 and pre-meeting offline discussion among concerning parties, following issues are raised and discussed.

# Issue 1) Frequency to derive SNR

# Issue 2) Power level to derive SNR

# As a result, for Issue 1, 32.125GHz instead of 30.2GHz (which was used in [4]) is adopted for FR2a but assume the core requirement level as the worst one among those of upper and lower band of 32.125GHz.  Issue 2 is discussed in detail in the separated document [4] and it is proposed to use core requirement – 3dB to derive SNR for the purpose of low PSD testability discussion.

- Estimated SNR

# SNR for Priority 1 and 2 Tx TCs are given in Table 1.

Table 1: SNR for Priority 1 and 2 Tx test cases

| Test case | Applicable Frequency Range [GHz] | Estimated SNR [dB] | UL signal level [dBm] | Required relaxation to ensure SNR=10dB |
| --- | --- | --- | --- | --- |
| MOP (Min Peak EIRP, PC3) | 23.45GHz ≤ f ≤ 32.125 GHz | 8.8 (BW ≤ 400MHz) | 15.9dBm/ChBW (20.6 – 3 – 1.7) | 1.2 - ChBW |
|  | 30.3 GHz ≤ f ≤ 40.8 GHz | 5.4 (BW ≤ 400MHz) | 1.7 : Multi-band relaxation | 4.6 - ChBW |
| Off power | 23.45 GHz ≤ f ≤ 32.125 GHz | -18.0 + ChBW | -38dBm/ChBW (-35-3) | 28.0 - ChBW |
|  | 30.3 GHz < f ≤ 40.8 GHz | -26.1 + ChBW | -38dBm/ChBW (-35-3) | 36.1 - ChBW |
| Rx spurious | 6 GHz ≤ f ≤ 20 GHz | 3.3 | -50dBm/1MHz | 6.7 |
|  | 20 GHz < f ≤ 40 GHz | -3.7 | (-47-3) | 13.7 |
|  | 40 GHz < f ≤ 80 GHz | -19.6 |  | 29.6 |
| Tx spurious | 6 GHz ≤ f ≤ 20 GHz | > 20 | -16dBm/1MHz | - |
|  | 20GHz ≤ f ≤ 80 GHz | > 10 | (-13-3) | - |
| UE co-existence (1) | 26.5 GHz ≤ f ≤ 29.5 GHz | 2.7 | -5dBm/100MHz (-5-3) | 7.3 |
| UE co-existence (2) | 37 GHz ≤ f ≤ 40 GHz | -0.7 | -5dBm/100MHz (-2-3) | 10.7 |
| UE co-existence (3) | 26.5 GHz ≤ f ≤ 29.5 GHz | -22.3 | -53dBm/1MHz (-50-3) | 32.3 |
| SEM | 23.45 GHz ≤ f ≤ 32.125 GHz | 8.1 | -16dBm/1MHz (-13-3) | 1.9 |
|  | 30.3 GHz < f ≤ 40.8 GHz | 1.2 | -16dBm/1MHz (-13-3) | 8.8 |
| ACLR | 23.45 GHz ≤ f ≤ 32.125 GHz | 1.0 + ChBW | 23dBm(TRP)-17dBc-3dB=3dBm/ChBW | 9.0 - ChBW (NOTE2, NOTE3) |
|  | 30.3 GHz < f ≤ 40.8 GHz | -4.8 + ChBW | 23dBm(TRP)-16dBc-3dB=4dBm/ChBW | 14.8 - ChBW (NOTE2) |
| OBW, Frequency Error | No impact from SNR |  |  |  |
| NOTE ChBW = 10log10(400MHz/ChBW), 9.03dB for ChBW=50MHz. NOTE 2 : Relaxation for the dBc value with relaxed adjacent channel power and no change for carrier power. NOTE 3 : No relaxation required for smaller ChBw(50MHz and100MHz) if BW dependent relaxation is considered. |  |  |  |  |

# Assumptions

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurement. 6GHz – 80GHz for spurious measurement. |
| #3 | Size of QZ for IFF | 30 cm |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | +43dBm – Path loss tolerable (PC3 max beam peak) |
| #6 | Temperature variation impact | +18 to +28 degrees C |
| #7 | UE power class | PC3 |
| #9 | Characterization for QoQZ for spurious measurements | N/A |

# Other assumptions:

- Path loss (= Free space path loss + Antenna gain) = 52.5 dB @ 29.5GHz 
 (Delta for other frequency follows the delta from Friis' s equation)

- Channel Bandwidth: up to 400MHz

- Spurious measurement path is separated from in-band measurement path

In case LNA can be used, loss between antenna and 1st LNA much depends on the frequency and also test cases, approx. 6dB - 20dB are assumed. For LNA, commercial availability (https://www.sagemillimeter.com/amplifiers/low-noise-amplifiers/) is considered in this estimation. Each LNA has different characteristic of Gain, support frequency, etc then various combination can be possible with the actual implementation considering the cost, space, power consumption, etc, then not always the ones with minimum NF can be used. In this estimation, NF of 3.2 - 5.5dB are used depending on the frequency in the estimation.

For co-existence, SEM and ACLR, with which the low power requirement needs to be measured with the presence of carrier (could be maximum power) in vicinity, LNA cannot be used due to the maximum limit for input power for the LNA. Also, the presence of the carrier limits the maximum gain of power amplifier used or pre-amp of SA etc. Hence, SNR tends to become worse compared to the TC with same level of requirement but for which LNA can be used . ( This is one example why SNR is not always one-to-one relationship with requirement level )

# 4.	Conclusion

In this paper, SNR estimation for Priority 1 and 2 Tx test cases are provided. RAN5 is asked to endorse the values in Table 1 used for determining the relaxation amount.

# 5.	References

[1] R5-185806, “Testability issue of low PSD test cases in FR2 (2)”, Anritsu, RAN5 NR#3 AdHoc

[2] R5-187273, “Consideration on MU and TT for FR2 low PSD tests”, Anritsu, RAN5#81,

[3] R5-188229, “SNR estimation for priority 1 and 2 FR2 Tx test cases”, Anritsu, RAN5#82

[4] R5-190446, “On the power level to derive SNR for FR2 TRx tests”, Anritsu, RAN5 NR#4 AdHoc