3GPP TSG-RAN5 Meeting #4-5G-NR Adhoc	R5-190666

# Singapore, Singapore, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 0426 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of USIM Profiles for Signaling TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 01-14-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1)Addition of Test Case specific USIM Configurations for Signaling TC 2)Appended USIM Configuration for more test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1)Introduction of new Subsection which will contain the USIM Configurations 2)Added USIM Configuration for following 38.523-1 Test Cases: 6.1.1.8, 6.1.1.2, 6.1.1.4, 6.1.1.5, 6.2.1.2, 6.2.1.3, 6.2.1.5, 6.3.1.2, 6.1.1.6, 6.2.1.1 and 6.2.1.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case Specific USIM Configuration definition will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-190174r2 --> Changed profile to configuration, updates all tables and alignment with Idle mode TC R5-190666 is final version of R5-190174r2 |  |  |  |  |  |  |  |  |

# 6	Test environments for Signalling test

<newly added section>

## 6.4	Signaling Test Case specific USIM Configurations

### 6.4.1	General

The default USIM fields are specified in section 4.8.3. Specific USIM fields are set according to the USIM configuration specified in the tables below. PLMN settings are defined in TS 36.523-1 [x] table 6.0.1-1

Table 6.4.1 : USIM Configuration 1

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 2 3 | Default PLMN3 PLMN2 Remaining mandatory entries use default values | Default All specified NG-RAN |
| EFOPLMNwACT | 1 | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | 1 | PLMN4 | NG-RAN |
| EFUST |  | Services 20, 42, 43 and 74 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) |  |
| Note: This USIM configuration is applied to Test Case 6.1.1.1 of TS 38.523-1[12] |  |  |  |

Table 6.4.2: USIM Configuration 2

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°71 and n°74 are "available" |  |
| EFEHPLMN | 1 2 | PLMN15 PLMN1 |  |
| EFLRPLMNSI |  | 01 |  |
| Note: This USIM configuration is applied to Test Case 6.1.1.7 and 6.1.1.8 (Part 1) of TS 38.523-1[12] |  |  |  |

Table 6.4.3: USIM Configuration 3

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service n°74 is "available" |  |
| EFEHPLMN |  | Empty |  |
| EFLRPLMNSI |  | 01 |  |
| Note: This USIM configuration is applied to Test Case 6.1.1.7 and 6.1.1.8 (Part 2) of TS 38.523-1[12] |  |  |  |

Table 6.4.4: USIM configuration 4

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFEHPLMN | 1 | PLMN1 |  |
|  |  | Remaining mandatory entries use default values |  |
| EFPLMNwAcT | 1 | PLMN2 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFOPLMNwACT | 1 | PLMN3 | NG-RAN |
|  |  | Remaining mandatory entries use default values |  |
| EFUST |  | Services 20, 42 and 71 are supported. |  |
| Note: This USIM configuration is applied to Test Case 6.1.1.2 of TS 38.523-1[12] |  |  |  |

Table 6.4.5: USIM configuration 5

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN4 (See preamble) |  |
| EFPLMNwAcT |  | Empty |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN1. |  |
| EFUST |  | Service 71 is not supported (Note). Service 74 is supported. |  |
| EFLRPLMNSI |  | 00 |  |
| EFEHPLMN |  | 0xFF..FF (Note 1) |  |
| Note 1: This USIM configuration is applied to Test Case 6.1.1.4 of TS 38.523-1. Either EFUST or EFEHPLMN or both can be set to the specified value in Table 6.1.1.4.3.1–2[12] |  |  |  |

Table 6.4.6: USIM configuration 6

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN3. |  |
| EFPLMNwAcT | 1 | PLMN1 Remaining mandatory entries use default values | NG-RAN |
| EFOPLMNwACT | 1 | PLMN2 | NG-RAN |
|  | 2 | PLMN4 | NG-RAN |
|  |  | Remaining defined entries use default values |  |
| EFUST |  | Service 71 is not supported |  |
| Note: This USIM configuration is applied to Test Case 6.1.1.5 of TS 38.523-1[12] |  |  |  |

Table 6.4.7: USIM configuration 7

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT | 1 | PLMN13 | NG-RAN |  |
|  | 2 | PLMN14 | E-UTRAN |  |
| EFOPLMNwAcT | 1 | PLMN2 | All |  |
|  | 2 | PLMN14 | E-UTRAN |  |
|  | 4 | PLMN13 | NG-RAN |  |
| Note: This USIM configuration is applied to Test Case 6.2.1.2 of TS 38.523-1[12] |  |  |  |  |

Table 6.4.8: USIM configuration 8

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN15 | NG-RAN |  |
|  | 2 | PLMN15 | E-UTRAN |  |
|  | 3 | PLMN17 | E-UTRAN |  |
|  | 4 | PLMN6 | NG-RAN |  |
| Note: This USIM configuration is applied to Test Case 6.2.1.3 of TS 38.523-1[12] |  |  |  |  |

Table 6.4.9: USIM configuration 9

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT | 1 | PLMN1 | NG-RAN |  |
|  | 2 | PLMN15 | E-UTRAN |  |
| EFHPPLMN |  | 1(=6 min) |  | The HPLMN Search Period on the USIM shall be set to 6 minutes. |
| Note: This USIM configuration is applied to Test Case 6.2.1.5 of TS 38.523-1[12] |  |  |  |  |

Table 6.4.10: USIM configuration 10

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFOPLMNwACT | 1 2 3 | PLMN4 PLMN3 PLMN2 Remaining defined entries use default values | NG-RAN NG-RAN NG-RAN |
| EFUST |  | Service n°127 is "available" |  |
| EFHPPLMN |  | 1(=6 min) | The HPLMN Search Period on the USIM shall be set to 6 minutes. |
| Note: This USIM configuration is applied to Test Case 6.3.1.2 of TS 38.523-1[12] |  |  |  |

Table 6.4.11: USIM configuration 11

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EF5GS3GPPLOCI |  | PLMN1 (See preamble) |  |
| EFIMSI |  | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | 1 | Default | NG-RAN |
|  | 2 | PLMN2 | NG-RAN |
| EFOPLMNwACT | 1 | PLMN1 Remaining defined entries use default values |  |
| EFHPLMNwAcT | 1 | PLMN4 | NG-RAN |
| EFUST |  | Services 20, 42, 43, 74 and 96 are supported. Service 71 is not supported (there is no EHPLMN list). |  |
| EFHPPLMN |  | 1 (6 minutes) | NG-RAN |
| EFNASCONFIG |  | MinimumPeriodicSearchTimer set to 7 minutes |  |
| Note: This USIM configuration is applied to Test Case 6.1.1.6 of TS 38.523-1[12] |  |  |  |

Table 6.4.12: USIM configuration 12

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFPLMNwAcT |  | 3GPP TS 31.102, Annex E |  | The EF is empty. |
| EFOPLMNwAcT | 1 | PLMN2 | NG-RAN |  |
|  | 2 | PLMN13 | E-UTRAN |  |
|  | 3 | PLMN13 | NG-RAN |  |
| Note: This USIM configuration is applied to Test Case 6.2.1.1 of TS 38.523-1[12] |  |  |  |  |

Table 6.4.13: USIM configuration 13

| USIM field | Priority | Value | Access technology | Comment |
| --- | --- | --- | --- | --- |
| EFOPLMNwAcT | 1 | PLMN2 | NG-RAN |  |
|  | 3 | PLMN2 | E-UTRAN |  |
|  | 4 | PLMN13 | NG-RAN |  |
| Note: This USIM configuration is applied to Test Case 6.2.1.4 of TS 38.523-1[12] |  |  |  |  |