3GPP TSG-RAN5 Meeting #4-5G-NR Adhoc	R5-190171r1

# Singapore, Singapore, 21st Jan 2019 - 25th Jan 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 0347 | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Idle mode TC 6.1.2.7 Cell reselection / Equivalent PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2019-14-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of 5GS Idle Mode TC 6.1.2.7 in TS 38.523-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new Test Case as part of Idle Mode SA Option 2 WP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA Option 2 Idle Mode WP implementation will not be complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-190171r1: 1)TC number is 6.1.2.7 in WP, updated the TC number - In Steps 3,4 and 8 since all the steps of Table 4.9.3-1 is used , “generic test procedure” is used. - The Power level unit was incorrectly mentioned as dBm/15KHz - Note 3 in table 6.1.2.7.3.2.3 as same as Note 1 - Derivation path of Specific message contents incorrect Note 2 in Table 6.1.2.7.3.2-3 referred to the wrong table |  |  |  |  |  |  |  |  |

### 6.1.2	NG-RAN only cell Selection

##### 6.1.2.7	Cell reselection / Equivalent PLMN

# 6.1.2.7.1	Test Purpose (TP)

(1)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Registration procedure}

ensure that {

when { a suitable neighbour cell on an equivalent PLMN becomes better ranked than the suitable Serving cell}

then { UE reselects to this equivalent PLMN cell}

}

(2)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Registration procedure}

ensure that {

when { the Serving cell becomes unsuitable due to S<0 and a suitable cell exists on an equivalent PLMN}

then {UE reselects to this equivalent PLMN cell}

}

(3)

with { UE camped normally on a cell and network has downloaded a list of equivalent PLMNs during the Registration procedure}

ensure that {

when { the UE evaluates cells for reselection}

then { UE ignores suitable cells for which the UE has no reselection priority provided}

}

6.1.2.7.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 23.122, clause 4.4.3, TS 38.3.4, clause 5.2.4.1.

[TS 38.304, clause 5.2.4.1]

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

[TS 23.122, clause 4.4.3]

The Mobile Equipment stores a list of "equivalent PLMNs". This list is replaced or deleted at the end of each location update procedure, routing area update procedure, GPRS attach procedure, tracking area update procedure, EPS attach procedure, and registration procedure. The list is deleted by an MS attached for emergency bearer services after detach or registered for emergency services after deregistration. The stored list consists of a list of equivalent PLMNs as downloaded by the network plus the PLMN code of the registered PLMN that downloaded the list. All PLMNs in the stored list, in all access technologies supported by the PLMN, are regarded as equivalent to each other for PLMN selection, cell selection/re-selection and handover.

# 6.1.2.7.3	Test Description

6.1.2.7.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN NR Cells as specified in TS 38.508-1 Table 4.4.2.1 are configured broadcasting PLMNs as indicated in Table 6.1.2.7.3.1-1.

-	The PLMNs are identified in the test by the identifiers in Table 6.1.2.7.3.1-1.

Table 6.1.2.7.3.1-1: PLMN identifiers

| NR Cell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 11 | PLMN1 | 001 | 11 |
| 12 | PLMN2 | 002 | 21 |
| 13 | PLMN3 | 003 | 31 |

-	All NR cells are high quality.

-	All cells are suitable cells.

-	System information combination NR-4 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used in NR cells

UE:-

-    The UE is in Automatic PLMN selection mode.

Preamble:

- The UE is registered on PLMN1 (NR Cell 11) using the procedure described in TS 38.508-1[4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message indicates PLMN3 in the Equivalent PLMN list as described in Table 6.1.2.7.3.3-1.

- The UE is in state Registered, Idle Mode (State 1N-A) on NR Cell 11 according to 38.508-1[4];

# 6.1.2.7.3.2	Test procedure sequence

# Table 6.1.2.7.3.2-1 for FR1 and Table 6.1.2.7.3.2-2 for FR2 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" and "T2" are to be applied subsequently in the Main behaviour. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 6.1.2.7.3.2-1: Cell configuration changes over time for FR1

|  | Parameter | Unit | NR Cell 11 | NR Cell 12 | NR Cell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -67 | -82 | -97 | All NR cells S>0 |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -115 | -82 | -97 | NR Cell 11 S < 0 as described in TS 38.508-1 clause FFS |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -67 | -97 | -82 | All NR cells S>0 |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 38.508-1 clauses FFS. |  |  |  |  |  |  |

Table 6.1.2.7.3.2-2: Cell configuration changes over time for FR2

|  | Parameter | Unit | NR Cell 11 | NR Cell 12 | NR Cell 13 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | FFS | FFS | FFS | FFS | All NR cells S>0 |
| T1 | SS/PBCH SSS EPRE | FFS | FFS | FFS | FFS | NR Cell 1 S < 0 as described in TS 38.508-1 clause FFS |
| T2 | SS/PBCH SSS EPRE | FFS | FFS | FFS | FFS | All NR cells S>0 |
| Note 1: The default values (including “not present”) for all other parameters influencing cell reselection are suitable for this test. The values are defined in TS 38.508-1 clauses FFS. |  |  |  |  |  |  |

# Table 6.1.2.7.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit an RRCSetupRequest message within 100s? (Note 1) | --> | RRCSetupRequest | 1 | F |
| 2 | SS adjusts cell levels according to row T1 of table 6.1.2.7.3.2-1 for FR1 (or table 6.1.2.7.3.2-2 for FR2) | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.3-1 indicate that the UE is camped on NR Cell 13? NOTE: The UE performs a " REGISTRATION REQUEST procedure with type "mobility registration updating” with PLMN1 and PLMN2 listed as an Equivalent PLMN in the REGISTRATION ACCEPT message; the RRC connection is released. | - |  | 2 | P |
| 4 | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.3-1 indicate that the UE is camped on NR Cell 12? NOTE: The UE performs a " REGISTRATION REQUEST procedure with type "mobility registration updating” with PLMN1 and PLMN3 listed as an Equivalent PLMN in the REGISTRATION ACCEPT message; the RRC connection is released. | - |  | 1 | P |
| 5 | SS adjusts cell levels according to row T0 of table 6.1.2.7.3.2-1 for FR1 (or table 6.1.2.7.3.2-2 for FR2) | - |  | - | - |
| 6 | Check: Does the UE transmit an RRCSetupRequest message within 100s? (Note 1) | --> | RRCSetupRequest | 1,3 | F |
| 7 | SS adjusts cell levels according to row T2 of table 6.1.2.7.3.2-1 for FR1 (or table 6.1.2.7.3.2-2 for FR2) | - | - | - | - |
| 8 | Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.3-1 indicate that the UE is camped on NR Cell 13? NOTE: The UE performs a " REGISTRATION REQUEST procedure with type "mobility registration updating” with PLMN1 and PLMN2 listed as an Equivalent PLMN in the REGISTRATION ACCEPT message; the RRC connection is released. (Note 2) | - |  | 3 | P |
| Note 1: In Steps 1 and 6, a wait time long enough to ensure that the UE has detected, measured and evaluated the neighbour inter-frequency NR cells is provided, see 38.133 clause 4.2.2.4. Note 2: In Step 8, the UE moves to Cell 13 because no reselection priority is defined for Cell 1, see Table 6.1.2.7.3.3-4. |  |  |  |  |  |

# 6.1.2.7.3.3	Specific message contents

# Table 6.1.2.7.3.3-1: REGISTRATION ACCEPT for NR Cell 11 (preamble)

| Derivation path: 38.508-1[4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Equivalent PLMNs | PLMN3 |  |  |

# Table 6.1.2.7.3.3-2: REGISTRATION ACCEPT for NR Cell 13 (step 3 and 18, Table 6.1.2.7.3.2-3)

| Derivation path: 38.508-1[4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Equivalent PLMNs | PLMN1, 2 |  |  |

# Table 6.1.2.7.3.3-3: REGISTRATION ACCEPT for NR Cell 12 (step 4, Table 6.1.2.7.3.2-3)

| Derivation path: 38.508-1[4] Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Equivalent PLMNs | PLMN1, 3 |  |  |

Table 6.1.2.7.3.3-4: SystemInformationBlockType4 for Cell 12 (preamble and all steps, Table 6.1.2.7.3.2-3)

| Derivation Path: TS 38.508-1[6] Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 2 entries | n denotes the index of the entry |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 11 |  |  |
| cellReselectionPriority[1] | Not present | UE does not have a priority for frequency of Cell 11 |  |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 13 |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

Table 6.1.2.7.3.3-5: SystemInformationBlockType4 for Cell 13 (preamble and all steps, Table 6.1.2.7.3.2-3)

| Derivation Path: TS 38.508-1[6] Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 2 entries | n denotes the index of the entry |  |
| dl-CarrierFreq[1] | Same downlink EARFCN as used for Cell 11 |  |  |
| cellReselectionPriority[1] | Not present | UE does not have a priority for frequency of Cell 11 |  |
| dl-CarrierFreq[2] | Same downlink EARFCN as used for Cell 12 |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |