3GPP TSG-RAN5 Meeting #100	R5-235239

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2920 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common test environments for NR-NTN testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Common test environ for NR NTN testing is currently missing in the spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added common test environ for NR NTN testing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.2	Procedure to configure SCC for EN-DC RF CA testing

5.5.2.1	Scope

The purpose of this procedure is to establish one or more SCC for EN-DC CA testing.

5.5.2.2	Procedure description

5.5.2.2.1	Initial conditions

The UE is in RRC_CONNECTED state.

5.5.2.2.2	Procedure sequence

Table 5.5.2.2.2-1: Procedure to configure SCC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message using n. | <-- | RRC: RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| - | EXCEPTION: Steps 3a1 to 3a3 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value | - | - | - | - |
| 3a1-3a3 | IF E-UTRA SCC > 0, same as TS 36.508 [2] table 5.2A.4-1, steps 1-3. | - | - | - | - |
| Note 1: n > 0 in step 1. |  |  |  |  |  |

5.5.2.2.3	Specific message contents

Table 5.5.2.2.2.3-1: RRCReconfiguration (step 1, Table 5.5.2.2.2-1)

| Derivation Path: Table 4.6.1-13 with condition EN-DC_SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig (n) | n is number of SCC to be added. |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 5.5.2.2.2.3-2: CellGroupConfig (n) (Table 5.5.2.2.2.3-1)

| Derivation Path: Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | n entries | n is equal to the number of SCCs to be added |  |
| SCellConfig[ k, k=1..n] |  | entry (1..n) |  |
| sCellIndex |  |  |  |
| sCellConfigCommon | ServingCellConfigCommon with condition SCell_add |  |  |
| sCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## 5.6	Test environment for NTN testing

This section defines the test environment which applies to all RF and Demod test cases executed for NR NTN UEs, unless otherwise specified.

### 5.6.1	UE location

UE shall determine its location during the test using any of the following means.

1.	UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test.

2.	Other options such as providing UE location via AT command are not precluded.

UE location provided to the UE before the start of the test is as follows:

Longitude: depends on the ephemeris generated files

Latitude: depends on the ephemeris generated files

Altitude: depends on the ephemeris generated files

### 5.6.2	Ephemeris Generation

This section provides the means to generate Ephemeris files for GSO and NGSO orbits.

#### 5.6.2.0	Assumptions and Steps for Ephemeris generation

FFS

#### 5.6.2.1	Ephemeris Generation for GSO

FFS

#### 5.6.2.2	Ephemeris Generation for NSGO

FFS

### 5.6.3	ntn-config-r17 values in SIB19

The ntn-config-r17 values including the EphemerisInfo-r17 values in SIB19 shall be periodically signaled to the UE during test based on the Ephemeris files generated in clause 5.6.2