3GPP TSG-RAN5 Meeting #102	R5-240151

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4165 | rev | - | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap MAC TC 7.1.1.8.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To align the RRC IE with TS 38.331 and update the referred table number. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specific message contents were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

7.1.1.8.4	Separate BWP / RedCap-specific initial DL BWP without CORESET#0 / NCD-SSB

7.1.1.8.4.1	Test Purpose (TP)

(1)

with { UE supporting RedCap and in NR RRC_IDLE state }

ensure that {

when { initialDownlinkBWP-RedCap does not contain the entire CORESET#0 }

then { UE uses this BWP for receiving DL messages during initial access and after initial access }

}

(2)

with { UE in NR RRC_CONNECTED and the active BWP is RedCap-specific initial downlink BWP and this BWP does not include CD-SSB and the entire CORESET#0 }

ensure that {

when { NCD-SSB is configured for a RedCap UE in RRCSetup }

then { UE replies RRCSetupComplete }

}

(3)

with { UE in NR RRC_CONNECTED and the active BWP is RedCap-specific initial downlink BWP and this BWP does not include CD-SSB and the entire CORESET#0 }

ensure that {

when { SS sends RRCRelease to trigger UE move to RRC_IDLE }

then { UE shall monitor paging in the initial DL BWP that includes CORESET#0 }

}

7.1.1.8.4.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.300, clauses 16.13.5, and TS 38.331, clause 6.3.2. Unless otherwise stated these are Rel-17 requirements.

[TS 38.321, clause 5.15.1]

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured for the selected carrier:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	if the Random Access procedure was initiated for SI request (as specified in TS 38.331 [5]) and the Random Access Resources for SI request have been explicitly provided by RRC, and if the selected carrier is SUL carrier:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

2>	else:

3>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

[TS 38.300, clause 16.13.5]

A RedCap UE may be configured with multiple NCD-SSBs provided that each BWP is configured with at most one SSB. NCD-SSB may be configured for a RedCap UE in RRC_CONNECTED to perform RLM, BFD, and RRM measurements when the active BWP does not contain CD-SSB.

[TS 38.331, clause 6.3.2]

DownlinkConfigCommonSIB field descriptions

initialDownlinkBWP-RedCap If present, RedCap UEs use this DL BWP instead of initialDownlinkBWP. If the locationAndBandwidth of this BWP contains the entire CORESET#0, the UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment. Otherwise, i.e., if the locationAndBandwidth of this BWP does not contain the entire CORESET#0, the UE uses this BWP for receiving DL messages during initial access (Msg2, MsgB, Msg4) and after initial access. If absent, RedCap UEs use initialDownlinkBWP provided that it does not exceed the RedCap UE maximum bandwidth (see also clause 5.2.2.4.2).

BWP-DownlinkDedicated field descriptions

nonCellDefiningSSB If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the cell-defining SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

PDCCH-ConfigCommon field descriptions

commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0. If the RedCap-specific initial downlink BWP does not contain the entire CORESET#0, the network configures the commonControlResourceSet in the RedCap-specific initial downlink BWP in SIB1 for RedCap such that it does not have to be contained in the bandwidth of CORESET#0.

pagingSearchSpace ID of the search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial downlink BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE in RRC_IDLE or RRC_INACTIVE while SDT procedure is not ongoing, shall monitor paging in the initial DL BWP that includes CORESET#0.

ra-SearchSpace ID of the Search space for random access procedure (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive RAR in this BWP. This field is mandatory present in the DL BWP(s) if the conditions described in TS 38.321 [3], clause 5.15 are met.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive other system information in this BWP. This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive other system information using searchSpaceOtherSystemInformation in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

searchSpaceSIB1 ID of the search space for SIB1 message. In the initial DL BWP of the UE′s PCell, the network sets this field to 0. If the field is absent, the UE does not receive SIB1 in this BWP. (see TS 38.213 [13], clause 10). This field is absent for the RedCap-specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0. In that case, a RedCap UE in RRC_IDLE or RRC_INACTIVE shall monitor PDCCH to receive SIB1 using searchSpaceSIB1 in the initial DL BWP that includes CD-SSB and the entire CORESET#0.

7.1.1.8.4.3	Test description

7.1.1.8.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is configured to use Test frequency NRf1 as specified in TS 38.508-1 [4] table 6.2.3.1-X and table 6.2.3.1-Y using the common highest mandatory UL and DL channel bandwidth and using the default subcarrier spacing specified in TS 38.508-1 [4] clause 6.2.3.1.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR Cell 1 with SIB1 content as specified in Table 7.1.1.8.4.3.3-1 and SS broadcasts CD-SSB and NCD-SSB. MIB for NCD-SSB is specified in Table 7.1.1.8.4.3.3-0.

UE:

-	None

Preamble:

-	The UE is in state Switched OFF (state 0-A) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.8.4.3.2	Test procedure sequence

Table 7.1.1.8.4.3.2-1: Void

Table 7.1.1.8.4.3.2-2: Void

Table 7.1.1.8.4.3.2-3: Void

Table 7.1.1.8.4.3.2-4: Void

Table 7.1.1.8.4.3.2-5: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void. | - | - | - | - |
| 2 | Power on the UE. | - | - | - | - |
| 3 | The UE transmits a preamble on PRACH in initialUplinkBWP-RedCap? | --> | PRACH Preamble | - | - |
| 4 | The SS transmits Random Access Response in initialDownlinkBWP-RedCap. | <-- | Random Access Response | - | - |
| 5 | Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 6 | The SS transmits RRCSetup including NCD-SSB. | <-- | NR RRC: RRCSetup | - | - |
| 7 | Check: Does the UE transmit an RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete | 2 | P |
| 8-23a1 | Steps 5 to 20a1 of the registration procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed on NR Cell 1. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 24 | The SS transmits a Paging message including a matched UE identity. | <-- | NR RRC: Paging | - | - |
| 25 | The UE transmits a preamble on PRACH in initialUplinkBWP-RedCap? | --> | PRACH Preamble | - | - |
| 26 | The SS transmits Random Access Response in initialDownlinkBWP-RedCap. | <-- | Random Access Response | - | - |
| 27 | Check: Does the UE transmit a MAC PDU containing an RRCSetupRequest message? | --> | NR RRC: RRCSetupRequest | 1,3 | P |
| 28 | The SS transmits RRCSetup including NCD-SSB. | <-- | NR RRC: RRCSetup | - | - |
| 29 | Check: Does the UE transmit an RRCSetupComplete message? | --> | NR RRC: RRCSetupComplete | 2 | P |
| 30-34a1 | Steps 5-9a1 of Generic procedure as specified in TS 38.508-1 [4] Table 4.9.4.2.2-1 are performed. | - | - | - | - |

7.1.1.8.4.3.3	Specific message contents

Table 7.1.1.8.4.3.3-0: MIB for NCD-SSB (preamble and all steps, Table 7.1.1.8.4.3.2-5)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 with condition NCD-SSB

Table 7.1.1.8.4.3.3-1: SIB1 (preamble and all steps, Table 7.1.1.8.4.3.2-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 7.1.1.8.4.3.3-2 |  |
| } |  |  |  |  |

Table 7.1.1.8.4.3.3-2: ServingCellConfigCommonSIB (Table 7.1.1.8.4.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 7.1.1.8.4.3.3-3 |  |
| uplinkConfigCommon-v1700 SEQUENCE { |  |  |  |
| initialUplinkBWP-RedCap-r17 | BWP-UplinkCommon-RedCap | Table 7.1.1.8.4.3.3-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-3: DownlinkConfigCommonSIB (Table 7.1.1.8.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| pei-Config-r17 | Not present |  |  |
| initialDownlinkBWP-RedCap-r17 | BWP-DownlinkCommon-RedCap | Table 7.1.1.8.4.3.3-4 |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-4: BWP-DownlinkCommon-RedCap (Table 7.1.1.8.4.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP-RedCap |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon-RedCap | Table 7.1.1.8.4.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-5: PDCCH-ConfigCommon-RedCap (Table 7.1.1.8.4.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| controlResourceSetZero | Not present |  |  |
| commonControlResourceSet | ControlResourceSet | Table 7.1.1.8.4.3.3-6 |  |
| searchSpaceZero | Not present |  |  |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace | entry 1 Table 7.1.1.8.4.3.3-7 |  |
| } |  |  |  |
| searchSpaceSIB1 | Not present |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| pagingSearchSpace | Not present |  |  |
| ra-SearchSpace | SearchSpaceId with condition CSS |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-6: ControlResourceSet (Table 7.1.1.8.4.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-7: SearchSpace (Table 7.1.1.8.4.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition CSS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-8: BWP-RedCap (Table 7.1.1.8.4.3.3-4 and Table 7.1.1.8.4.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set to value of locationAndBandwidth in TS 38.508-1 [4] Table 6.2.3.1-X |  | FR1 |
|  | Set to value of locationAndBandwidth in TS 38.508-1 [4] Table 6.2.3.1-Y |  | FR2 |
| } |  |  |  |

Table 7.1.1.8.4.3.3-9: BWP-UplinkCommon-RedCap (Table 7.1.1.8.4.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP-RedCap | Table 7.1.1.8.4.3.3-8 |  |
| pucch-ConfigCommon CHOICE { |  |  |  |
| setup | PUCCH-ConfigCommon-RedCap | Table 7.1.1.8.4.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-10: PUCCH-ConfigCommon-RedCap (Table 7.1.1.8.4.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not present |  |  |
| pucch-ResourceCommonRedCap-r17 | 0 |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-11: RRCSetup (step 6, Table 7.1.1.8.4.3.2-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-RedCap | Table 7.1.1.8.4.3.3-12 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.8.4.3.3-12: CellGroupConfig-RedCap (Table 7.1.1.8.4.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig-RedCap | Table 7.1.1.8.4.3.3-13 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-13: ServingCellConfig-RedCap (Table 7.1.1.8.4.3.3-12)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkDedicated-RedCap | Table 7.1.1.8.4.3.3-14 |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated-RedCap | Table 7.1.1.8.4.3.3-15 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-14: BWP-DownlinkDedicated-RedCap (Table 7.1.1.8.4.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config | Table 7.1.1.8.4.3.3-16 |  |
| } |  |  |  |
| nonCellDefiningSSB-r17 | NonCellDefiningSSB | Table 7.1.1.8.4.3.3-18 |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-15: BWP-UplinkDedicated-RedCap (Table 7.1.1.8.4.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config | Table 7.1.1.8.4.3.3-19 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-16: PDCCH-Config (Table 7.1.1.8.4.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config ::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList | Not present |  |  |
| controlResourceSetToReleaseList | Not present |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace | entry 1 Table 7.1.1.8.4.3.3-17 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-17: SearchSpace (Table 7.1.1.8.4.3.3-16)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-18: NonCellDefiningSSB (Table 7.1.1.8.4.3.3-14)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-84AAA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| NonCellDefiningSSB-r17 ::= SEQUENCE { |  |  |  |  |
| absoluteFrequencySSB-r17 |  | Set to value of absoluteFrequencySSB for NCD-SSB in TS 38.508-1 [4] Table 6.2.3.1-X/Y |  |  |
| ssb-Periodicity-r17 |  | Not present | The periodicity of this NCD-SSB is same as SSB periodicity of the CD-SSB |  |
| ssb-TimeOffset-r17 |  | Not present | The time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this NCD-SSB transmitted is zero |  |
| } |  |  |  |  |

Table 7.1.1.8.4.3.3-19: PUCCH-Config (Table 7.1.1.8.4.3.3-15)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..maxNrofPUCCH-Resources)) OF PUCCH-Resource { | 16 entries |  |  |
| PUCCH-Resource[n] SEQUENCE { |  | entry 1- 16 n=1,2,…16 |  |
| secondHopPRB | PRB-Id | Table 7.1.1.8.4.3.3-20 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.4.3.3-20: PRB-Id (Table 7.1.1.8.4.3.3-19)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-109 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRB-Id | Set to value of the L_RBs - nrofPRBs where L_RBs is LRB of initialDownlinkBWP-RedCap found in TS 38.508-1 [4] Table 6.2.3.1-X or TS 38.508-1 [4] Table 6.2.3.1-Y and nrofPRBs is defined for the corresponding PUCCH-Resource (1 otherwise). |  |  |

[End of change]