3GPP TSG-RAN5 Meeting #102	 R5-240521

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 2896 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of NOTE 1 from applicability of FR1 and FR2 MRDC test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-25-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MRDC test cases have been completed in TS 38.5333 hence removing NOTE1 from TS 38.522. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of NOTE 1 from MRDC test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.2	RRM conformance test cases

----Start of Change ----

Table 4.2-1: Applicability of RRM EN-DC FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Test Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 4.3 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 4.3.2 | RRC connection mobility control |  |  |  |  |  |  |
| 4.3.2.2 | Random access |  |  |  |  |  |  |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.3.2.2.1 has been executed. | 2Rx 4Rx |  |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.3.2.2.2 has been executed. | 2Rx 4Rx | Subtest 2: F001 |
| 4.3.2.2.3 | EN-DC FR1 2-step contention based random access | Rel-16 | C157 | UEs supporting EN-DC FR1 and 2-step RACH | Test execution not necessary if test 6.3.2.2.3 has been executed. | 2Rx 4Rx |  |
| 4.3.2.2.4 | EN-DC FR1 2-step non-contention based random access | Rel-16 | C158 | UEs supporting EN-DC FR1 and 2-step RACH | Test execution not necessary if test 6.3.2.2.4 has been executed. | 2Rx 4Rx |  |
| 4.4 | Timing |  |  |  |  |  |  |
| 4.4.1 | UE Transmit Timing |  |  |  |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.4.1.1 has been executed. | 2Rx 4Rx | Subtest 2: F002 |
| 4.4.2 | UE timer accuracy |  |  |  |  |  |  |
| 4.4.3 | Timing Advance |  |  |  |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.4.3.1 has been executed. | 2Rx 4Rx |  |
| 4.5 | Signalling characteristics |  |  |  |  |  |  |
| 4.5.1 | Radio link monitoring |  |  |  |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.5.1.1 has been executed. | 2Rx 4Rx |  |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.5.1.2 has been executed. | 2Rx 4Rx |  |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.5.1.3 has been executed. | 2Rx 4Rx |  |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.5.1.4 has been executed. | 2Rx 4Rx |  |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM | Test execution not necessary if test 6.5.1.5 has been executed. | 2Rx 4Rx |  |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM | Test execution not necessary if test 6.5.1.6 has been executed. | 2Rx 4Rx |  |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if test 6.5.1.7 has been executed. | 2Rx 4Rx |  |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if test 6.5.1.8 has been executed. | 2Rx 4Rx |  |
| 4.5.1.9 | EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | Rel-17 | C021e | UEs supporting EN-DC FR1, long DRX cycle and RLM relaxed measurements |  | 2Rx 4Rx |  |
| 4.5.2 | Interruption |  |  |  |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |  |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx |  |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |  |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |  |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx |  |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx |  |
| 4.5.2.10 | EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | Rel-17 | C2xxy->C288 | UEs supporting EN-DC FR1 and activation and deactivation on SCG |  | 2Rx 4Rx |  |
| 4.5.3 | Scell activation and deactivation delay |  |  |  |  |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms Scell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |  |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms Scell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |  |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx |  |
| 4.5.3.5 | EN-DC FR1 direct SCell activation at SCell addition of known SCell | Rel-16 | C243 | UEs supporting EN-DC FR1 and 2DL CA in NR and direct SCell activation |  | 2Rx 4Rx |  |
| 4.5.3.6 | EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-17 | C267 | UEs supporting EN-DC FR1 and 2DL CA in NR and fast SCell activation |  | 2Rx 4Rx |  |
| 4.5.3.7 | EN-DC FR1 fast SCell Activation of known SCell for 640 ms SCell measurement cycle | Rel-17 | C267 | UEs supporting EN-DC FR1 and 2DL CA in NR and fast SCell activation |  | 2Rx 4Rx |  |
| 4.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C032 | UEs supporting EN-DC FR1 and SUL | Test execution not necessary if test 6.5.4.1 has been executed. | 2Rx 4Rx |  |
| 4.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C082 | UEs supporting EN-DC FR1 and link recovery | Test execution not necessary if test 6.5.5.1 has been executed. | 2Rx 4Rx |  |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C082a | UEs supporting EN-DC FR1 and long DRX cycle and link recovery | Test execution not necessary if test 6.5.5.2 has been executed. | 2Rx 4Rx |  |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C083 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and link recovery | Test execution not necessary if test 6.5.5.3 has been executed. | 2Rx 4Rx |  |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C083a | UEs supporting EN-DC FR1 and long DRX cycle and CSI-RS-based RLM and link recovery | Test execution not necessary if test 6.5.5.4 has been executed. | 2Rx 4Rx |  |
| 4.5.5.5 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Rel-16 | C175 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and SSB link recovery | Test execution not necessary if test 6.5.5.5 has been executed. | 2Rx 4Rx |  |
| 4.5.5.6 | EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Rel-16 | C176 | UEs supporting EN-DC FR1 and long DRX cycle and CSI-RS-based RLM and SSB link recovery | Test execution not necessary if test 6.5.5.6 has been executed. | 2Rx 4Rx |  |
| 4.5.6 | Active BWP switch delay |  |  |  |  |  |  |
| 4.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C065 | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |  |
| 4.5.6.1.2 | EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | Rel-15 | C065a | UEs supporting EN-DC FR1 and (DCI and timer based active BWP switching delay Type1 or Type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  | 2Rx 4Rx |  |
| 4.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |  |
| 4.5.6.2.1 | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C065b | UEs supporting EN-DC FR1 and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |  |
| 4.5.6.3 | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |  |  |  |  |  |  |
| 4.5.6.3.1 | Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs | Rel-16 | C065d | UEs supporting EN-DC FR1, incremental delay for DCI and timer based active BWP switching on multiple CCs simultaneously and 2DL CA | NOTE 1 | 2Rx 4Rx |  |
| 4.5.6.4 |  |  |  |  |  |  |  |
| 4.5.6.5 | Simultaneous RRC-based Active BWP Switch on multiple CCs |  |  |  |  |  |  |
| 4.5.6.5.1 | E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs | Rel-16 | C065d | UEs supporting EN-DC FR1, incremental delay for DCI and timer based active BWP switching on multiple CCs simultaneously and 2DL CA | NOTE 1 | 2Rx 4Rx |  |
| 4.5.7 | PSCell addition and release delay |  |  |  |  |  |  |
| 4.5.7.1 | EN-DC FR1 addition and release delay of known PSCell | Rel-15 | C021 | UEs supporting EN-DC FR1 |  | 2Rx 4Rx |  |
| 4.5.8 | UL switching |  |  |  |  |  |  |
| 4.5.8.1 | EN-DC FR1 interruptions at switching between two uplink carriers | Rel-16 | C126a | UEs supporting EN-DC and dynamic UL Tx switching in case of inter-band EN-DC |  | 2Rx 4Rx |  |
| 4.5.10 | PSCell activation and deactivation delay |  |  |  |  |  |  |
| 4.5.10.1 | EN-DC FR1 PSCell activation and deactivation delay | Rel-17 | C2xxy->C288 | UEs supporting EN-DC FR1 and activation and deactivation on SCG |  | 2Rx 4Rx |  |
| 4.5.11 | Conditional PSCell addition and release delay (FR1 EN-DC) |  |  |  |  |  |  |
| 4.5.11.1 | EN-DC FR1 Addition and Release Delay of PSCell | Rel-17 | C268 | UEs supporting EN-DC FR1 and conditional PSCell addition in EN-DC |  | 2Rx 4Rx |  |
| 4.6 | Measurement procedures |  |  |  |  |  |  |
| 4.6.1 | Intra-frequency measurements |  |  |  |  |  |  |
| 4.6.1.1 | EN-DC FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.6.1.1 has been executed. | 2Rx 4Rx |  |
| 4.6.1.2 | EN-DC FR1 event-triggered reporting without gap in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.6.1.2 has been executed. | 2Rx 4Rx |  |
| 4.6.1.3 | EN-DC FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C042 | UEs supporting EN-DC FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction | Test execution not necessary if test 6.6.1.3 has been executed. | 2Rx 4Rx |  |
| 4.6.1.4 | EN-DC FR1 event-triggered reporting with gap in DRX | Rel-15 | C042a | UEs supporting EN-DC FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle | Test execution not necessary if test 6.6.1.4 has been executed. | 2Rx 4Rx |  |
| 4.6.1.5 | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | Rel-15 | C021b | UEs supporting EN-DC FDD FR1 | Test execution not necessary if test 6.6.1.5 has been executed. | 2Rx 4Rx |  |
| 4.6.1.6 | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | Rel-15 | C042b | UEs supporting EN-DC FDD FR1 and CSI-RS based RLM and BWP operation without bandwidth restriction | Test execution not necessary if test 6.6.1.6 has been executed. | 2Rx 4Rx |  |
| 4.6.1.7 | EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C097 | UEs supporting EN-DC FR1 and long DRX cycle and measurement enhancements in HST | Test execution not necessary if test 6.6.1.7 has been executed. | 2Rx 4Rx |  |
| 4.6.1.8 | EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17 | Rel-16 | C021c | UEs supporting EN-DC FR1 and CA measurement enhancements in HST | Test execution not necessary if test 6.6.1.8 has been executed. | 2Rx 4Rx |  |
| 4.6.2 | Inter-frequency measurements |  |  |  |  |  |  |
| 4.6.2.1 | EN-DC FR1-FR1 event-triggered reporting in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.6.2.1 has been executed. | 2Rx 4Rx |  |
| 4.6.2.2 | EN-DC FR1-FR1 event-triggered reporting in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.6.2.2 has been executed. | 2Rx 4Rx |  |
| 4.6.2.5 | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.6.2.5 has been executed. | 2Rx 4Rx |  |
| 4.6.2.6 | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.6.2.6 has been executed. | 2Rx 4Rx |  |
| 4.6.2.9 | EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Rel-16 | C021d | UEs supporting EN-DC FR1 and inter-frequency measurement enhancements in HST | Test execution not necessary if test 6.6.2.12 has been executed. | 2Rx 4Rx |  |
| 4.6.4 | L1-RSRP for beam reporting |  |  |  |  |  |  |
| 4.6.4.1 | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.6.4.1 has been executed. | 2Rx 4Rx |  |
| 4.6.4.2 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.6.4.2 has been executed. | 2Rx 4Rx |  |
| 4.6.4.3 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.6.4.3 has been executed. | 2Rx 4Rx |  |
| 4.6.4.4 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.6.4.4 has been executed. | 2Rx 4Rx |  |
| 4.6.4.5 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C098 | UEs supporting EN-DC FR1, long DRX cycle and intra-NR measurement enhancement in HST | Test execution not necessary if test 6.6.4.5 has been executed. | 2Rx 4Rx |  |
| 4.6.7 | L1-SINR for beam reporting |  |  |  |  |  |  |
| 4.6.7.1 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C141 | UEs supporting EN-DC FR1 and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx 4Rx |  |
| 4.6.7.2 | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX | Rel-16 | C142 | UEs supporting EN-DC FR1 and long DRX cycle and L1-SINR measurement based on SSB as CMR and dedicated CSI-IM as IMR |  | 2Rx 4Rx |  |
| 4.6.7.3 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX | Rel-16 | C143 | UEs supporting EN-DC FR1 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-RS as IMR |  | 2Rx 4Rx |  |
| 4.7 | Measurement performance requirements |  |  |  |  |  |  |
| 4.7.1 | SS-RSRP |  |  |  |  |  |  |
| 4.7.1.1 | Intra-frequency measurements |  |  |  |  |  |  |
| 4.7.1.1.1 | EN-DC FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.1.1.1 has been executed. | 2Rx 4Rx |  |
| 4.7.1.1.2 | EN-DC FR1 SS-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.1.1.2 has been executed. | 2Rx 4Rx |  |
| 4.7.1.2 | Inter-frequency measurements |  |  |  |  |  |  |
| 4.7.1.2.1 | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.1.2.1 has been executed. | 2Rx 4Rx |  |
| 4.7.1.2.2 | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.1.2.2 has been executed. | 2Rx 4Rx |  |
| 4.7.2 | SS-RSRQ |  |  |  |  |  |  |
| 4.7.2.1 | EN-DC FR1 SS-RSRQ measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.2.1 has been executed. | 2Rx 4Rx |  |
| 4.7.2.2.1 | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.2.2.1 has been executed. | 2Rx 4Rx |  |
| 4.7.2.2.2 | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.2.2.2 has been executed. | 2Rx 4Rx |  |
| 4.7.3 | SS-SINR |  |  |  |  |  |  |
| 4.7.3.1 | EN-DC FR1 SS-SINR measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas | Test execution not necessary if test 6.7.3.1 has been executed. | 2Rx 4Rx |  |
| 4.7.3.2.1 | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas | Test execution not necessary if test 6.7.3.2.1 has been executed. | 2Rx 4Rx |  |
| 4.7.3.2.2 | EN-DC FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 | C035 | UEs supporting EN-DC FR1 and SS-SINR-meas | Test execution not necessary if test 6.7.3.2.2 has been executed. | 2Rx 4Rx |  |
| 4.7.4 | L1-RSRP |  |  |  |  |  |  |
| 4.7.4.1.1 | EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.4.1.1 has been executed. | 2Rx 4Rx |  |
| 4.7.4.1.2 | EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.4.1.2 has been executed. | 2Rx 4Rx |  |
| 4.7.4.2.1 | EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.4.2.1 has been executed. | 2Rx 4Rx |  |
| 4.7.4.2.2 | EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.7.4.2.2 has been executed. | 2Rx 4Rx |  |
| 4.7.5 | SFTD |  |  |  |  |  |  |
| 4.7.5.1 | EN-DC FR1 SFTD measurement accuracy | Rel-15 | C043 | UEs supporting EN-DC FR1 and SFTD measurements between E-UTRA Pcell and NR PSCell |  | 2Rx 4Rx |  |
| 4.7.7 | L1-SINR |  |  |  |  |  |  |
| 4.7.7.1.1 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Rel-16 | C135 | UEs supporting EN-DC FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured | Test execution not necessary if test 6.7.9.1.1 has been executed. | 2Rx 4Rx |  |
| 4.7.7.1.2 | EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Rel-16 | C135 | UEs supporting EN-DC FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured | Test execution not necessary if test 6.7.9.1.2 has been executed. | 2Rx 4Rx |  |
| 4.7.7.2 | EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C136 | UEs supporting EN-DC FR1 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR | Test execution not necessary if test 6.7.9.2 has been executed. | 2Rx 4Rx |  |
| 4.7.7.3.1 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C137 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR | Test execution not necessary if test 6.7.9.3.1 has been executed. | 2Rx 4Rx |  |
| 4.7.7.3.2 | EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Rel-16 | C137 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR | Test execution not necessary if test 6.7.9.3.2 has been executed. | 2Rx 4Rx |  |
| 4A | NE-DC with all NR cells in FR1 |  |  |  |  |  |  |
| 4A.1 | Signalling characteristics |  |  |  |  |  |  |
| 4A.1.1 | E-UTRA PSCell addition |  |  |  |  |  |  |
| 4A.1.1.1 | NE-DC FR1 addition and release delay of known PSCell | Rel-15 | FFS | FFS | NOTE 1 | 2Rx 4Rx |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. |  |  |  |  |  |  |  |

Table 4.2-2: Applicability of RRM EN-DC FR2 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Test Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 5.3 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 5.3.2 | RRC connection mobility control |  |  |  |  |  |  |
| 5.3.2.2 | Random access |  |  |  |  |  |  |
| 5.3.2.2.1 | EN-DC FR2 contention based random access | Rel-16 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.3.2.2.2 | EN-DC FR2 non-contention based random access | Rel-16 | C022 | UEs supporting EN-DC FR2 |  | 2Rx | Subtest 2: F003 |
| 5.3.2.2.3 | EN-DC FR2 2-step contention based random access | Rel-16 | C158 | UEs supporting EN-DC FR2 and 2-step RACH | NOTE 1 | 2Rx |  |
| 5.3.2.2.4 | EN-DC FR2 2-step non-contention based random access | Rel-16 | C158 | UEs supporting EN-DC FR2 and 2-step RACH | NOTE 1 | 2Rx |  |
| 5.4 | Timing |  |  |  |  |  |  |
| 5.4.1 | UE transmit timing |  |  |  |  |  |  |
| 5.4.1.1 | EN-DC FR2 UE transmit timing accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx | Subtest 2: F004 |
| 5.4.2 | UE timer accuracy |  |  |  |  |  |  |
| 5.4.3 | Timing advance |  |  |  |  |  |  |
| 5.4.3.1 | EN-DC FR2 timing advance adjustment accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.5 | Signalling characteristics |  |  |  |  |  |  |
| 5.5.1 | Radio link monitoring |  |  |  |  |  |  |
| 5.5.1.1 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |  |
| 5.5.1.2 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C022 | UEs supporting EN-DC FR2 | NOTE 1 | 2Rx |  |
| 5.5.1.3 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.5.1.4 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.5.1.5 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C161 | UEs supporting EN-DC FR2 and CSI-RS based RLM | NOTE 1 | 2Rx |  |
| 5.5.1.6 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C161 | UEs supporting EN-DC FR2 and CSI-RS based RLM | NOTE 1 | 2Rx |  |
| 5.5.1.7 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C162 | UEs supporting EN-DC FR2, CSI-RS-based RLM and long DRX cycle |  | 2Rx |  |
| 5.5.1.8 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C162 | UEs supporting EN-DC FR2, CSI-RS-based RLM and long DRX cycle |  | 2Rx |  |
| 5.5.1.9 | EN-DC FR2 radio link monitoring UE scheduling restrictions | Rel-15 | C022n | UEs supporting EN-DC FR2 and PDCCH monitoring in any symbol of the slot (with or without span gap) | NOTE 1 | 2Rx |  |
| 5.5.1.10 | EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | Rel-17 | C022e | UEs supporting EN-DC FR2, long DRX cycle and RLM relaxed measurements |  | 2Rx |  |
| 5.5.2 | Interruption |  |  |  |  |  |  |
| 5.5.2.1 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.5.2.2 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.5.2.3 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Rel-15 | C180 | UEs supporting EN-DC FR2 and 2DL CA in NR |  | 2Rx |  |
| 5.5.2.4 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Rel-15 | C180 | UEs supporting EN-DC FR2 and 2DL CA in NR |  | 2Rx |  |
| 5.5.2.5 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |  |
| 5.5.2.6 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |  |
| 5.5.3 | Scell activation and deactivation delay |  |  |  |  |  |  |
| 5.5.3.1 | EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | Rel-15 | C180 | UEs supporting EN-DC FR2 and 2DL CA in NR |  | 2Rx |  |
| 5.5.3.7 | EN-DC FR2 direct SCell activation at SCell addition of known SCell | Rel-16 | C242 | UEs supporting EN-DC FR2 and 2DL CA in NR and direct SCell activation |  | 2Rx |  |
| 5.5.3.8 | EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band | Rel-17 | C269 | UEs supporting EN-DC FR2 and 2DL CA in NR and direct SCell activation |  | 2Rx |  |
| 5.5.3.13 | EN-DC FR2 Addition and Release Delay of NR PSCell | Rel-17 | C268a | UEs supporting EN-DC FR2 and conditional PSCell addition in EN-DC | NOTE 1 | 2Rx |  |
| 5.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |
| 5.5.5 | Beam failure detection and link recovery procedures |  |  |  |  |  |  |
| 5.5.5.1 | EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.5.5.2 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.5.5.3 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C161 | UEs supporting EN-DC FR2 and CSI-RS-based RLM |  | 2Rx |  |
| 5.5.5.4 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C162 | UEs supporting EN-DC FR2 and long DRX cycle and CSI-RS-based RLM |  | 2Rx |  |
| 5.5.5.5 | EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.5.5.6 | EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX | Rel-16 | C149 | UEs supporting EN-DC FR2 and CSI-RS based BFR on Scell |  | 2Rx |  |
| 5.5.5.7 | EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Rel-16 | C150 | UEs supporting EN-DC FR2 and long DRX cycle and CSI-RS based BFR on Scell |  | 2Rx |  |
| 5.5.5.9 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion | Rel-17 | FFS | UEs supporting EN-DC FR2 and long DRX cycle and BFD relaxation criteria bfd-Relaxation-r17 |  | 2Rx |  |
| 5.5.6 | Active BWP switch delay |  |  |  |  |  |  |
| 5.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |  |
| 5.5.6.1.1 | EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | FFS | FFS | FFS | NOTE 1 | 2Rx |  |
| 5.5.6.1.2 | EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | Rel-15 | N/A | Not recommended due to LTE – FR2 testability issue | NOTE 1 | 2Rx |  |
| 5.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |  |
| 5.5.6.2.1 | EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | Rel-15 | C065c | UEs supporting EN-DC FR2 and (Support of BWP adaptation upto2 or upto4) |  | 2Rx |  |
| 5.5.7 | Void |  |  |  |  |  |  |
| 5.5.8 | Active TCI state switch delay |  |  |  |  |  |  |
| 5.5.8.1 | EN-DC FR2 MAC-CE based active TCI state switch | Rel-15 | C022m | UEs supporting EN-DC FR2, maximum number of active TCI states per BWP greater than 1, and maximum number of simultaneously trackable TRS resource sets per CC greater than 1 | NOTE 1 | 2Rx |  |
| 5.5.8.2 | EN-DC FR2 RRC based active TCI state switch | Rel-15 | C022m | UEs supporting EN-DC FR2, maximum number of active TCI states per BWP greater than 1, and maximum number of simultaneously trackable TRS resource sets per CC greater than 1 | NOTE 1 | 2Rx |  |
| 5.5.11 | Unified TCI state switch delay |  |  |  |  |  |  |
| 5.5.11.1 | EN-DC FR2 MAC-CE based active joint TCI state switch | Rel-17 | C278 | UEs supporting EN-DC FR2, and unified TCI state operation with joint DL/UL TCI update for intra-cell beam management | NOTE 1 | 2Rx |  |
| 5.5.12 | PSCell activation and deactivation delay |  |  |  |  |  |  |
| 5.5.12.1 | EN-DC FR2 PSCell activation and deactivation delay | Rel-17 | C2xxz->C289 | UEs supporting EN-DC FR2 and activation and deactivation on SCG |  | 2RX |  |
| 5.5.11.2 | EN-DC FR2 MAC-CE based active uplink TCI state switch | Rel-17 | C279 | UEs supporting EN-DC FR2, and unified TCI state operation with separate DL/UL TCI update for intra-cell beam management | NOTE 1 | 2Rx |  |
| 5.5.11.3 | EN-DC FR2 MAC-CE based active downlink TCI state switch |  | C280 | UEs supporting EN-DC FR2, and unified TCI state operation with separate DL/UL TCI update for intra-cell beam management, and supporting RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info, and unified TCI with separate DL/UL TCI update for inter-cell beam management | NOTE 1 | 2Rx |  |
| 5.6 | Measurement procedures |  |  |  |  |  |  |
| 5.6.1 | Intra-frequency measurements |  |  |  |  |  |  |
| 5.6.1.1 | EN-DC FR2 event-triggered reporting without gap in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.6.1.2 | EN-DC FR2 event-triggered reporting without gap in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.6.1.3 | EN-DC FR2 event-triggered reporting with gap in non-DRX | Rel-15 | C163 | UEs supporting EN-DC FR2, CSI-RS-based RLM and BWP operation without bandwidth restriction |  | 2Rx |  |
| 5.6.1.4 | EN-DC FR2 event-triggered reporting with gap in DRX | Rel-15 | C043a | UEs supporting EN-DC FR2, long DRX cycle, CSI-RS based RLM and BWP operation without BW restriction |  | 2Rx |  |
| 5.6.2 | Inter-frequency measurements |  |  |  |  |  |  |
| 5.6.2.1 | EN-DC FR2-FR2 event-triggered reporting in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.6.2.2 | EN-DC FR2-FR2 event-triggered reporting in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.6.2.3 | EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.6.2.4 | EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 | 2Rx |  |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 | 2Rx |  |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 | 2Rx |  |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | Rel-15 | N/A | Not recommended due to FR1 – FR2 testability issue | NOTE 1 | 2Rx |  |
| 5.6.3 | L1-RSRP for beam reporting |  |  |  |  |  |  |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C022a | UEs supporting EN-DC FR2 and long DRX cycle |  | 2Rx |  |
| 5.6.4.1 | EN-DC FR2 SRS-RSRP measurement in non-DRX | Rel-16 | C022b | UEs supporting EN-DC FR2 and SRS-RSRP measurements |  | 2Rx |  |
| 5.6.3 | L1-SINR measurement for beam reporting |  |  |  |  |  |  |
| 5.6.6.1 | EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Rel-16 | C141a | UEs supporting EN-DC FR2 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx |  |
| 5.6.6.2 | EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C142a | UEs supporting EN-DC FR2 and L1-SINR measurement based on SSB as CMR and dedicated CSI-RS as IMR |  | 2Rx |  |
| 5.6.6.3 | EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C143a | UEs supporting EN-DC FR2 and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | 2Rx |  |
| 5.7 | Measurement performance requirements |  |  |  |  |  |  |
| 5.7.1 | SS-RSRP |  |  |  |  |  |  |
| 5.7.1.1 | EN-DC FR2 SS-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.7.1.2 | EN-DC FR2-FR2 SS-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.7.1.3 | EN-DC FR1-FR2 SS-RSRP measurement accuracy | Rel-15 | N/A | Not recommended due to E-UTRA/FR1 – FR2 testability issue | NOTE 1 | 2Rx |  |
| 5.7.2 | SS-RSRQ |  |  |  |  |  |  |
| 5.7.2.1 | EN-DC FR2 SS-RSRQ measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.7.2.2 | EN-DC FR2-FR2 SS-RSRQ measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.7.3 | SS-SINR |  |  |  |  |  |  |
| 5.7.3.1 | EN-DC FR2 SS-SINR measurement accuracy | Rel-15 | C069 | UEs supporting EN-DC FR2 and SS-SINR-meas |  | 2Rx |  |
| 5.7.3.2 | EN-DC FR2-FR2 SS-SINR measurement accuracy | Rel-15 | C069 | UEs supporting EN-DC FR2 and SS-SINR-meas |  | 2Rx |  |
| 5.7.4 | L1-RSRP |  |  |  |  |  |  |
| 5.7.4.1 | EN-DC FR2 SSB based L1-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.7.4.2 | EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy | Rel-15 | C022 | UEs supporting EN-DC FR2 |  | 2Rx |  |
| 5.7.5.1 | EN-DC FR2 SRS-RSRP measurement accuracy | Rel-16 | C022b | UEs supporting EN-DC FR2 and SRS-RSRP measurements |  | 2Rx |  |
| 5.7.6 | L1-SINR measurement for beam reporting |  |  |  |  |  |  |
| 5.7.6.1 | EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Rel-16 | C141b | UEs supporting EN-DC FR2 and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx |  |
| 5.7.6.2 | EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C142a | UEs supporting EN-DC FR2 and L1-SINR measurement based on SSB as CMR and dedicated CSI-RS as IMR |  | 2Rx |  |
| 5.7.6.3 | EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Rel-16 | C143a | UEs supporting EN-DC FR2 and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | 2Rx |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |  |

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Test Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6.1 | RRC_IDLE state mobility |  |  |  |  |  |  |
| 6.1.1 | NR cell re-selection |  |  |  |  |  |  |
| 6.1.1.1 | NR SA FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  | 2Rx 4Rx |  |
| 6.1.1.8 | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | Rel-16 | C052a | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST |  | 2Rx 4Rx |  |
| 6.1.2 | NR – E-UTRA cell re-selection |  |  |  |  |  |  |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |  |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |  |
| 6.1.2.3 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C094 | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  | 2Rx 4Rx |  |
| 6.1.2.4 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C094 | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  | 2Rx 4Rx |  |
| 6.1.2.5 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C025b | UEs supporting 5GS NR SA FR1 and E-UTRA and E-UTRA inter-RAT measurement enhancements in HST |  | 2Rx 4Rx |  |
| 6.2 | RRC_INACTIVE state mobility |  |  |  |  |  |  |
| 6.2.1 | NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT) | Rel-17 | C001m | UEs supporting 5GS NR SA FR1, CG-SDT and long DRX cycle |  | 2Rx 4Rx |  |
| 6.3 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 6.3.1 | Handover |  |  |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.3.1.3 | NR SA FR1-FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |  |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |  |
| 6.3.1.6 | NR SA FR1 – UTRAN FDD handover with known target cell | Rel-16 | C096 | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  | 2Rx 4Rx |  |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | Rel-16 | C101 | UEs supporting 5GS NR SA FR1 and intra-frequency DAPS handover |  | 2Rx 4Rx |  |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | Rel-16 | C102 | UEs supporting 5GS NR SA FR1 and intra-frequency async DAPS handover |  | 2Rx 4Rx |  |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Rel-16 | C107 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover |  | 2Rx 4Rx |  |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Rel-16 | C108 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover |  | 2Rx 4Rx |  |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | Rel-16 | C107 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx |  |
|  |  |  | C109 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover and supporting different SCSs in source Pcell and inter-frequency target Pcell | For test configuration 3, 6, 7, 8 | 2Rx 4Rx |  |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | Rel-16 | C108 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx |  |
|  |  |  | C110 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover and supporting different SCSs in source Pcell and inter-frequency target Pcell | For test configuration 3, 6, 7, 8 | 2Rx 4Rx |  |
| 6.3.2 | RRC connection mobility control |  |  |  |  |  |  |
| 6.3.2.1 | RRC re-establishment |  |  |  |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.3.2.2 | Random access |  |  |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.3.2.2.1 has been executed. | 2Rx 4Rx |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.3.2.2.2 has been executed. | 2Rx 4Rx | Subtest 2: F005 |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | Rel-16 | C159 | UEs supporting 5GS NR SA FR1and 2-step RACH | Test execution not necessary if test 4.3.2.2.3 has been executed. | 2Rx 4Rx |  |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | Rel-16 | C159 | UEs supporting 5GS NR SA FR1and 2-step RACH | Test execution not necessary if test 4.3.2.2.4 has been executed. | 2Rx 4Rx |  |
| 6.3.2.3 | RRC connection release with redirection |  |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.3.2.3.2 | NR SA FR1 - E-UTRA RRC connection release with redirection | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx |  |
| 6.3.3 | Conditional handover |  |  |  |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | Rel-16 | C105 | UEs supporting 5GS NR SA FR1 and Conditional handover |  | 2Rx 4Rx |  |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | Rel-16 | C105 | UEs supporting 5GS NR SA FR1 and Conditional handover |  | 2Rx 4Rx |  |
| 6.4 | Timing |  |  |  |  |  |  |
| 6.4.1 | UE transmit timing |  |  |  |  |  |  |
| 6.4.1.1 | NR SA FR1 UE transmit timing accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.4.1.1 has been executed. | 2Rx 4Rx | Subtest 2: F006 |
| 6.4.2 | UE timer accuracy |  |  |  |  |  |  |
| 6.4.3 | Timing advance |  |  |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.4.3.1 has been executed. | 2Rx 4Rx |  |
| 6.5 | Signalling characteristics |  |  |  |  |  |  |
| 6.5.1 | Radio Link Monitoring |  |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.5.1.1 has been executed. | 2Rx 4Rx |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.5.1.2 has been executed. | 2Rx 4Rx |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.5.1.3 has been executed. | 2Rx 4Rx |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.5.1.4 has been executed. | 2Rx 4Rx |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM | Test execution not necessary if test 4.5.1.5 has been executed. | 2Rx 4Rx |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM | Test execution not necessary if test 4.5.1.6 has been executed. | 2Rx 4Rx |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if test 4.5.1.7 has been executed. | 2Rx 4Rx |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if test 4.5.1.8 has been executed. | 2Rx 4Rx |  |
| 6.5.1.9 | SA FR1 radio link monitoring out-of-sync Test for PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | Rel-17 | C037b | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and RLM relaxation criteria rlm-Relaxation-r17 |  | 2Rx 4Rx |  |
| 6.5.2 | Interruption |  |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |  |
| 6.5.3 | Scell activation and deactivation delay |  |  |  |  |  |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |  |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |  |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx |  |
| 6.5.3.4 | NR SA FR1 direct SCell activation at SCell addition of known SCell | Rel-16 | C244 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation |  | 2Rx 4Rx |  |
| 6.5.3.5 | NR SA FR1 direct SCell activation at handover with known SCell | Rel-16 | C244 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation |  | 2Rx 4Rx |  |
| 6.5.3.10 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-17 | C270 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation |  | 2Rx 4Rx |  |
| 6.5.3.11 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle | Rel-17 | C270 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation |  | 2Rx 4Rx |  |
| 6.5.4 | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | Rel-15 | C002 | UEs supporting 5GS NR SA FR1 and SUL | Test execution not necessary if test 4.5.4.1 has been executed. | 2Rx 4Rx |  |
| 6.5.5 | Link recovery procedures |  |  |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | Rel-15 | C084 | UEs supporting 5GS NR SA FR1 and link recovery | Test execution not necessary if test 4.5.5.1 has been executed. | 2Rx 4Rx |  |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | Rel-15 | C084a | UEs supporting 5GS NR SA FR1 and long DRX cycle and link recovery | Test execution not necessary if test 4.5.5.2 has been executed. | 2Rx 4Rx |  |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | Rel-15 | C085 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and link recovery | Test execution not necessary if test 4.5.5.3 has been executed. | 2Rx 4Rx |  |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | Rel-15 | C085a | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and link recovery | Test execution not necessary if test 4.5.5.4 has been executed. | 2Rx 4Rx |  |
| 6.5.5.5 | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | Rel-16 | C173 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and SSB-based link recovery on Scell | Test execution not necessary if test 4.5.5.5 has been executed. | 2Rx 4Rx |  |
| 6.5.5.6 | NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Rel-16 | C174 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and SSB-based link recovery on Scell and long DRX cycle | Test execution not necessary if test 4.5.5.6 has been executed. | 2Rx 4Rx |  |
| 6.5.6 | Active BWP switch delay |  |  |  |  |  |  |
| 6.5.6.1 | DCI-based and timer-based active BWP switch |  |  |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | Rel-15 | C066a | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) and 2DL CA |  | 2Rx 4Rx |  |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | Rel-15 | C066 | UEs supporting 5GS NR SA FR1 and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |  |
| 6.5.6.2 | RRC-based active BWP switch |  |  |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | Rel-15 | C066b | UEs supporting 5GS NR SA FR1 and (Support of BWP adaptation upto2 or upto4) |  | 2Rx 4Rx |  |
| 6.5.6.3 | Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs |  |  |  |  |  |  |
| 6.5.6.3.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | Rel-16 | C066e | UEs supporting 5GS NR SA FR1, incremental delay for DCI and timer based active BWP switching on multiple CCs and 3DL CA | NOTE 1 | 2Rx 4Rx |  |
| 6.5.6.4 |  |  |  |  |  |  |  |
| 6.5.6.5 | Simultaneous RRC-based Active BWP Switch on multiple CCs |  |  |  |  |  |  |
| 6.5.6.5.1 | RRC based BWP switch on multiple CCs | Rel-16 | C066d | UEs supporting 5GS NR SA FR1, incremental delay for DCI and timer based active BWP switching on multiple CCs and 2DL CA | NOTE 1 | 2Rx 4Rx |  |
| 6.5.7 | DL interruptions at switching between two uplink carriers |  |  |  |  |  |  |
| 6.5.7.1 | NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA | Rel-16 | C051 | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching |  | 2Rx 4Rx |  |
| 6.5.7.2 | NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA | Rel-16 | C051a | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching |  | 2Rx 4Rx |  |
| 6.5.7A.2 | NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA | Rel-17 | C051c | UEs supporting 5GS NR SA FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching |  | 2Rx 4Rx |  |
| 6.5.7B.2 | NR SA FR1 DL interruptions at switching between two uplink bands in TDD-TDD CA | Rel-17 | C051e | UEs supporting 5GS NR SA FR1 and Inter-band/intra-band CA (3UL CA) and dynamic UL Tx switching |  | 2Rx 4Rx |  |
| 6.5.7C.1 | NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA | Rel-17 | C051f | UEs supporting 5GS NR SA FR1 and Inter-band/intra-band CA (3UL CA) and dynamic UL Tx switching |  | 2Rx 4Rx |  |
| 6.5.7C.2 | NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA | Rel-17 | C051g | UEs supporting 5GS NR SA FR1 and Inter-band/intra-band CA (3UL CA) and dynamic UL Tx switching |  | 2Rx 4Rx |  |
| 6.5.8 | UE specific CBW change |  |  |  |  |  |  |
| 6.5.8.1 | UE specific CBW change on PCell in FR1 in non-DRX | Rel-16 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.6 | Measurement procedures |  |  |  |  |  |  |
| 6.6.1 | Intra-frequency measurements |  |  |  |  |  |  |
| 6.6.1.1 | NR SA FR1 event-triggered reporting without gap in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.6.1.1 has been executed. | 2Rx 4Rx |  |
| 6.6.1.2 | NR SA FR1 event-triggered reporting without gap in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.6.1.2 has been executed. | 2Rx 4Rx |  |
| 6.6.1.3 | NR SA FR1 event-triggered reporting with gap in non-DRX | Rel-15 | C041 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction | Test execution not necessary if test 4.6.1.3 has been executed. | 2Rx 4Rx |  |
| 6.6.1.4 | NR SA FR1 event-triggered reporting with gap in DRX | Rel-15 | C041a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle | Test execution not necessary if test 4.6.1.4 has been executed. | 2Rx 4Rx |  |
| 6.6.1.5 | NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading | Rel-15 | C024 | UEs supporting 5GS NR FDD SA FR1 | Test execution not necessary if test 4.6.1.5 has been executed. | 2Rx 4Rx |  |
| 6.6.1.6 | NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading | Rel-15 | C041b | UEs supporting 5GS NR FDD SA FR1 and CSI-RS-based RLM and BWP operation without bandwidth restriction | Test execution not necessary if test 4.6.1.6 has been executed. | 2Rx 4Rx |  |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST | Test execution not necessary if test 4.6.1.7 has been executed. | 2Rx 4Rx |  |
| 6.6.1.8 | NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17 | Rel-16 | C052b | UEs supporting 5GS NR SA FR1 and CA measurement enhancements in HST | Test execution not necessary if test 4.6.1.8 has been executed. | 2Rx 4Rx |  |
| 6.6.2 | Inter-frequency measurements |  |  |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.6.2.1 has been executed. | 2Rx 4Rx |  |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.6.2.2 has been executed. | 2Rx 4Rx |  |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.6.2.5 has been executed. | 2Rx 4Rx |  |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.6.2.6 has been executed. | 2Rx 4Rx |  |
| 6.6.2.9 | NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern | Rel-16 | C001 | UEs supporting 5GS NR SA FR1 | NOTE 1 | 2Rx 4Rx |  |
| 6.6.2.10 | NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used | Rel-16 | CZZZ->C287 | UEs supporting 5GS NR SA FR1 and long DRX cycle and inter-frequency SSB based measurements without measurement gaps | NOTE 1 | 2Rx 4Rx |  |
| 6.6.2.11 | NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used | Rel-16 | CXXX->C286 | UEs supporting 5GS NR SA FR1 and inter-frequency SSB based measurements without measurement gaps | NOTE 1 | 2Rx 4Rx |  |
| 6.6.2.12 | NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17 | Rel-16 | C052c | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST | Test execution not necessary if test 4.6.2.9 has been executed. | 2Rx 4Rx |  |
| 6.6.3 | Inter-RAT measurements |  |  |  |  |  |  |
| 6.6.3.1 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRAN |  | 2Rx 4Rx |  |
| 6.6.3.2 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX | Rel-15 | C025a | UEs supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle |  | 2Rx 4Rx |  |
| 6.6.3.3 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C025c | UEs supporting 5GS NR SA FR1 and E-UTRAN, long DRX cycle and E-UTRA inter-RAT measurement enhancements in HST |  | 2Rx 4Rx |  |
| 6.6.4 | L1-RSRP measurement for beam reporting |  |  |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.6.4.1 has been executed. | 2Rx 4Rx |  |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.6.4.2 has been executed. | 2Rx 4Rx |  |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.6.4.3 has been executed. | 2Rx 4Rx |  |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.6.4.4 has been executed. | 2Rx 4Rx |  |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | Rel-15 | C001f | UEs supporting 5GS NR SA FR1, long DRX cycle and intra-NR measurement enhancements in HST | Test execution not necessary if test 4.6.4.5 has been executed. | 2Rx 4Rx |  |
| 6.6.5 | FFS |  |  |  |  |  |  |
| 6.6.5.1 | NR SA FR1 – UTRAN event-triggered reporting in non-DRX | Rel-16 | C096 | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  | 2Rx 4Rx |  |
| 6.6.8 | L1-SINR measurement for beam reporting |  |  |  |  |  |  |
| 6.6.8.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX | Rel-16 | C144 | UEs supporting 5GS NR SA FR1 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  | 2Rx 4Rx |  |
| 6.6.8.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C145 | UEs supporting 5GS NR SA FR1 and L1-SINR measurement based on SSB as CMR and dedicated CSI-RS as IMR |  | 2Rx 4Rx |  |
| 6.6.8.3 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX | Rel-16 | C146 | UEs supporting 5GS NR SA FR1 and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  | 2Rx 4Rx |  |
| 6.6.9 | Idle Mode CA/DC Measurements |  |  |  |  |  |  |
| 6.6.9.1 | NR SA FR1 Idle mode CA/DC measurement for FR1 | Rel-16 | C031a | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and NR SSB measurements in RRC_IDLE/RRC_INACTIVE |  |  |  |
| 6.6.15 | Idle Mode inter-RAT CA/DC Measurements |  |  |  |  |  |  |
| 6.6.15.1 | NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting | Rel-16 | C031b | UEs supporting NE-DC FR1 and E-UTRA measurements in RRC_IDLE/RRC_INACTIVE |  |  |  |
| 6.6.17 | SA event triggered reporting tests with Pre-MG |  |  |  |  |  |  |
| 6.6.17.1 | NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG | Rel-17 | C259 | UEs supporting 5GS NR SA FR1, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredUE-AutonomousMeasGap-r17 |  | 2Rx 4Rx |  |
| 6.6.17.2 | NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | Rel-17 | C260 | UEs supporting 5GS NR SA FR1, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredNW-ControlledMeasGap-r17 |  | 2Rx 4Rx |  |
| 6.6.18 | SA event triggered reporting tests with concurrent gaps |  |  |  |  |  |  |
| 6.6.18.1 | NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers | Rel-17 | C264 | UEs supporting 5GS NR SA FR1 and more than 1 per-UE measurement gap configurations |  | 2Rx 4Rx |  |
| 6.6.18.2 | NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers | Rel-17 | C264 | UEs supporting 5GS NR SA FR1 and more than 1 per-UE measurement gap configurations |  | 2Rx 4Rx |  |
| 6.6.18.3 | NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1 | Rel-17 | C265 | UEs supporting 5GS NR SA FR1 and E-UTRA and more than 1 per-UE measurement gap configurations and the configurations of E-UTRAN measurement objectives associated with more than 1 concurrent measurement gaps |  | 2Rx 4Rx |  |
| 6.6.18.4 | NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used | Rel-17 | C266 | UEs supporting 5GS NR SA FR1 and more than 1 per-UE measurement gap configurations and two independent measurement gap configurations for FR1 and FR2 for PRS measurement |  | 2Rx 4Rx |  |
| 6.6.19 | SA event triggered reporting tests with NCSG |  |  |  |  |  |  |
| 6.6.19.1 | NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1 | Rel-17 | C253 | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction, NR only NCSG patterns and reporting of NCSG requirement information but don’t support per-FR NCSG | For sub-test 1 | 2Rx 4Rx |  |
|  |  |  | C254 | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction, per-FR NCSG, reporting of NCSG requirement information and NR only NCSG patterns | For sub-test 2 | 2Rx 4Rx |  |
| 6.6.19.2 | NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement | Rel-17 | C255 | UEs supporting 5GS NR SA FR1, NR only NCSG patterns and reporting of NCSG requirement information but don’t support per-FR NCSG | For sub-test 1 | 2Rx 4Rx |  |
|  |  |  | C256 | UEs supporting 5GS NR SA FR1, per-FR NCSG, reporting of NCSG requirement information and NR only NCSG patterns | For sub-test 2 | 2Rx 4Rx |  |
| 6.6.19.3 | NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG | Rel-17 | C257 | UEs supporting 5GS NR SA FR1, E-UTRAN, reporting of NCSG requirement information for E-UTRA and NCSG patterns |  | 2Rx 4Rx |  |
| 6.6.19.4 | NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX | Rel-17 | C258 | UEs supporting 5GS NR SA FR1, CA (2DL CA), reporting of NCSG requirement information and NR only NCSG patterns |  | 2Rx 4Rx |  |
| 6.6.20 | UE Rx-Tx time difference measurement for propagation delay compensation |  |  |  |  |  |  |
| 6.6.20.1 | UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA | Rel-17 | C212 | UEs supporting 5GS NR SA FR1 and RTT-based PDC for Rx-Tx measurement with PRS |  | 2Rx |  |
| 6.6.21 | UE Rx-Tx time difference measurement for propagation delay compensation with TRS |  |  |  |  |  |  |
| 6.6.21.1 | UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA | Rel-17 | C213 | UEs supporting 5GS NR SA FR1 and RTT-based PDC for Rx-Tx measurement with TRS |  | 2Rx |  |
| 6.7 | Measurement performance requirements |  |  |  |  |  |  |
| 6.7.1 | SS-RSRP |  |  |  |  |  |  |
| 6.7.1.1 | Intra-frequency measurements |  |  |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.1.1.1 has been executed. | 2Rx 4Rx |  |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.1.1.2 has been executed. | 2Rx 4Rx |  |
| 6.7.1.2 | Inter-frequency measurements |  |  |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.1.2.1 has been executed. | 2Rx 4Rx |  |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.1.2.2 has been executed. | 2Rx 4Rx |  |
| 6.7.2 | SS-RSRQ |  |  |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.2.1 has been executed. | 2Rx 4Rx |  |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.2.2.1 has been executed. | 2Rx 4Rx |  |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.2.2.2 has been executed. | 2Rx 4Rx |  |
| 6.7.3 | SS-SINR |  |  |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas | Test execution not necessary if test 4.7.3.1 has been executed. | 2Rx 4Rx |  |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas | Test execution not necessary if test 4.7.3.2.1 has been executed. | 2Rx 4Rx |  |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | Rel-15 | C034 | UEs supporting 5GS NR SA FR1 and SS-SINR-meas | Test execution not necessary if test 4.7.3.2.2 has been executed. | 2Rx 4Rx |  |
| 6.7.4 | L1-RSRP for beam reporting |  |  |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.4.1.1 has been executed. | 2Rx 4Rx |  |
| 6.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.4.1.2 has been executed. | 2Rx 4Rx |  |
| 6.7.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.4.2.1 has been executed. | 2Rx 4Rx |  |
| 6.7.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.7.4.2.2 has been executed. | 2Rx 4Rx |  |
| 6.7.5 | E-UTRAN RSRP |  |  |  |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.7.6 | E-UTRAN RSRQ |  |  |  |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx |  |
| 6.7.7 | E-UTRAN RS-SINR |  |  |  |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Rel-15 | C168 | UEs supporting 5GS NR SA FR1 and E-UTRA RS-SINR measurements |  | 2Rx 4Rx |  |
| 6.7.9 | L1-SINR |  |  |  |  |  |  |
| 6.7.9.1.1 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy | Rel-16 | C132 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured | Test execution not necessary if test 4.7.7.1.1 has been executed. | 2Rx 4Rx |  |
| 6.7.9.1.2 | NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy | Rel-16 | C132 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured | Test execution not necessary if test 4.7.7.1.2 has been executed. | 2Rx 4Rx |  |
| 6.7.9.2 | NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C133 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR | Test execution not necessary if test 4.7.7.2 has been executed. | 2Rx 4Rx |  |
| 6.7.9.3.1 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy | Rel-16 | C134 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR | Test execution not necessary if test 4.7.7.3.1 has been executed. | 2Rx 4Rx |  |
| 6.7.9.3.2 | NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy | Rel-16 | C134 | UEs supporting 5GS NR SA FR1 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR | Test execution not necessary if test 4.7.7.3.2 has been executed. | 2Rx 4Rx |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Test X refers to the corresponding Sub-Test as defined in TS 38.533 [5]. NOTE 3: Test cases in TS 38.533 [5] clause 6 only apply to FR1 non-RedCap UEs. For FR1 RedCap UEs, Test cases in TS 38.533 [5] clause 16 apply. |  |  |  |  |  |  |  |

Table 4.2-4: Applicability of RRM NR SA FR2 conformance test cases, ref. TS 38.533 [5]

| Clause |  | TC Title |  | Release |  | Applicability |  |  |  | Additional Information |  | Branch |  | Test Selection Criteria |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Condition |  | Comment |  |  |  |  |  |  |  |
| 7.1 |  | RRC_IDLE state mobility |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.1 |  | NR cell re-selection |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1.1.1 |  | NR SA FR2 cell re-selection |  | FFS |  | FFS |  | FFS |  |  |  | 2Rx |  |  |  |
| 7.1.1.2 |  | NR SA FR2-FR2 cell re-selection |  | FFS |  | FFS |  | FFS |  |  |  | 2Rx |  |  |  |
| 7.1.1.3 |  | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Rel-16 |  | C095 |  | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  | 2Rx |  |  |  |
| 7.1.1.4 |  | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Rel-16 |  | C095 |  | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  | 2Rx |  |  |  |
| 7.1.1.5 |  | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Rel-16 |  | C095 |  | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  | 2Rx |  |  |  |
| 7.1.1.6 |  | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Rel-16 |  | C095 |  | UEs supporting 5GS NR SA FR2 and relaxed RRM measurement |  |  |  | 2Rx |  |  |  |
| 7.1.1.7 |  | NR SA FR2 cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 |  | Rel-17 |  | C240 |  | UEs supporting 5GS NR SA FR2 and intra-frequency RRC_IDLE measurements in HST |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.2 |  | RRC_INACTIVE state mobility |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.2.1 |  | Small Data Transmission |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.2.1.1 |  | TA Validation for CG-SDT in FR2 |  | Rel-17 |  | C095a |  | UEs supporting 5GS NR SA FR2 and TA Validation for CG-SDT in FR2 |  | NOTE 1 |  | 2RX |  |  |  |
| 7.3 |  | RRC_CONNECTED state mobility |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.1 |  | Handover |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.1.4 |  | NR SA FR1-FR2 synchronous DAPS handover |  | Rel-16 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.3.1.5 |  | NR SA FR1-FR2 asynchronous DAPS handover |  | Rel-16 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.3.2 |  | RRC connection mobility control |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.1 |  | RRC re-establishment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.1.1 |  | NR SA FR2 RRC re-establishment |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.3.2.1.2 |  | NR SA FR2 - FR2 RRC re-establishment |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.3.2.1.3 |  | NR SA FR2 RRC re-establishment without serving cell timing |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.3.2.2 |  | Random access |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.2.1 |  | NR SA FR2 contention based random access |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.3.2.2.2 |  | NR SA FR2 non-contention based random access |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  | NOTE 1 |  | 2Rx |  | Subtest 2: F007 |  |
| 7.3.2.2.4 |  | NR SA FR2 2-step non-contention based random access |  | Rel-16 |  | C160 |  | UEs supporting 5GS NR SA FR2 and 2-step RACH |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.3.2.3 |  | RRC connection release with redirection |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.3.1 |  | NR SA FR2-FR2 RRC connection release with redirection |  | FFS |  | FFS |  | FFS |  |  |  | 2Rx |  |  |  |
| 7.3.3 |  | Conditional Handover |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.3.3.1 |  | NR SA FR2 conditional handover |  | Rel-16 |  | C106 |  | UEs supporting 5GS NR SA FR2 and conditional handover |  |  |  | 2Rx |  |  |  |
| 7.3.3.2 |  | NR SA FR2-FR2 conditional handover |  | Rel-16 |  | C106 |  | UEs supporting 5GS NR SA FR2 and conditional handover |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.4 |  | Timing |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.4.1 |  | UE transmit timing |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.4.1.1 |  | NR SA FR2 UE transmit timing accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  | Subtest 2: F008 |  |
| 7.4.2 |  | UE timer accuracy |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.4.3 |  | Timing advance |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.4.3.1 |  | NR SA FR2 timing advance adjustment accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.5 |  | Signalling characteristics |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.1 |  | Radio Link Monitoring |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.1 |  |  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSBbased RLM RS in non-DRX mode |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.1.2 |  |  | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.1.3 |  |  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSBbased RLM RS in DRX mode |  | Rel-15 |  | C006a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.5.1.4 |  |  | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode |  | Rel-15 |  | C006a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.5.1.5 |  | NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  | Rel-15 |  | C164 |  | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.1.6 |  | NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode |  | Rel-15 |  | C164 |  | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.1.7 |  | NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM RS in DRX mode |  | Rel-15 |  | C165 |  | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM and log DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.5.1.8 |  | NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode |  | Rel-15 |  | C165 |  | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM and log DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.5.1.9 |  | NR SA FR2 radio link monitoring UE scheduling restrictions |  | Rel-15 |  | C006n |  | UEs supporting 5GS NR SA FR2 and PDCCH monitoring in any symbol of the slot (with or without span gap) |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.2 |  | Interruption |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.2.1 |  | NR SA FR2 interruptions during measurements on deactivated NR SCC |  | Rel-15 |  | C006b |  | UEs supporting 5GS NR SA FR2 and 2DL CA in NR |  |  |  | 2Rx |  |  |  |
| 7.5.3 |  | Scell activation and deactivation delay |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.3.1 |  | NR SA FR2-FR2 intra-band SCell activation and deactivation delay |  | FFS |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.3.2 |  | NR SA FR1-FR2 inter-band SCell activation and deactivation delay |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.3.3 |  | SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX |  | Rel-17 |  | C031c |  | UEs supporting 5GS NR SA FR2 and 2DL CA in NR |  |  |  | 2Rx |  |  |  |
| 7.5.3.4 |  | NR SA FR2 direct SCell activation at SCell addition of known SCell |  | Rel-16 |  | C241 |  | UEs supporting 5GS NR SA FR2 and 2DL CA in NR and direct SCell activation |  |  |  | 2Rx |  |  |  |
| 7.5.3.5 |  | NR SA FR2 direct SCell activation at handover with known SCell |  | Rel-16 |  | C241 |  | UEs supporting 5GS NR SA FR2 and 2DL CA in NR and direct SCell activation |  |  |  | 2Rx |  |  |  |
| 7.5.3.13 |  | NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX |  | Rel-17 |  | C271 |  | UEs supporting 5GS NR SA intra-band FR2 and 2DL CA in NR and fast SCell activation |  |  |  | 2Rx |  |  |  |
| 7.5.3.14 |  | NR SA FR2 SCell Activation for known SCell in FR2 inter-band |  | Rel-17 |  | C271a |  | UEs supporting 5GS NR SA inter-band FR2 and 2DL CA in NR and fast SCell activation |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.5 |  | Beam failure detection and link recovery procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.1 |  | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.5.5.2 |  | NR SA FR2 SSB-based beam failure detection and link recovery in DRX |  | Rel-15 |  | C006a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.5.5.3 |  | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C164 |  | UEs supporting 5GS NR SA FR2 and CSI-RS based RLM |  |  |  | 2Rx |  |  |  |
| 7.5.5.4 |  | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX |  | Rel-15 |  | C165 |  | UEs supporting 5GS NR SA FR2, long DRX cycle and CSI-RS based RLM |  |  |  | 2Rx |  |  |  |
| 7.5.5.5 |  | NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.5.5.6 |  | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX |  | Rel-16 |  | C147 |  | UEs supporting 5GS NR SA FR2 and CSI-RS based BFR on Scell |  |  |  | 2Rx |  |  |  |
| 7.5.5.7 |  | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX |  | Rel-16 |  | C148 |  | UEs supporting 5GS NR SA FR2 and long DRX cycle and CSI-RS based BFR on Scell |  |  |  | 2Rx |  |  |  |
| 7.5.6 |  | Active BWP switch delay |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.6.1 |  | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.6.1.1 |  | NR SA FR2 DCI-based DL active BWP switch in non-DRX |  | FFS |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.6.1.2 |  | NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.6.1.3 |  | NR SA FR2 DCI-based DL active BWP switch in non-DRX |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.6.2 |  | RRC-based active BWP switch |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.6.2.1 |  | NR SA FR2 RRC-based DL active BWP switch in non-DRX |  | FFS |  | FFS |  | FFS |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.7 |  | PSCell addition and release delay |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.7.1 |  | NR SA FR2 addition and release delay of known PSCell |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.7.2 |  | NR SA FR2 addition and release delay of unknown PSCell |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.8 |  | Active TCI state switch delay |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.8.1 |  | NR SA FR2 MAC-CE based active TCI state switch |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.8.1.1 |  | NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state |  | Rel-15 |  | C006m |  | UEs supporting 5GS NR SA FR2, maximum number of active TCI states per BWP greater than 1, and maximum number of simultaneously trackable TRS resource sets per CC greater than 1 |  |  |  | 2Rx |  |  |  |
| 7.5.8.2 |  | NR SA FR2 RRC based active TCI state switch |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.8.2.1 |  | NR SA Pcell FR2 RRC based active TCI state switch for a known TCI state |  | Rel-15 |  | C006m |  | UEs supporting 5GS NR SA FR2, maximum number of active TCI states per BWP greater than 1, and maximum number of simultaneously trackable TRS resource sets per CC greater than 1 |  |  |  | 2Rx |  |  |  |
| 7.5.8.3 |  | MAC-CE based active TCI state switch for HST FR2 scenario |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.8.3.1 |  | NR PCell FR2 HST active TCI state switch for a known TCI state |  | Rel-17 |  | C240a |  | UEs supporting 5GS NR SA FR2 and one-shot large UL timing adjustment.in HST |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.11 |  | UE UL carrier RRC reconfiguration delay |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.11.1 |  | UE UL carrier RRC reconfiguration delay |  | Rel-17 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.5.12 |  | Conditional PSCell addition and release delay |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.12.1 |  | Addition and Release Delay of PSCell |  | Rel-17 |  | C268b |  | UEs supporting Inter-band NR-DC between FR1 and FR2 and conditional PSCell addition in NR-DC |  |  |  |  |  |  |  |
| 7.5.13 |  | Unified TCI state switch delay |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.5.13.1 |  | NR SA FR2 MAC-CE based active joint TCI state switch |  | Rel-17 |  | C281 |  | UEs supporting 5GS NR SA FR2, and unified TCI state operation with joint DL/UL TCI update for intra-cell beam management |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.13.2 |  | NR SA FR2 MAC-CE based active uplink TCI state switch |  | Rel-17 |  | C282 |  | UEs supporting 5GS NR SA FR2, and unified TCI state operation with separate DL/UL TCI update for intra-cell beam management |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.13.3 |  | NR SA FR2 MAC-CE based active downlink TCI state switch |  | Rel-17 |  | C283 |  | UEs supporting 5GS NR SA FR2, and supporting RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info, and unified TCI with separate DL/UL TCI update for inter-cell beam management |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.5.14 |  | NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |  | Rel-17 |  | Cxx1->C284 |  | UEs supporting Inter-band NR-DC between FR1 and FR2 and activation and deactivation on SCG in NR-DC |  |  |  |  |  |  |  |
| 7.6 |  | Measurement procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.1 |  | Intra-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.1.1 |  | NR SA FR2 event-triggered reporting without gap in non-DRX |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.6.1.2 |  | NR SA FR2 event-triggered reporting without gap in DRX |  | Rel-15 |  | C006a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.6.1.3 |  | NR SA FR2 event-triggered reporting with gap in non-DRX |  | Rel-15 |  | C166 |  | UEs supporting 5GS NR SA FR2, CSI-RS-based RLM and BWP operation without bandwidth restriction |  |  |  | 2Rx |  |  |  |
| 7.6.1.4 |  | NR SA FR2 event-triggered reporting with gap in DRX |  | Rel-15 |  | C167 |  | UEs supporting 5GS NR SA FR2 long DRX cycle, CSI-RS-based RLM and BWP operation without bandwidth restriction |  |  |  | 2Rx |  |  |  |
| 7.6.1.5 |  | NR SA FR2 event-triggered reporting without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17 |  | Rel-17 |  | C240 |  | UEs supporting 5GS NR SA FR2 and intra-frequency measurements in HST |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.6.2 |  | Inter-frequency measurements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.2.1 |  | NR SA FR2-FR2 event-triggered reporting in non-DRX |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.6.2.2 |  | NR SA FR2-FR2 event-triggered reporting in DRX |  | Rel-15 |  | C006a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.6.2.3 |  | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.6.2.4 |  | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection |  | Rel-15 |  | C006a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.6.2.5 |  | NR SA FR1-FR2 event-triggered reporting in non-DRX |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.6.2.6 |  | NR SA FR1-FR2 event-triggered reporting in DRX |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.6.2.7 |  | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.6.2.8 |  | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.6.3 |  | L1-RSRP for beam reporting |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.3.1 |  | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.6.3.2 |  | NR SA FR2 SSB-based L1-RSRP measurement in DRX |  | Rel-15 |  | C006a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.6.3.3 |  | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.6.3.4 |  | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX |  | Rel-15 |  | C006a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle |  |  |  | 2Rx |  |  |  |
| 7.6.3.6 |  | NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |  | Rel-17 |  | C006zb |  | UEs supporting 5GS NR SA FR2 and Inter-band CA (2UL CA) |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.6.3.5 |  | NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17 |  | Rel-17 |  | C240 |  | power class 6 UEs supporting 5GS NR SA FR2 configured and HST intra-frequency measurements |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.6.4.1 |  | NR SA FR2 SRS-RSRP measurement in non-DRX |  | Rel-16 |  | C006l |  | UEs supporting 5GS NR SA FR2 and SRS-RSRP measurements |  |  |  | 2Rx |  |  |  |
| 7.6.6 |  | L1-SINR measurement for beam reporting |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.6.1 |  | NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX |  | Rel-16 |  | C144a |  | UEs supporting 5GS NR SA FR2 and L1-SINR measurement based on CSI-RS as CMR without dedicated IMR configured |  |  |  | 2Rx 4Rx |  |  |  |
| 7.6.6.2 |  | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX |  | Rel-16 |  | C145a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle and L1-SINR measurement based on SSB as CMR and dedicated CSI-IM as IMR |  |  |  | 2Rx 4Rx |  |  |  |
| 7.6.6.3 |  | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX |  | Rel-16 |  | C146a |  | UEs supporting 5GS NR SA FR2 and long DRX cycle and L1-SINR measurement based on CSI-RS as CMR and dedicated CSI-RS as IMR |  |  |  | 2Rx 4Rx |  |  |  |
| 7.6.13 |  | UE Rx-Tx time difference measurements for PDC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.13.1 |  | NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS |  | Rel-17 |  | C212 |  | UEs supporting 5GS NR SA FR2 and RTT-based PDC for Rx-Tx measurement with PRS |  |  |  | 2Rx |  |  |  |
| 7.6.13.2 |  | NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS |  | Rel-17 |  | C213 |  | UEs supporting 5GS NR SA FR2 and RTT-based PDC for Rx-Tx measurement with TRS |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.6.14 |  |  | SA event triggered reporting tests with Pre-MG |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.14.1 |  |  | NR SA FR2 Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2 |  | Rel-17 |  | Cxx1->C290 |  | UEs supporting 5GS NR SA FR2, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredUE-AutonomousMeasGap-r17 |  | NOTE 1 |  | 2RX |  |  |  |
| 7.6.14.2 |  |  | NR SA FR2 Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2 |  | Rel-17 |  | Cxx2->C291 |  | UEs supporting 5GS NR SA FR2, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredNW-ControlledMeasGap-r17 |  | NOTE 1 |  | 2RX |  |  |  |
| 7.6.15 |  |  | SA event triggered reporting tests with concurrent gaps |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.15.1 |  |  | NR SA FR2 event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements |  | Rel-17 |  | Cxx3->C292 |  | UEs supporting 5GS NR SA FR2 and more than 1 per-UE measurement gap configurations |  | NOTE 1 |  | 2RX |  |  |  |
| 7.6.15.2 |  |  | NR SA FR2 event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2) |  | Rel-17 |  | Cxx3->C292 |  | UEs supporting 5GS NR SA FR2 and more than 1 per-UE measurement gap configurations |  | NOTE 1 |  | 2RX |  |  |  |
| 7.6.15.3 |  |  | NR SA FR2 event triggered reporting tests for FR2 concurrent gap with partially overlapping scenario for SSB-based measurements and PRS-based measurement |  | Rel-17 |  | Cxx4->C293 |  | UEs supporting 5GS NR SA FR2 and more than 1 per-UE measurement gap configurations and two independent measurement gap configurations for FR1 and FR2 for PRS measurement |  | NOTE 1 |  | 2RX |  |  |  |
| 7.6.16 |  |  | SA event triggered reporting tests with NCSG |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.6.16.1 |  |  | NR SA FR2 event triggered reporting test with per-UE NCSG under non-DRX |  | Rel-17 |  | Cxx5->C294 |  | UEs supporting 5GS NR SA FR2, CSI-RS-based RLM, BWP operation without bandwidth restriction, per-FR NCSG, reporting of NCSG requirement information and NR only NCSG patterns |  | NOTE 1 |  | 2RX |  |  |  |
| 7.6.16.2 |  |  | NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2) |  | Rel-17 |  | Cxx6->C295 |  | UEs supporting 5GS NR SA FR2, reporting of NCSG requirement information and NR only NCSG patterns but not supporting ncsg-MeasGapPerFR-r17 |  | For sub-test 1 NOTE 1 |  | 2RX |  |  |  |
|  |  |  |  |  |  |  | Cxx7->C296 |  | UEs supporting 5GS NR SA FR2, per-FR NCSG, reporting of NCSG requirement information and NR only NCSG patterns |  | For sub-test 2 NOTE 1 |  | 2RX |  |  |  |
| 7.6.16.3 |  |  | NR SA FR2 Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX |  | Rel-17 |  | Cxx8->C297 |  | UEs supporting 5GS NR SA FR2, CA (2DL CA), reporting of NCSG requirement information and NR only NCSG patterns |  | NOTE 1 |  | 2RX |  |  |  |
| 7.7 |  | Measurement performance requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.1 |  | SS-RSRP |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.1.1 |  | NR SA FR2 SS-RSRP measurement accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.7.1.2 |  | NR SA FR2-FR2 SS-RSRP measurement accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.7.1.3 |  | Inter-frequency measurements between FR1 and FR2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.1.3.1 |  | NR SA FR1-FR2 SS-RSRP measurement accuracy |  | Rel-15 |  | N/A |  | Not recommended due to FR1 – FR2 testability issue |  | NOTE 1 |  | 2Rx |  |  |  |
| 7.7.2 |  | SS-RSRQ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.2.1 |  | NR SA FR2 SS-RSRQ measurement accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.7.2.2 |  | NR SA FR2-FR2 SS-RSRQ measurement accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.7.3 |  | SS-SINR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.3.1 |  | NR SA FR2 SS-SINR measurement accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.7.3.2 |  | NR SA FR2-FR2 SS-SINR measurement accuracy |  | Rel-16 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.7.4 |  | L1-RSRP for beam reporting |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.4.1 |  | NR SA FR2 SSB based L1-RSRP measurement accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.7.4.2 |  | NR SA FR2 CSI-RS based L1-RSRP measurement accuracy |  | Rel-15 |  | C006 |  | UEs supporting 5GS NR SA FR2 |  |  |  | 2Rx |  |  |  |
| 7.7.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.5.1 |  | NR SA FR2 SRS-RSRP measurement accuracy |  | Rel-16 |  | C006l |  | UEs supporting 5GS NR SA FR2 and SRS-RSRP measurements |  |  |  | 2Rx |  |  |  |
| 7.7.6 |  | L1-SINR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.6.1 |  | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | Rel-16 |  | C138 |  | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on CSI-RS as CMR without dedicated IMR configured |  |  |  | 2Rx |  |  |  |
| 7.7.6.2 |  | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | Rel-16 |  | C139 |  | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on SSB as CMR and dedicated CSI-IM as IMR |  |  |  | 2Rx |  |  |  |
| 7.7.6.3 |  | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | Rel-16 |  | C140 |  | UEs supporting 5GS NR SA FR2 and L1-SINR-measurement based on CSI-RS as CMR and dedicated CSI-IM as IMR |  |  |  | 2Rx |  |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.533. NOTE 2: Void. NOTE 3: Void. NOTE 4: Test cases in TS 38.533 [5] clause 7 only apply to FR2 non-RedCap UEs. For FR2 RedCap UEs, Test cases in TS 38.533 [5] clause 17 apply. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

----End of Change ----