3GPP TSG-RAN5 Meeting #102	R5-240818

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5285 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT NTN testcase 22.4.13a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose, in the Tble 22.4.13a.3.3-2, the IE mixedOperationMode-r15 appears twice, once within IE ue-RadioPagingInfo-r13, and once within IE phyLayerParameters-v1530. if the UE has reported the support of mixedOperationMode-r15 within UE-RadioPagingInfo-NB-r13 IE and the UE supports solely this capability out of those listed within PhyLayerParameters-NB-v1530 IE, the UE may consider not to include PhyLayerParameters-NB-v1530 in the report. But the current prose does not consider this. The option to allow this behaviour has to be introduced. The proposed change in the CR does not directly omit PhyLayerParameters-NB-v1530 IE, but it would produce the same effect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the check for IE mixedOperationMode-r15 within IE phyLayerParameters-v1530 based upon the PICS pc_SR_WithHARQ_ACK, pc_SR_WithoutHARQ_ACK and pc_NB_nprach_Format2. The idea is that if any of these IEs are present, then the UE should also include the IE mixedOperationMode-r15. If none of these are present, then it means that IE phyLayerParameters-v1530 is absent, and hence setting of mixedOperationMode-r15 need not be checked. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.13a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Associated with R5s240161 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 22.4.13a	NB-IoT / NTN / UE capability transfer / Success

22.4.13a.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry-NB message }

then { UE transmits an UECapabilityInformation-NB message including UE radio access capability parameters }

}

22.4.13a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 5.6.3.1 and 5.6.3.3. Unless and otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.6.3.1]

Figure 5.6.3.1-1: UE capability transfer

The purpose of this procedure is to transfer UE radio access capability information from the UE to E-UTRAN.

If the UE is NTN capable, the UE reports its E-UTRAN radio access capabilities for the network type (TN or NTN) to which it is connected.

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	for NB-IoT, set the contents of UECapabilityInformation message as follows:

2>	include the UE Radio Access Capability Parameters within the ue-Capability;

2>	include ue-RadioPagingInfo;

2>	submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends;

22.4.13a.3	Test description

22.4.13a.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 is configured according to Table 8.1.4.2-1 and Table 8.1.4.2-2 in TS 36.508 [18].

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State Connected Mode (State 2-NB) according to [18].

22.4.13a.3.2	Test procedure sequence

Table 22.4.13a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UECapabilityEnquiry-NB message to request UE radio access capability information. | <-- | UECapabilityEnquiry-NB | - | - |
| 2 | Check: Does the UE transmit a UECapabilityInformation-NB message? | --> | UECapabilityInformation-NB | 1 | P |

22.4.13a.3.3	Specific message contents

Table 22.4.13a.3.3-1: UEcapabilityEnquiry-NB (step 1)

Derivation path: 36.508 clause 8.1.6 Table 8.1.6.1-16

Table 22.4.13a.3.3-2: UECapabilityInformation-NB (step 2)

| Derivation path: 36.508 clause 8.1.6 Table 8.1.6.1-17 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityInformation-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation-r13 SEQUENCE { |  |  |  |
| ue-Capability-Container-r13 SEQUENCE { |  |  |  |
| accessStratumRelease-r13 | Not checked | Value should be based on Rel of Access stratum supported |  |
| ue-Category-NB-r13 | nb1 |  |  |
| multipleDRB-r13 | Checked |  | pc_NB_MultiDRB |
| pdcp-Parameters-r13 SEQUENCE {} | Not present |  | NOT (pc_User_Plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) |
| pdcp-Parameters-r13 SEQUENCE {} | Not present or Present | Up to UE implementation (i.e this case is not exactly specified in TS 36.331 / TS 36.306 | (pc_User_Plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) AND NOT (pc_ROHC_profile0x0002 OR pc_ROHC_profile0x0003 OR pc_ROHC_profile0x0004 OR pc_ROHC_profile0x0006 OR pc_ROHC_profile0x0102 OR pc_ROHC_profile0x0103 OR OR pc_ROHC_profile0x0104) |
| pdcp-Parameters-r13 SEQUENCE { |  |  | (pc_User_Plane_CIoT_Optimisation OR pc_S1_U_DataTransfer) AND (pc_ROHC_profile0x0002 OR pc_ROHC_profile0x0003 OR pc_ROHC_profile0x0004 OR pc_ROHC_profile0x0006 OR pc_ROHC_profile0x0102 OR pc_ROHC_profile0x0103 OR OR pc_ROHC_profile0x0104) |
| supportedROHC-Profiles-r13 SEQUENCE { |  |  |  |
| profile0x0002 | Checked |  | pc_ROHC_profile0x0002 |
| profile0x0003 | Checked |  | pc_ROHC_profile0x0003 |
| profile0x0004 | Checked |  | pc_ROHC_profile0x0004 |
| profile0x0006 | Checked |  | pc_ROHC_profile0x0006 |
| profile0x0102 | Checked |  | pc_ROHC_profile0x0102 |
| profile0x0103 | Checked |  | pc_ROHC_profile0x0103 |
| profile0x0104 | Checked |  | pc_ROHC_profile0x0104 |
| } |  |  |  |
| maxNumberROHC-ContextSessions | Not checked |  |  |
| } |  |  |  |
| phyLayerParameters-r13 SEQUENCE { |  |  |  |
| multiTone-r13 | Checked |  | pc_NB_MultiTone |
| multiCarrier-r13 | Checked |  | pc_NB_MultiCarrier |
| } |  |  |  |
| rf-Parameters-r13 SEQUENCE { |  |  |  |
| supportedBandList-r13 SEQUENCE (SIZE (1.. maxBands)) OF SEQUENCE { | n1 entries where n1 is the sum of pc_eBandα_Supp for α= 255 to maxFBI2(256) |  |  |
| band-r13 [α = 1..2] | Any value β such that pc_eBandβ_Supp is TRUE and different from all eutra-Band[k] where k = 1 to α - 1 | β = 255, 256 |  |
| powerClassNB-20dBm-r13 [1] | Not checked |  |  |
| } |  |  |  |
| multiNS-Pmax-r13 | Not checked |  |  |
| } |  |  |  |
| dummy SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| ue-RadioPagingInfo-r13 SEQUENCE { |  |  |  |
| ue-Category-NB-r13 | nb1 |  |  |
| multiCarrierPaging-r14 | Checked |  | pc_NB_MultiCarrier_Paging |
| mixedOperationMode-r15 | Checked |  | pc_NB_mixedOperationMode |
| wakeUpSignal-r15 | Checked |  | pc_wakeUpSignal |
| wakeUpSignalMinGap-eDRX-r15 | Not checked |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-Capability-ContainerExt-r14 OCTET STRING { |  |  |  |
| ue-Category-NB-r14 | Checked |  | pc_ue_Category_NB2 |
| mac-Parameters-r14 SEQUENCE { |  |  |  |
| dataInactMon-r14 | Not checked |  |  |
| rai-Support-r14 | Checked |  | pc_NB_Rai_Support |
| } |  |  |  |
| phyLayerParameters-v1430 SEQUENCE { |  |  |  |
| multiCarrier-NPRACH-r14 | Checked |  | pc_NB_MultiCarrier_NPRACH |
| twoHARQ-Processes-r14 | Checked |  | pc_NB_TwoHARQ_Processes |
| } |  |  |  |
| rf-Parameters-v1430 SEQUENCE { |  |  |  |
| powerClassNB-14dBm-r14 | Not checked |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| phyLayerParameters-v1440 SEQUENCE { |  |  |  |
| interferenceRandomisation-r14 | Checked |  | pc_NB_InterferenceRandomisation |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| earlyData-UP-r15 | Not checked |  |  |
| rlc-Parameters-r15 SEQUENCE { |  |  |  |
| rlc-UM-r15 | Checked |  | pc_NB_RLC_UM |
| } |  |  |  |
| mac-Parameters-v1530 SEQUENCE { |  |  |  |
| sr-SPS-BSR-r15 | Not checked |  |  |
| } |  |  |  |
| phyLayerParameters-v1530 SEQUENCE { |  |  | pc_SR_WithHARQ_ACK OR pc_SR_WithoutHARQ_ACK OR pc_NB_nprach_Format2 |
| mixedOperationMode-r15 | Checked |  | pc_NB_mixedOperationMode |
| sr-WithHARQ-ACK-r15 | Checked |  | pc_SR_WithHARQ_ACK |
| sr-WithoutHARQ-ACK-r15 | Checked |  | pc_SR_WithoutHARQ_ACK |
| nprach-Format2-r15 | Checked |  | pc_NB_nprach_Format2 |
| additionalTransmissionSIB1-r15 | Not checked |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ntn-Parameters-r17 SEQUENCE { |  |  |  |
| ntn-Connectivity-EPC-r17 | Checked |  | pc_NB_ntn_Connectivity_EPC OR pc_NB_ntn_only_Connectivity_EPC |
| ntn-TA-Report-r17 | Checked |  | pc_NB_ntn_TA_Report |
| ntn-PUR-TimerDelay-r17 | Checked |  | pc_NB_ntn_PUR_TimerEnhancement |
| ntn-OffsetTimingEnh-r17 | Checked |  | pc_NB_ntn_OffsetTimingEnh |
| ntn-ScenarioSupport-r17 | Checked |  | GSO: pc_NB_ntn_GSO_ScenarioSupport NGSO: pc_NB_ntn_NGSO_ScenarioSupport |
| ntn-ScenarioSupport-r17 | Not present |  | GSO AND NGSO: pc_NB_ntn_GSO_ScenarioSupport AND pc_NB_ntn_NGSO_ScenarioSupport |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |