3GPP TSG-RAN WG5 Meeting # 102	R5-240855

# Athens, Greece, February 26 – 01 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0018 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of chapter 4 for RF general description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The general description for the relationship between minimum requirements and test requirements for NTN UE is inaccurate. Furthermore, to align with the core spec, the terms for Geostationary satellite and Geostationary-Satellite Orbit, and the abbreviations for GSO and NGSO should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the general description for the relationship between minimum requirements and test requirements for NTN UE in clause 4.1. - Correct the applicability of minimum requirements for NTN UE in clause 4.2. - Add the terms for Geostationary satellite and Geostationary-Satellite Orbit in clause 3.1. - Add the abbreviations for GSO and NGSO in clause 3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The general description for NTN UE RF will remain inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, 4.1, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Geostationary satellite: A geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth’s equator and which thus remains fixed relative to the Earth; by extension, a geosynchronous satellite which remains approximately fixed relative to the Earth.

Geostationary-Satellite Orbit: The orbit of a geosynchronous satellite whose circular and direct orbit lies in the plane of the Earth's equator.

Geosynchronous Earth Orbit: Earth-centered orbit at approximately 35786 kilometres above Earth's surface and synchronised with Earth's rotation. A geostationary orbit is a non-inclined geosynchronous orbit, i.e. in the Earth's equator plane.

Geosynchronous satellite: An earth satellite whose period of revolution is equal to the period of rotation of the Earth about its axis.

Low Earth Orbit: Orbit around the Earth with an altitude between 300 km, and 1500 km.

Non-terrestrial networks: Networks, or segments of networks, using an airborne or space-borne vehicle to embark a transmission equipment relay node or base station.

Satellite: A space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO), Medium-Earth Orbit (MEO), or Geostationary Earth Orbit (GEO).

Satellite Access Node: see definition in TS 38.108 [4].

UE transmission bandwidth configuration: Set of resource blocks located within the UE channel bandwidth which may be used for transmitting or receiving by the UE.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFGlobal	Granularity of the global frequency raster

ΔFRaster	Band dependent channel raster granularity

BWChannel	Channel bandwidth

BWinterferer	Bandwidth of the interferer

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FInterferer	Frequency of the interferer

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FOOB	The boundary between the NR out of band emission and spurious emission domains

FREF	RF reference frequency

FREF-Offs	Offset used for calculating FREF

Fuw (offset)	The frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NREF-Offs	Offset used for calculating NREF

PInterferer	Modulated mean power of the interferer

Puw	Power of an unwanted DL signal

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

BW	Bandwidth

BWP	Bandwidth Part

CP-OFDM	Cyclic Prefix-OFDM

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

DTX	Discontinuous Transmission

EIRP	Equivalent Isotropically Radiated Power

EVM	Error Vector Magnitude

FR	Frequency Range

FRC	Fixed Reference Channel

GEO	Geosynchronous Earth Orbit

GSCN	Global Synchronization Channel Number

GSO	Geostationary-Satellite Orbit

IBB	In-band Blocking

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LEO	Low Earth Orbiting

MBW	Measurement bandwidth defined for the protected band

MEO	Medium Earth Orbiting

MOP	Maximum Output Power

MPR	Allowed maximum power reduction

MSD	Maximum Sensitivity Degradation

NGEO	Non-Geostationary Earth Orbiting

NGSO 	Non-Geostationary-Satellite Orbit

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

NS	Network Signalling

NTN	Non-Terrestrial Network

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

PRB	Physical Resource Block

QAM	Quadrature Amplitude Modulation

RAN	Radio Access Network

RE	Resource Element

REFSENS	REFerence SENSitivity

RF	Radio Frequency

RMS	Root Mean Square (value)

RSRP	Reference Signal Receive Power

RSRQ	Reference Signal Receive Quality

RX	Receiver

SAN	Satellite Access Node

SC	Single Carrier

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SNR	Signal-to-Noise Ratio

SRS	Sounding Reference Symbol

SS	Synchronization Symbol

TN	Terrestrial Network

TX	Transmitter

TxD	Tx Diversity

UE	User Equipment

4	General

4.1	Relationship between minimum requirements and test requirements

TS 38.101-5 [11] is a Single-RAT specification for satellite NR UE, covering RF characteristics and minimum performance requirements. Conformance to the TS 38.101-5 [11] is demonstrated by fulfilling the test requirements specified in the present document.

The Minimum Requirements given in TS 38.101-5 [11] makes no allowance for measurement uncertainty (MU). The present document defines test tolerances (TT). These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in TS 38.101-5 [11] to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the various levels of "shared risk" principle as described below.

a)	Core specification value is not relaxed by any relaxation value (TT=0). For each single measurement, the probability of a borderline good UE being judged as FAIL equals the probability of a borderline bad UE being judged as PASS.

-	Test tolerances equal to 0 (TT=0) are considered in this specification.

b)	Core specification value is relaxed by a relaxation value (TT>0). For each single measurement, the probability of a borderline bad UE being judged as PASS is greater than the probability of a borderline good UE being judged as FAIL.

-	Test tolerances lower than measurement uncertainty and greater than 0 (0 < TT < MU) are considered in this specification.

-	Test tolerances high up to measurement uncertainty (TT = MU) are considered in this specification which is also known as "Never fail a good DUT" principle.

c)	Core specification value is tightened by a stringent value (TT<0). For each single measurement, the probability of a borderline good UE being judged as FAIL is greater than the probability of a borderline bad UE being judged as PASS.

Test tolerances lower than 0 (TT<0) are not considered in this specification. The "Never fail a good DUT" and the "Shared Risk" principles are defined in Recommendation ITU-R M.1545 [3].

4.2	Applicability of minimum requirements

The Minimum Requirements are specified as general requirements and additional requirements. The applicability of each requirement is described under clauses 6.1, 7.1, 8.1 of TS 38.101-5 [11].

The conducted minimum requirements specified in TS 38.101-5 [11] as a general requirement, the requirement shall be met in all applicable scenarios for FR1. For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty, it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal.

4.3	Specification suffix information

Specification suffix information is not defined for the time being in Release 17.

4.4	Relationship with core specifications

TS 38.101-5 [11] specifies the minimum RF and performance requirements for NR User Equipment (UE) operating in a Non-Terrestrial Network. TS 38.108 [4] specifies the minimum RF and performance requirements of Satellite Access Node (SAN).

==============================================================

<< End of changes >>