3GPP TSG-RAN5 Meeting #102 	R5-241922

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2707 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Rel-15 A-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Table 6.2A.3.0.3-1 refers incorrect clauses and tables, and has incorrect column title. - NOTE 2 in Table 6.2A.3.1.4.1-7 does not match the correction of R5-233186 (implementation error). - Incorrect A-MPR values in Table 6.2D.3.5-10 - Incorrect A-MPR values in Table 6.2D.3.5-11 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected reference and column title in Table 6.2A.3.0.3-1 - Corrected NOTE 2 in Table 6.2A.3.1.4.1-7 according to R5-233186. - Corrected test requirements in Table 6.2D.3.5-10 - Corrected test requirements in Table 6.2D.3.5-11 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR values in the test requirements will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.0.3, 6.2A.3.1, 6.2D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-241100 and the outcome of 1 revision to this document. r1: Corrections to Table 6.2A.3.1.5-1 and Table 6.2A.3.1.5-2 were reversed. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2A.3	UE additional maximum output power reduction for CA

6.2A.3.0	Minimum conformance requirements

<<Unchanged sections skipped>>

6.2A.3.0.3	UE additional maximum output power reduction for inter-band CA

Unless otherwise stated, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.3 apply for each uplink component carrier.

Unless specified in Table 6.2A.3.0.3-1, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply only to the indicated carrier. The requirements in Table 6.2A.3.0.3-1 are specified in terms of an additional spectrum emission requirement with their associated network signalling values and the allowed A-MPR. Unless otherwise stated, the combined requirements and allowed A-MPR are applicable on both bands when both component carriers are active. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet the additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

The emission requirements specified in Table 6.2A.3.0.3-1 also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.2A.3.0.3-1: Additional Requirements for uplink inter-band carrier aggregation (two-bands)

| NR CA combination | Band | Applied NS | Requirements (clause) | A-MPR (table/clause) | Note |
| --- | --- | --- | --- | --- | --- |
|  | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 |  |
| CA_n1-n3 |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1 |
|  | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 |  |
|  | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 |  |
| CA_n1-n8 |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 | 1 |
|  | n8 | 43 | 6.5.3.3.3.5 | Clause 6.2.3.3.6 |  |
|  |  | 43U | 6.5.3.3.3.5, 6.5.2.4.2 | Clause 6.2.3.3.6 |  |
| CA_n1-n78 | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 | 1 |
|  |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 |  |
| CA_n1-n79 | n1 | 05 | 6.5.3.3.3.4 | Clause 6.2.3.3.4 | 1 |
|  |  | 05U | 6.5.3.3.3.4, 6.5.2.4.2 | Clause 6.2.3.3.4 |  |
| CA_n3-n41 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
|  | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| CA_n3-n78 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n5-n77 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n5-n78 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.3.1-2 | 1 |
| CA_n8-n78 | n8 | 43 | 6.5.3.3.3.5 | Clause 6.2.3.3.6 | 1 |
|  |  | 43U | 6.5.3.3.3.5, 6.5.2.4.2 | Clause 6.2.3.3.6 |  |
| CA_n24-n41 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n41 | 04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | Clause 6.2.3.3.2 |  |
| CA_n24-n48 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n48 | 27 | 6.5.2.3.3.8, 6.5.3.3.3.14 | Clause 6.2.3.3.16 |  |
| CA_n24-n77 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n77 | 55 | NOTE 6 | N/A |  |
| CA_n28-n41 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
|  | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| CA_n28-n78 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
| CA_n28-n79 | n28 | 17 | 6.5.3.3.3.2 | N/A | 2 |
| CA_n41-n79 | n41 | 47 | 6.5.3.3.3.15 | Table 6.2.3.3.18-2 |  |
| NOTE 1: NS_05U, NS_43U and NS_100 can be signalled for NR bands that have UTRA services deployed and the requirements in clause 6.5.2.4.2 are only applicable to the signalling carrier. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 3: TBD NOTE 4: TBD NOTE 5: TBD NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. |  |  |  |  |  |

6.2A.3.1	UE additional maximum output power reduction for CA (2UL CA)

Editor’s note: This clause is complete for AMPR testing. But the following aspects are either missing for not yet determined:

FFS is left in Test applicability since there are no requirements for 6.5A.2.3 Additional Spectrum Emission mask for CA and 6.5A.3.3 Additional Spurious Emissions for CA in In Release-15.

6.2A.3.1.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the CA maximum output power as specified in Table 6.2A.1.0.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2. Outer and inner allocation notation used in clause 6.2A.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2A.3.1.2	Test applicability

The requirements of this test apply in test case 6.5A.2.3 Additional Spectrum Emission mask for CA for network signalling values FFS to all types of NR UE release 15 and forward.

The requirements of this test apply in test case 6.5A.3.3 Additional Spurious Emissions for CA for network signalling values FFS to all types of NR UE release 15 and forward.

6.2A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in 6.2A.3.0.

6.2A.3.1.4	Test description

6.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2A.3.0.3-1, the power limits specified in subclause 6.2A.4.1.3 apply.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 6.2A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

<<Unchanged tables skipped>>

Table 6.2A.3.1.4.1-7: Test Configuration Table for intra-band contiguous CA for CA_NS_04 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Refering to Test Frequency Column |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Test Frequency | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation for PCC&SCC (NOTE 1) |
| 1 | High range |  | CP-OFDM QPSK | Outer_Full |
| 2 | High range |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Low range |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low range |  | CP-OFDM QPSK | Edge_1RB_Left |
| 5 | Low range |  | DFT-s-OFDM Pi/2 BPSK | Inner_Full |
| 6 | Low range |  | DFT-s-OFDM Pi/2 QPSK | Inner_Full |
| 7 | Low range |  | DFT-s-OFDM Pi/2 16QAM | Inner_Full |
| 8 | Low range |  | DFT-s-OFDM Pi/2 64QAM | Inner_Full |
| 9 | Low range |  | DFT-s-OFDM Pi/2 256QAM | Inner_Full |
| 10 | Low range |  | CP-OFDM QPSK | Inner_Full |
| 11 | Low range |  | CP-OFDM 16QAM | Inner_Full |
| 12 | Low range |  | CP-OFDM 64QAM | Inner_Full |
| 13 | Low range |  | CP-OFDM 256QAM | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

<<Unchanged sections skipped>>

6.2D.3	UE additional maximum output power reduction for UL MIMO

Editor’s Note:

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U.

<<Unchanged sections skipped>>

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

<<Unchanged tables skipped>>

Table 6.2D.3.5-10: UE Power Class 3 test requirements (NS_03/NS_03U) for band n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

<<Unchanged tables skipped>>

Table 6.2D.3.5-11: UE Power Class 3 test requirements (NS_03/NS_03U) for band n2, n25

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 3 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 4,5 | NS_03, NS_03U | 23 | 3 | 4 | 1.5 | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 6 | NS_03, NS_03U | 23 | 3 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 7,8 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 1.5 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | NS_03, NS_03U | 23 | 3.5 | 4.5 | 0 | 18.5 | 5 | 3 | 25+TT | 13.5-TT |
| 10,11 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 1.5 | 14 | 6 | 3 | 25+TT | 8-TT |
| 12 | NS_03, NS_03U | 23 | 6.5 | 7.5 | 0 | 15.5 | 6 | 3 | 25+TT | 9.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

<<End of change>>