3GPP TSG-RAN5 Meeting #103	R5-242819

# Fukuoka, Japan, 20th May 2024 – 24th May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4418 | rev |  | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR eCall TC 11.5.5-Limited service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In this TC, UE register on an acceptable cell,after the emergency PDU session released in step 5 (i.e emergency services are no longer required), UE may perform de-registration immediatedly which is unexpected by SS. [24.501 5.5.2.1] When upper layers indicate that emergency services are no longer required, the UE if still registered for emergency services, may perform UE-initiated de-registration procedure followed by a re-registration to regain normal services, if the UE is in or moves to a suitable cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.PDU session release procedure in Step5A/5B is listed in the specific test procedure instead of embeding it in the common procedure TS 38.508-1 [4], subclause 4.9.12B 2.Add note so that SS allowed UE to send de-registration request immediatedly after step5B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail this test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.5.5	eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted / 5GS

11.5.5.1	Test Purpose (TP)

(1)

with { UE in eCALL-INACTIVE.LIMITED-SERVICE state }

ensure that {

when { UE is requested to make an eCall to URI for test service }

then { UE does not transmit any RRCSetupRequest message }

}

(2)

with { UE in eCALL-INACTIVE.LIMITED-SERVICE state }

ensure that {

when { UE is requested to make a manual eCall }

then { UE transmits a REGISTRATION REQUEST message with registration type set to “emergency” }

}

11.5.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122 [38], subclause 2

[TS 23.122 clause 2]

If the MS is in eCall only mode, it attempts to camp on a suitable cell and enters an "eCall inactive" state in which it can only attempt an eCall over IMS, or a call to a non-emergency MSISDN or URI for test or terminal reconfiguration services as specified in 3GPP TS 31.102 [40].

If the MS is in eCall only mode and is unable to find a suitable cell to camp on, it attempts to camp on an acceptable cell in limited service state, and enters an "eCall inactive" state in which it can only attempt an eCall over IMS.

11.5.5.3	Test description

11.5.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	The PLMN is defined in Table 11.5.5.3.1-1.

Table 11.5.5.3.1–1: PLMN identifier

| NR Cell | PLMN name | MCC / MNC |
| --- | --- | --- |
| 1 | PLMN4 | 004 / 31 |

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-26.

Preamble:

-	 The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2.

11.5.5.3.2	Test procedure sequence

Table 11.5.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Wait 60s for the UE to camp on NR Cell 1 as an acceptable cell. | - | - | - | - |
| 3 | A manual eCall is initiated. (Note 1) | - | - | - | - |
| 4A-4B | Steps 1-2 of test procedure 4.9.12 in TS 38.508-1 [4] are performed. | - | - | - | - |
| 4C | Check: Does the UE transmit an RRCSetupComplete message and a REGISTRATION REQUEST message with ‘Registration type' set to 'emergency’? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 2 | P |
| 4D-4O | Steps 4-15 of test procedure 4.9.12 in TS 38.508-1 [4] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 4P-4R below the events specified Table 11.5.5.3.2-2 take place. | - | - | - | - |
| 4P-4R | Steps 16-18 of test procedure 4.9.12 in TS 38.508-1 [4] are performed. | - | - | - | - |
| 5 | Step 1-2 of test procedure for IMS MT Emergency call release is performed as specified in TS 38.508-1 [4], subclause 4.9.12B on NR Cell 1. | - | - | - | - |
| 5A | The SS transmits a PDU SESSION RELEASE COMMAND message. | <-- | NR RRC: DLInformationTransfer 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMMAND | - | - |
| 5B | The UE transmit a PDU SESSION RELEASE COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION RELEASE COMPLETE | - | - |
| 6 | Void | - | - | - | - |
| 7 | The SS releases the RRC connection. | <-- | NR RRC: RRCRelease | - | - |
| 8 | Wait for 5 seconds | - | - | - | - |
| 9 | An eCall to URI for test service is initiated. (Note 2) | - | - | - | - |
| 10 | Check: Does UE send an RRCConnectionRequest on NR Cell 1 within 120 seconds? | --> | NR RRC: RRCSetupRequest | 1 | F |
| Note 1: The request to originate a manual eCall may be performed by MMI or AT command. Note 2: The request to originate an eCall to URI for test service may be performed by MMI or AT command. Note 3: The UE may optionally transmit a De-registration REQ during 5B – 7. |  |  |  |  |  |

Table 11.5.5.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1-3 | Steps 1-3 of generic test procedure for setting up eCall as defined in TS 34.229-5 [47] annex A.23 are performed. | - | - | - | - |

11.5.5.3.3	Specific message contents

Table 11.5.5.3.3-1: SIB1 for NR Cell 1 (All steps, Table 11.5.5.3.2-1)

Derivation path: TS 38.508-1 [4] table 4.6.1-28 Condition eCalloverIMSforNR

Table 11.5.5.3.3-2: REGISTRATION REQUEST (step 4C, Table 11.5.5.3.2-1)

Derivation Path: Table 4.7.1-6, condition EMERGENCY.

Table 11.5.5.3.3-3: INVITE (step 1, Table 11.5.5.3.2-2)

Derivation Path: Step 1 in Annex A.23 TS 34.229-5 [47], Condition A6

<End of modified section 1>

<Start of modified section 2>

<End of modified section 2>