3GPP TSG-RAN5 Meeting #104	R5-244394

# Maastricht, Netherlands, 19 - 23 Aug 2024

Title:							Discussion on FR2 UE Beamlock test function

Source:			Nokia

Agenda Item:			4.2.3

Document for:			Discussion

# 1.	Background

The UE Beamlock test function was specified in TS 38.509 to make the UE locking the UE TX and/or RX beam(s). RAN4 is not clear if current Beamlock test function can support locking two TX beams from two panels simultaneously. For transmission with multi-panel (STxMP) measurement, RAN4 agreed it is necessary to make sure UE Beamlock test function can lock UE two Tx beams from two panels simultaneously [1].

In the LS [1] RAN4 ask how to extend the current FR2 UE Beamlock test function to lock UE two Tx beams from two panels simultaneously.

This document provides our view and proposal on the question in RAN4 LS [1].

# 2	Discussion

# 2.1	Related Work and Study Items

Under the ongoing Release 19 study item on NR FR2 OTA (Over the Air) testing enhancement Phase 3, RAN4 sent a LS to RAN5 [1] to check whether and how to extend the current FR2 UE Beamlock test function to lock UE two Tx beams from two panels simultaneously.

On the other hand, UE need to lock two Rx beams from two panels simultaneously for OTA testing of the Release 18 work item on Requirement for NR FR2 multi-Rx chain DL reception [2].

Several enhancements to enable efficient and robust DL multi-TRP/panel operation were introduced in the Rel-16 NR eMIMO WI. For instance, DL transmission schemes with simultaneous and non-simultaneous multi-beam reception from multiple TRPs/panels were introduced. The simultaneous reception may require support of simultaneous multi-panel operation with several independent RX beams/chains at the UE side. As part of this item, a new FR2 UE capability for simultaneous multi-beam reception was introduced (simultaneousReceptionDiffTypeD-r16). However, no RF, RRM or performance requirements were defined in Rel-16 and Rel-17 for FR2 UEs with simultaneousReceptionDiffTypeD-r16 capability. Enhanced NR FR2 UEs with multi-beam simultaneous reception and multiple RX chains can provide a meaningful performance improvement in FR2 improving both demodulation performance (4-layer DL MIMO), RRM performance and improve RF spherical coverage. This work item aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier to achieve improved RF, RRM and UE demodulation performance.

# 2.2	UE special conformance testing functions

To support 5GS conformance testing, UE special conformance test functions are required. They form a part of the core requirements and thus have a direct impact on the design of the UE. The UE special conformance test functions vary depending on the conformance testing functionality they are designed to support and are broadly classified into the following two groups:

- Test Loop Functions: Functions which require a loop to be established between the UE and the System Simulator (SS) to allow e.g., DL data packets sent by the SS to be looped back UL by the UE

- General Test Functions: Commands send by the SS e.g., to trigger a certain UE behavior which may be a behavior determined by 3GPP core spec requirements or such needed to facilitate conformance testing and not being part of any 3GPP core spec requirements, or, to provide to the UE information needed for the conformance testing.

The utilization of any UE special conformance test functions shall be considered as putting the UE in a test mode. TS 38.509 defines the UE special conformance test functions including any relevant procedure and the Test Mode Control (TMC) message contents used for information exchange.

The TMC messages for UE Beamlock Test Function (UBF) are defined in TS 38.509 [3].

The default TMC messages for UBF and test procedures to activate / deactivate UBF are defined in TS 38.508-1 [4].

# 2.3	Proposed solution

The current UBF only support locking ONE antenna pattern once it has formed ONE beam. Therefore, the UBF need to be extended to support locking at least two beams to fulfil the request in RAN4 LS [1].

Note that only bits 1 and 2 in the octet of ‘UE Beamlock test Function’ IE are currently used to set whether ‘Tx’ or ‘Rx’ or both ‘Tx and Rx’ antenna pattern forming the beam need to be locked by the UE. Up to 3 Tx and 8 Rx beams have been specified in the current 3GPP standards for UE and different number of Tx and Rx beams may be used in the UE according to the feature being tested, e.g., multi-Tx and /or multi-Rx, which need to be locked by the UE for OTA testing.

Our proposal is to set independently the number of Tx antenna patterns forming the Tx beams and the number of Rx antenna patterns forming the Rx beams to be locked by the UE. Below we propose two alternative solutions.

Alternative A:

We propose an alternative to independently set the number of Tx antenna patterns forming the Tx beams and the number of Rx antenna patterns forming the Rx beams to be locked by the UE as following:

- Use 2 bits (e.g., bits 3 and 4) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 3 (4 possible combinations) to set the number of Tx antenna patterns forming the Tx beams to be locked.

- Use 3 bits (e.g., bits 3 to 5) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 7 (8 possible combinations) to set the number of Tx antenna patterns forming the Tx beams to be locked.

- Use 3 bits (e.g., bits 5 to 7 or bits 6 to 8) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 7 (8 possible combinations) to set the number of Rx antenna patterns forming the Rx beams to be locked.

- Use 1 bit (e.g., bit 8) in the octet of ‘UE Beamlock test Function’ IE to set all active Tx and / or Rx antenna patterns forming the Tx and / or Rx beams to be locked by the UE.

Alternative B:

As more than one beams can be formed by the same antenna patterns, we propose another alternative to independently set the number of Tx beams formed by the Tx antenna pattern(s) and the number of Rx beams formed by the Rx antenna pattern(s) to be locked by the UE as following:

- Use 2 bits (e.g., bits 3 and 4) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 3 (4 possible combinations) to set the number of Tx beams formed by the Tx antenna pattern(s) to be locked.

- Use 3 bits (e.g., bits 3 and 5) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 7 (8 possible combinations) to set the number of Tx beams formed by the Tx antenna pattern(s) to be locked.

- Use 3 bits (e.g., bits 5 to 7 or bits 6 to 8) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 7 (8 possible combinations) to set the number of Rx beams formed by the Rx antenna pattern(s) to be locked.

- Use 1 bit (e.g., bit 8) in the octet of ‘UE Beamlock test Function’ IE to set all active Tx and / or Rx beams formed by the Tx and / or Rx antenna pattern(s) to be locked by the UE.

Note that bits 1 and 2 in the octet of ‘UE Beamlock test Function’ IE shall continue to be used to set whether ‘Tx’ or ‘Rx’ or both ‘Tx and Rx’ antenna pattern forming the beam need to be locked by the UE, to ensure no backward compatibility issue for legacy UE.

# 2.4	Proposed changes to TS 38.508-1

Proposal can be implemented as new default TMC messages and test procedures into TS 38.508-1 as following. Modifications are shown by track changes for the current text. Below we propose two alternative solutions.

Alternative A:

### 4.7A.3 Beamlock messages #### - ACTIVATE BEAMLOCK This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message. Table 4.7A.3-1: ACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.1. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 0. UE Beamlock test Function; x x x x x x 0 1; Tx Only. UE Beamlock test Function; x x x x x x 1 0; Rx Only. UE Beamlock test Function; x x x x x x 1 1; Tx and Rx. UE Beamlock test Function; x x x x 0 0 x x; Best with Tx. UE Beamlock test Function; x x x x 0 1 x x; Best 2 with Tx. UE Beamlock test Function; x x x x 1 0 x x; Best 3 with Tx. UE Beamlock test Function; x x x x 1 1 x x; Best 4 with Tx. UE Beamlock test Function; x 0 0 0 x x x x; Best with Rx. UE Beamlock test Function; x 0 0 1 x x x x; Best 2 with Rx. UE Beamlock test Function; x 0 1 0 x x x x; Best 3 with Rx. UE Beamlock test Function; x 0 1 1 x x x x; Best 4 with Rx. UE Beamlock test Function; x 1 0 0 x x x x; Best 5 with Rx. UE Beamlock test Function; x 1 0 1 x x x x; Best 6 with Rx. UE Beamlock test Function; x 1 1 0 x x x x; Best 7 with Rx. UE Beamlock test Function; x 1 1 1 x x x x; Best 8 with Rx. UE Beamlock test Function; 0 x x x x x x x; Lock selected. UE Beamlock test Function; 1 x x x x x x x; All. Condition; Explanation. Tx Only; Activation UE beamlock function for Tx only. Rx Only; Activation UE beamlock function for Rx only. Tx and Rx; Activation UE beamlock function for both Tx and Rx. Best with Tx; Lock the best UE antenna pattern with Tx. Best 2 with Tx; Lock the best two UE antenna pattern with Tx. Best 3 with Tx; Lock the best three UE antenna pattern with Tx. Best 4 with Tx; Lock the best four UE antenna pattern with Tx. Best with Rx; Lock the best UE antenna pattern with Rx. Best 2 with Rx; Lock the best two UE antenna pattern with Rx. Best 3 with Rx; Lock the best three UE antenna pattern with Rx. Best 4 with Rx; Lock the best four UE antenna pattern with Rx. Best 5 with Rx; Lock the best five UE antenna pattern with Rx. Best 6 with Rx; Lock the best six UE antenna pattern with Rx. Best 7 with Rx; Lock the best seven UE antenna pattern with Rx. Best 8 with Rx; Lock the best eight UE antenna pattern with Rx. Lock selected; Lock the UE antenna patterns according to bits 7 to 3. All; Lock all active UE antenna pattern(s) with Tx or Rx or TxRx. #### - ACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message. Table 4.7A.3-2: ACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.2. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 1.

#### - DEACTIVATE BEAMLOCK This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message. Table 4.7A.3-3: DEACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.3. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 0. #### - DEACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message. Table 4.7A.3-4: DEACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.4. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 1.

### 4.9.2 Test procedure to activate UE Beamlock Test Function (UBF) 4.9.2.1 Initiation UE is operating in FR2 in RRC_CONNECTED state. 4.9.2.2 Procedure Table 4.9.2.2-1: Test procedure Sequence St; Procedure; Message Sequence; TP; Verdict. U - S; Message/PDU/SDU. 1; SS request UE to activate UE beamlock function.; <--; ACTIVATE BEAMLOCK; -; -. 2; UE confirms that UE beamlock function is activated; -->; ACTIVATE BEAMLOCK COMPLETE; -; -. 4.9.2.3 Specific Message contents Table 4.9.2.3-1: ACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.1. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 0. UE Beamlock test Function; x x x x x x 0 1; Tx Only. UE Beamlock test Function; x x x x x x 1 0; Rx Only. UE Beamlock test Function; x x x x x x 1 1; Tx and Rx. UE Beamlock test Function; x x x x 0 0 x x; Best with Tx. UE Beamlock test Function; x x x x 0 1 x x; Best 2 with Tx. UE Beamlock test Function; x x x x 1 0 x x; Best 3 with Tx. UE Beamlock test Function; x x x x 1 1 x x; Best 4 with Tx. UE Beamlock test Function; x 0 0 0 x x x x; Best with Rx. UE Beamlock test Function; x 0 0 1 x x x x; Best 2 with Rx. UE Beamlock test Function; x 0 1 0 x x x x; Best 3 with Rx. UE Beamlock test Function; x 0 1 1 x x x x; Best 4 with Rx. UE Beamlock test Function; x 1 0 0 x x x x; Best 5 with Rx. UE Beamlock test Function; x 1 0 1 x x x x; Best 6 with Rx. UE Beamlock test Function; x 1 1 0 x x x x; Best 7 with Rx. UE Beamlock test Function; x 1 1 1 x x x x; Best 8 with Rx. UE Beamlock test Function; 0 x x x x x x x; Lock selected. UE Beamlock test Function; 1 x x x x x x x; All.

Condition; Explanation. Tx Only; Activation UE beamlock function for Tx only. Rx Only; Activation UE beamlock function for Rx only. Tx and Rx; Activation UE beamlock function for both Tx and Rx. Best with Tx; Lock the best UE antenna pattern with Tx. Best 2 with Tx; Lock the best two UE antenna pattern with Tx. Best 3 with Tx; Lock the best three UE antenna pattern with Tx. Best 4 with Tx; Lock the best four UE antenna pattern with Tx. Best with Rx; Lock the best UE antenna pattern with Rx. Best 2 with Rx; Lock the best two UE antenna pattern with Rx. Best 3 with Rx; Lock the best three UE antenna pattern with Rx. Best 4 with Rx; Lock the best four UE antenna pattern with Rx. Best 5 with Rx; Lock the best five UE antenna pattern with Rx. Best 6 with Rx; Lock the best six UE antenna pattern with Rx. Best 7 with Rx; Lock the best seven UE antenna pattern with Rx. Best 8 with Rx; Lock the best eight UE antenna pattern with Rx. Lock selected; Lock the UE antenna patterns according to bits 7 to 3. All; Lock all active UE antenna pattern(s) with Tx or Rx or TxRx. Table 4.9.2.3-2: ACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.2. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 1. ### 4.9.3 Test procedure to deactivate UE Beamlock Test Function (UBF) 4.9.3.1 Initiation UE is operating in FR2 in RRC_CONNECTED state with UE beamlock test function activated. 4.9.3.2 Procedure Table 4.9.3.2-1: Test procedure Sequence St; Procedure; Message Sequence; TP; Verdict. U - S; Message/PDU/SDU. 1; SS request UE to deactivate UE beamlock function.; <--; DEACTIVATE BEAMLOCK; -; -. 2; UE confirms that UE beamlock function is activated; -->; DEACTIVATE BEAMLOCK COMPLETE; -; -. 4.9.3.3 Specific Message contents Table 4.9.3.3-1: DEACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.3. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 0. Table 4.9.3.3-2: DEACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.4. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 1.

Alternative B:

### 4.7A.3 Beamlock messages #### - ACTIVATE BEAMLOCK This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message. Table 4.7A.3-1: ACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.1. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 0. UE Beamlock test Function; x x x x x x 0 1; Tx Only. UE Beamlock test Function; x x x x x x 1 0; Rx Only. UE Beamlock test Function; x x x x x x 1 1; Tx and Rx. UE Beamlock test Function; x x x x 0 0 x x; Best with Tx. UE Beamlock test Function; x x x x 0 1 x x; Best 2 with Tx. UE Beamlock test Function; x x x x 1 0 x x; Best 3 with Tx. UE Beamlock test Function; x x x x 1 1 x x; Best 4 with Tx. UE Beamlock test Function; x 0 0 0 x x x x; Best with Rx. UE Beamlock test Function; x 0 0 1 x x x x; Best 2 with Rx. UE Beamlock test Function; x 0 1 0 x x x x; Best 3 with Rx. UE Beamlock test Function; x 0 1 1 x x x x; Best 4 with Rx. UE Beamlock test Function; x 1 0 0 x x x x; Best 5 with Rx. UE Beamlock test Function; x 1 0 1 x x x x; Best 6 with Rx. UE Beamlock test Function; x 1 1 0 x x x x; Best 7 with Rx. UE Beamlock test Function; x 1 1 1 x x x x; Best 8 with Rx. UE Beamlock test Function; 0 x x x x x x x; Lock selected. UE Beamlock test Function; 1 x x x x x x x; All. Condition; Explanation. Tx Only; Activation UE beamlock function for Tx only. Rx Only; Activation UE beamlock function for Rx only. Tx and Rx; Activation UE beamlock function for both Tx and Rx. Best with Tx; Lock the best beam formed by the UE antenna pattern with Tx. Best 2 with Tx; Lock the best two beams formed by the UE antenna pattern(s) with Tx. Best 3 with Tx; Lock the best three beams formed by the UE antenna pattern(s) with Tx. Best 4 with Tx; Lock the best four beams formed by the UE antenna pattern(s) with Tx. Best with Rx; Lock the best beam formed by the UE antenna pattern with Rx. Best 2 with Rx; Lock the best two beams formed by the UE antenna pattern(s) with Rx. Best 3 with Rx; Lock the best three beams formed by the UE antenna pattern(s) with Rx. Best 4 with Rx; Lock the best four beams formed by the UE antenna pattern(s) with Rx. Best 5 with Rx; Lock the best five beams formed by the UE antenna pattern(s) with Rx. Best 6 with Rx; Lock the best six beams formed by the UE antenna pattern(s) with Rx. Best 7 with Rx; Lock the best seven beams formed by the UE antenna pattern(s) with Rx. Best 8 with Rx; Lock the best eight beams formed by the UE antenna pattern(s) with Rx. Lock selected; Lock the beam(s) formed by the UE antenna pattern(s) according to bits 7 to 3. All; Lock all active beam(s) formed by the UE antenna pattern(s) with Tx or Rx or TxRx. #### - ACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message. Table 4.7A.3-2: ACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.2. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 1.

#### - DEACTIVATE BEAMLOCK This message is only sent in the direction SS to UE, embedded in a RRC DLInformationTransfer message. Table 4.7A.3-3: DEACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.3. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 0. #### - DEACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS, embedded in a RRC ULInformationTransfer message. Table 4.7A.3-4: DEACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.4. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 1.

### 4.9.2 Test procedure to activate UE Beamlock Test Function (UBF) 4.9.2.1 Initiation UE is operating in FR2 in RRC_CONNECTED state. 4.9.2.2 Procedure Table 4.9.2.2-1: Test procedure Sequence St; Procedure; Message Sequence; TP; Verdict. U - S; Message/PDU/SDU. 1; SS request UE to activate UE beamlock function.; <--; ACTIVATE BEAMLOCK; -; -. 2; UE confirms that UE beamlock function is activated; -->; ACTIVATE BEAMLOCK COMPLETE; -; -. 4.9.2.3 Specific Message contents Table 4.9.2.3-1: ACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.1. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 0. UE Beamlock test Function; x x x x x x 0 1; Tx Only. UE Beamlock test Function; x x x x x x 1 0; Rx Only. UE Beamlock test Function; x x x x x x 1 1; Tx and Rx. UE Beamlock test Function; x x x x 0 0 x x; Best with Tx. UE Beamlock test Function; x x x x 0 1 x x; Best 2 with Tx. UE Beamlock test Function; x x x x 1 0 x x; Best 3 with Tx. UE Beamlock test Function; x x x x 1 1 x x; Best 4 with Tx. UE Beamlock test Function; x 0 0 0 x x x x; Best with Rx. UE Beamlock test Function; x 0 0 1 x x x x; Best 2 with Rx. UE Beamlock test Function; x 0 1 0 x x x x; Best 3 with Rx. UE Beamlock test Function; x 0 1 1 x x x x; Best 4 with Rx. UE Beamlock test Function; x 1 0 0 x x x x; Best 5 with Rx. UE Beamlock test Function; x 1 0 1 x x x x; Best 6 with Rx. UE Beamlock test Function; x 1 1 0 x x x x; Best 7 with Rx. UE Beamlock test Function; x 1 1 1 x x x x; Best 8 with Rx. UE Beamlock test Function; 0 x x x x x x x; Lock selected. UE Beamlock test Function; 1 x x x x x x x; All.

Condition; Explanation. Tx Only; Activation UE beamlock function for Tx only. Rx Only; Activation UE beamlock function for Rx only. Tx and Rx; Activation UE beamlock function for both Tx and Rx. Best with Tx; Lock the best beam formed by the UE antenna pattern with Tx. Best 2 with Tx; Lock the best two beams formed by the UE antenna pattern(s) Tx. Best 3 with Tx; Lock the best three beams formed by the UE antenna pattern(s) with Tx. Best 4 with Tx; Lock the best four beams formed by the UE antenna pattern(s) with Tx. Best with Rx; Lock the best beam formed by the UE antenna pattern with Rx. Best 2 with Rx; Lock the best two beams formed by the UE antenna pattern(s) with Rx. Best 3 with Rx; Lock the best three beams formed by the UE antenna pattern(s) with Rx. Best 4 with Rx; Lock the best four beams formed by the UE antenna pattern(s) with Rx. Best 5 with Rx; Lock the best five beams formed by the UE antenna pattern(s) with Rx. Best 6 with Rx; Lock the best six beams formed by the UE antenna pattern(s) with Rx. Best 7 with Rx; Lock the best seven beams formed by the UE antenna pattern(s) with Rx. Best 8 with Rx; Lock the best eight beams formed by the UE antenna pattern(s) with Rx. Lock selected; Lock the beam(s) formed by the UE antenna pattern(s) according to bits 7 to 3. All; Lock all active beam(s) formed by the UE antenna pattern(s) with Tx or Rx or TxRx. Table 4.9.2.3-2: ACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.2. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 0 1. ### 4.9.3 Test procedure to deactivate UE Beamlock Test Function (UBF) 4.9.3.1 Initiation UE is operating in FR2 in RRC_CONNECTED state with UE beamlock test function activated. 4.9.3.2 Procedure Table 4.9.3.2-1: Test procedure Sequence St; Procedure; Message Sequence; TP; Verdict. U - S; Message/PDU/SDU. 1; SS request UE to deactivate UE beamlock function.; <--; DEACTIVATE BEAMLOCK; -; -. 2; UE confirms that UE beamlock function is activated; -->; DEACTIVATE BEAMLOCK COMPLETE; -; -. 4.9.3.3 Specific Message contents Table 4.9.3.3-1: DEACTIVATE BEAMLOCK Derivation Path: 38.509 clause 6.4.3. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 0. Table 4.9.3.3-2: DEACTIVATE BEAMLOCK COMPLETE Derivation Path: 38.509 clause 6.4.4. Information Element; Value/remark; Comment; Condition. Protocol discriminator; 1 1 1 1. Skip indicator; 0 0 0 0. Message type; 1 0 1 0 0 0 1 1.

# 2.5	Proposed changes to TS 38.509

Proposal can be implemented as new test procedures and TMC messages into TS 38.509 as following. Modifications are shown by track changes for the current text. Below we propose two alternative solutions.

Alternative A:

## 5.4 UE Beamlock test Function (UBF) ### 5.4.1 General The UE Beamlock test function is intended for making the UE to lock the UE antenna pattern(s) once it has formed beam(s) towards the base station (SS) direction(s) following the cell identification procedure in preparation for subsequent test procedures. Activating the UBF shall lock the antenna pattern(s) of all active intra-band component carriers and all MIMO layers affected by the test function. The Beamlock test function is mandatory for applicable UEs operating in Frequency Range 2 (FR2). The SS uses the UE Beamlock test mode activation procedure to command the UE to lock the UE antenna pattern(s). The Beamlock activation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently. Figure 5.4.1-1: UE Beamlock test mode activation procedure The SS uses the UE Beamlock test mode deactivation procedure to command the UE to re-tracking the beam(s) towards the base station direction(s). The Beamlock deactivation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently. Figure 5.4.1-2: UE Beamlock test mode deactivation procedure ### 5.4.2 Activate Beamlock procedure #### 5.4.2.1 Initiation The SS requests the UE to activate beamlock by transmitting an ACTIVATE BEAMLOCK message. #### 5.4.2.2 Reception of ACTIVATE BEAMLOCK message by UE When UE receives ACTIVATE BEAMLOCK message then the UE shall: 1> if the UE is operating in FR2 AND is in RRC_CONNECTED state: 2> if bits 2 and 1 in UE Beamlock test Function = 01 3> Lock the UE antenna pattern with Tx only 2> else if bits 2 and 1 in UE Beamlock test Function = 10 3> Lock the UE antenna pattern with Rx only 2> else if bits 2 and 1 in UE Beamlock test Function = 11 3> Lock the UE antenna pattern with both TxRx 2> if bits 4 and 3 in UE Beamlock test Function = 00 3> Lock the best UE antenna pattern with Tx 2> else if bits 4 and 3 in UE Beamlock test Function = 01 3> Lock the best two UE antenna patterns with Tx 2> else if bits 4 and 3 in UE Beamlock test Function = 10 3> Lock the best three UE antenna patterns with Tx 2> else if bits 4 and 3 in UE Beamlock test Function = 11 3> Lock the best four UE antenna patterns with Tx 2> if bits 7, 6 and 5 in UE Beamlock test Function = 000 3> Lock the best UE antenna pattern with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 001 3> Lock the best two UE antenna patterns with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 010 3> Lock the best three UE antenna patterns with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 011 3> Lock the best four UE antenna patterns with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 100 3> Lock the best five UE antenna patterns with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 101 3> Lock the best six UE antenna patterns with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 110 3> Lock the best seven UE antenna patterns with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 111 3> Lock the best eight UE antenna patterns with Rx 2> if bit 8 in UE Beamlock test Function = 0 3> Lock the UE antenna patterns according to bits 7 to 3 2> else if bit 8 in UE Beamlock test Function = 1 3> Lock all active UE antenna pattern(s) with Tx or Rx or TxRx 2> Transmit ACTIVATE BEAMLOCK COMPLETE message 1> else: 2> the UE behaviour is unspecified. ### 5.4.3 Deactivate Beamlock procedure #### 5.4.3.1 Initiation The SS requests the UE to deactivate beamlock by transmitting a DEACTIVATE BEAMLOCK message. The SS should do this when the UE is in RRC_CONNECTED state. #### 5.4.3.2 Reception of DEACTIVATE BEAMLOCK message by UE When UE receives DEACTIVATE BEAMLOCK message then the UE shall: 1> if the UE is operating in FR2 AND is in RRC_CONNECTED state AND the UE Beamlock test function is active: 2> unlock all active UE antenna pattern(s) and transmit DEACTIVATE BEAMLOCK COMPLETE message; 1> else: 2> the UE behaviour is unspecified. #### 5.4.3.3 Release of antenna beamlock by UE When the UE leaves the RRC_CONNECTED state, the UE shall: 1> if the UE is operating in FR2 AND the UE Beamlock test Function is active 2> unlock all active UE antenna pattern(s);

## 6.4 Beamlock messages ### 6.4.1 ACTIVATE BEAMLOCK This message is only sent in the direction SS to UE. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. UE Beamlock test Function; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 0; 0; octet 1. where UE Beamlock test Function is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. A1; R1; R2; R3; T1; T2; X1; X2; octet 1. where X1,X2 = 01 for activate beamlock of Tx only, 10 for activate beamlock of Rx only and 11 for activate beamlock of both TxRx, where T1,T2 = 00 for activate beamlock of best Tx antenna pattern, 01 for activate beamlock of best two Tx antenna patterns, 10 for activate beamlock of best three Tx antenna patterns and 11 for activate beamlock of best four Tx antenna patterns, where R1,R2,R3 = 000 for activate beamlock of best Rx antenna pattern, 001 for activate beamlock of best two Rx antenna patterns, 010 for activate beamlock of best three Rx antenna patterns and 011 for activate beamlock of best four Rx antenna patterns, 100 for activate beamlock of best five Rx antenna pattern, 101 for activate beamlock of best six Rx antenna patterns, 110 for activate beamlock of best seven Rx antenna patterns and 111 for activate beamlock of best eight Rx antenna patterns, where A1 = 0 for activate beamlock of according to bits 7 to 3, 1 for activate beamlock of all active Tx/Rx/TxRx antenna patterns. NOTE: X1,X2 = 00 is not used ### 6.4.2 ACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 0; 1; octet 1. ### 6.4.3 DEACTIVATE BEAMLOCK This message is only sent in the direction SS to UE. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 1; 0; octet 1. ### 6.4.4 DEACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 1; 1; octet 1.

Alternative B:

## 5.4 UE Beamlock test Function (UBF) ### 5.4.1 General The UE Beamlock test function is intended for making the UE to lock the UE antenna pattern(s) once it has formed beam(s) towards the base station (SS) direction(s) following the cell identification procedure in preparation for subsequent test procedures. Activating the UBF shall lock the antenna pattern(s) of all active intra-band component carriers and all MIMO layers affected by the test function. The Beamlock test function is mandatory for applicable UEs operating in Frequency Range 2 (FR2). The SS uses the UE Beamlock test mode activation procedure to command the UE to lock the UE antenna pattern(s). The Beamlock activation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently. Figure 5.4.1-1: UE Beamlock test mode activation procedure The SS uses the UE Beamlock test mode deactivation procedure to command the UE to re-tracking the beam(s) towards the base station direction(s). The Beamlock deactivation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently. Figure 5.4.1-2: UE Beamlock test mode deactivation procedure ### 5.4.2 Activate Beamlock procedure #### 5.4.2.1 Initiation The SS requests the UE to activate beamlock by transmitting an ACTIVATE BEAMLOCK message. #### 5.4.2.2 Reception of ACTIVATE BEAMLOCK message by UE When UE receives ACTIVATE BEAMLOCK message then the UE shall: 1> if the UE is operating in FR2 AND is in RRC_CONNECTED state: 2> if bits 2 and 1 in UE Beamlock test Function = 01 3> Lock the UE antenna pattern with Tx only 2> else if bits 2 and 1 in UE Beamlock test Function = 10 3> Lock the UE antenna pattern with Rx only 2> else if bits 2 and 1 in UE Beamlock test Function = 11 3> Lock the UE antenna pattern with both TxRx 2> if bits 4 and 3 in UE Beamlock test Function = 00 3> Lock the best beam formed by the UE antenna pattern with Tx 2> else if bits 4 and 3 in UE Beamlock test Function = 01 3> Lock the best two beams formed by the UE antenna pattern(s) with Tx 2> else if bits 4 and 3 in UE Beamlock test Function = 10 3> Lock the best three beams formed by the UE antenna pattern(s) with Tx 2> else if bits 4 and 3 in UE Beamlock test Function = 11 3> Lock the best four beams formed by the UE antenna pattern(s) with Tx 2> if bits 7, 6 and 5 in UE Beamlock test Function = 000 3> Lock the best beam formed by the UE antenna pattern with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 001 3> Lock the best two beams formed by the UE antenna pattern(s) with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 010 3> Lock the best three beams formed by the UE antenna pattern(s) with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 011 3> Lock the best four beams formed by the UE antenna pattern(s) with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 100 3> Lock the best five beams formed by the UE antenna pattern(s) with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 101 3> Lock the best six beams formed by the UE antenna pattern(s) with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 110 3> Lock the best seven beams formed by the UE antenna pattern(s) with Rx 2> else if bits 7, 6 and 5 in UE Beamlock test Function = 111 3> Lock the best eight beams formed by the UE antenna pattern(s) with Rx 2> if bit 8 in UE Beamlock test Function = 0 3> Lock the beam(s) formed by the UE antenna pattern(s) according to bits 7 to 3 2> else if bit 8 in UE Beamlock test Function = 1 3> Lock all active beam(s) formed by the UE antenna pattern(s) with Tx or Rx or TxRx 2> Transmit ACTIVATE BEAMLOCK COMPLETE message 1> else: 2> the UE behaviour is unspecified. ### 5.4.3 Deactivate Beamlock procedure #### 5.4.3.1 Initiation The SS requests the UE to deactivate beamlock by transmitting a DEACTIVATE BEAMLOCK message. The SS should do this when the UE is in RRC_CONNECTED state. #### 5.4.3.2 Reception of DEACTIVATE BEAMLOCK message by UE When UE receives DEACTIVATE BEAMLOCK message then the UE shall: 1> if the UE is operating in FR2 AND is in RRC_CONNECTED state AND the UE Beamlock test function is active: 2> unlock all active beam(s) formed by the UE antenna pattern(s) and transmit DEACTIVATE BEAMLOCK COMPLETE message; 1> else: 2> the UE behaviour is unspecified. #### 5.4.3.3 Release of antenna beamlock by UE When the UE leaves the RRC_CONNECTED state, the UE shall: 1> if the UE is operating in FR2 AND the UE Beamlock test Function is active 2> unlock all active beam(s) formed by the UE antenna pattern(s);

## 6.4 Beamlock messages ### 6.4.1 ACTIVATE BEAMLOCK This message is only sent in the direction SS to UE. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. UE Beamlock test Function; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 0; 0; octet 1. where UE Beamlock test Function is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. A1; R1; R2; R3; T1; T2; X1; X2; octet 1. where X1,X2 = 01 for activate beamlock of Tx only, 10 for activate beamlock of Rx only and 11 for activate beamlock of both TxRx, where T1,T2 = 00 for activate beamlock of best beam formed by the Tx antenna pattern, 01 for activate beamlock of best two beams formed by the Tx antenna pattern(s), 10 for activate beamlock of best three beams formed by the Tx antenna pattern(s) and 11 for activate beamlock of best four beams formed by the Tx antenna pattern(s), where R1,R2,R3 = 000 for activate beamlock of best beam formed by the Rx antenna pattern, 001 for activate beamlock of best two beams formed by the Rx antenna pattern(s), 010 for activate beamlock of best three beams formed by the Rx antenna pattern(s) and 011 for activate beamlock of best beams formed by the Rx antenna pattern(s), 100 for activate beamlock of best five beams formed by the Rx antenna pattern(s), 101 for activate beamlock of best six beams formed by the Rx antenna pattern(s), 110 for activate beamlock of best seven beams formed by the Rx antenna pattern(s) and 111 for activate beamlock of best eight beams formed by the Rx antenna pattern(s), where A1 = 0 for activate beamlock of the beams(s) formed by the Tx/Rx/TxRx antenna pattern(s) according to bits 7 to 3, 1 for activate beamlock of all active beam(s) formed by the Tx/Rx/TxRx antenna pattern(s). NOTE: X1,X2 = 00 is not used ### 6.4.2 ACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 0; 1; octet 1. ### 6.4.3 DEACTIVATE BEAMLOCK This message is only sent in the direction SS to UE. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 1; 0; octet 1. ### 6.4.4 DEACTIVATE BEAMLOCK COMPLETE This message is only sent in the direction UE to SS. Information Element; Reference; Presence; Format; Length. Protocol discriminator; TS 24.007 [5], sub clause 11.2.3.1.1; M; V; ½. Skip indicator; TS 24.007 [5], sub clause 11.2.3.1.2; M; V; ½. Message type; M; V; 1. where message type is: 8; 7; 6; 5; 4; 3; 2; 1; bit no.. 1; 0; 1; 0; 0; 0; 1; 1; octet 1.

# 3.	Conclusion

To fulfil the request in RAN4 LS [1] our proposal is to set independently the number of Tx antenna patterns forming the Tx beams and the number of Rx antenna patterns forming the Rx beams to be locked by the UE. Below we propose two alternative solutions.

Alternative A:

We propose an alternative to independently set the number of Tx antenna patterns forming the Tx beams and the number of Rx antenna patterns forming the Rx beams to be locked by the UE as following:

- Use 2 bits (e.g., bits 3 and 4) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 3 (4 possible combinations) to set the number of Tx antenna patterns forming the Tx beams to be locked.

- Use 3 bits (e.g., bits 3 to 5) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 7 (8 possible combinations) to set the number of Tx antenna patterns forming the Tx beams to be locked.

- Use 3 bits (e.g., bits 5 to 7 or bits 6 to 8) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 7 (8 possible combinations) to set the number of Rx antenna patterns forming the Rx beams to be locked.

- Use 1 bit (e.g., bit 8) in the octet of ‘UE Beamlock test Function’ IE to set all active Tx and / or Rx antenna patterns forming the Tx and / or Rx beams to be locked by the UE.

Alternative B:

As more than one beams can be formed by the same antenna patterns, we propose another alternative to independently set the number of Tx beams formed by the Tx antenna pattern(s) and the number of Rx beams formed by the Rx antenna pattern(s) to be locked by the UE as following:

- Use 2 bits (e.g., bits 3 and 4) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 3 (4 possible combinations) to set the number of Tx beams formed by the Tx antenna pattern(s) to be locked.

- Use 3 bits (e.g., bits 3 and 5) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 7 (8 possible combinations) to set the number of Tx beams formed by the Tx antenna pattern(s) to be locked.

- Use 3 bits (e.g., bits 5 to 7 or bits 6 to 8) in the octet of ‘UE Beamlock test Function’ IE to denote a binary number from 0 to 7 (8 possible combinations) to set the number of Rx beams formed by the Rx antenna pattern(s) to be locked.

- Use 1 bit (e.g., bit 8) in the octet of ‘UE Beamlock test Function’ IE to set all active Tx and / or Rx beams formed by the Tx and / or Rx antenna pattern(s) to be locked by the UE.

Note that bits 1 and 2 in the octet of ‘UE Beamlock test Function’ IE shall continue to be used to set whether ‘Tx’ or ‘Rx’ or both ‘Tx and Rx’ antenna pattern forming the beam need to be locked by the UE, to ensure no backward compatibility issue for legacy UE.

We propose RAN5 to consider this solution and provide the response LS to RAN4.

# 4.	Reference

[1]	R4-2409999, “LS on FR2 UE Beamlock test function”

[2]	TS 38.751, “User Equipment (UE) RF and demodulation requirements for NR frequency range 2 (FR2) multi-Rx chain DL reception (Release 18)”

[3] TS 38.509, “SGS; Special conformance testing functions for User Equipment (UE) (Release 17)”

[4] TS 38.508-1, “User Equipment (UE) conformance specification; Part 1: Common test environment (Release 18)”