3GPP TSG-RAN5 Meeting #106	R5-250230

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508 | CR | 3419 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding NR CA_n78A-n258A test frequencies to NR CA Capability table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Adding NR CA_n78A-n258A configuration to NR CA Capability table missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding CA_n78A-n258A configuration to NR CA Capability table 4.3.1.3.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_CADC_NR_LTE_DC_R16-UEConTest work item cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.3.1-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

#### 4.3.1.3	Test frequencies for NR band combinations between FR1 and FR2

##### 4.3.1.3.1	NR inter-band CA configurations between FR1 and FR2

Table 4.3.1.3.1-1: NR-CA configurations between FR1 and FR2 (two bands)

| NR-CA configuration | Uplink NR-CA configuration | NR FR1 downlink configuration | NR FR2 downlink configuration | NR FR1 uplink configuration | NR FR2 uplink configuration | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n258A | CA_n1A-n258A | n1A | n258A | n1A | n258A | Yes |
| CA_n1A-n258D | CA_n1A-n258A | n1A | CA_n258D | n1A | n258A | No |
| CA_n1A-n258E | CA_n1A-n258A | n1A | CA_n258E | n1A | n258A | No |
| CA_n1A-n258F | CA_n1A-n258A | n1A | CA_n258F | n1A | n258A | No |
| CA_n1A-n258G | CA_n1A-n258A | n1A | CA_n258G | n1A | n258A | No |
| CA_n1A-n258H | CA_n1A-n258A | n1A | CA_n258H | n1A | n258A | No |
| CA_n1A-n258I | CA_n1A-n258A | n1A | CA_n258I | n1A | n258A | No |
| CA_n1A-n258J | CA_n1A-n258A | n1A | CA_n258J | n1A | n258A | No |
| CA_n1A-n258K | CA_n1A-n258A | n1A | CA_n258K | n1A | n258A | No |
| CA_n1A-n258L | CA_n1A-n258A | n1A | CA_n258L | n1A | n258A | No |
| CA_n1A-n258M | CA_n1A-n258A | n1A | CA_n258M | n1A | n258A | No |
| CA_n2A-n260A | CA_n2A-n260A | n2A | n260A | n2A | n260A | Yes |
| CA_n2A-n261A | CA_n2A-n261A | n2A | n261A | n2A | n261A | Yes |
| CA_n5A-n260A | CA_n5A-n260A | n5A | n260A | n5A | n260A | Yes |
| CA_n5A-n261A | CA_n5A-n261A | n5A | n261A | n5A | n261A | Yes |
| e | CA_n48A-n260A | n48A | n260A | n48A | n260A | Yes |
| CA_n48A-n261A | CA_n48A-n261A | n48A | n261A | n48A | n261A | Yes |
| CA_n66A-n260A | CA_n66A-n260A | n66A | n260A | n66A | n260A | Yes |
| CA_n66A-n261A | CA_n66A-n261A | n66A | n261A | n66A | n261A | Yes |
| CA_n77A-n260A | CA_n77A-n260A | n77A | n260A | n77A | n260A | Yes |
| CA_n77A-n261A | CA_n77A-n261A | n77A | n261A | n77A | n261A | Yes |
| CA_n78A-n258A | CA_n78A-n258A | n78A | n258A | n78A | n258A | Yes |
| CA_n78A-n258D | CA_n78A-n258A | n78A | CA_n258D | n78A | n258A | No |
| CA_n78A-n258E | CA_n78A-n258A | n78A | CA_n258E | n78A | n258A | No |
| CA_n78A-n258F | CA_n78A-n258A | n78A | CA_n257F | n78A | n258A | No |
| Note 1: Protocol testing is limited to NR-CA configurations with 2CC. |  |  |  |  |  |  |

###### 4.3.1.3.2.1	NR-DC configurations between FR1 and FR2 (two bands)

Table 4.3.1.3.2.1-1: NR-DC configurations between FR1 and FR2 (two bands)

#### 4.3.1.4	Test frequencies for EN-DC band combinations within FR1

<<< STOP CHANGES >>>