3GPP TSG-RAN5 Meeting #106	R5-250494

Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3694 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR SA FR2 DCI based active TCI state switch with m-DCI test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # NR_FR2_multiRX_DL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case NR SA FR2 DCI based active TCI state switch with m-DCI for simultaneous reception is present in RAN4 specification but missing in RAN5 specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case NR SA FR2 DCI based active TCI state switch with m-DCI for simultaneous reception has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be missing in spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.8.0.4(new), 7.5.8.0.5(new), 7.5.8.4(new), E.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.5.8	Active TCI state switch delay

#### 7.5.8.0	Minimum conformance requirements

-------- Start of change 1--------

-------- Unchanged Clauses Skipped --------

##### 7.5.8.0.4             Known conditions for TCI state for UE operating in FR2-1 and configured with    groupBasedBeamReporting-r17

Editor’s note: FFS whether additional conditions are needed for tests.

Dual TCI states will be indicated in one or two TCI state switch commands. The dual target TCI states are known if the following conditions are met:

-	Dual target TCI states are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report

-	The dual target TCI states and all the RSs in the two QCL chains remain detectable during the TCI state switching period

-	SNR of the TCI state ≥ -3dB

-	RS resource group configured for dual target TCI states is reported in the last [1280] ms.

##### 7.5.8.0.5             DCI based dual DL TCI state switching delay for mDCI for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17

For mDCI, if the dual target TCI states are known, when a UE is configured with the higher layer parameter tci-PresentInDCI from two TRPs which is set as 'enabled' for the DCIs scheduling two PDSCHs at slot n1 and n2 respectively, UE shall be able to receive PDSCHs with target TCI states of the serving cell on which TCI state switch occurs at the first slot that is after slot n1 + timeDurationForQCL and slot n2 + timeDurationForQCL respectively, where, timeDurationForQCL is the time required by the UE to perform PDCCH reception and apply spatial QCL information received in DCI for PDSCHs processing as described in TS 38.214 [19], the value of timeDurationForQCL is defined in TS 38.331 [13]. UE shall be able to simultaneously receive PDSCHs with target TCI states after slot max(n1, n2) + timeDurationForQCL.

For mDCI, if the dual target TCI states are known, when a UE is configured with the higher layer parameter tci-PresentInDCI from one TRP which is set as 'enabled' for the DCI scheduling PDSCHs at slot n1, while the indicated TCI for the other TRP remains unchanged, UE shall be able to simultaneously receive PDSCHs with target dual TCI states of the serving cell on which TCI state switch occurs at the first slot that is after slot n1 + timeDurationForQCL.

The known condition for dual target TCI states defined in clause 7.5.8.0.4 is applied.

-------- End of change 1--------

-------- Unchanged Clauses Skipped --------

-------- Start of change 2--------

#### 7.5.8.4	NR SA FR2 DCI based active TCI state switch with m-DCI for simultaneous reception

Editor’s note: This test case is incomplete. The following aspect is either missing or TBD:

- Test tolerance analysis and measurement uncertainty are missing

- Test procedure is subject to change

7.5.8.4.1	Test Purpose

The purpose of this test is to verify the DCI based active TCI state switch delay requirement specified in clause 7.5.8.0.5.

7.5.8.4.2	Test Applicability

This test applies to all types of NR UE release 18 onwards supporting SA FR2, more than one TCI state configuration, more than one simultaneously trackable TRS resource set, multi-DCI based multi-TRP, simultaneous reception with different QCL Type D, and group based beam reporting.

7.5.8.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.8.0.4 and 7.5.8.0.5.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.8.4.

7.5.8.4.4	Test description

7.5.8.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.8.4.4.1-1.

Table 7.5.8.4.4.1-1: Supported test configurations for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Config | Description |
| --- | --- |
| 7.5.8.4-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.5.8.4.4.1-2.

Table 7.5.8.4.4.1-2: Initial conditions for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.8.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1-1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 7.5.8.4.4.3.

2.	The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 7.5.8.4.4.1-3.

Table 7.5.8.4.4.1-3: General test parameters for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.8 |  |
| T2 | s | 0.2 |  |

7.5.8.4.4.2	Test procedure

The test scenario comprises of one NR PCell (Cell 1) as given in Table 7.5.8.4.4.1-3. Cell-specific parameters of NR PCell are specified in Table 7.5.8.4.5-1. The OTA related test parameters for FR2 are shown in Table 7.5.8.4.5-2.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with the following TCI states for Cell 1 (PCell) before starting the test:

-	CORESETPoolIndex 0:

-	PDCCH TCI state 0 (QCL’d to SSB0)

-	PDSCH TCI state 0 (QCL’d to SSB0)

-	PDSCH TCI state 2 (QCL’d to SSB2)

-	CORESETPoolIndex 1:

-	PDCCH TCIstate 1 (QCL’d to SSB1)

-	PDSCH TCI state 1 (QCL’d to SSB1)

-	PDSCH TCI state 3 (QCL’d to SSB3)

-	SSB0 and SSB2 are transmitted from the same probe. SSB2 will be transmitted with a time offset of [25% CP] from SSB0 and shall be transmitted with a higher power as compared to SSB0.

-	UE is configured with groupBasedBeamReporting-r17 for SSB index 2 and SSB index 3.

-	tci-PresentInDCI is configured in the PDSCH configuration.

The test consists of two time periods, T1 and T2. During T1, the time multiplexed (allocation in Frequency is symbolic) downlink transmissions from each Angle of Arrival is shown in Figure 7.5.8.4.5-1. UE is configured to transmit periodic L1-RSRP group-based beam reports for SSB index 2 and SSB index 3. After UE transmits the first L1-RSRP group-based beam report with SSB index 2 and SSB index 3 as a resource group, TE sends TCI state activation MAC-CEs to activate TCI state 2 and TCI state 3 for PDSCHs.

At the beginning of T2, at slot n, UE receives DCI to switch PDSCH TCI state on CORESETPoolIndex 0 for indicating the TCI state switch of PDSCH to TCI state 2. At slot n+1, UE receives DCI to switch PDSCH TCI state switch on CORESETPoolIndex 1 for indicating the TCI state switch of PDSCH to TCI state 3. During T2, the time multiplexed (allocation in Frequency is symbolic) downlink transmissions from each Angle of Arrival is shown in Figure 7.5.8.4.5-2.

The test equipment verifies the TCI state switch time in PCell by scheduling the UE on TCI state 2 and TCI state 3 simultaneously after slot n + 1 + timeDurationForQCL.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.8.4.5-1. Propagation conditions are set according to clause C.2.3.

3.	The SS transmits an RRCReconfiguration message to configure 2 different TCI states for Cell 1, PDCCH TCI state 0 (QCL’d to SSB0) and TCI state 1 (QCL’d to SSB1), as indicated in clause 7.5.8.4.4.3. This message also configures tci-PresentInDCI in the PDSCH configuration. This message also configures 4 different TCI states for Cell 1 PDSCH, TCI state 0 (QCL’d to SSB0), TCI state 1 (QCL’d to SSB1), TCI state 2 (QCL’d to SSB2) and TCI state 3 (QCL’d to SSB3).

4.	The UE transmits an RRCReconfigurationComplete message.

5.	The SS shall send a MAC-CE to the UE indicating TCI state 0 and TCI state 1 as the active PDCCH TCI states, and TCI state 0 and TCI state 1 as the active PDSCH TCI states. The SS shall continuously transmit PDCCHs and PDSCHs to the UE indicating new transmissions on PCell to ensure that the UE is transmitting the corresponding ACK/NACK feedback.

6.  T1 starts. UE is configured to transmit periodic L1-RSRP group-based beam reports for SSB index 2 and SSB index 3. The UE transmits the first L1-RSRP group-based beam report with SSB index 2 and SSB index 3 as a resource group.

7.	The SS sends TCI state activation MAC-CEs to activate TCI state 3 and TCI state 4 for PDSCHs. The SS shall continuously transmit PDCCHs and PDSCHs to the UE indicating new transmissions on PCell to ensure that the UE is transmitting the corresponding ACK/NACK feedback.

8.	When T1 expires the SS transmits an RRCReconfiguration message to configure periodic L1-RSRP reports for both SSB2 and SSB3 as indicated in clause 7.5.8.4.4.3. The start of T2 is the instant when the last TTI containing the RRCReconfiguration message is sent to the UE, at that instant the SS shall switch the power settings from T1 to T2 as specified in Table 7.5.8.4.5-2 and SSB2, and SSB3, corresponding to TCI state 2, and TCI state3, starts transmitting. T2 starts.

9.	The SS sends, in slot n, DCI to switch PDSCH TCI state on CORESETPoolIndex 0 for indicating the TCI state switch of PDSCH to TCI state 2. Slot n is within 1280 ms of UE providing group-based L1-RSRP report with results for SSB2 and SSB3 as a reference group.

10.  The SS sends, in slot n+1, DCI to switch PDSCH TCI state switch on CORESETPoolIndex 1 for indicating the TCI state switch of PDSCH to TCI state 3.

11.	During T2, if the SS receives ACK/NACK on TCI state 2 and TCI state 3, corresponding to the next scheduled DL slot after slot n + 1 + timeDurationForQCL, the number of successful tests is increased by one, otherwise the number of failed tests is increased by one.

12.	If the SS receives ACK/NACK corresponding to PDSCH transmissions scheduled on TCI state 2 and TCI state 3 continue to step 13. Otherwise switch the UE off and on and continue to step 1.

13.	Repeat steps 2-12 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.8.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.8.4.4.3-1: Common Exception messages for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.6-2 with conditions PERIODIC and SS-RSRP and groupBasedBeamReporting enabled Table H.3.6-3 with condition SSB |

Table 7.5.8.4.4.3-2: ControlResourceSet for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| tci-StatesPDCCH-ToAddList SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF { | 2 entries |  |  |
| TCI-StateId[0] | 0 | TCI.State.2 |  |
| TCI-StateId[1] | 1 | TCI.State.3 |  |
| } |  |  |  |
| tci-PresentInDCI | present |  |  |
| } |  |  |  |

Table 7.5.8.4.4.3-3: PDSCH-Config for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| tci-StatesToAddModList SEQUENCE(SIZE (1.. maxNrofTCI-States)) OF TCI-State { | 4 entries |  |  |
| TCI-State[0] | TCI.State.2 | TCI-StateId 2 |  |
| TCI-State[1] | TCI.State.3 | TCI-StateId 3 |  |
| TCI-State[2] | TCI.State.2 | TCI-StateId 2 |  |
| TCI-State[3] | TCI.State.3 | TCI-StateId 3 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.5.8.4.4.3-4: TCI-State for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-190 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TCI-State ::= SEQUENCE { | 4 entries |  |  |
| { |  |  |  |
| tci-StateId | 0 | TCI.State.2 |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 3 | TRS.2.1 TDD Resource #4 Note1 |  |
| } |  |  |  |
| qcl-Type | typeA |  |  |
| } |  |  |  |
| qcl-Type2 SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 3 | TRS.2.1 TDD Resource #4 Note1 |  |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| }, |  |  |  |
| { |  |  |  |
| tci-StateId | 1 | TCI.State.3 |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 7 | TRS.2.2 TDD Resource #4 Note1 |  |
| } |  |  |  |
| qcl-Type | typeA |  |  |
| } |  |  |  |
| qcl-Type2 SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 7 | TRS.2.2 TDD Resource #4 Note1 |  |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| }, |  |  |  |
| { |  |  |  |
| tci-StateId | 3 | TCI.State.2 |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 3 | TRS.2.1 TDD Resource #4 Note1 |  |
| } |  |  |  |
| qcl-Type | typeA |  |  |
| } |  |  |  |
| qcl-Type2 SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 3 | TRS.2.1 TDD Resource #4 Note1 |  |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| }, |  |  |  |
| { |  |  |  |
| tci-StateId | 1 | TCI.State.3 |  |
| qcl-Type1 SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 7 | TRS.2.2 TDD Resource #4 Note1 |  |
| } |  |  |  |
| qcl-Type | typeA |  |  |
| } |  |  |  |
| qcl-Type2 SEQUENCE { |  |  |  |
| bwp-Id | 1 |  |  |
| referenceSignal CHOICE { |  |  |  |
| csi-rs | 7 | TRS.2.2 TDD Resource #4 Note1 |  |
| } |  |  |  |
| qcl-Type | typeD |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Both TRS.2.1 TDD and TRS.2.2 TDD are configured on the UE. The assumption to configure this field is that nzp-CSI-RS-ResourceId indices are configured in the following order: TRS.2.1 TDD = {0,1,2,3}, TRS.2.2 TDD = {4,5,6,7}, so resource #4 for each set corresponds to index 3 and 7, accordingly. |  |  |  |

7.5.8.4.5	Test requirement

Tables 7.5.8.4.4.1-3, 7.5.8.4.5-1 and 7.5.8.4.5-2 define the primary level settings including test tolerances for NR SA PCell FR2 DCI based active TCI state switch with m-DCI for simultaneous reception.

Table 7.5.8.4.5-1: NR Cell specific test parameters for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Data RBs allocated |  | 24 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3. 2 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP. 5 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State 0 |  | TCI. State.2 |
| TCI State 1 |  | TCI.State.3 |
| TCI State 2 |  | TCI. State.2 |
| TCI State 3 |  | TCI.State.3 |
| TRS Configuration |  | TRS.2.1 TDD TRS.2.2 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | No external noise (Note 2) |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |

Table 7.5.8.4.5-2: OTA related test parameter for NR SA FR2 PCell DCI based active TCI state switch with m-DCI for simultaneous reception

| Parameter | Unit | Cell 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  | SSB2 |  | SSB3 |  |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | - | Setup 6 according to A.9.6 |  |  |  |  |  |  |  |
|  |  | AoA1 |  | AoA2 |  | AoA3 |  | AoA1 |  |
| Assumption for UE beams Note 3 | - | Rough |  |  |  | Rough |  |  |  |
| Ês | dBm/SCS | -80.6+TT | -80.6+TT | -80.6+TT | -80.6+TT | -Infinity | -80.6+TT | -Infinity | -85.6+TT |
| SS B_RP Note 1 | dBm/ SCS | -80.6+TT | -80.6+TT | -80.6+TT | -80.6+TT | -Infinity | -80.6+TT | -Infinity | -85.6+TT |
| BB Note 4. | dB | 8.3+TT | 8.3+TT | 8.3+TT | 8.3+TT | -Infinity | 8.3+TT | -Infinity | 7.63+TT |
| IoNote2 | dBm/95.04 MHz Note2 | -55.41+TT | -55.41+TT | -55.41+TT | -55.41+TT | - Infinity | -55.41+TT | - Infinity | -60.41+TT |
| Note 1: SS B_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 3: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. Note 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 5: Three probes are used to test dual to dual TCI state switch. One probe shall be used to emulate two different SSBs. SSB3 is transmitted with same AoA as SSB0 with a time offset of [25% CP] and a different power from SSB0 |  |  |  |  |  |  |  |  |  |

Figure 7.5.8.4.5-1: Time multiplexed downlink transmissions during T1

Figure 7.5.8.4.5-2: Time multiplexed downlink transmissions during T2

After receiving two DCIs in slot n and slot n+1 during T2, UE shall:

-	be able to start receiving on TCI state 3 and TCI state 4 simultaneously after slot n + timeDurationForQCL

The rate of correct events observed during repeated tests shall be at least 90%.

-------- Unchanged Clauses Skipped --------

-------- Start of change 3--------

-------- Unchanged Clauses Skipped --------

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 | NR SA FR2 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.1.1.3 | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.4 | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.1.1.5 | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | NR Cell 1 | NR Cell 2 |  |  |  |
| … |  |  |  |  |  |  |
| 7.5.8.1.1 | NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |
| 7.5.8.2.1 | NR SA PCell FR2 RRC based active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |
| 7.5.8.3.1 | NR SA FR2 HST active TCI state switch for a known TCI state | NR Cell 1 |  |  |  |  |
| 7.5.8.3.2 | NR SA FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 a known TCI state | NR Cell 1 |  |  |  |  |
| 7.5.8.3.4 | NR SA FR2 DCI based active TCI state switch with m-DCI for simultaneous reception | NR Cell 1 |  |  |  |  |
| … |  |  |  |  |  |  |
| 7.7.6.2 | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |
| 7.7.6.3 | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | NR Cell 1 |  |  |  |  |

-------- End of change 3--------