# Title:	Test Tolerances for FR1 RRC-based DL Active BWP Switch on Multiple CCs

Source:	Apple

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for FR1 RRC-based DL active BWP switch on multiple CCs.

The test cases in question, from 38.533, are:

- 4.5.6.4.1		E-UTRAN - NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs

This document describes the process to derive the Test Tolerances. The calculations are provided in Section 4.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v16.22.0 Annex A.

##### A.4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs

A.4.5.6.5.1.1	Test Purpose and Environment

The purpose of this test is to verify the DL BWP switch delay requirement on multiple CCs for RRC-based BWP switch defined in clause 8.6.3A.1. Supported test configurations for LTE PCell and NR PSCell are shown in Table A.4.5.6.5.1.1-1. Supported test configurations for NR SCell are shown in Table A.4.5.6.5.1.1-1A. Test configuration for LTE PCell and NR PSCell and test configuration for NR SCell are chosen independently.

The test scenario comprises of one E-UTRA PCell (Cell 1), one NR PSCell (Cell 2) and one NR SCell(Cell 3) as given in Table A.4.5.6.5.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell and NR SCell are specified in Table A.4.5.6.5.1.1-3 and Table A.4.5.6.5.1.1-4 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), PSCell (Cell 2) on radio channel 2 (PSCC) and SCell (Cell 3) on radio channel 3 (SCC).

-	UE has bandwidth part BWP-1 in its RRC-configuration for PSCell (Cell 2) and SCell (Cell 3)

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PSCell (Cell 2) and SCell (Cell 3).

All cells have constant signal levels throughout the test.

The test consists of 1 time period, with duration of T1.

During T1,

Time period T1 starts when a RRCReconfiguration with updated bandwidth part configuration for both PSCell(Cell 2) and SCell(Cell 3), sent from the test equipment to the UE, is completely received at the UE side in PSCell’s slot # denoted i. The UE shall reconfigure its bandwidth part with the updated bandwidth part configuration on PSCell(Cell 2) and SCell(Cell 3).

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length) as defined in clause 8.6.3A.1 and be ready for the reception of uplink grant for the PSCell(Cell 2) and SCell(Cell 3) no later than at the beginning of the DL slot right after slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length). The UE shall be continuously scheduled on PSCell’s BWP-1 and SCell’s BWP-1 starting from the beginning of the DL slot right after slot (i +TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length).

TRRCprocessingDelay , TBWPswitchDelayRRC, DRRC are defined in clause 8.6.3A.1 .

The test equipment verifies the DL BWP switch time in PSCell(Cell 2) and SCell(Cell 3) by counting the time from the time when the RRC Reconfiguration message including updated BWP configuration sent till the time when RRC Reconfiguration Complete message is received.

Table A.4.5.6.5.1.1-1: DL BWP switch supported test configurations for LTE PCell and NR PSCell

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.4.5.6.5.1.1-1A: DL BWP switch supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration |  |

Table A.4.5.6.5.1.1-2: General test parameters for DL BWP switch in synchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2,3 | Two NR radio channel is used for this test |
| Active PCell |  | Cell 1 | Pcell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| Active SCell |  | Cell 3 | SCell on RF channel number 3. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| Cell3 timing offset to cell2 | s | 3 | Synchronous cells |
| T1 | s | 0.2 |  |

Table A.4.5.6.5.1.1-3: NR Cell specific test parameters for NR PSCell for DL BWP switch in synchronous EN-DC

| Parameter |  |  | Unit | Cell 2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | Config 1,4 |  | FDD |
|  |  | Config 2,3,5,6 |  | TDD |
| TDD configuration |  | Config 1,4 |  | Not Applicable |
|  |  | Config 2,5 |  | TDDConf.1.1 |
|  |  | Config 3,6 |  | TDDConf.1.2 |
| BWchannel |  | Config 1,4 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 2,5 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 3,6 |  | 40 MHz: NRB,c = 106 |
| Active DL BWP ID |  |  |  | 1 |
| Initial DL BWP Configuration |  | Config 1,4 |  | DLBWP.0.2 |
|  |  | Config 2,5 |  |  |
|  |  | Config 3,6 |  |  |
| Initial UL BWP Configuration |  | Config 1,4 |  | ULBWP.0.2 |
|  |  | Config 2,5 |  |  |
|  |  | Config 3,6 |  |  |
| Initial Condition | Active DL BWP-1 Configuration | Config 1,4 |  | DLBWP.1.3 |
|  |  | Config 2,5 |  |  |
|  |  | Config 3,6 |  |  |
|  | Active UL BWP-1 Configuration | Config 1,4 |  | ULBWP.1.3 |
|  |  | Config 2,5 |  |  |
|  |  | Config 3,6 |  |  |
| Final Condition | Active DL BWP-1 Configuration | Config 1,4 |  | DLBWP.1.1 |
|  |  | Config 2,5 |  |  |
|  |  | Config 3,6 |  |  |
|  | Active UL BWP-1 Configuration | Config 1,4 |  | ULBWP.1.1 |
|  |  | Config 2,5 |  |  |
|  |  | Config 3,6 |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |
|  |  | Config 2,5 |  | SR.1.1 TDD |
|  |  | Config 3,6 |  | SR2.1 TDD |
| RMSI CORESET parameters |  | Config 1,4 |  | CR.1.1 FDD |
|  |  | Config 2,5 |  | CR.1.1 TDD |
|  |  | Config 3,6 |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | Config 1,4 |  | CCR.1.1 FDD |
|  |  | Config 2,5 |  | CCR.1.1 TDD |
|  |  | Config 3,6 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |
|  |  | Config 3,6 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| TRS Configuration |  | Config 1,4 |  | TRS.1.1 FDD |
|  |  | Config 2,5 |  | TRS.1.1 TDD |
|  |  | Config 3,6 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  | 1x2 |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NocNote 2 |  |  | dBm/15 kHz | -104 |
| SS-RSRP Note 3 |  |  | dBm/15 kHz | -87 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | Config 1,2,4,5 | dBm/ 9.36MHz | -59 |
|  |  | Config 3,6 | dBm/ 38.16MHz | -61.9 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

Table A.4.5.6.5.1.1-4: NR Cell specific test parameters for NR SCell for DL BWP switch in synchronous EN-DC

| Parameter |  |  | Unit | Cell 3 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode |  | ConfigSCell 1 |  | FDD |
|  |  | ConfigSCell 2,3 |  | TDD |
| TDD configuration |  | ConfigSCell 1 |  | Not Applicable |
|  |  | ConfigSCell 2 |  | TDDConf.1.1 |
|  |  | ConfigSCell 3 |  | TDDConf.1.2 |
| BWchannel |  | ConfigSCell 1 |  | 10 MHz: NRB,c = 52 |
|  |  | ConfigSCell 2 |  | 10 MHz: NRB,c = 52 |
|  |  | ConfigSCell 3 |  | 40 MHz: NRB,c = 106 |
| Active DL BWP ID |  |  |  | 1 |
| Initial DL BWP Configuration |  | ConfigSCell 1 |  | DLBWP.0.2 |
|  |  | ConfigSCell 2 |  |  |
|  |  | ConfigSCell 3 |  |  |
| Initial UL BWP Configuration |  | ConfigSCell 1 |  | ULBWP.0.2 |
|  |  | ConfigSCell 2 |  |  |
|  |  | ConfigSCell 3 |  |  |
| Initial Condition | Active DL BWP-1 Configuration | ConfigSCell 1 |  | DLBWP.1.3 |
|  |  | ConfigSCell 2 |  |  |
|  |  | ConfigSCell 3 |  |  |
|  | Active UL BWP-1 Configuration | ConfigSCell 1 |  | ULBWP.1.3 |
|  |  | ConfigSCell 2 |  |  |
|  |  | ConfigSCell 3 |  |  |
| Final Condition | Active DL BWP-1 Configuration | ConfigSCell 1 |  | DLBWP.1.1 |
|  |  | ConfigSCell 2 |  |  |
|  |  | ConfigSCell 3 |  |  |
|  | Active UL BWP-1 Configuration | ConfigSCell 1 |  | ULBWP.1.1 |
|  |  | ConfigSCell 2 |  |  |
|  |  | ConfigSCell 3 |  |  |
| PDSCH Reference measurement channel |  | ConfigSCell 1 |  | SR.1.1 FDD |
|  |  | ConfigSCell 2 |  | SR.1.1 TDD |
|  |  | ConfigSCell 3 |  | SR2.1 TDD |
| RMSI CORESET parameters |  | ConfigSCell 1 |  | CR.1.1 FDD |
|  |  | ConfigSCell 2 |  | CR.1.1 TDD |
|  |  | ConfigSCell 3 |  | CR2.1 TDD |
| Dedicated CORESET parameters |  | ConfigSCell 1 |  | CCR.1.1 FDD |
|  |  | ConfigSCell 2 |  | CCR.1.1 TDD |
|  |  | ConfigSCell 3 |  | CCR.2.1 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SSB Configuration |  | ConfigSCell 1,2 |  | SSB.1 FR1 |
|  |  | ConfigSCell 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| TRS Configuration |  | ConfigSCell 1 |  | TRS.1.1 FDD |
|  |  | ConfigSCell 2 |  | TRS.1.1 TDD |
|  |  | ConfigSCell 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  | 1x2 |
| Propagation Condition |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NocNote 2 |  |  | dBm/15 kHz | -104 |
| SS-RSRP Note 3 |  |  | dBm/15 kHz | -87 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 |  | ConfigSCell 1,2 | dBm/ 9.36MHz | -59 |
|  |  | ConfigSCell 3 | dBm/ 38.16MHz | -61.9 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [3]. |  |  |  |  |

A.4.5.6.5.1.2	Test Requirements

During T1, the UE shall be ready for the reception of uplink grant for PSCell and SCell in the beginning of the DL slot right after  slot (i+TRRCprocessingDelay+TBWPswitchDelayRRC+DRRCNR slot length) .

All of the above test requirements shall be fulfilled in order for the observed PSCell and SCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

# 3. Discussion

In the test there are two NR cells (PSCell and SCell). All cells have constant signal levels throughout the test. The test consists of one time period: T1.

During the test, the UE shall be able to reliably demodulate the PDSCH to reconfigure the BWP as the RRC message is transmitted on the PDSCH.

The UE shall have the capability to demodulate the PDSCH correctly.

# 4. Calculation of Test Tolerances

To calculate the overall test system uncertainty, it is proposed to adopt the same approach as performance and demodulation test cases.

Overall system uncertainty for AWGN condition comprises two quantities:

- Signal-to-noise ratio uncertainty

- Effect of AWGN flatness and signal flatness

Items 1 and 2 are assumed to be uncorrelated so can be root sum squared.

AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution.

Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2)

For test case 4.5.6.4.1, the SS provides AWGN and two cells to the UE. The following parameters are proposed to be controlled:

- AWGN flatness and signal flatness ±2.0 dB

- Signal-to-noise ratio uncertainty ±0.3 dB

Test System uncertainty on Cell1 = SQRT (0.32 + (0.25 * 2.0)2) = 0.58dB, round to 0.6dB

Considering the test system uncertainty, the minimum SNR (Es/Iot) of this test case is decreased to 17-0.6=16.4dB

The UE is required to demodulate the PDSCH correctly. The demodulation performance of the control channel used in this test case is specified for AWGN. However, there are no AWGN channels in Performance specifications for PDSCH demodulation, so the fading channels are chosen as reference. If the SNR meets the condition of the Test 1-1 in TS 38.521-4 Table 5.2.2.1.1.0-3 for 15kHz (-0.8dB) or Table 5.2.2.2.1.0-3 for 30kHz (-1.1dB), then the PDSCH can be demodulated easily in AWGN channel.

For NR PSCell and SCell, the minimum SNR (Es/Iot) is 16.4dB, much higher than the reference value -0.8dB of the Test 1-1 in TS 38.521-4 Table 5.2.2.1.1.0-3 or the reference value -1.1dB of the Test 1-1 in TS 38.521-4 Table 5.2.2.2.1.0-3.

All cells have high Es/Iot (SNR) and the propagation condition is AWGN, so the requirement of PDSCH demodulation is easily to be met and the parameter values are not intended to be critical. The uncertainties will not have any significant effect, so there is no need for any offsets, and Test Tolerance of 0dB can be applied.