3GPP TSG-RAN5 Meeting #106	R5-250030r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1499 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NB-IoT NTN RRM enhanced coverage TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek India Technology Pvt. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Inplementation CR of R5-250026. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a set of parameters for RRM enh TCs in 36.521-3 Clause 13. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inappropriate values remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.3, 8.2.3, 8.1.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Inplementation CR of R5-250026. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: - Merge IE name cells - Add binary value for Repetition number - Remove reference table number from condition row - Add additional changes to RadioResourceConfigCommonSIB-NB |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1 – SIB + q-QualMin-r13>>>

#### 8.1.4.3	NB-IoT Common parameters for simulated cells

The parameters specified in this sub clause apply to all simulated cells unless otherwise specified.

##### 8.1.4.3.1	NB-IoT Common configurations of system information blocks

###### 8.1.4.3.1.1	NB-IoT Combinations of system information blocks

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, the following combinations of system information blocks are defined.

Combination 1 is the default combination which applies to the following test case scenarios:

-	single cell scenario

Combination 2 applies to the following test case scenarios:

-	intra-frequency multi cell scenario

-	intra-frequency multi cell scenario with neighbouring cell related information

Combination 3 applies to the following test case scenarios:

-	inter-frequency multi cell scenario

Combination 4 applies to the following test case scenarios:

- access barring single cell scenario

Combination 5 applies to the following test case scenarios:

-	SC-PTM single cell scenario (SC-PTM cell broadcasting SIB20-NB)

Combination 6 applies to the following test case scenarios:

-	single cell on multi carriers scenario

Combination 7 applies to the following test case scenarios:

-	single cell NPRACH resources using preamble format 2 on non-anchor carriers scenario

Combination 8 is the default combination which applies to the following test case scenarios:

-	single NTN cell scenario

Combination 9 is the default combination which applies to the following test case scenarios:

-	intra-frequency multi NTN cell scenario

Combination 10 is the default combination which applies to the following test case scenarios:

-	intra-frequency multi NTN cell scenario with neighbouring NTN cell related information

Combination 11 is the default combination which applies to the following test case scenarios:

-	inter-frequency multi NTN cell scenario

Combination 12 is the default combination which applies to the following test case scenarios:

-	single NTN cell on multi NTN carriers scenario

Combination 13 applies to the following test case scenarios:

-	access barring single NTN cell scenario

Table 8.1.4.3.1.1-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB2-NB | SIB3-NB | SIB4-NB | SIB5-NB | SIB14-NB | SIB16-NB | SIB20-NB | SIB22-NB | SIB23-NB | SIB31-NB | SIB32-NB |
| 1 | X |  |  |  |  |  |  |  |  |  |  |
| 2 | X | X | X |  |  |  |  |  |  |  |  |
| 3 | X | X |  | X |  |  |  |  |  |  |  |
| 4 | X | X |  |  | X |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  | X |  |  |  |  |
| 6 | X |  |  |  |  |  |  | X |  |  |  |
| 7 | X |  |  |  |  |  |  | X | X |  |  |
| 8 | X |  |  |  |  |  |  |  |  | X |  |
| 9 | X | X |  |  |  |  |  |  |  | X |  |
| 10 | X | X | X |  |  |  |  |  |  | X |  |
| 11 | X | X |  | X |  |  |  |  |  | X |  |
| 12 | X |  |  |  |  |  |  | X |  | X |  |
| 13 | X | X |  |  | X |  |  |  |  | X |  |

###### 8.1.4.3.1.2	NB-IoT Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables.

Table 8.1.4.3.1.2-1: Scheduling for combination 1

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |

Table 8.1.4.3.1.2-2: Scheduling for combination 2

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB4-NB |

Table 8.1.4.3.1.2-3: Scheduling for combination 3

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB5-NB |

Table 8.1.4.3.1.2-4: Scheduling for combination 4

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB14-NB |

Table 8.1.4.3.1.2-5: Scheduling for combination 5

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB20-NB |

Table 8.1.4.3.1.2-6: Scheduling for combination 6

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB22-NB |

Table 8.1.4.3.1.2-7: Scheduling for combination 7

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB22-NB |
| 3 | 64 | every16thRF | 256 | SIB23-NB |

Table 8.1.4.3.1.2-8: Scheduling for combination 8

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |

Table 8.1.4.3.1.2-8a: Scheduling for combination 8 under RRM enh coverage

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 256 | every2thRF | 552 | SIB2-NB |
| 2 | 256 | every2thRF | 256 | SIB31-NB |

Table 8.1.4.3.1.2-9: Scheduling for combination 9

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB, SIB31-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |

Table 8.1.4.3.1.2-9a: Scheduling for combination 9 under RRM enh coverage

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 256 | every2thRF | 552 | SIB2-NB, SIB31-NB |
| 2 | 256 | every2thRF | 256 | SIB3-NB |

Table 8.1.4.3.1.2-10: Scheduling for combination 10

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB3-NB |
| 4 | 64 | every16thRF | 256 | SIB4-NB |

Table 8.1.4.3.1.2-11: Scheduling for combination 11

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB3-NB |
| 4 | 64 | every16thRF | 256 | SIB5-NB |

Table 8.1.4.3.1.2-12: Scheduling for combination 12

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB22-NB |

Table 8.1.4.3.1.2-12a: Scheduling for combination 12 under RRM enh coverage

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 256 | every2thRF | 552 | SIB2-NB |
| 2 | 256 | every2thRF | 256 | SIB31-NB |
| 3 | 256 | every2thRF | 256 | SIB22-NB |

Table 8.1.4.3.1.2-13: Scheduling for combination 13

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB3-NB |
| 4 | 64 | every16thRF | 256 | SIB14-NB |

##### 8.1.4.3.2	NB-IoT Common contents of system information messages

<<< Skip unchanged part>>>

###### -	SystemInformationBlockType1-NB

SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information.

Table 8.1.4.3.2-3: SystemInformationBlockType1-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| hyperSFN-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| plmn-IdentityList-r13 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF PLMN-IdentityInfo-NB-r13 SEQUENCE { | 1 entry |  |  |
| PLMN-IdentityInfo-NB-r13[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity-r13 SEQUENCE { |  |  |  |
| mcc | See table 8.1.4.2-2 |  |  |
| mnc | See table 8.1.4.2-2 |  |  |
| } |  |  |  |
| cellReservedForOperatorUse-r13 | notReserved |  |  |
| attachWithoutPDN-Connectivity-r13 | true |  | ATTACH_WITHOUT_PDN |
|  | Not present |  | ATTACH_WITH_PDN |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r13 | See table 8.1.4.2-2 |  |  |
| cellIdentity-r13 | Cell ID for the simulated cell |  |  |
| cellBarred-r13 | notBarred |  |  |
|  | barred |  | NTN |
| intraFreqReselection-r13 | notAllowed |  |  |
| } |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) | For RF/RRM test cases | RF |
|  | -106 dBm | For signalling test cases | Signaling |
| q-QualMin-r13 | -20 (-20dB) |  |  |
|  | -34 (-34dB) | For RRM NB-IoT NTN enhanced coverage test cases | RRM_NTN_Enh |
| } |  |  |  |
| p-Max-r13 | Not Present |  |  |
| freqBandIndicator-r13 | Operating band under test. |  |  |
| freqBandInfo-r13 | Not Present |  |  |
| multiBandInfoList-r13 | Not Present |  |  |
| downlinkBitmap-r13 | Not Present |  |  |
| eutraControlRegionSize-r13 | n2 |  | Inband-Same, Inband-Different |
|  | Not Present |  | Standalone, Guard |
| nrs-CRS-PowerOffset-r13 | dB0 |  | Inband-Same |
|  | Not Present |  | Inband-Different, Standalone, Guardband |
| schedulingInfoList-r13 | See subclause 8.1.4.3.1 |  |  |
| si-WindowLength-r13 | ms160 |  |  |
|  | ms640 | For RRM NB-IoT NTN enhanced coverage test cases | RRM_NTN_Enh |
| si-RadioFrameOffset-r13 | Not Present |  |  |
| systemInfoValueTagList-r13 | Not Present |  |  |
| lateNonCriticalExtension | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1350 | TDD, SibType>22, NTN |
| cellSelectionInfo-v1350 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1430 |  |
| cellSelectionInfo-v1430 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1450 |  |
| nrs-CRS-PowerOffset-v1450 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1530 |  |
| tdd-Parameters-r15 | Not Present |  |  |
| tdd-Parameters-r15 SEQUENCE { |  |  | TDD |
| tdd-Config-r15 SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa1 |  |  |
| specialSubframePatterns-r15 | ssp6 |  |  |
| } |  |  |  |
| tdd-SI-CarrierInfo-r15 | anchor |  |  |
| tdd-SI-SubframesBitmap-r15 | Not Present |  |  |
| } |  |  |  |
| schedulingInfoList-v1530 | Not Present |  |  |
| schedulingInfoList-v1530 SEQUENCE (SIZE (1..maxSI-Message-NB-r13)) OF SchedulingInfo-NB-v1530 { | Actual number of entries depends on system information combination in clause 8.1.4.3.1.1 | Includes the same number of entries, and listed in the same order, as in schedulingInfoList-r13 | SibType>22 |
| SchedulingInfo-NB-v1530[x] | See clause 8.1.4.3.1.2 | entry x (x denotes the index of the entry) |  |
| } |  |  |  |
| nonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1610 | NTN |
| cellAccessRelatedInfo-5GC-r16 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1700 |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | notBarred |  |  |
| plmn-IdentityList-v1700 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| ATTACH_WITH_PDN | If the UE shall perform ATTACH with PDN connectivity procedure. |
| ATTACH_WITHOUT_PDN | If the UE may perform ATTCH without PDN connectivity procedure. |
| Inband_Same | In-band with same PCI test environment |
| Inband_Different | In-band with different PCI test environment |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |
| RF | For RF/RRM test cases |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |
| Signaling | For Signaling test cases |
| NTN | Non-Terrestrial Networks test environment |
| SibType>22 | When the system information combination in use (see clause 8.1.4.3.1) includes at least one SIB with index exceeding 22 (i.e. 23 or more) |

<<< END OF CHANGES 1 - SIB + q-QualMin-r13>>>

<<< START OF CHANGES 2 - NPDSCH>>>

8.1.3.6.1.2	NB-IoT Physical layer parameters for DCI format N1

<<< Skip unchanged part>>>

Table 8.1.3.6.1.2-2: NB-IoT Physical layer parameters for DCI format N1 (for the scheduling of one NPDSCH codeword in one cell scrambled by RA-RNTI/C-RNTI)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for format N0/format N1 differentiation | format N1 | “1” |
| NPDCCH order indicator | 0 | “0” |
| Scheduling delay | 0 (3 bits as defined in clause16.4.1 of 36.213 [29]) | “000” |
| Resource assignment | Depending on test parameters (3 bits as defined in section 16.4.1.3 of 36.213 [29]) | - |
| Modulation and coding scheme | Depending on test parameters (4 bits as defined in section 16.4.1.5 of 36.213 [29]) | - |
| Repetition number | NRep = 1 for normal coverage and 32 for enhanced coverage (4 bits as defined in section 16.4.1.3 of 36.213 [29]) | “0000” for normal coverage and “0101” for enhanced coverage. |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 [14] (1 bit) | - |
| HARQ-ACK resource | 0 if not specified otherwise depending on test case (4 bits as defined in section 16.4.2 of 36.213 [29]) | - |
| DCI subframe repetition number | R = 2 (according to NPRACH configuration of Table 8.1.6.3-5) (2 bits as defined in section 16.6 in 36.213 [29]) | “00” |
| HARQ process number | Mandatory present if 2 HARQ processes are configured in the UE, default value (HARQ process) "0". Otherwise not present (1 bit as defined in section 16.4.1.5 of 36.213 [29]) | - |

<<< END OF CHANGES 2 - NPDSCH >>>

<<< START OF CHANGES 3 - NPDCCH>>>

### 8.2.3	NB-IoT Default RRC message and information elements contents

#### 8.2.3.1	NB-IoT Radio resource control information elements

As defined in clause 8.1.6.3 with the following exceptions.

Table 8.2.3.1-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: Clause 8.1.6.3 Table 8.1.6.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r1 |  |  |
|  | r32 |  | RRM_NTN_Enh |
| npdcch-StartSF-USS-r13 | v8 |  |  |
| npdcch-Offset-USS-r13 | oneFourth |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |

<<< END OF CHANGES 3 - NPDCCH>>>

<<< START OF CHANGES 4 - NPDCCH>>>

#### 8.1.6.3	NB-IoT Radio resource control information elements

<<< Skip unchanged part>>>

#### –	PCCH-Config-NB-DEFAULT

Table 8.1.6.3-2: PCCH-Config-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCCH-Config-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| defaultPagingCycle-r13 | rf128 | Typical value in real network. |  |
| nB-r13 | oneT |  |  |
| npdcch-NumRepetitionPaging-r13 | r8 |  |  |
|  | r32 |  | RRM_NTN_Enh |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |

<<< Skip unchanged part>>>

#### –	NPRACH-ConfigSIB-NB-DEFAULT

Table 8.1.6.3-5: NPRACH-ConfigSIB-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | us66dot7 |  |  |
| rsrp-ThresholdsPrachInfoList-r13 | Not present | Only one resource |  |
| nprach-ParametersList-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 1 entry |  |  |
| nprach-Periodicity-r13 | ms80 |  |  |
| nprach-StartTime-r13 | ms32 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n6 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n1 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
|  | r32 |  | RRM_NTN_Enh |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |

<<< Skip unchanged part>>>

#### RadioResourceConfigCommonSIB-NB-DEFAULT

Table 8.1.6.3-9: RadioResourceConfigCommonSIB-NB-DEFAULT

| Derivation Path: TS 36.331 [17], clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon-r13 | RACH-ConfigCommon-NB-DEFAULT |  |  |
| bcch-Config-r13 | BCCH-Config-NB-DEFAULT |  |  |
| pcch-Config-r13 | PCCH-Config-NB-DEFAULT |  |  |
|  | PCCH-Config-NB-DEFAULT with condition RRM_NTN_Enh |  | RRM_NTN_Enh |
| nprach-Config-r13 | NPRACH-ConfigSIB-NB-DEFAULT |  |  |
|  | NPRACH-ConfigSIB-NB-DEFAULT with condition RRM_NTN_Enh |  | RRM_NTN_Enh |
| npdsch-ConfigCommon-r13 | NPDSCH-ConfigCommon-NB-DEFAULT |  |  |
| npusch-ConfigCommon-r13 | NPUSCH-ConfigCommon-NB-DEFAULT |  |  |
| dl-Gap-r13 | Not present |  |  |
| uplinkPowerControlCommon-r13 | UplinkPowerControlCommon-NB-DEFAULT |  |  |
| nprach-Config-v1330 | Not present |  |  |
| nprach-Config-v1450 | Not present |  |  |
| nprach-Config-v1530 | NPRACH-ConfigSIB-NB-v1530-DEFAULT |  | TDD |
| dl-Gap-v1530 | Not present |  |  |
| wus-Config-v1530 | Not present |  |  |
| nprach-Config-v1550 | NPRACH-ConfigSIB-NB-v1550-DEFAULT |  | TDD |
| gwus-Config-r16 | Not present |  |  |
| nrs-NonAnchorConfig-r16 | Not present |  |  |
| ue-SpecificDRX-CycleMin-r16 | Not present |  |  |
| ntn-ConfigCommon-r17 | Not present |  |  |
| ntn-ConfigCommon-r17 SEQUENCE { |  |  | NTN |
| ta-Report-r17 | enabled |  |  |
| t318-r17 | ms8000 |  |  |
| nprach-TxDurationFmt01-r17 | Not present |  |  |
| nprach-TxDurationFmt01-r17 SEQUENCE { |  |  | NGSO |
| nprach-TxDurationFmt01-r17 | n32 |  |  |
| } |  |  |  |
| nprach-TxDurationFmt2-r17 | Not present |  |  |
| nprach-TxDurationFmt2-r17 SEQUENCE { |  |  | NGSO |
| nprach-TxDurationFmt2-r17 | n8 |  |  |
| } |  |  |  |
| npusch-TxDuration-r17 | Not present |  |  |
| npusch-TxDuration-r17 SEQUENCE { |  |  | NGSO |
| npusch-TxDuration-r17 | ms128 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN | Non-Terrestrial Networks test environment |
| NGSO | Non-Geosynchronous Orbit test environment |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |

<<< END OF CHANGES 4 - NPDCCH>>>