3GPP TSG-RAN5 Meeting #106	R5-250043r1

# Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3410 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE LTM-TCI-Info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2024-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To align the IE LTM-TCI-Info with core specification updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The ASN.1 field ltm-DL-OrJointTCI-StateToAddModList-r18 has been updated with a condition for the IE CandidateTCI-State. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-250043r1. ASN.1 fields ltm-DL-OrJointTCI-StateToAddModList-r18 and ltm-UL-TCI-StateToAddModList-r18 updated with not present possibilities. |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	LTM-TCI-Info

Table 4.6.3-67I: LTM-TCI-Info

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-TCI-Info-r18 ::= SEQUENCE { |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList-r18 | Not present |  |  |
| ltm-DL-OrJointTCI-StateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18 { | 1 entry |  | DL TCI |
| CandidateTCI-State-r18[1] | CandidateTCI-State | entry 1 |  |
| } |  |  |  |
| ltm-DL-OrJointTCI-StateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCandidateTCI-State-r18)) OF CandidateTCI-State-r18 { | 1 entry |  | Joint |
| CandidateTCI-State-r18[1] | CandidateTCI-State with condition Joint | entry 1 |  |
| } |  |  |  |
| ltm-DL-OrJointTCI-StateToReleaseList-r18 | Not present |  |  |
| ltm-UL-TCI-StateToAddModList-r18 | Not present |  |  |
| ltm-UL-TCI-StateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCandidateUL-TCI-r18)) OF CandidateTCI-UL-State-r18 { | 1 entry |  | UL TCI |
| CandidateTCI-UL-State-r18[1] | CandidateTCI-UL-State | entry 1 |  |
| } |  |  |  |
| ltm-UL-TCI-StateToReleaseList-r18 | Not present |  |  |
| ltm-NZP-CSI-RS-ResourceToAddModList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | 1 entry |  |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource | entry 1 |  |
| } |  |  |  |
| ltm-NZP-CSI-RS-ResourceToReleaseList-r18 | Not present |  |  |
| ltm-NZP-CSI-RS-ResourceSetToAddModList-r18 SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | 1 entry |  |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet | entry 1 |  |
| } |  |  |  |
| ltm-NZP-CSI-RS-ResourceSetToReleaseList-r18 | Not present |  |  |
| pathlossReferenceRS-ToAddModList-r18 SEQUENCE (SIZE (1..maxNrofPathlossReferenceRSs-r17)) OF PathlossReferenceRS-r17 { | 1 entry |  |  |
| PathlossReferenceRS-r17[1] | PathlossReferenceRS | entry 1 |  |
| } |  |  |  |
| pathlossReferenceRS-ToReleaseList-r18 | Not present |  |  |
| unifiedTCI-StateType-r18 | separate |  | Separate |
|  | joint |  | Joint |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DL TCI | Downlink TCI state the UE is configured |
| UL TCI | Uplink TCI state the UE is configured |
| Separate | LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for DL TCI states and ltm-UL-TCI-StatesToAddModList for UL TCI states |
| Joint | LTM candidate configuration is configured with ltm-DL-OrJointTCI-StateToAddModList for joint TCI states for UL and DL operation |

<End of modified section>