3GPP TSG-RAN5 Meeting #106	R5-250090r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1502 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SIB19 for Inter-RAT TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To align the message content of SIB19 with TS 25.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SIB19 was updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect IE will exist in this message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: missing value/remark was added for vb80NonCriticalExtensions. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

4.4.4.1	Common contents of system information blocks for UTRA cells

-	System Information Block type 19

The system information block type 19 contains Inter-RAT frequency and priority information to be used in the cell.

Table 4.4.4.1-1: System Information Block type 19

| Derivation Path: 25.331 clause 11.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 0 (0dB) |  |  |
| s-PrioritySearch2 | Not present | default value is 0 |  |
| threshServingLow | 0 (0dB) |  |  |
| } |  |  |  |
| utran-FDD-FrequencyList | Not present |  |  |
| utran-TDD-FrequencyList | Not present |  |  |
| } |  |  |  |
| gsm-PriorityInfoList | Not present |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { | The same number of entries as the configured eutra carriers For Signalling test cases, see table 6.3.1.7-1 | n denotes the index of the entry |  |
| earfcn[n] | Downlink EARFCN under test For Signalling test cases, see table 6.3.1.7-1 |  |  |
| measurementBandwidth[n] | Set according to EUTRA band ChBw in Table 5.6-1 of TS36.104. |  |  |
| priority[n] | 4 |  |  |
| qRxLevMinEUTRA[n] | -53 (-106 dBm) | For signalling test cases |  |
|  | -70 (-140 dBm) | For RF/RRM test cases |  |
| threshXhigh[n] | 2 (4 dB) |  |  |
| threshXlow[n] | 1 (2 dB) |  |  |
| eutra-exclude-ListedCellList[n] | Not present |  |  |
| eutraDetection[n] | TRUE |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList | Not present |  | Band > 64 |
| v920NonCriticalExtensions SEQUENCE { |  |  | QBASED |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 7 (7dB) | default value is 0 |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| qQualMinEUTRA[n] | -20 (-20dB) |  |  |
| threshXhigh2[n] | 5 (5dB) |  |  |
| threshXlow2[n] | 5 (5dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| sysInfoType19-va80ext SEQUENCE { |  |  |  |
| multipleEutraFrequencyInfoList | Not present |  |  |
| } |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | Not present |  |  |
| } |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| gsmTreselectionScalingFactor | Not present |  |  |
| eutraTreselectionScalingFactor | Not present |  |  |
| numberOfApplicableEARFCN | 0 |  |  |
| eutra-FrequencyAndPriorityInfoList-vb50ext | Not present |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn | Downlink EARFCN under test For Signalling test cases, see table 6.3.1.7-1 |  |  |
| measurementBandwidth | Set according to EUTRA band ChBw in Table 5.6-1 of TS 36.104. |  |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -53 (-106 dBm) | For signalling test cases |  |
|  | -70 (-140 dBm) | For RF/RRM test cases |  |
| threshXhigh | 2 (4 dB) |  |  |
| threshXlow | 1 (2 dB) |  |  |
| qqualMinEUTRA | Not present |  |  |
| threshXhigh2 | Not present |  |  |
| threshXlow2 | Not present |  |  |
| eutra-exclude-ListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| multipleEutraFrequencyInfoExtensionList | Not present |  |  |
| eutra-FrequencyRACHReportingInfo | Not present |  |  |
| } |  |  |  |
| vb80NonCriticalExtensions | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  | Band > 64 AND NOT (QBASED) |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | Not present |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| qQualMinEUTRA[n] | Not present |  |  |
| threshXhigh2[n] | Not present |  |  |
| threshXlow2[n] | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-va80ext SEQUENCE { |  |  |  |
| multipleEutraFrequencyInfoList | Not present |  |  |
| } |  |  |  |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb30ext SEQUENCE { |  |  |  |
| cell-fach-meas-ind | Not present |  |  |
| } |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| gsmTreselectionScalingFactor | Not present |  |  |
| eutraTreselectionScalingFactor | Not present |  |  |
| numberOfApplicableEARFCN | 0 |  |  |
| eutra-FrequencyAndPriorityInfoList-vb50ext | Not present |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn | Downlink EARFCN under test For Signalling test cases, see table 6.3.1.7-1 |  |  |
| measurementBandwidth | Set according to EUTRA band ChBw in Table 5.6-1 of TS 36.104. |  |  |
| priority | 4 |  |  |
| qRxLevMinEUTRA | -53 (-106 dBm) | For signalling test cases |  |
|  | -70 (-140 dBm) | For RF/RRM test cases |  |
| threshXhigh | 2 (4 dB) |  |  |
| threshXlow | 1 (2 dB) |  |  |
| qqualMinEUTRA | Not present |  |  |
| threshXhigh2 | Not present |  |  |
| threshXlow2 | Not present |  |  |
| eutra-exclude-ListedCellList | Not present |  |  |
| eutraDetection | TRUE |  |  |
| } |  |  |  |
| multipleEutraFrequencyInfoExtensionList | Not present |  |  |
| eutra-FrequencyRACHReportingInfo | Not present |  |  |
| } |  |  |  |
| vb80NonCriticalExtensions | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| QBASED | See the definition below table 4.4.3.2-3. |
| Band > 64 | If band > 64 is selected |

[End of change]