3GPP TSG-RAN5 Meeting #106	R5-250094r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5403 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UP EDT TC 23.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To satisfy the test requirements in TP1 and TP2, steps 4 to 6 are not needed to avoid redundant steps. FFS in step 1 needs to be clarified. According to below core spec requirement, the edt-Parameters shall be specified in SIB2. 1> SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes edt-Parameters; According to below core spec requirement, the nextHopChainingCount shall be specified in RRCConnectionRelease. The nextHopChainingCount parameter is used upon handover, connection re-establishment, connection resume, UP-EDT and UP transmission using PUR by the UE when deriving the new KeNB that is used to generate KRRCint, KRRCenc and KUPenc (see TS 33.401 [32]). The RRC message contents need to be corrected based on TS 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Step 1 was updated, and step 0 was added to remove the FFS. Steps 4-6 are set to void. The RRC specific message contents were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 23.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: set earlySecurityReactivation-r16 to ture according to TS 36.331 Cl. 5.3.3.18. font in step 0 was updated. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

23.2.4	CIoT Optimization / User Plane / EDT

23.2.4.1	Test Purpose (TP)

(1)

with { UE with CIoT Optimization and with UP-EDT supported, with RRC Connection suspended }

ensure that {

when { EDT data transfer is triggered }

then { UE performs resumption of a suspended RRC connection by transmitting RRCConnectionResumeRequest and transfer the EDT user data on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH }

}

(2)

with { UE with UP-EDT supported initiated by sending RRCConnectionResumeRequest }

ensure that {

when { UE receives the RRCConnectionResume message }

then { UE performs UP-EDT fallback by sending the RRCConnectionResumeComplete message }

}

23.2.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300, clause 7.3b.3, TS 36.331, clause 5.3.1.1, 5.3.3.1b and 5.3.3.3a. Unless otherwise stated these are Rel-15 requirements.

[TS 36.331, clause 5.3.1.1]

In case of CP-EDT, the data are appended in the RRCEarlyDataRequest and RRCEarlyDataComplete messages, if available, and sent over SRB0. In case of UP-EDT, security is re-activated prior to transmission of RRC message using the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure and the radio bearers are re-established. The uplink data are transmitted ciphered on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. In the downlink, the data, if available, are transmitted on DTCH multiplexed with the RRCConnectionRelease message on DCCH. In response to a request for EDT, E-UTRAN may also choose to establish or resume the RRC connection.

[TS 36.331, clause 5.3.3.1b]

A BL UE, UE in CE or NB-IoT UE can initiate EDT when all of the following conditions are fulfilled:

1>	for CP-EDT, the upper layers request establishment of an RRC connection, the UE supports CP-EDT, and SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-EDT; or

1>	for UP-EDT, the upper layers request resumption of an RRC connection, the UE supports UP-EDT, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-EDT, and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure;

1>	the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess;

1>	the establishment or resumption request is suitable for EDT as specified in TS 36.300 [9], clause 7.3b.1;

1>	SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes edt-Parameters;

1>	the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in edt-TBS as specified in TS 36.321 [6], clause 5.1.1;

1>	EDT fallback indication has not been received from lower layers for this establishment or resumption procedure;

NOTE 1:	Upper layers request or resume an RRC connection. The interaction with NAS is up to UE implementation.

NOTE 2:	It is up to UE implementation how the UE determines whether the size of UL data is suitable for EDT.

[TS 36.331, clause 5.3.3.3a]

1>	if the UE is initiating UP-EDT in accordance with conditions in 5.3.3.1b:

2>	restore the PDCP state and re-establish PDCP entities for all SRBs and all DRBs;

2>	if drb-ContinueROHC has been provided in immediately preceding RRC connection release message, and the UE is requesting to resume RRC connection in the same cell:

3>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

3>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

2>	else:

3>	indicate to lower layers that stored UE AS context is used;

3>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

2>	resume all SRBs and all DRBs;

2>	derive the KeNB key based on the KASME key to which the current KeNB is associated, using the stored value of nextHopChainingCount received in the RRCConnectionRelease message in the preceding connection, as specified in TS 33.401 [32];

2>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

2>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

2>	configure lower layers to resume integrity protection using the previously configured algorithm and the KRRCint key derived in this clause to all subsequent messages received and sent by the UE;

2>	configure lower layers to resume ciphering and to apply the ciphering algorithm and the KRRCenc key derived in this clause to all subsequent messages received and sent by the UE;

2>	configure lower layers to resume ciphering and to apply the ciphering algorithm and the KUPenc key derived in this clause immediately to the user data sent and received by the UE;

2>	configure the lower layers to use EDT;

[TS 36.300, clause 7.3b.3]

EDT for User Plane CIoT EPS optimizations, as defined in TS 24.301 [20], is characterized as below:

-	The UE has been provided with a NextHopChainingCount in the RRCConnectionRelease message with suspend indication;

-	Uplink user data are transmitted on DTCH multiplexed with UL RRCConnectionResumeRequest message on CCCH;

-	Downlink user data are optionally transmitted on DTCH multiplexed with DL RRCConnectionRelease message on DCCH;

-	The short resume MAC-I is reused as the authentication token for RRCConnectionResumeRequest message and is calculated using the integrity key from the previous connection;

-	The user data in uplink and downlink are ciphered. The keys are derived using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous RRC connection;

-	The RRCConnectionRelease message is integrity protected and ciphered using the newly derived keys;

-	There is no transition to RRC CONNECTED.

The EDT procedure for User Plane CIoT EPS optimizations is illustrated in Figure 7.3b-2.

Figure 7.3b-2: EDT for User Plane CIoT EPS Optimizations

0.	Upon connection resumption request for Mobile Originated data from the upper layers, the UE initiates the early data transmission procedure and selects a random access preamble configured for EDT.

1.	The UE sends an RRCConnectionResumeRequest to the eNB, including its Resume ID, the establishment cause, and an authentication token. The UE resumes all SRBs and DRBs, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous connection and re-establishes the AS security. The user data are ciphered and transmitted on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH.

2.	The eNB initiates the S1-AP Context Resume procedure to resume the S1 connection and re-activate the S1-U bearers.

3.	The MME requests the S-GW to re-activate the S1-U bearers for the UE.

4.	The MME confirms the UE context resumption to the eNB.

5.	The uplink data are delivered to the S-GW.

6.	If downlink data are available, the S-GW sends the downlink data to the eNB.

7.	If no further data are expected from the S-GW, the eNB can initiate the suspension of the S1 connection and the deactivation of the S1-U bearers.

8.	The eNB sends the RRCConnectionRelease message to keep the UE in RRC_IDLE. The message includes the releaseCause set to rrc-Suspend, the resumeID, the NextHopChainingCount and drb-ContinueROHC which are stored by the UE. If downlink data were received in step 6, they are sent ciphered on DTCH multiplexed with the RRCConnectionRelease message on DCCH.

NOTE 1:	If the MME or eNB decides the UE to move in RRC_CONNECTED mode, RRCConnectionResume message is sent in step 7 to fall back to the RRC Connection resume procedure. In that case, the RRCConnectionResume message is integrity protected and ciphered with the keys derived in step 1 and the UE ignores the NextHopChainingCount included in the RRCConnectionResume message. Downlink data can be transmitted on DTCH multiplexed with the RRCConnectionResume message. In addition, an RRCConnectionSetup can also be sent in step 7 to fall back to the RRC Connection establishment procedure.

NOTE 2:	If neither RRCConnectionRelease nor, in case of fallback, RRCConnectionResume is received in response to RRCConnectionResumeRequest for EDT, the UE considers the UL data transmission not successful.

23.2.4.3	Test description

23.2.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

-	System information combination 1 as defined in TS 36.508 [18] clause 4.4.3.1 is used in Cell 1;

UE:

-	User Plane CIoT Optimisation is supported by UE (UE capability), pc_User_Plane_CIoT_Optimisation.

- User Plane Early Data Transmission is supported by the UE.

- The UE is configured to utilise EDT as transport mechanism for user data of 1 octet.

Preamble:

-	UE is in State 4A-UP, Loopback Activation User Plane according to TS 36.508 [18].

23.2.4.3.2	Test procedure sequence

Table 23.2.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits 1 IP packet to the UE. | <-- | IP packet |  |  |
| 1 | The SS transmits an RRCConnectionRelease message including resumeIdentity and rrc-Suspend as releaseCause. | <-- | RRC: RRCConnectionRelease | - | - |
| 2 | Void . | - | - | - | - |
| 3 | Check: Does the UE transmit an RRCConnectionResumeRequest message including the resumeIdentity and AS security context stored at Step 1 and transmit the EDT data on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH? | --> | RRC: RRCConnectionResumeRequest On DTCH: EDT data | 1 | P |
| 4-6 | Void | - | - | - | - |
| 7 | The SS transmits an RRCConnectionResume message. | <-- | RRC: RRCConnectionResume | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionResumeComplete message? | --> | RRC: RRCConnectionResumeComplete | 2 | P |

23.2.4.3.3	Specific message contents

Table 23.2.4.3.3-0: CLOSE UE TEST LOOP (preamble)

| Derivation path: TS 36.508 [18] Table 4.7A-3 with condition UE TEST LOOP MODE B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode B LB setup |  |  |  |
| IP PDU delay | '00000011'B | 3 seconds delay |  |

Table 23.2.4.3.3-1: SystemInformationBlockType2 (preamble and all steps of Table 23.2.4.3.2-1)

| Derivation Path: TS 36.508 [18] clause 4.4.3.3-1 with condition CIoT-test |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { | RACH-ConfigCommon-DEFAULT according to TS 36.508 [18], Table 4.6.3-12 with condition EDT |  |  |
| } |  |  |  |
| cIoT-EPS-OptimisationInfo-r13 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF CIOT-OptimisationPLMN-r13 { | 1 entry |  |  |
| CIOT-OptimisationPLMN-r13 SEQUENCE { |  | entry 1 |  |
| cp-CIoT-EPS-Optimisation-r13 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| cp-EDT-r15 | Not present |  |  |
| up-EDT-r15 | true |  |  |
| idleModeMeasurements-r15 | Not present |  |  |
| reducedCP-LatencyEnabled-r15 | Not present |  |  |
| rlos-Enabled-r16 | Not present |  |  |
| earlySecurityReactivation-r16 | true |  |  |
| cp-EDT-5GC-r16 | Not present |  |  |
| up-EDT-5GC-r16 | Not present |  |  |
| cp-PUR-EPC-r16 | Not present |  |  |
| up-PUR-EPC-r16 | Not present |  |  |
| cp-PUR-5GC-r16 | Not present |  |  |
| up-PUR-5GC-r16 | Not present |  |  |
| mpdcch-CQI-Reporting-r16 | Not present |  |  |
| rai-ActivationEnh-r16 | Not present |  |  |
| idleModeMeasurementsNR-r16 | Not present |  |  |
| } |  |  |  |

Table 23.2.4.3.3-2: RRCConnectionRelease (step 1, table 23.2.4.3.2-1)

| Derivation Path: TS 36.331 [17] clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| releaseCause | rrc-Suspend-v1320 |  |  |
| redirectedCarrierInfo | Not present |  |  |
| idleModeMobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| resumeIdentity-r13 | BIT STRING (SIZE(40)) | Any value |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nextHopChainingCount-r15 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 23.2.4.3.3-3: RRCConnectionResumeRequest (step 3, Table 23.2.4.3.2-1)

| Derivation Path: TS 36.331 [17] clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResumeRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionResumeRequest-r13 SEQUENCE { |  |  |  |
| resumeIdentity-r13 CHOICE { |  |  |  |
| resumeID-r13 | BIT STRING (SIZE(40)) | Same value as Step 1 |  |
| } |  |  |  |
| shortResumeMAC-I-r13 | The same value as the 16 least significant bits of the MAC-I value calculated by SS. |  |  |
| resumeCause-r13 | mo-Data |  |  |
| spare | Present but contents not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 23.2.4.3.3-4: Void

[End of change]