3GPP TSG-RAN WG5 Meeting #106	R5-250144r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3155 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to R18 Coverage enh AMPR TC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - R18 Coverage enh AMPR TC needs to be updated considering ΔPPowerBoost impact. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NS_04 test configuration table and test requirement have been updated. - Remove the editor note of R18 coverage enh. - Fix the PCMAX value of test point 14 of NS_04 PC2 in Table 6.2.3.5-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Update test point 65 and 71 RB allocation to EDGE_1RB_Right due to high range frequency. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

<< Unchanged Parts Skipped >>

Table 6.2.3.4.1-2: Test Configuration table for NS_04 (Power Class 1.5, 2 and 3)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 (Note 3, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 3 (Note 3) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 4 (Note 3) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 5 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 6 (Note 3) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 7 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 8 (Note 4, 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 9 (Note 4) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 10 (Note 4) | Low |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 11 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 12 (Note 4) | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 14 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 16 | Low |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 17 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 18 | High |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 23 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 25 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 26 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 27 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 28 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 29 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 30 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 31 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 32 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 33 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 34 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 35 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 36 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 37 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 38 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 39 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 40 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 41 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 42 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 43 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 44 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 45 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 46 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 47 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 48 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 49 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |

| A-MPR test parameters for NS_04 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 50 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 51 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 52 | Low |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 53 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 54 | High |  |  |  | CP-OFDM 64 QAM | Outer Full |
| 55 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 56 (Note 5) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 57 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 58 | Low |  |  |  | CP-OFDM 256 QAM | Outer Full |
| 59 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 60 | High |  |  |  |  |  | CP-OFDM 256 QAM |  | Outer Full |  |
| 61 (Note 6) | Low |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |  |
| 62 (Note 6) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |  |
| 63 (Note 6) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |  |
| 64 (Note 6) | Low |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |  |
| 65 (Note 6) | High |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |  |
| 66 (Note 6) | High |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |  |
| 67 (Note 7) | Low |  |  |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |  |
| 68 (Note 7) | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |  |
| 69 (Note 7) | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |  |
| 70 (Note 7) | Low |  |  |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |  |
| 71 (Note 7) | High |  |  |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |  |
| 72 (Note 7) | High |  |  |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth NOTE 6: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost for band n41. NOTE 7: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost for band n41. |  |  |  |  |  |  |

<< Unchanged Parts Skipped >>

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | ΔPPowerBoost (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26 | 3.5 | 5.5 | 1.5 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 8 | 26 | 3.5 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 9 | 26 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 10 | 26 | 0.5 | 3.5 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 11 | 26 | 3.5 | 0 | 1.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 12 | 26 | 0.5 | 0 | 0 | 0 | 25.5 | 2.0 | 3 | 28+TT | 22.5-TT |
| 13 | 26 | 3.5 | 6 | 1.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 14 | 26 | 3.5 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 15 | 26 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 16 | 26 | 1 | 4.5 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 17 | 26 | 3.5 | 0 | 1.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 18 | 26 | 1 | 0 | 0 | 0 | 25 | 2.0 | 3 | 28+TT | 22-TT |
| 19 | 26 | 3.5 | 6 | 1.5 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 20 | 26 | 3.5 | 5 | 0 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 21 | 26 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 22 | 26 | 2 | 5 | 0 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 23 | 26 | 3.5 | 0 | 1.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 24 | 26 | 2 | 0 | 0 | 0 | 24 | 2.0 | 3 | 28+TT | 21-TT |
| 25 | 26 | 3.5 | 6.5 | 1.5 | 0 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 26 | 26 | 3.5 | 5 | 0 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 27 | 26 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 28 | 26 | 2.5 | 5 | 0 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 29 | 26 | 3.5 | 0 | 1.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 30 | 26 | 2.5 | 0 | 0 | 0 | 23.5 | 2.0 | 3 | 28+TT | 20.5-TT |
| 31 | 26 | 4.5 | 8 | 1.5 | 0 | 16.5 | 5.0 | 3 | 28+TT | 11.5-TT |
| 32 | 26 | 4.5 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 33 | 26 | 4.5 | 0 | 0 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 34 | 26 | 4.5 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 35 | 26 | 4.5 | 0 | 1.5 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 36 | 26 | 4.5 | 0 | 0 | 0 | 21.5 | 2.0 | 3 | 28+TT | 18.5-TT |
| 37 | 26 | 3.5 | 7.5 | 1.5 | 0 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 38 | 26 | 3.5 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 39 | 26 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 3 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 41 | 26 | 3.5 | 0 | 1.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 42 | 26 | 3 | 0 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 43 | 26 | 3.5 | 7.5 | 1.5 | 0 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 44 | 26 | 3.5 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 45 | 26 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 46 | 26 | 3 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 47 | 26 | 3.5 | 0 | 1.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 48 | 26 | 3 | 0 | 0 | 0 | 23 | 2.0 | 3 | 28+TT | 20-TT |
| 49 | 26 | 3.5 | 7.5 | 1.5 | 0 | 17 | 5.0 | 3 | 28+TT | 12-TT |
| 50 | 26 | 3.5 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 51 | 26 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 3.5 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 53 | 26 | 3.5 | 0 | 1.5 | 0 | 21 | 2.0 | 3 | 28+TT | 18-TT |
| 54 | 26 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 3 | 28+TT | 19.5-TT |
| 55 | 26 | 6.5 | 10 | 1.5 | 0 | 14.5 | 5.0 | 3 | 28+TT | 9.5-TT |
| 56 | 26 | 6.5 | 7.5 | 0 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 57 | 26 | 6.5 | 0 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 58 | 26 | 6.5 | 7.5 | 0 | 0 | 18.5 | 4.0 | 3 | 28+TT | 14.5-TT |
| 59 | 26 | 6.5 | 0 | 1.5 | 0 | 18 | 4.0 | 3 | 28+TT | 14-TT |
| 60 | 26 | 6.5 | 0 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 614 | 26 | 3.5 | 5.5 | 1.5 | 0.5 | 19.5 | 3.5 | 3 | 28.5+TT | 16 |
| 624 | 26 | 3.5 | 3.5 | 0 | 0.5 | 23 | 2 | 3 | 28.5+TT | 20 |
| 634 | 26 | 3.5 | 0 | 0 | 0.5 | 23 | 2 | 3 | 28.5+TT | 20 |
| 644 | 26 | 0.5 | 3.5 | 0 | 0.5 | 23 | 2 | 3 | 28.5+TT | 20 |
| 654 | 26 | 3.5 | 0 | 1.5 | 0.5 | 21.5 | 2 | 3 | 28.5+TT | 18.5 |
| 664 | 26 | 0.5 | 0 | 0 | 0.5 | 26 | 2 | 3 | 28.5+TT | 23 |
| 675 | 26 | 3.5 | 6 | 1.5 | 0.5 | 19 | 3.5 | 3 | 28.5+TT | 15.5 |
| 685 | 26 | 3.5 | 4.5 | 0 | 0.5 | 22 | 2 | 3 | 28.5+TT | 19 |
| 695 | 26 | 3.5 | 0 | 0 | 0.5 | 23 | 2 | 3 | 28.5+TT | 20 |
| 705 | 26 | 1 | 4.5 | 0 | 0.5 | 22 | 2 | 3 | 28.5+TT | 19 |
| 715 | 26 | 3.5 | 0 | 1.5 | 0.5 | 21.5 | 2 | 3 | 28.5+TT | 18.5 |
| 725 | 26 | 1 | 0 | 0 | 0.5 | 25.5 | 2 | 3 | 28.5+TT | 22.5 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. NOTE 4: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost for band n41. NOTE 5: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost for band n41. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_04) for band n41 (Step 3) and UE Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | ΔPPowerBoost (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 5.5 | 1.5 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 2 | 23 | -3 | 3.5 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 3 | 23 | -3 | 3.5 | 0 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 4 | 23 | -3 | 1.2 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 5 | 23 | -3 | 3.5 | 0 | 1.5 | 0 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 0 | 24.8 | 2.0 | 2 | 28+TT | 22.8-TT |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |
| 8 | 23 | 0 | 0.5 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 9 | 23 | 0 | 0.5 | 0 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 10 | 23 | 0 | 0.5 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 11 | 23 | 0 | 0.5 | 0 | 1.5 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 0 | 22.5 | 2.0 | 2 | 25+TT | 20.5-TT |
| 13 | 23 | 0 | 1 | 4 | 1.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 14 | 23 | 0 | 1 | 4 | 0 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 15 | 23 | 0 | 1 | 0 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 16 | 23 | 0 | 1 | 4 | 0 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 17 | 23 | 0 | 1 | 0 | 1.5 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 18 | 23 | 0 | 1 | 0 | 0 | 0 | 22 | 2.0 | 2 | 25+TT | 20-TT |
| 19 | 23 | 0 | 2 | 4 | 1.5 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |
| 20 | 23 | 0 | 2 | 4 | 0 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | 23 | 0 | 2 | 0 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 22 | 23 | 0 | 2 | 4 | 0 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 23 | 23 | 0 | 2 | 0 | 1.5 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 24 | 23 | 0 | 2 | 0 | 0 | 0 | 21 | 2.0 | 2 | 25+TT | 19-TT |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.5 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 26 | 23 | 0 | 2.5 | 4 | 0 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 27 | 23 | 0 | 2.5 | 0 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 28 | 23 | 0 | 2.5 | 4 | 0 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 29 | 23 | 0 | 2.5 | 0 | 1.5 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 0 | 20.5 | 2.5 | 2 | 25+TT | 18-TT |
| 31 | 23 | 0 | 4.5 | 6 | 1.5 | 0 | 15.5 | 5.0 | 2 | 25+TT | 10.5-TT |
| 32 | 23 | 0 | 4.5 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 4.5 | 0 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 4.5 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 35 | 23 | 0 | 4.5 | 0 | 1.5 | 0 | 17 | 5.0 | 2 | 25+TT | 12-TT |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 1.5 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |
| 38 | 23 | 0 | 3 | 5.5 | 0 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | ΔPPowerBoost (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39 | 23 | 0 | 3 | 0 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |  |
| 40 | 23 | 0 | 3 | 5.5 | 0 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |  |
| 41 | 23 | 0 | 3 | 0 | 1.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |  |
| 42 | 23 | 0 | 3 | 0 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |  |
| 43 | 23 | 0 | 3 | 5.5 | 1.5 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |  |
| 44 | 23 | 0 | 3 | 5.5 | 0 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |  |
| 45 | 23 | 0 | 3 | 0 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |  |
| 46 | 23 | 0 | 3 | 5.5 | 0 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |  |
| 47 | 23 | 0 | 3 | 0 | 1.5 | 0 | 18.5 | 4.0 | 2 | 25+TT | 14.5-TT |  |
| 48 | 23 | 0 | 3 | 0 | 0 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |  |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.5 | 0 | 16 | 5.0 | 2 | 25+TT | 11-TT |  |
| 50 | 23 | 0 | 3.5 | 5.5 | 0 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |  |
| 51 | 23 | 0 | 3.5 | 0 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |  |
| 52 | 23 | 0 | 3.5 | 5.5 | 0 | 0 | 17.5 | 5.0 | 2 | 25+TT | 12.5-TT |  |
| 53 | 23 | 0 | 3.5 | 0 | 1.5 | 0 | 18 | 4.0 | 2 | 25+TT | 14-TT |  |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |  |
| 55 | 23 | 0 | 6.5 | 8 | 1.5 | 0 | 13.5 | 5.0 | 2 | 25+TT | 8.5-TT |  |
| 56 | 23 | 0 | 6.5 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |  |
| 57 | 23 | 0 | 6.5 | 0 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |  |
| 58 | 23 | 0 | 6.5 | 6.5 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |  |
| 59 | 23 | 0 | 6.5 | 0 | 1.5 | 0 | 15 | 5.0 | 2 | 25+TT | 10-TT |  |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 0 | 16.5 | 5.0 | 2 | 25+TT | 11.5-TT |  |
| 614 | 23 | 0 | 0.5 | 3.5 | 1.5 | 1 | 19 | 3.5 | 2 | 26+TT | 15.5-TT |  |
| 624 | 23 | 0 | 0.5 | 3.5 | 0 | 1 | 20.5 | 2.5 | 2 | 26+TT | 18-TT |  |
| 634 | 23 | 0 | 0.5 | 0 | 0 | 1 | 23.5 | 2.0 | 2 | 26+TT | 21.5-TT |  |
| 644 | 23 | 0 | 0.5 | 3.5 | 0 | 1 | 20.5 | 2.5 | 2 | 26+TT | 18-TT |  |
| 654 | 23 | 0 | 0.5 | 0 | 1.5 | 1 | 22 | 2.0 | 2 | 26+TT | 20-TT |  |
| 664 | 23 | 0 | 0.5 | 0 | 0 | 1 | 23.5 | 2.0 | 2 | 26+TT | 21.5-TT |  |
| 675 | 23 | 0 | 1 | 4 | 1.5 | 1 | 18.5 | 4.0 | 2 | 26+TT | 14.5-TT |  |
| 685 | 23 | 0 | 1 | 4 | 0 | 1 | 20 | 2.5 | 2 | 26+TT | 17.5-TT |  |
| 695 | 23 | 0 | 1 | 0 | 0 | 1 | 23 | 2.0 | 2 | 26+TT | 21-TT |  |
| 705 | 23 | 0 | 1 | 4 | 0 | 1 | 20 | 2.5 | 2 | 26+TT | 17.5-TT |  |
| 715 | 23 | 0 | 1 | 0 | 1.5 | 1 | 21.5 | 2.0 | 2 | 26+TT | 19.5-TT |  |
| 725 | 23 | 0 | 1 | 0 | 0 | 1 | 23 | 2.0 | 2 | 26+TT | 21-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Void NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. NOTE 4: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost for band n41. NOTE 5: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is enabled, A-MPR, if larger than zero, is increased by ΔPPowerBoost for band n41. |  |  |  |  |  |  |  |  |  |  |  |  |

<< END OF CHANGES >>