# 3GPP TSG-RAN5 Meeting #106	R5-250153r1

# Athens, Greece, Feb 17th - Feb 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4810 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 8.1.5.10.10 for R18 IDC enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IDC_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of a new IDC TC 8.1.5.10.10 UE Assistance Information / IDC / TDM assistance configuration for R18 IDC enhancement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new TC 8.1.5.10.10 UE Assistance Information / IDC / TDM assistance configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.10.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-2 CR 0542 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Add a new Time instances “T3” in Table 8.1.5.10.10.3.2-1, and a new step 9 in test procedure to ensure UE connecting to WLAN AP2 after handover to cell 1, as the handover procedure may also waste some time that affects UE connecting to WLAN AP2. Reordering the test procedure. |  |  |  |  |  |  |  |  |

<Start of modified section>

8.1.5.10.10	UE Assistance Information / IDC / TDM assistance configuration

8.1.5.10.10.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE configured to provide IDC assistance information based on idc-TDM-AssistanceConfig and UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information }

}

(2)

with { UE in NR RRC_CONNECTED state and UE already transmits a UEAssistanceInformation message with idc-TDM-Assistance }

ensure that {

when { UE configured to provide IDC assistance information based on idc-TDM-AssistanceConfig and the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message and UE is experiencing IDC problems that it cannot solve by itself }

then { The UE transmits a UEAssistanceInformation message to provide IDC assistance information }

}

8.1.5.10.10.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in TS 38.331, clause 5.7.4.2 and 5.4.7.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.7.4.2]

Upon initiating the procedure, the UE shall:

1>	if configured to provide delay budget report:

……

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

1>	if configured to provide IDC assistance information based on idc-TDM-AssistanceConfig included in idc-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or NR-DC combination comprising of carrier frequencies included in candidateServingFreqListNR or frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

……

[TS 38.331, clause 5.7.4.3]

The UE shall set the contents of the UEAssistanceInformation message as follows:

……

1>	if transmission of the UEAssistanceInformation message is initiated to provide IDC assistance information according to 5.7.4.2 or 5.3.5.3:

……

2>	if there is at least one affected frequency range overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or NR-DC combinations comprising of candidate frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or NR-DC combination overlapping with one candidate frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the candidate frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or NR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or NR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one candidate carrier frequency included in candidateServingFreqListNR or candidate frequency range included in candidateServingFreqRangeListNR or one supported UL CA or NR-DC combination comprising of candidate carrier frequencies included in candidateServingFreqListNR or candidate frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself, and affectedCarrierFreqList or affectedCarrierFreqCombList or affectedCarrierFreqRangeList or affectedCarrierFreqRangeCombList is included, and idc-TDM-AssistanceConfig is set to setup:

3>	include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems;

……

8.1.5.10.10.3	Test description

8.1.5.10.10.3.1	Pre-test conditions

System Simulator:

-	NR Cell 14/NRf4 (2390-2400MHz for band n40) and WLAN AP 1 (Cell 27) as specified in TS 38.508-1[4] clause 4.13.

-	NR Cell 1/NRf1 (2496-2556MHz for band n41) and WLAN AP 2 (Cell 27b) as specified in TS 38.508-1[4] clause 4.13.

UE:

-	UE’s WLAN is on.

Preamble:

-	The UE is in state 3N-A on NR Cell 14 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

8.1.5.10.10.3.2	Test procedure sequence

Table 8.1.5.10.10.3.2-1 for FR1 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", “T2” and "T3" are applied at the points indicated in the Main behaviour description in Table 8.1.5.10.10.3.2-2.

Table 8.1.5.10.10.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 14 | WLAN AP 1 (Cell 27) | NR Cell 1 | WLAN AP 2 (Cell 27b) | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Off |  | Cell configuration for non-IDC. |
|  | BeaconRSSI | dBm |  | Off |  | Off |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | Off |  | Cell configuration for IDC in NR Cell 14. |
|  | BeaconRSSI | dBm |  | -80 |  | Off |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | -79 |  | Cell configuration for non-IDC in NR Cell 1. |
|  | BeaconRSSI | dBm |  | Off |  | Off |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 |  | -79 |  | Cell configuration for IDC in NR Cell 1. |
|  | BeaconRSSI | dBm |  | Off |  | -80 |  |

Table 8.1.5.10.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a RRCReconfiguration message including OtherConfig with idc-FDM-AssistanceConfig-r18 including CandidateServingFreqRangeListNR-r18 including center frequency and bandwidth for NR Cell 14 and NR Cell 1, and idc-TDM-AssistanceConfig-r18. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS re-adjusts the power levels according to row "T1" in Table 8.1.5.10.10.3.2-1. | - | - | - | - |
| 4 | The UE associates with the WLAN AP1 and obtains the local IP address of WLAN AP1. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T1” in RRC_CONNECTED in NR Cell 14. | - | - | - | - |
| 5 | Check: Does the UE transmit an an UEAssistanceInformation message including idc-TDM-Assistance-r18? | --> | NR RRC: UEAssistanceInformation | 1 | P |
| 6 | The SS re-adjusts the power levels according to row "T2" in Table 8.1.5.10.10.3.2-1. | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message including to order the UE to perform inter-frequency handover to NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmit RRCReconfigurationComplete message in NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | The SS re-adjusts the power levels according to row "T3" in Table 8.1.5.10.10.3.2-1. | - | - | - | - |
| 10 | The UE associates with the WLAN AP2 and obtains the local IP address of WLAN AP2. Note: The UL signal to WLAN AP shall generate IDC problem. The UE shall detect IDC problem with “T2” in RRC_CONNECTED in NR Cell 1. | - | - | - | - |
| 11 | Check: Does the UE transmit an an UEAssistanceInformation message including idc-TDM-Assistance-r18, which information is different from the information in Step 5? | --> | NR RRC: UEAssistanceInformation | 2 | P |

8.1.5.10.10.3.3	Specific message contents

Table 8.1.5.10.10.3.3-1: RRCReconfiguration (step 1, Table 8.1.5.10.10.3.2-2)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtensionotherConfig-v1610 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| otherConfig-v1800 SEQUENCE { |  |  |  |  |  |  |  |
| idc-AssistanceConfig-v1800 CHOICE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| idc-FDM-AssistanceConfig-r18 CHOICE { |  |  |  |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| candidateServingFreqRangeListNR-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF CandidateServingFreqRangeNR-r18 { |  | 2 entries |  |  |  |  |  |
| CandidateServingFreqRangeNR-r18[1] SEQUENCE { |  |  |  |  |  |  |  |
| candidateCenterFreq-r18 |  | ARFCN-ValueNR of NR Cell 14 |  |  |  |  |  |
| candidateBandwidth-r18 |  | mhz10 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| CandidateServingFreqRangeNR-r18[2] SEQUENCE { |  |  |  |  |  |  |  |
| candidateCenterFreq-r18 |  | ARFCN-ValueNR of NR Cell 1 |  |  |  |  |  |
| candidateBandwidth-r18 |  | mhz60 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| idc-TDM-AssistanceConfig-r18 |  | setup |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.5.10.10.3.3-2: UEAssistanceInformation (step 5, Table 8.1.5.10.10.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-FDM-Assistance-r18 SEQUENCE { |  |  |  |
| affectedCarrierFreqRangeList-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18 { | 1 entry |  |  |
| AffectedCarrierFreqRange-r18 SEQUENCE { |  |  |  |
| affectedFreqRange-r18 SEQUENCE { |  |  |  |
| centerFreq-r18 | Present |  |  |
| affectedBandwidth-r18 | Present |  |  |
| } |  |  |  |
| interferenceDirection-r18 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| idc-TDM-Assistance-r18 SEQUENCE { |  | The actual IE values depend on UE reporting |  |
| cycleLength-r18 | Present |  |  |
| startOffset-r18 | Present |  |  |
| slotOffset-r18 | Present |  |  |
| activeDuration-r18 | Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.10.3.3-3: RRCReconfiguration (step 7, Table 8.1.5.10.10.3.2-2)

| Derivation Path: TS 38.508-1 [4] 4.8.1-1A with condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.10.3.3-4: CellGroupConfig (Table 8.1.5.10.10.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical cell Id of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5.10.10.3.3-5: UEAssistanceInformation (step 11, Table 8.1.5.10.10.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-30 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEAssistanceInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueAssistanceInformation SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| overheatingAssistance | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idc-FDM-Assistance-r18 SEQUENCE { |  |  |  |
| affectedCarrierFreqRangeList-r18 SEQUENCE (SIZE (1..maxFreqIDC-r16)) OF AffectedCarrierFreqRange-r18 { | 1 entry |  |  |
| AffectedCarrierFreqRange-r18 SEQUENCE { |  |  |  |
| affectedFreqRange-r18 SEQUENCE { |  |  |  |
| centerFreq-r18 | Present |  |  |
| affectedBandwidth-r18 | Present |  |  |
| } |  |  |  |
| interferenceDirection-r18 | nr |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| idc-TDM-Assistance-r18 SEQUENCE { |  | This entry only exists when at least one IE value is different from the IE values in step 5. Otherwise, this entry will not exist. |  |
| cycleLength-r18 | Present |  |  |
| startOffset-r18 | Present |  |  |
| slotOffset-r18 | Present |  |  |
| activeDuration-r18 | Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>