3GPP TSG-RAN5 Meeting #106	R5-250292r2

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	5.3.16.9

Source:	Keysight Technologies, Anritsu

Title:	Discussion on MU and TT for ATG RF testing

Document for:	Discussion and Endorsement

# Introduction

The aim of this paper is to continue MU and TT analysis for Air-To-Ground (ATG) devices initiated in [1]. In particular, this paper will resubmit proposals made for OFF power test case and Out-of-band blocking test case, with certain updates after consensus of multiple TE vendors, and it will also do the analysis and proposals for the power control, frequency error, the spectrum emission mask and the spurious test cases.

# Discussion

## Background

Proposals 1 to 8 made in [1] were endorsed in previous meeting. From them, proposals 1 to 4 were generic while proposals 5 to 8 were test case specific. Therefore, to help understanding the proposals to be made in this document, new or resubmitted, it is worth to recover here those endorsed proposals that will condition them:

Proposal 1. In the dynamic range analysis of the test systems to support ATG, the discrete value of the nominal attenuation and the tolerance in the frequency response of the extra attenuators need to be considered. Proposal 2. If a test system requires extra attenuation to run ATG test cases, RAN5 shall consider the highest required attenuation to be added to all ATG test cases. That is, the same level of extra attenuation will be used for the MU and TT analysis of all the ATG test cases. Proposal 3. Create an AP to ATG UE manufacturers to clarify the expected implementation in terms of number of omni-directional antennas or, in case of using an antenna array, the number of TAB connectors. Proposal 4. While it is unclear the number of UL connectors for ATG, for requirements that apply to the sum of the mean power from all antenna connectors or all TAB connectors, limit MTSU definition for ATG assuming no more than 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, otherwise MTSU will be FFS.

## Test cases specific analysis

#### OFF power test case

- With all the background presented and following the endorsed proposals, we have analyzed the test requirement in our test systems and concluded that, for ATG UEs with maximum nominal power in the range {31dBm (PC1) < maxOutputPowerATG-r18 ≤ 40dBm}, MTSU and TT should be increased, and even certain relaxation would be required depending on the CBW to test compared to regular OFF power test case 6.3.2, otherwise, values from test case 6.3.2 can apply, assuming required attenuators remain connected to enable testing automation and test system protection of ATG devices.

| - MTSU proposal | - TT Proposal | - Relaxation Proposal |
| --- | --- | --- |
| For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): Same as 6.3.2 for each UE antenna/TAB connector Otherwise, for each UE antenna/TAB connector: f ≤ 3.0GHz ±2.15 dB, BW ≤ 40MHz ±2.35 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 4.2GHz ±2.45 dB, BW ≤ 40MHz ±2.55 dB, 40MHz < BW ≤ 80MHz ±2.85 dB, 80MHz < BW ≤ 100MHz 4.2GHz < f ≤ 6.0GHz ±2.65 dB, BW ≤ 20MHz ±2.75 dB, 20MHz < BW ≤ 80MHz ±2.85 dB, 80MHz < BW ≤ 100MHz | For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): Same as 6.3.2 for each UE antenna/TAB connector Otherwise, for each UE antenna/TAB connector: f ≤ 3.0GHz 2.15 dB, BW ≤ 40MHz 2.35 dB, 40MHz < BW ≤ 100MHz 3.0GHz < f ≤ 6.0GHz 2.45 dB, BW ≤ 100MHz | For ATG UEs with maxOutputPowerATG-r18 up to 31dBm (up to PC1): 0dB Otherwise: 3dB, BW ≤ 25MHz 6dB, 25MHz < BW ≤ 50MHz 9dB, 50MHz < BW ≤ 100MHz |

Table 2.2.1-1. MTSU, TT and relaxations proposal for ATG OFF power test case 6.3J.2.

- Proposal 1. For ATG OFF power test case 6.3J.2, define MTSU, TT and relaxation according to proposal in Table 2.2.1-1.

#### Out-of-band blocking test case

This is the test case requiring the highest power in DL, in particular, for CW signal, required power in Range 3 is -15dBm at DUT plane operating at frequencies from 1MHz up to 12.75GHz. This is quite remarkable high DL power compared to other test cases. For example, it is 15dB higher than the highest DL power required for ATG Maximum input level test 7.4J. Therefore, the presence of any extra attenuation would have a significant impact on the maximum power a test system can transmit at DUT plane.

After evaluating our test systems under all premises presented in section 2.1, we conclude that, for ATG UEs with maximum nominal power in the range {31dBm (PC1) < maxOutputPowerATG-r18 ≤ 40dBm}, on the need of certain relaxation to apply to the CW power. In particular 9dB.

In this type of test cases, the most important is the power difference between the interferer and the wanted signal so if CW signal power level is decreased by certain relaxation, the wanted signal power level should be decreased accordingly. However, the required wanted signal power level is P ≤ REFSENS +9dB. In no case, wanted signal should be set below REFSENS. Therefore, in those cases, either should not be tested for Range 3 or tested with less power reduction in the wanted signal. Given only highest channel BW will be tested for Range 3, in practice there will no deviation in the delta power between the interferer and the wanted signal vs the delta required, we propose to test them in the way captured in Table 2.2.2-1.

| - Relaxation Proposal - (CW) | - Relaxation Proposal - (Wanted Signal) |
| --- | --- |
| For Range 3 for ATG UEs with maxOutputPowerATG-r18 greater than 31dBm (greater than PC1): 9dB Otherwise: 0dB | For Range 3 for ATG UEs with maxOutputPowerATG-r18 greater than 31dBm (greater than PC1): 6dB, BW ≤ 10MHz 7dB, BW = 15MHz 9dB, 15MHz < BW ≤ 100MHz Otherwise: 0dB |

Table 2.2.2-1. Relaxations proposal for ATG Out-of-band blocking test case 7.6J.3, Range 3.

- Proposal 2. For ATG Out-of-band blocking test case 7.6J.3, Range 3, apply relaxations according to proposal in Table 2.2.2-1.

#### Spurious Emissions test cases

We have been evaluating our test systems with all the considerations and endorsed proposals made in [1] and concluded that for spurious test cases for ATG, MU and TT can be leveraged from the equivalent non-ATG tests limiting, in case of Spurious Tx test, to ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors. Such a condition does not apply to Rx Spurious test case where requirements apply per connector.

Proposal 3. For ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, in ATG General Spurious test 6.5.J.3.1, define MU and TT same as defined for General Spurious test 6.5.3.1.

Proposal 4. For ATG UEs, in ATG Rx Spurious test 7.9J, define MU and TT same as defined for Rx Spurious test 7.9.

#### Power control test cases

During last meeting, MU and TT were defined for ATG power control test cases under section 6.3J.4. Given there was no previous analysis from any test vendor, MU and TT values were left in [ ]. After detailed analysis in our test systems, we conclude that values can be accepted and the [ ] removed.

Proposal 5. For ATG UEs, for ATG power control test cases defined in section 6.3J.4, accept current MU and TT values by removing the [ ].

#### Frequency error test case

Frequency error test for ATG is expected to be defined during this meeting. For the analysis of the required test system MU, we have considered 2 aspects: the need of extra attenuation as per background captured in section 2.1 and the minimum requirement definition.

We consider that the presence of extra attenuators will not impact the frequency error measurement uncertainty.

Regarding the frequency error requirement definition for ATG in section 6.4J.1 [2] vs requirement for regular non-ATG in section 6.4.1 [2], it can be checked there are certain differences  at UE side regarding the UE ‘pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift’. However, such a requirement or clarification at UE side is also made for NTN devices and it is not considered to have an impact on measurement uncertainty and so frequency error measurement uncertainty for NTN devices defined in [4] directly reuses MU definition made for regular NR devices for test case 6.4.1 in [3].

Therefore, for measurement frequency error for ATG UEs in test case 6.4J.1, it is proposed to reuse MU and TT defined for regular non-ATG UEs in frequency error test 6.4.1.

Proposal 6. For ATG UEs, in ATG frequency error test 6.4J.1, define MU and TT same as defined for frequency error test 6.4.1.

#### Spectrum emission mask test case

Spectrum emission mask (SEM) test for ATG is expected to be defined during this meeting. For the analysis of the required test system MU, we have considered 2 aspects: the need of extra attenuation as per background captured in section 2.1 and the minimum requirement definition.

Regarding the requirements definition for SEM for ATG in section 6.5J.2.2 [2], they are the same as those defined for non-ATG UEs in section 6.5.2.2[2] with the peculiarity that in case ATG, ‘if the actual transmission power of ATG UE is larger than 31dBm, the requirements of spectrum emission mask in clause 6.5.2.2 shall be relaxed with scaling factor equal to (the actual transmission power minus 31) dB’. That will limit the relative ratio of the maximum power of the UE vs the minimum power required to be measured during the test. In addition, the SEM requirement for ATG UEs applies to ‘the sum of the emissions from all UE transmit antenna connectors or all TAB connectors’.

In such a scenario, we have evaluated our test systems and concluded that, for ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, without any restriction in the DUT maximum output power, MU and TT for SEM ATG test case 6.5J.2.2 can be defined same as defined for SEM test case 6.5.2.2.

Proposal 7. For ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, MU and TT for SEM ATG test case 6.5J.2.2 can be defined same as defined for SEM test case 6.5.2.2.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1. For ATG OFF power test case 6.3J.2, define MTSU, TT and relaxation according to proposal in Table 2.2.1-1.

Proposal 2. For ATG Out-of-band blocking test case 7.6J.3, Range 3, apply relaxations according to proposal in Table 2.2.2-1.

Proposal 3. For ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, in ATG General Spurious test 6.5.J.3.1, define MU and TT same as defined for General Spurious test 6.5.3.1.

Proposal 4. For ATG UEs, in ATG Rx Spurious test 7.9J, define MU and TT same as defined for Rx Spurious test 7.9.

Proposal 5. For ATG UEs, for ATG power control test cases defined in section 6.3J.4, accept current MU and TT values by removing the [ ].

Proposal 6. For ATG UEs, in ATG frequency error test 6.4J.1, define MU and TT same as defined for frequency error test 6.4.1.

Proposal 7. For ATG UEs with up to 2 transmit antenna connectors / transceiver array boundary (TAB) connectors, MU and TT for SEM ATG test case 6.5J.2.2 can be defined same as defined for SEM test case 6.5.2.2.

# References

- R5-246445, “Discussion on MU and TT for ATG RF testing”, Keysight, RAN5#105, Nov’24.

- TS 38.101-1, “User Equipment (UE) Radio transmission and reception; Part 1: Range 1 Standalone”, v18.7.0 (2024-09)

- TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v18.5.0 (2024-12)

- TS 38.521-5, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance”, v18.5.0 (2024-12)

- R5-195141, “MU for sum of power”, Anritsu, RAN5#83, May’19.