3GPP TSG-RAN5 Meeting #106	R5-250301

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3186 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for CA_n41A-n77A combo in test case 7.3A.1_1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is a Rel-17 combo with no Rel-18 assigned WPs assciated to it in PRD21. Howerver, tests ID 3, 4, 8 and 9 associated requirements are only defined in 38.101-1 Rel-18. After offline discussion it has been concluded the reason is that requirements are paired to 3CC scenarios that are not yet in plan so these test IDs should be removed along with their minimum requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleted test IDs 3,4 (PC3) and associated minimum requirements for CA_n41A-n77A combo. Next 3 test IDs renumbered accoridngly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.5, 7.3A.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: -Reverted all changes and notes related to the test IDs 3,4 (PC3) for CA_n41A-n77A combo. Instead, after offline discussion, those test IDs and their associated minimum requirements have been removed. Test IDs 8 and 9 (PC2) and associated minimum requirements being removed in a revision of R5-250952 (CR 3228). |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 7.3A.0.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands given in Table 7.3A.0.5-1, Table 7.3A.0.5-1a, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a. For these test points the reference sensitivity requirement specified in Table 7.3.2.3-1a, Table 7.3.2.3-1b, Table 7.3.2.3-2 and Table 7.3.2.3-2a are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.0.5-1, Table 7.3A.0.5-1a, Table 7.3A.0.5-2 and Table 7.3A.0.5-2a.

Table 7.3A.0.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) |  | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 |  | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 |  | N/A | FDD | N/A |
| CA_n1-n8 | n1 | 1965 | 5 | 25 | 2155 |  | 6.0 | FDD | IMD4 |
|  | n8 | 887.5 | 5 | 25 | 932.5 |  | N/A | FDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 |  | 8.0 | FDD | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 |  | N/A | TDD | N/A |
| CA_n2-n48 | n2 | 1852.5 | 5 | 25 | 1932.5 |  | 12 | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 |  | N/A | TDD | N/A |
| CA_n2-n66 | n2 | 1855 | 5 | 25 | 1935 |  | 20 | FDD | IMD3 |
|  | n66 | 1775 | 5 | 25 | 2175 |  | N/A | FDD | N/A |
|  | n2 | 1883.3 | 5 | 25 | 1963.3 |  | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 |  | 4 | FDD | IMD5 |
| CA_n2-n77 | n2 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
|  | n2 | 1900 | 5 | 25 | 1980 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A | TDD | N/A |
|  | n2 | 1885 | 5 | 25 | 1965 |  | 5 | FDD | IMD5 |
|  | n77 | 3810 | 10 | 50 | 3810 |  | N/A | TDD | N/A |
|  | n2 | N/A | 5 | N/A | 1987.5 |  | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 RBSTART=10 | 3455 |  | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 1 RBSTART=0 | 3945 |  |  |  |  |
| CA_n3-n5 | n3 | 1771 | 10 | 50 | 1866 |  | 4 | FDD | IMD4 |
|  | n5 | 838 | 5 | 25 | 883 |  | N/A | FDD | N/A |
| CA_n3-n8 | n3 | 1755 | 10 | 50 | 1850 |  | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 |  | 8 | FDD | IMD44 |
|  | n3 | 1747.5 | 10 | 50 | 1842.5 |  | 6.4 | FDD | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 |  | N/A | FDD | N/A |
| CA_n3-n41 | n3 | 1740 | 5 | 25 | 1835 |  | 8.2 | FDD | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 |  | N/A | TDD | N/A |
| CA_n3-n5 | n3 | 1721 | 10 | 50 | 1816 |  | N/A | FDD | N/A |
|  | n5 | 838 | 5 | 25 | 883 |  | 24 | FDD | IMD23 |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 |  | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 |  | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 |  | 8.0 | FDD | IMD44 |
|  | n77 | 3435 | 10 | 50 | 3435 |  | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 |  | 26 | FDD | IMD24 |
|  | n78 | 3575 | 10 | 25 | 3575 |  | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 |  | 8.0 | FDD | IMD44 |
|  | n78 | 3435 | 10 | 25 | 3435 |  | N/A | TDD | N/A |
| CA_n5-n66 | n5 | 838 | 5 | 25 | 883 |  | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 |  | N/A | FDD | N/A |
| CA_n5-n776 | n5 | 844 | 5 | 25 | 889 |  | 8.3 | FDD | IMD4 |
|  | n77 | 3421 | 10 | 50 | 3421 |  | N/A | TDD | N/A |
|  | n5 | 829 | 5 | 25 | 874 |  | 5.5 | FDD | IMD5 |
|  | n77 | 4190 | 10 | 50 | 4190 |  | N/A | TDD | N/A |
| CA_n5-n78 | n5 | 844 | 5 | 25 | 889 |  | 8.3 | FDD | IMD4 |
|  | n78 | 3421 | 10 | 50 | 3421 |  | N/A | TDD | N/A |
| CA_n8-n78 | n8 | 897.5 | 5 | 25 | 942.5 |  | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 |  | N/A | TDD | N/A |
| CA_n14A-n77A | n14 | 793 | 5 | 20 | 763 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3935 | 10 | 50 | 3935 |  | N/A | TDD |  |
| CA_n20A-n78A | n20 | 850 | 5 | 25 | 809 |  | 11 | FDD | IMD4 |
|  | n78 | 3359 | 10 | 50 | 3359 |  | N/A | TDD | N/A |
| CA_n24-n7710 | n24 | N/A | N/A | N/A | N/A |  | N/A | FDD | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A |  | N/A | TDD | N/A |
| CA_n25-n66 | n66 | 1775 | 5 | 25 | 2175 |  | N/A | FDD | N/A |
|  | n25 | 1855 | 5 | 25 | 1935 |  | 20 | FDD | IMD3 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 |  | 23 | FDD | IMD3 |
|  | n25 | 1912.5 | 5 | 25 | 1992.5 |  | N/A | FDD | N/A |
|  | n66 | 1750 | 5 | 25 | 2150 |  | 4 | FDD | IMD5 |
|  | n25 | 1883.3 | 5 | 25 | 1963.3 |  | N/A | FDD | N/A |
| CA_n25-n77 | n25 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD2 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
|  | n25 | 1900 | 5 | 25 | 1980 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3690 | 10 | 50 | 3690 |  | N/A | TDD | N/A |
|  | n25 | 1885 | 5 | 25 | 1965 |  | 5 | FDD | IMD5 |
|  | n77 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
| CA_n25-n78 | n25 | 1855 | 5 | 25 | 1935 |  | 26 | FDD | IMD24 |
|  | n78 | 3790 | 10 | 50 | 3790 |  | N/A | TDD | N/A |
| CA_n26-n66 | n26 | 838 | 5 | 25 | 883 |  | 30 | FDD | IMD24 |
|  | n66 | 1721 | 5 | 25 | 2121 |  | N/A | FDD | N/A |
| CA_n26-n70 | n26 | 831 | 5 | 25 | 876 |  | 30 | FDD | IMD24 |
|  | n70 | 1707.5 | 5 | 25 | 2007.5 |  | N/A | FDD | N/A |
| CA_n28-n77 | n28 | 705.5 | 5 | 25 | 760.5 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 |  | N/A | TDD | N/A |
| CA_n30-n77 | n30 | 2310 | 5 | 25 | 2355 |  | 8.0 | FDD | IMD4 |
|  | n77 | 3487.5 | 10 | 50 | 3487.5 |  | N/A | TDD | N/A |
| CA_n48-n66 | n48 | 3660 | 5 | 25 | 3660 |  | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 |  | 5.0 | FDD | IMD5 |
| CA_n48-n70 | n70 | 1697.5 | 25/15 | 25 | 1997.5 |  | 26 | FDD | IMD24 |
|  | n48 | 3695 | 10 | 50 | 3695 |  | N/A | TDD | N/A |
| CA_n66-n71 | n66 | 1750 | 5 | 25 | 2150 |  | 5 | FDD | IMD4 |
|  | n71 | 675 | 5 | 25 | 629 |  | N/A | FDD | N/A |
| CA_n66-n77 | n66 | 1775 | 5 | 25 | 2175 |  | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 |  | N/A | TDD | N/A |
|  | n66 | 1760 | 5 | 25 | 2160 |  | 5.0 | FDD | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 |  | N/A | TDD | N/A |
|  | n66 | N/A | 5 | N/A | 2197.5 |  | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBSTART=0) | 3305 |  | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBSTART=8) | 3855 |  |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 |  | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=10) | 3455 |  | N/A | TDD | N/A |
|  |  | 3875 | 10 | 1 (RBSTART=0) | 3875 |  |  |  |  |
| CA_n66-n78 | n66 | 1730 | 5 | 25 | 2130 |  | 5.0 | FDD | IMD5 |
|  | n78 | 3660 | 10 | 50 | 3660 |  | N/A | TDD | N/A |
| CA_n70-n71 | n70 | 1697.5 | 5 | 25 | 1997.5 |  | 5 | FDD | IMD4 |
|  | n71 | 695.5 | 5 | 25 | 649.5 |  | N/A | FDD | N/A |
| CA_n71-n77 | n71 | 671 | 5 | 25 | 625 |  | 5.5 | FDD | IMD5 |
|  | n77 | 3309 | 10 | 50 | 3309 |  | N/A | TDD | N/A |
| CA_n71-n78 | n71 | 681.5 | 5 | 25 | 635.5 |  | 5.5 | FDD | IMD5 |
|  | n78 | 3361.5 | 10 | 50 | 3361.5 |  | N/A | TDD | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in subclause 6.2A.4 NOTE 2: RBSTART = 0, 15kHz SCS is assumed. NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void. NOTE 6: TBD NOTE 7: TBD NOTE 8: TBD NOTE 9: TBD NOTE 10: There is no IMD4 product in band n24 downlink for n77 operating in 3450 – 3980 MHz and n24 uplink restricted to between 1627.5 – 1637.5 MHz and between 1646.5 – 1656.5 MHz. NOTE 11: TBD NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

## <<< Skip unchanged sections >>>

### 7.3A.1_1	Reference sensitivity power level for 2DL CA exceptions

Editor’s Note: The following aspects are either missing or not yet determined:

- Test point analysis for CA_n3A-n5A IMD2 and IMD4 is currently missing in TR 38.905.

7.3A.1_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.1_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support NR 2DL CA

7.3A.1_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.1_1.4	Test description

7.3A.1_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.1_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.1_1.4.1-1: Test Configuration Table for inter-band 2DL CA exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.1-1 for the CA Configuration across bandwidth combination sets supported by the UE |  |  |  |  |  |  | Refer to “PCC”and “SCC” columns |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1950 MHz (UL) | n3 | 1760 MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n1 | Low | n3 | High | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 3 | n1 | Low | n3 | High | 50MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n1A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1965 MHz (UL) | n8 | 887.5 MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n1A-n28A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | 713 MHz (UL) | n1 | 2139 MHz | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n28 | 713 MHz (UL) | n1 | 2139 MHz | 5 MHz | 50 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n1A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1955 MHz (UL) | n41 | 2501 MHz | 50 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 2 | n1 | 1970 MHz (UL) | n41 | 2546 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 3 | n41 | 2546 MHz | n1 | 2167.5 MHz (UL) | 30 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n1A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | Mid | n77 | 3900 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n1 | Mid | n77 | 3900 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n1 | Mid | n77 | 3875 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n1A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | 1950 MHz (UL) | n78 | 3710 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n48A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | 1860 MHz (UL) | n48 | 3690 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n2 | UL 1852.5/DL 1932.5 | n48 | 3625 MHz | 5 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/DL 1935 | n66 | UL 1775/DL 2175 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n2 | UL 1883.3/DL 1963.3 | n66 | UL 1750/DL 2150 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n2A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | 1880 MHz (UL) | n77 | 3760 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n2 | 1880 MHz (UL) | n77 | 3760 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n2 | 1880 MHz (UL) | n77 | 3735 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n77 | 3920 MHz | n2 | DL Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 5 | n77 | 3920 MHz | n2 | DL Mid | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 6 | n2 | UL 1855/DL 1935 | n77 | 3790 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 7 | n2 | UL 1900/DL 1980 | n77 | 3720 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 8 | n2 | UL 1885/DL 1965 | n77 | 3810 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 95 | n2 | 1987.5 MHz (DL) | n77 | 3305 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_4 |
| Test Settings for CA_n3A-n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 1771 MHz (UL) | n5 | 838 MHz | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n3A-n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 1755MHz | n8 | 900MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | Mid | n8 | Mid | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n3A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 1740 MHz (UL) | n41 | 2657.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n41 | 2546 MHz (UL) | n3 | 1877.5 MHz | 100 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n3A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | Mid | n77 | 3495 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n3 | Mid | n77 | 3495 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n3 | Mid | n77 | 3470 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n3 | 1740 MHz (UL) | n77 | 3575 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n3 | 1765 MHz (UL) | n77 | 3435 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 65 | n77 | 3685 MHz | n3 | DL Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 75 | n77 | 3685 MHz | n3 | DL Mid | 40 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| Test Settings for CA_n3A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | Mid | n78 | 3495 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n3 | Mid | n78 | 3495 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n3 | Mid | n78 | 3470 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n3 | 1740 MHz (UL) | n78 | 3575 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n3 | 1765 MHz (UL) | n78 | 3435 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 65 | n78 | 3685 MHz | n3 | DL Mid | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 75 | n78 | 3685 MHz | n3 | DL Mid | 40 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| Test Settings for a CA_n5A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 838/DL 883 | n66 | UL 1721/DL 2121 | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for a CA_n5A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | 836.5 MHz (UL) | n77 | 3346 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n5 | 846.5 MHz (UL) | n77 | 3386 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n5 | 826.5 MHz (UL) | n77 | 4132.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n5 | 826.5 MHz (UL) | n77 | 4132.5 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 5 | n77 | 3526 MHz | n5 | DL Mid | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 6 | n77 | 3526 MHz | n5 | DL Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 7 | n5 | UL 844/ DL 889 | n77 | 3421 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 8 | n5 | UL 829/ DL 874 | n77 | 4190 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for CA_n5A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | Mid | n78 | 3346 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n5 | 846.5 MHz (UL) | n78 | 3386 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n5 | 844 MHz (UL) | n78 | 3421 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n7A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3350 MHz | n7 | 2687.5 MHz | 100 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n8A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n8 | Mid | n78 | 3590 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n8 | Mid | n78 | 3590 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n8 | 897.5 MHz (UL) | n78 | 3635 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n14A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n14 | Mid | n77 | 3965 MHz | 5 Mhz | 10 Mhz | CP-OFDM QPSK | 10@0 |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n14 | Mid | n77 | 3965 MHz | 5 Mhz | 100 Mhz | CP-OFDM QPSK | 20@0 |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n14 | 793 MHz (UL) | n77 | 3935 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 4 | n77 | 3815 MHz | n14 | Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | 25@0 |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 5 | n77 | 3815 MHz | n14 | Mid | 10 MHz | 10 MHz | CP-OFDM QPSK | 50@0 |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| Test Settings for CA_n20A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n20 | Mid | n78 | High | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 2 | n20 | Mid | n78 | 3388 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 3 | n20 | 850 MHz (UL) | n78 | 3359 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n25A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n25 | 1855 MHz (UL) | n66 | 1775 MHz | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n25 | 1912.5 MHz | n66 | 1712.5 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 3 | n25 | 1883.3 MHz | n66 | 1750 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n25A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n25 | 1882.5 MHz (UL) | n77 | 3765 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n25 | 1882.5 MHz (UL) | n77 | 3765 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n25 | 1852.5 MHz (UL) | n77 | 3680 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n25 | 1962.5 MHz | n77 | 3925 MHz (UL) | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 5 | n25 | 1962.5 MHz | n77 | 3925 MHz (UL) | 40 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| 6 | n25 | 1855 MHz (UL) | n77 | 3790 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 7 | n25 | 1900 MHz (UL) | n77 | 3690 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 8 | n25 | 1885 MHz (UL) | n77 | 3790 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n25A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n25 | 1882.5 MHz (UL) | n78 | 3765 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n25 | 1882.5 MHz (UL) | n78 | 3765 MHz | 10 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n25 | 1852.5 MHz (UL) | n78 | 3680 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n25 | 1855 MHz (UL) | n78 | 3790 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n26A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1721 MHz (UL) | n26 | 838 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n26A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n70 | 1707.5 MHz (UL) / 2007.5 MHz (DL) | n26 | 831 MHz (UL) / 876 MHz (DL) | 5 MHz UL / 5 MHz DL | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n28A-n40A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n40 | 2341.5 MHz | n28 | 780.5 MHz (DL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 2 | n40 | 2341.5 MHz | n28 | 780.5 MHz (DL) | 20 MHz | 20 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| Test Settings for CA_n28A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | 705.5 MHz (UL) | n77 | 3527.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 2 | n28 | 705.5 MHz (UL) | n77 | 3527.5 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 |  |
| 3 | n28 | 705.5 MHz (UL) | n77 | 3587.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n28A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | 705.5 MHz (UL) | n78 | 3527.5 MHz | 5 MHz | 10MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n28 | 705.5 MHz (UL) | n78 | 3527.5 MHz | 5 MHz | 100MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n28 | 705.5 MHz | n78 | 3527.5 MHz (UL) | 5 MHz | 10MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | - | REFSENS_CA_2 |
| Test Settings for CA_n29A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | High | n29 | Low | 20 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |
| Test Settings for CA_n30A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n30 | 2310 MHz (UL) | n66 | 2197.5 MHz | 10 MHz | 15 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |
| Test Settings for CA_n30A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n77 | 3532.5 MHz | n30 | Mid | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK |  | REFSENS_CA_2 |
| 2 | n77 | 3487.5 MHz | n30 | Mid | 10 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK |  | REFSENS_CA_2 |
| 3 | n30 | Mid | n77 | 3487.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |

| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | CA Configuration / CBW |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2) |  |  |
|  | CA Configuration |  |  |  | PCC | SCC | CC MOD | PCC & SCC RB allocation |  | CC MOD | PCC & SCC RB allocations (LCRB @ RBstart) |  |
|  | PCC |  | SCC |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Test Settings for a CA_n39A-n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n39 | 1912.5 MHz | n41 | 2550 MHz | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 |  |
| 2 | n39 | 1912.5 MHz | n41 | 2550 MHz | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 |  |
| 3 | n41 | 2530 MHz | n39 | 1897.5 MHz | 10 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 |  |
| 4 | n41 | 2530 MHz | n39 | 1897.5 MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 |  |
| 5 | n39 | 1900 MHz | n41 | 2501 MHz | 40 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 |  |
| 6 | n41 | 2546 MHz | n39 | 1917.5 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for a CA_n40A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n77 | 3525 MHz | n40 | Mid | 20 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 2 | n77 | 3525 MHz | n40 | Mid | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 3 | n77 | 3350 MHz | n40 | 2397.5 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 4 | n77 | 3350 MHz | n40 | Mid | 100 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for a CA_n41A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2546 MHz | n66 | 2197.5 MHz | 100 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n41A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | Low | n41 | 2662 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n71 | Low | n41 | High | 5 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n41A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n77 | 3960 MHz | n41 | 2640 MHz | 20 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 2 | n77 | 3960 MHz | n41 | 2640 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_2 | - |
| 3 | n41 | 2640 MHz | n77 | 3305 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 4 | n77 | 3350 MHz | n41 | 2685 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 5 | n77 | 3350 MHz | n41 | 2640 MHz | 100 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 85 | n41 | 2545 MHz | n77 | 3305 MHz | 10 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 95 | n41 | 2625 MHz | n77 | 3305 MHz | 10 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n41A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | n41 | 2640 MHz | n79 | 4405 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 25 | n79 | 4450 MHz | n41 | 2685 MHz | 100 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Test Settings for CA_n48A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1730 MHz (UL) | n48 | 3660 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n66 | High | n48 | 3555 MHz | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | High | n48 | Low | 40 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n66 | 1777.5 MHz | n48 | 3580 MHz | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| Test Settings for CA_n48A-n70A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n48 | 3695 MHz (UL) | n70 | 1697.5 MHz (UL) | 10 MHz | 15 MHz UL / 25 MHz DL | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1750 MHz (UL) | n71 | 675 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1750 MHz (UL) | n77 | 3500 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n66 | 1750 MHz (UL) | n77 | 3500 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | 1712.5 MHz (UL) | n77 | 3400 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 4 | n66 | UL 1775/ DL 2175 | n77 | 3950 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 5 | n66 | UL 1760/ DL 2160 | n77 | 3720 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n66A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | 1750 MHz (UL) | n78 | 3500 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n66 | 1750 MHz (UL) | n78 | 3500 MHz | 20 MHz | 100 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n66 | 1712.5 MHz (UL) | n78 | 3400 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB |  | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |

| Test Settings for CA_n70A-n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n71 | Low | n70 | Low | 5 MHz | 25 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n71 | Low | n70 | Low | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 3 | n70 | 1697.5 MHz (UL) | n71 | 695.5 MHz (UL) | 5 MHz | 5 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n71A-n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | 671 MHz (UL) | n77 | 3309 MHz (UL) | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n71A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n71 | 680.5 MHz (UL) | n78 | 3402.5 MHz | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_1 | - |
| 2 | n71 | 681.5 MHz (UL) | n78 | 3361.5MHz (UL) | 5 MHz | 10 MHz | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Test Settings for CA_n78A-n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n78 | 3750 | n79 | 4420 | 100 | 40 | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 2 | n79 | 4450 | n78 | 3795 | 100 | 10 | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 3 | n79 | 4450 | n78 | 3750 | 100 | 100 | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| 45 | n78 | 3750 | n79 | 4450 | 100 | 100 | CP-OFDM QPSK | Full RB | DFT-s-OFDM QPSK | REFSENS_CA_4 | - |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3. REFSENS_CA_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3A.0.4-1. REFSENS_CA_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3A.0.4-4b for PC3 and Table 7.3A.0.4-4c for PC2. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference due to 2UL CA according to table 7.3A.0.5-1 for PC3 and table 7.3A.0.5-1a for PC2. REFSENS_CA_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation for NR CA FR1 according to table 7.3A.0.6-1 for PC3 and Table 7.3A.0.6-3a for PC2. Note 3: In a band where UE supports 4Rx but not supports 8Rx, the test needs to be performed only with 4Rx antennas connected. In a band where UE supports 8Rx, the test needs to be performed only with 8Rx antennas connected. Note 4: The test points are executed only when UEs support simultaneous Rx/Tx capability as indicated in Table A.4.3.2A.4.1-3 of TS 38.508-2. Note 5: This test ID is for UL PC2 only. Note 6: This test ID is for UL PC3 only. |  |  |  |  |  |  |  |  |  |  |  |

## <<< Skip unchanged sections >>>

7.3A.1_1.5	Test requirement

For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier indicating power class 2 in TS 38.508-2 table A.4.3.2A.4.1-3. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 7.3A.1_1.5-1: Reference sensitivity requirement for inter band CA with PC3 single UL carrier or PC3 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 9, Note 10, Note 11) |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | 1 | n1 | 5 | REF_victim +23 |
|  |  | n3 | 5 | REF_aggressor |
|  | 2 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_victim +3 |
|  | 3 | n1 | 5 | REF_aggressor |
|  |  | n3 | 5 | REF_victim +19.7 |
| CA_n1A-n8A | 1 | n1 | 5 | REF_victim +6 |
|  |  | n8 | 5 | REF_aggressor |
| CA_n1A-n28A | 1 | n1 | 5 | REF_victim +10.2 |
|  |  | n28 | 5 | REF_aggressor |
|  | 2 | n1 | 50 | REF_victim +1.1 |
|  |  | n28 | 5 | REF_aggressor |
| CA_n1A-n41A | 1 | n1 | 50 | REF_aggressor |
|  |  | n41 | 10 | REF_victim +6.1 |
|  | 2 | n1 | 20 | REF_aggressor |
|  |  | n41 | 100 | REF_victim +0.7 |
|  | 3 | n1 | 5 | REF_victim +18.1 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n1A-n77A | 1 | n1 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +27.1 |
|  | 2 | n1 | 20 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n1 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.9 |
| CA_n1A-n78A | 1 | n1 | 5 | REF_victim +8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n2A-n48A | 1 | n2 | 5 | REF_aggressor |
|  |  | n48 | 10 | REF_victim +1.9 |
|  | 2 | n2 | 5 | REF_victim +12 |
|  |  | n48 | 20 | REF_aggressor |
| CA_n2A-n66A | 1 | n2 | 5 | REF_victim +20 |
|  |  | n66 | 5 | REF_aggressor |
|  | 2 | n2 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +4 |
| CA_n2A-n77A | 1 | n2 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n2 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n2 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n2 | 5 | REF_victim +6.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n2 | 20 | REF_victim +3.7 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n2 | 5 | REF_victim +26 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n2 | 5 | REF_victim +8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n2 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n3A-n8A | 1 | n3 | 10 | REF_aggressor |
|  |  | n8 | 5 | REF_victim +8 |
|  | 2 | n3 | 10 | REF_victim +6.4 |
|  |  | n8 | 5 | REF_aggressor |
| CA_n3A-n41A | 1 | n3 | 5 | REF_victim +8.2 |
|  |  | n41 | 10 | REF_aggressor |
|  | 2 | n3 | 5 | REF_victim +0.6 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n3A-n77A | 1 | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n3 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n3 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n3 | 5 | REF_victim +26.0 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +8.0 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n3A-n78A | 1 | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n3 | 10 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n3 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
|  | 4 | n3 | 5 | REF_victim +26 |
|  |  | n78 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n5A-n66A | 1 | n5 | 5 | REF_victim +30 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n5A-n77A | 1 | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.5 |
|  | 2 | n5 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +1.4 |
|  | 3 | n5 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 4 | n5 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 5 | n5 | 20 | REF_victim +2.7 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n5 | 5 | REF_victim +5.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n5 | 5 | REF_victim +8.3 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n5 | 5 | REF_victim +5.5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n5A-n78A | 1 | n5 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.5 |
|  | 2 | n5 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n5 | 5 | REF_victim +8.3 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n7A-n78A | 1 | n7 | 5 | REF_victim +4.5 |
|  |  | n78 | 100 | REF_aggressor |
| CA_n8A-n78A | 1 | n8 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.8 |
|  | 2 | n8 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n8 | 5 | REF_victim +8.3 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n14A-n77A | 1 | n14 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 2 | n14 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 3 | n14 | 5 | REF_victim + 5.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 4 | n14 | 5 | REF_victim + 31 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n14 | 10 | REF_victim + 28 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n20A-n78A | 1 | n20 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.8 |
|  | 2 | n20 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +1.4 |
|  | 3 | n20 | 5 | REF_victim + 11 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n25A-n66A | 1 | n66 | 5 | REF_aggressor |
|  |  | n25 | 5 | REF_victim +20 |
|  | 2 | n66 | 5 | REF_victim +23 |
|  |  | n25 | 5 | REF_aggressor |
|  | 3 | n66 | 5 | REF_victim +4 |
|  |  | n25 | 5 | REF_aggressor |
| CA_n25A-n77A | 1 | n25 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n25 | 10 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n25 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n25 | 5 | REF_victim +6.7 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n25 | 40 | REF_victim +1.1 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n25 | 5 | REF_victim +26 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n25 | 5 | REF_victim +8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 8 | n25 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n25A-n78A | 1 | n25 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n25 | 10 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n25 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
|  | 4 | n25 | 5 | REF_victim +26 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n26A-n66A | 1 | n26 | 5 | REF_victim + 30 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n26A-n70A | 1 | n26 | 5 | REF_victim + 30 |
|  |  | n70 | 5 | REF_aggressor |
| CA_n28A-n40A | 1 | n28 | 5 | REF_victim +37.8 |
|  |  | n40 | 5 | REF_aggressor |
|  | 2 | n28 | 20 | REF_victim +30.3 |
|  |  | n40 | 20 | REF_aggressor |
| CA_n28A-n77A | 1 | n28 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.4 |
|  | 2 | n28 | 5 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +0.7 |
|  | 3 | n28 | 5 | REF_victim +0.7 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n28A-n78A | 1 | n28 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.4 |
|  | 2 | n28 | 5 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +0.7 |
|  | 3 | n28 | 5 | REF_victim + 31 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n29A-n71A | 1 | n29 | 5 | REF_victim + 17.5 |
|  |  | n71 | 20 | REF_aggressor |
| CA_n30A-n66A | 1 | n30 | 10 | REF_aggressor |
|  |  | n66 | 15 | REF_victim + 8.3 |
| CA_n30A-n77A | 1 | n30 | 5 | REF_victim + 10.4 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n30 | 10 | REF_victim + 8 |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n30 | 5 | REF_victim + 8 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n39A-n41A | 1 | n39 | 5 | REF_aggressor |
|  |  | n41 | 5 | REF_victim +8.1 |
|  | 2 | n39 | 5 | REF_aggressor |
|  |  | n41 | 100 | REF_victim +3.3 |
|  | 3 | n39 | 5 | REF_victim +4.3 |
|  |  | n41 | 10 | REF_aggressor |
|  | 4 | n39 | 40 | REF_victim + 0.8 |
|  |  | n41 | 10 | REF_aggressor |
|  | 5 | n39 | 40 | REF_aggressor |
|  |  | n41 | 10 | REF_victim +3.3 |
|  | 6 | n39 | 5 | REF_victim +1.6 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n40A-n77A | 1 | n40 | 10 | REF_victim +10.4 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n40 | 100 | REF_victim +0.9 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n40 | 10 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 4 | n40 | 100 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n41A-n66A | 1 | n41 | 100 | REF_aggressor |
|  |  | n66 | 5 | REF_victim +10.5 |
| CA_n41A-n71A | 1 | n41 | 10 | REF_victim + 10.8 |
|  |  | n71 | 5 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim + 1.4 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n41A-n77A | 1 | n41 | 10 | REF_victim +10.4 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim +6.3 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n41 | 100 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +8.3 |
|  | 4 | n41 | 10 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 5 | n41 | 100 | REF_victim +4.5 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n48A-n66A | 1 | n48 | 5 | REF_aggressor |
|  |  | n66 | 5 | REF_victim + 5.0 |
|  | 2 | n48 | 5 | REF_victim +27.1 |
|  |  | n66 | 5 | REF_aggressor |
|  | 3 | n48 | 100 | REF_victim + 13.8 |
|  |  | n66 | 40 | REF_aggressor |
|  | 4 | n48 | 10 | REF_victim + 1.9 |
|  |  | n66 | 5 | REF_aggressor |
| CA_n48A-n70A | 1 | n48 | 10 | REF_aggressor |
|  |  | n70 | 15 MHz UL /25 MHz DL | REF_victim +26 |
| CA_n66A-n71A | 1 | n66 | 5 | REF_victim +5 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n66A-n77A | 1 | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +23.9 |
|  | 2 | n66 | 20 | REF_aggressor |
|  |  | n77 | 100 | REF_victim +13.8 |
|  | 3 | n66 | 5 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +1.1 |
|  | 4 | n66 | 5 | REF_victim +31 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n66 | 5 | REF_victim +5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n66A-n78A | 1 | n66 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +23.9 |
|  | 2 | n66 | 20 | REF_aggressor |
|  |  | n78 | 100 | REF_victim +13.8 |
|  | 3 | n66 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +1.1 |
| CA_n70A-n71A | 1 | n70 | 25 | REF_victim +4.1 |
|  |  | n71 | 5 | REF_aggressor |
|  | 2 | n70 | 5 | REF_victim +9.9 |
|  |  | n71 | 5 | REF_aggressor |
|  | 3 | n70 | 5 | REF_victim +5 |
|  |  | n71 | 5 | REF_aggressor |
| CA_n71A-n77A | 1 | n71 | 5 | REF_victim +5.5 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n71A-n78A | 1 | n71 | 5 | REF_aggressor |
|  |  | n78 | 10 | REF_victim +10.4 |
|  | 2 | n71 | 5 | REF_victim +5.5 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n78A-n79A | 1 | n78 | 100 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +2 |
|  | 2 | n78 | 10 | REF_victim +2.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 3 | n78 | 100 | REF_victim +2.6 |
|  |  | n79 | 100 | REF_aggressor |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: Void Note 4: Void Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 10: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 11: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports. |  |  |  |  |

Table 7.3A.1_1.5-1a: Reference sensitivity requirement for inter band CA with PC2 single UL carrier or PC2 2UL CA

| CA configuration | Test ID | NR band | CBW (MHz) | REFSENS test requirement of NR band (dBm) (Note 8, Note 9, Note 10) |
| --- | --- | --- | --- | --- |
| CA_n1A-n78A | 1 | n1 | 5 | REF_victim +17.8 |
|  |  | n78 | 10 | REF_aggressor |
| CA_n2A-n77A | 4 | n2 | 5 | REF_victim +9.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n2 | 5 | REF_victim +6.7 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n2 | 5 | REF_victim +32.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n2 | 5 | REF_victim +19.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n2 | 5 | REF_victim +1.0 |
|  |  | n77 | 100 | REF_aggressor |
| CA_n3A-n41A | 1 | n3 | 5 | REF_victim +18.4 |
|  |  | n41 | 10 | REF_aggressor |
|  | 2 | n3 | 5 | REF_victim +2.3 |
|  |  | n41 | 100 | REF_aggressor |
| CA_n3A-n77A | 4 | n3 | 5 | REF_victim +31.9 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n3 | 5 | REF_victim +18.5 |
|  |  | n77 | 10 | REF_aggressor |
|  | 6 | n3 | 5 | REF_victim +8.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n3 | 40 | REF_victim +1.0 |
|  |  | n77 | 40 | REF_aggressor |
| CA_n3A-n78A | 6 | n3 | 5 | REF_victim + 8.1 |
|  |  | n78 | 5 | REF_aggressor |
|  | 7 | n3 | 40 | REF_victim + 1.0 |
|  |  | n78 | 40 | REF_aggressor |
| CA_n5A-n77A | 5 | n5 | 20 | REF_victim +4.3 |
|  |  | n77 | 20 | REF_aggressor |
|  | 6 | n5 | 10 | REF_victim +8.1 |
|  |  | n77 | 10 | REF_aggressor |
|  | 7 | n5 | 5 | REF_victim +18.6 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n14A_n77A | 4 | n14 | 5 | REF_victim + 34 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n14 | 10 | REF_victim + 31 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n30A-n77A | 1 | n30 | 5 | REF_victim + 13.2 |
|  |  | n77 | 10 | REF_aggressor |
|  | 2 | n30 | 10 | REF_victim + 10.6 |
|  |  | n77 | 10 | REF_aggressor |
|  | 3 | n30 | 5 | REF_victim + 17.6 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n41A-n77A | 1 | n41 | 10 | REF_victim +13.2 |
|  |  | n77 | 20 | REF_aggressor |
|  | 2 | n41 | 100 | REF_victim +8.8 |
|  |  | n77 | 20 | REF_aggressor |
|  | 3 | n41 | 100 | REF_aggressor |
|  |  | n77 | 10 | REF_victim +10.5 |
|  | 4 | n41 | 10 | REF_victim+6.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 5 | n41 | 100 | REF_victim+6.5 |
|  |  | n77 | 100 | REF_aggressor |
|  | 8 | n41 | 10 | REF_victim +32 |
|  |  | n77 | 10 | REF_aggressor |
|  | 9 | n41 | 10 | REF_victim +32 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n41A-n79A | 1 | n41 | 100 | REF_aggressor |
|  |  | n79 | 10 | REF_victim +3.1 |
|  | 2 | n41 | 10 | REF_victim +3.5 |
|  |  | n79 | 100 | REF_aggressor |
| CA_n66A-n77A | 4 | n66 | 5 | REF_victim +34.33 |
|  |  | n77 | 10 | REF_aggressor |
|  | 5 | n66 | 5 | REF_victim +11.27 |
|  |  | n77 | 10 | REF_aggressor |
| CA_n78A-n79A | 1 | n78 | 100 | REF_aggressor |
|  |  | n79 | 40 | REF_victim +5 |
|  | 2 | n78 | 10 | REF_victim +5.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 3 | n78 | 100 | REF_victim +5.6 |
|  |  | n79 | 100 | REF_aggressor |
|  | 4 | n78 | 100 | REF_aggressor |
|  |  | n79 | 100 | REF_victim +5 |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.3.5-2) Note 2: The reference measurement channel is specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2. Note 3: Void Note 4: Void Note 5: Void Note 6: Void Note 7: Void Note 8: The symbol “REF_victim” in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to reference sensitivity values for 2 antenna ports. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports. Note 9: The symbol "REF_aggressor" in this table can be used for cases of 2 antenna ports, 4 antenna ports, or 8 antenna ports. The values equal to the corresponding reference sensitivity values. Refer to Table 7.3.2.5-1a and Table 7.3.2.5-1b for reference sensitivity values for 2 antenna ports, Table 7.3.2.5-2a and Table 7.3.2.5-2b for reference sensitivity values for 4 antenna ports, and Table 7.3.2.5-2e and Table 7.3.2.5-2f for reference sensitivity values for 8 antenna ports. Note 10: The test requirement for 2Rx victim band is “REF_victim + MSD”, for 4Rx victim band is “REF_victim + ΔRIB,4R + MSD +¦ΔRIB,4R¦ = REF_victim + MSD”, and for 8Rx victim band is “REF_victim + ΔRIB,8R + MSD +¦ΔRIB,8R ¦ = REF_victim+ MSD”, where the “MSD” refers to the MSD requirements for 2Rx antenna ports. |  |  |  |  |

## <<< END OF CHANGES >>>