3GPP TSG-RAN5 Meeting #106		R5-250305r1

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Agenda item:	5.5.1.5

Source:   Verizon

Title:       Discussions on LTE NB-IoT NTN TxRx test procedures required test frequency ranges

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

At RAN5#105 in November 2024, two CR proposals [1] [2] have reached consolidated versions to clarify the LTE NB-IoT NTN test frequency ranges for Tx Rx (TRx) related test procedures. These test frequency ranges are defined as part of the Annex K, NB-IoT Test Frequencies, of [3]. LTE NB-IoT NTN test frequency ranges selection criteria for TRx related test procedures are following the same criteria as for LTE NB-IoT [4]. While LTE NB-IoT NTN is used in smart phones for mission critical services, there is a need to have LTE NB-IoT NTN TRx test procedures test the same frequency ranges as regular LTE devices.

# 2	Discussion

### 2.1	LTE NB-IoT NTN Test Frequencies Ranges

TS 36.521-4 [3] Annex K defines LTE NB-IoT NTN TRx test frequency ranges selection criteria as:

These test frequency range selection criteria for TRx related test procedures are following the similar criteria as for LTE NB-IoT UEs [4].

Criteria 1 has been referred by the following TRx test configuration tables:

6.2B.4.4.1-1, 6.3B.1.4.1-1, 6.3B.3.1.4.1-1, 6.3B.3.2.4.1-1, 6.3B.4.1.4.1-1, 6.3B.4.2.4.1-1, 6.3B.4.3.4.1-1, 6.4B.1_1.4.1-1, 6.4B.2.1.4.1-1, 6.4B.2.1.4.1-2, 6.4B.2.2.4.1-1, 6.4B.2.3.4.1-1, 6.5B.2.4.1-1, 6.6B.4.1-1, 7.6B.2.4.1-1, 7.6B.3.4.1-1, 7.8B.4.1-1, 7.9B.4.1-1

Criteria 2 has been referred by the following TRx test configuration tables:

6.2B.1.4.1-1, 6.2B.2.4.1-1, 6.2B.3.4.1-1, 6.2B.3.4.1-2, 6.5B.3.2.4.1-1, 6.5B.3.4.4.1-1, 6.5B.4.2.4.1-1, 6.5B.4.4.4.1-1, 6.5B.4.4.4.1-2, 7.3B.4.1-1, 7.4B.4.1-1, 7.5B.4.1-1

Criteria 3 has been referred by the following TRx test configuration table:

6.5B.4.3.4.1-1

Major observations:

Observation 1: For LTE NB-IoT NTN mission critical services on smart phones, instead of testing low, mid, and high frequency ranges on each supported band, TRx test procedures which follow the frequency selection criteria 1 are only performed on limited frequency ranges on lowest and highest bands supported by the UE

Observation 2: For LTE NB-IoT NTN mission critical services on smart phones, instead of testing low, mid, and high frequency ranges on each supported band, TRx test procedures which follow the frequency selection criteria 2 are only performed on limited frequency ranges on lowest, middle, and highest bands supported by the UE

Observation 3: For LTE NB-IoT NTN mission critical services on smart phones, instead of testing low, mid, and high frequency ranges on each supported band, TRx test procedure which follow the frequency selection criteria 3 is only performed on low and high frequency of each band supported by the UE.

Observation 4: For LTE NB-IoT NTN mission critical services on smart phones, instead of testing single frequency range of low or mid on each supported band, TRx test procedures follows the frequency selection criteria 1 or 2 and test extra frequency ranges comparing to regular LTE UE.

Here is the table which summarizes observations of the LTE NB-IoT NTN TRx test procedures test configuration test frequency range requirements as comparing to regular LTE UEs:

Table 1: TRx test cases test frequency ranges observations

| TRx test cases | 38.521-4 Test Configuration Table | 38.521-4 annex K Test frequency Selection Criteria | 38.521-1 Test Configuration Table for regular LTE UE | 38.521-1 required test frequency range for regular LTE UE |
| --- | --- | --- | --- | --- |
| UE maximum output power | Table 6.2B.1.4.1-1 | 2 | Table 6.2.2.4.1-1 | low mid high |
| UE maximum output power reduction | Table 6.2B.2.4.1-1 | 2 | Table 6.2.3.4.1-1 | low mid high |
| UE additional maximum output power reduction | Table 6.2B.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”) | 2 | There is no test configuration table for “NS_02” | NA |
|  | Table 6.2B.3.4.1-2: Test Configuration Table (network signalled value "NS_24") | 2 | Table 6.2.4.4.1-22: Test Configuration Table (network signalled value "NS_24") | Refer to FUL in test parameters with following: 5 MHz CH BW: - 1985 MHz (NUL = 131722) - 2007.5 MHz (NUL = high range) 10 MHz CH BW: - 1980 MHz (NUL = 131672) - 1990 MHz (NUL = 131772) - 2005 MHz (NUL = high range) 15 MHZ CH BW: - 1980 MHz (NUL = 131672) - 1990 MHz (NUL = 131772) - 2002.5 MHz (NUL = high range) 20 MHz CH BW: - 2000 MHz (NUL = high range) |
| Configured transmitted Power | Table 6.2B.4.4.1-1 | 1 | Table 6.2.5.4.1-1 | mid |
| UE Minimum output power | Table 6.3B.1.4.1-1 | 1 | Table 6.3.2.4.1-1 | Low mid high |
| General ON/OFF time mask | Table 6.3B.3.1.4.1-1 | 1 | Table 6.3.4.1.4.1-1 | Low mid high |
| (N)PRACH time mask | Table 6.3B.3.2.4.1-1 | 1 | Table 6.3.4.2.1.4.1-1 | mid |
| Power Control Absolute power tolerance | Table 6.3B.4.1.4.1-1 | 1 | Table 6.3.5.1.4.1-1 | mid |
| Power Control Relative power tolerance | Table 6.3B.4.2.4.1-1 | 1 | Table 6.3.5.2.4.1-1 | low |
| Aggregate power control tolerance | Table 6.3B.4.3.4.1-1 | 1 | Table 6.3.5.3.4.1-1 | mid |
| Transmit signal quality: Frequency error with GSO ephemeris | Table 6.4B.1_1.4.1-1 | 1 | Table 6.5.1.4.1-1 | Low mid high |
| Transmit signal quality: Frequency error with NGSO ephemeris | Table 6.4B.1_1.4.1-1 | 1 | Table 6.5.1.4.1-1 | Low mid high |
| Transmit modulation quality: Error Vector Magnitude (EVM) | Table 6.4B.2.1.4.1-1: Test Configuration for NPUSCH Table 6.4B.2.1.4.1-2: Test Configuration for NPRACH for FDD | 1 | Table 6.5.2.1.4.1-1: Test Configuration Table for PUSCH Table 6.5.2.1.4.1-2: Test Configuration Table for PUCCH | Low mid high |
| Transmit modulation quality: Carrier leakage | Table 6.4B.2.2.4.1-1 | 1 | Table 6.5.2.2.4.1-1 | Low mid high |
| Transmit modulation quality: In-band emissions | Table 6.4B.2.3.4.1-1 | 1 | Table 6.5.2.3.4.1-1 | Low mid high |
| Occupied bandwidth | Table 6.5B.2.4.1-1 | 1 | Table 6.6.1.4.1-1 | mid |
| Spectrum emission mask | Table 6.5B.3.2.4.1-1 | 2 | Table 6.6.2.1.4.1-1 | Low mid high |
| Adjacent Channel Leakage (power) Ratio | Table 6.5B.3.4.4.1-1 | 2 | Table 6.6.2.3.4.1-1 | Low mid high |
| Transmitter Spurious emissions | Table 6.5B.4.2.4.1-1 | 2 | Table 6.6.3.1.4.1-1 | Low mid high |
| Spurious emission band UE co-existence | Table 6.5B.4.3.4.1-1 | 3 | Table 6.6.3.2.4.1-1 | Low mid high |
| Additional spurious emissions | Table 6.5B.4.4.4.1-1: Test Configuration Table (network signalled value " NS_02N, NS_03N, NS_04N, NS_05N") | 2 | Table 6.6.3.3.4.1-2: Test Configuration Table (network signalled value "NS_04") | low mid high |
|  |  |  | Table 6.6.3.3.4.1-3: Test Configuration Table (network signalled value "NS_05") | Low mid |
|  | Table 6.5B.4.4.4.1-2 Test Configuration Table (network signalled value "NS_24") | 2 | Table 6.6.3.3.4.1-20: Test Configuration Table (network signalled value "NS_24") | Refer to FUL in test parameters with following: 5 MHz CH BW: - 1985 MHz (NUL = 131722) - 2007.5 MHz (NUL = high range) 10 MHz CH BW: - 1980 MHz (NUL = 131672) - 1990 MHz (NUL = 131772) - 2005 MHz (NUL = high range) 15 MHZ CH BW: - 1980 MHz (NUL = 131672) - 1990 MHz (NUL = 131772) - 2002.5 MHz (NUL = high range) 20 MHz CH BW: 2000 MHz (NUL = high range) |
| Transmit intermodulation | Table 6.6B.4.1-1 | 1 | Table 6.7.4.1-1 | mid |
| Reference sensitivity power level | Table 7.3B.4.1-1 | 2 | Table 7.3.4.1-1 | Low mid high |
| Maximum input level | Table 7.4B.4.1-1 | 2 | Table 7.4.4.1-1 | mid |
| Adjacent Channel Selectivity | Table 7.5B.4.1-1 | 2 | Table 7.5.4.1-1 | mid |
| In-band blocking | Table 7.6B.2.4.1-1 | 1 | Table 7.6.1.4.1-1 | Mid |
| Out-of-band blocking | Table 7.6B.3.4.1-1 | 1 with Note | Table 7.6.2.4.1-1 | One frequency chosen arbitrarily from low or high range |
| Wide Band Intermodulation characteristics | Table 7.8B.4.1-1 | 1 | Table 7.8.1.4.1-1 | mid |
| Spurious emissions | Table 7.9B.4.1-1 | 1 | Table 7.9.4.1-1 | Low mid high |

While LTE NB-IoT NTN is used in smart phones for mission critical services, the TRx test procedures should follow the same test frequency ranges as for regular LTE devices.

Proposal 1a: Same as for testing regular LTE UEs, test the same frequency ranges for all TRx test procedures listed in Table 1 for LTE NB-IoT NTN UEs. For test procedures with NS values, instead of following Criteria 2, suggest to test low, mid, and high frequency ranges.

Proposal 1b: Same as for testing regular LTE UEs, test the following critical test cases the same frequency ranges for LTE NB-IoT NTN UEs as for regular LTE UEs. For test procedures with NS values, instead of following Criteria 2, suggest to test low, mid, and high frequency ranges.

- 6.2B	Transmit power

- 6.4B	Transmit signal quality

- 7.3B	Reference sensitivity power level

Proposal 2: For test procedure of Out-of-band blocking (7.6B.3) which takes days to complete, test one frequency range, low or high, on each UE supported band

# 3	Summary

Observation 1: For LTE NB-IoT NTN mission critical services on smart phones, instead of testing low, mid, and high frequency ranges on each supported band, TRx test procedures which follow the frequency selection criteria 1 are only performed on limited frequency ranges on lowest and highest bands supported by the UE

Observation 2: For LTE NB-IoT NTN mission critical services on smart phones, instead of testing low, mid, and high frequency ranges on each supported band, TRx test procedures which follow the frequency selection criteria 2 are only performed on limited frequency ranges on lowest, middle, and highest bands supported by the UE

Observation 3: For LTE NB-IoT NTN mission critical services on smart phones, instead of testing low, mid, and high frequency ranges on each supported band, TRx test procedure which follow the frequency selection criteria 3 is only performed on low and high frequency of each band supported by the UE.

Observation 4: For LTE NB-IoT NTN mission critical services on smart phones, instead of testing single frequency range of low or mid on each supported band, TRx test procedures follows the frequency selection criteria 1 or 2 and test extra frequency ranges comparing to regular LTE UE.

Proposal 1a: Same as for testing regular LTE UEs, test the same frequency ranges for all TRx test procedures listed in Table 1 for LTE NB-IoT NTN UEs. For test procedures with NS values, instead of following Criteria 2, suggest to test low, mid, and high frequency ranges.

Proposal 1b: Same as for testing regular LTE UEs, test the following critical test cases the same frequency ranges for LTE NB-IoT NTN UEs as for regular LTE UEs. For test procedures with NS values, instead of following Criteria 2, suggest to test low, mid, and high frequency ranges.

- 6.2B	Transmit power

- 6.4B	Transmit signal quality

- 7.3B	Reference sensitivity power level

Proposal 2: For test procedure of Out-of-band blocking (7.6B.3) which takes days to complete, test one frequency range, low or high, on each UE supported band

CRs for proposal 1a: R5-250362, R5-250363, R5-250365, R5-250507, R5-250366, R5-250368

CRs for proposal 1b: R5-250362, R5-250365, R5-250366

CR for Proposal 2: R5-250367

# 4	References

- R5-246487 - Include Notes to further specify test frequency selection criteria of Annex K.1.1 and K.1.2 in the cases when UE supports only one band or supports two bands; 3GPP TSG RAN Meeting #106; Madrid, Spain, 9th Dec – 12th Dec 2024

- R5-246488 - Modifications of the table note to further clarify the required test frequencies; 3GPP TSG RAN Meeting #106; Madrid, Spain, 9th Dec – 12th Dec 2024

- TS 36.521-4: LTE; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing

TS 36.521-1: LTE; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance testing