3GPP TSG-RAN5 Meeting #106	R5-250343r1

# Athens, Greece, 17th - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1566 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of annex A.2.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The MT BYE for release of an eCall needs to have a Via header field according to the routing of an emergency call. - Condition A6 is specific for MO BYE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Condition added to specify the MT BYE for realease of an eCall or emercency call. - Clarification of explanation for A6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact (Via header field) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: - Due to further changes in A.2.19 the Via header shall not refer to A.2.19 anymore - Specification of the Via header field re-organised to be more precise - Editorial corrections |  |  |  |  |  |  |  |  |

## A.2.8	BYE

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | BYE |  |  |
| Request-URI |  | same URI value as the recipient of BYE has earlier sent in its Contact header within the same dialog |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via | A1,A2 |  |  | RFC 3261 [15] |
| via-parm1: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by |  | MO Call has been established: same value as in INVITE message MT Call has been established: as defined in A.2.1 |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| Via | (A3 OR A4) AND NOT A9 | as defined in A.2.9 (There is more than one value) |  | RFC 3261 [15] |
| Via | A9 |  |  | RFC 3261 [15] |
| via-parm1: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A3 | The SS P-CSCF address and the SS protected server port |  |  |
|  | A4 | The SS P-CSCF address and the SS unprotected server port |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| via-parm2 |  | SIP/2.0/UDP ecscf.3gpp.org ;branch=z9hG4bK… (NOTE 1) |  |  |
| via-parm3 |  | SIP/2.0/UDP psap.3gpp.org ;branch=z9hG4bK… (NOTE 1) |  |  |
| Route |  |  |  | RFC 3261 [15] |
| route-param | A1,A2 | MO Call has been established: URIs of the Record-Route header of 183 response in reverse order (or any other response creating the dialog according to RFC 3261 clause 12.1 [15]) MT Call has been established: value of Record-Route header as defined in A.2.9 |  |  |
| Route | A3,A4, A9 | not present |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE when BYE is sent by the UE. SIP URI of the SS when BYE is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS when BYE is sent by the UE. SIP URI of the UE when BYE is sent by the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as sent or received in INVITE message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the endpoint within its previous request in the same dialog but increased by one |  |  |
| method |  | BYE |  |  |
| Require | (A1 OR A5) AND NOT A6 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1,A5 | sec-agree |  |  |
| Require | A2, A6 | not present |  |  |
| Proxy-Require | (A1 OR A5) AND NOT A6 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag | A1,A5 | sec-agree |  |  |
| Proxy-Require | A2,A6 | not present |  |  |
| Security-Verify | (A1 OR A5) AND NOT A6 |  |  | RFC 3329 [21] |
| sec-mechanism | A1,A5 | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A2,A6 | not present |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A1,A2(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A7 | access network information and, if applicable, the cell ID |  |  |
|  | A8 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
| Proxy-Authorization | A5 |  |  | RFC 2617 [16] RFC 3310 [17] |
| username | A5 | preconfigured in the UE |  |  |
| realm | A5 | same value as received in the realm directive in the Proxy-Authorization header sent by SS |  |  |
| nonce | A5 | same value as in Proxy-Authorization header sent by SS |  |  |
| digest-uri | A5 | preconfigured in the UE |  |  |
| qop-value | A5 | auth |  |  |
| cnonce-value | A5 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A5 | counter to indicate how many times UE has sent the same value of nonce within successive INVITESs, initial value shall be 1 |  |  |
| response | A5 | response calculated by UE |  |  |
| algorithm | A5 | MD5 |  |  |
| Content-Length | A3,A4, A9 |  |  | RFC 3261 [15] |
| value |  | length of message body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | BYE sent by the UE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A2 | BYE sent by the UE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A3 | BYE sent by the SS (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A4 | BYE sent by the SS (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 2) |
| A5 | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A6 | BYE sent by the UE for emergency call with no registration |
| A7 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A8 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A9 | BYE sent by the SS for emergency call or eCall |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

NOTE 2:	When the UE supports IMS security then condition A1 or A3 is applied unless condition A2 or A4 is specified explicitly by the test case.