3GPP TSG-RAN5 Meeting #106	R5-250380r2

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0635 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of eCall test case 11.8 for CEN alignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New test case 11.8 needs to be added as part of eCallCEN-UEConTest WI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 11.8. eCall over IMS / Automatic initiation / Normal registration / Emergency registration / 200 OK with NACK |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WI will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.8 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0350 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Title Updated TP Updated Conformance requirement updated Specific message reference condition updated/added Editorial correction done Step16 IMS MT call release included "UE is configured to register for IMS after switch on" from the Pre-test conditions for the UE |  |  |  |  |  |  |  |  |

<Start of modified section>

## 11.8	eCall capable UE / Automatic initiation / Normal registration / Emergency registration / 200 OK with NACK

11.8.1	Test Purpose (TP)

(1)

with { UE having sent an INVITE for automatic initiated ecall over IMS }

ensure that {

when { UE receives 200 OK containing NACK for MSD transmission }

then { UE continue with the call establishment }

}

11.8.2	Conformance requirement

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-19 requirements.

[TS 24.229, clause 5.1.6.11.1]:

If the upper layers request establishment of an IMS emergency call of the manually initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.manual" as specified in RFC 8147 [244].

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service, the service URN shall be "urn:service:sos.ecall.automatic" as specified in RFC 8147 [244].

NOTE 1:	The manually initiated eCall type of emergency service is used when the eCall IMS emergency session is invoked with user input. The automatically initiated eCall type of emergency service is used if the eCall IMS emergency session is invoked without user input.

NOTE 2:	To ensure the identification and the related routing (e.g. to a test centre) of non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes, "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual" are not used for such calls. Additionally the usage of any other service URN to establish non-emergency calls initiated for eCall testing or eCall terminal reconfiguration purposes needs to be in accordance with local operator policy.

[TS 24.229, clause 5.1.6.11.2]:

If the upper layers request establishment of an IMS emergency call of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service and if allowed by IP-CAN specific annex, the UE shall send an INVITE request as specified in the procedures in subclause 5.1.6.8 with the following additions:

1)	the UE shall set the Request-URI to "urn:service:sos.ecall.automatic" or "urn:service:sos.ecall.manual"; and

2)	if the IP-CAN indicates the eCall support indication, the UE shall:

a)	insert a multipart/mixed body containing an "application/EmergencyCallData.eCall.MSD" MIME body part as defined in RFC 8147 [244], containing the MSD not exceeding 140 bytes and encoded in binary ASN.1 PER as specified in CEN EN 15722:2020 [245] and include a Content-Disposition header field with a "handling" header field parameter with an "optional" value, as described in RFC 3261 [26];

b)	insert an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244]; and

c)	insert a Recv-Info header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244].

NOTE 1:	Further content for the INVITE is as defined in RFC 8147 [244].

Then the UE shall proceed as follows:

…

2a)	if the UE receives a 200 (OK) response to the INVITE request containing:

a)	a multipart/mixed body containing an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] with an "ack" element containing:

i)	a "received" attribute set to "false"; and

ii)	a "ref" attribute set to the Content-ID of the MIME body part containing the MSD sent by the UE;

then the UE shall consider the initial MSD transmission as unsuccessful;

…

4)	in all other cases, the UE shall perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

NOTE 2:	For case 3-a-iv). upper layers can wait for an incoming call from the PSAP as defined in CEN EN 17184:2024 [300] if incoming IMS calls are possible or if the upper layers can instigate CS fallback to receive incoming calls over CS. If incoming calls cannot be received by the UE, upper layers can perform domain selection to re-attempt the eCall as specified in 3GPP TS 23.167 [4B].

Reference(s)

3GPP TS 24.229 [10], clauses 5.1.6.11.1 and 5.1.6.11.2.

11.8.3	Test description

11.8.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application with eCall subscription on UICC.

-	UE is configured to register for IMS after switch on

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

11.8.3.2	Test procedure sequence

Table 11.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is triggered to start a automatic eCall. | - | - | - | - |
| 2-11 | Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 [21] with condition ‘eCall’ are performed. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 12 and 14, steps 11 to 13 specified in 38.508-1 [21] Table 4.9.11.2.2-1 takes place. | - | - | - | - |
| 12 | Step 1 of Annex A.23 happens | --> | INVITE | - | - |
| 13 | SS sends 200 OK with NACK | <-- | 200 OK |  |  |
| 14 | Step 3 of Annex A.23 happens Check: Does the UE send ACK? | --> | ACK | 1 | P |
| 15 | SS waits for 5 seconds | - | - | - | - |
| 16 | Generic test procedure for IMS MT call release in 5GS described in TS 38.508-1 [21] subclause 4.9.18 takes place. | - | - | - | - |

11.8.3.3	Specific message contents

Table 11.8.3.3-1: INVITE (step 12, table 11.8.3.2-1)

Derivation path: Step 1 in Annex A.23 with Condition A21

Table 11.8.3.3-2: 200 OK for INVITE (step 13, table 11.8.3.2-1)

Derivation path: Step 2 in Annex A.23 with Conditions A14 instead of A13

Table 11.8.3.3-3: BYE (step 16, table 11.8.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.8 with condition A9.

<End of modified section>