3GPP TSG-RAN5 Meeting #106	R5-250402r2

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1574 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new test case 8.17 Initial Registration / IMS data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add new test case 8.17 Initial Registration / IMS data channel. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new test case 8.17 Initial Registration / IMS data channel. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0405 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A.15/yy is a new PICS defined in TS 34.229-2, CR 0405. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correct the related CR number in coverpage, delete UTRA related text, add specific message contents, correct the message in Expected Sequence, add new paramenters and new conditions. R2: Remove new condition Ayyy in Specific Message Contents, shorten the explanation for new condition Azzz. |  |  |  |  |  |  |  |  |

<Start of modifed section1>

2  References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or nonspecific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[xx01] 3GPP TS 24.186: " IMS Data Channel applications; Protocol specification".

[xx02] IETF RFC 5688: " A Session Initiation Protocol (SIP) Media Feature Tag for MIME Application Subtypes".

<End of modifed section1>

<Start of modifed section2>

8.17 Initial Registration / IMS data channel

8.17.1	Definition

Test to verify that the UE can correctly register to IMS data channel when equipped with UICC that contains either both ISIM and USIM applications or only USIM application but not ISIM. The process consists of sending initial registration to S-CSCF via the P-CSCF discovered, authenticating the user and finally subscribing the registration event package for the registered default public user identity.

8.17.2	Conformance requirement

[TS 26.114, clause 6.2.10.1]:

Support of data channel media is optional for an MTSI client and an MTSI client in terminal. For brevity, an MTSI client supporting data channel is henceforth denoted as a DCMTSI client or DCMTSI client in terminal, respectively.

To indicate support for the procedures in this clause, a DCMTSI client shall when including media feature tags as specified in TS 24.229 [7] include a +sip.app-subtype media feature tag, as specified by RFC 5688 [177], with a value of "webrtc-datachannel" (the application media format used by [172]), regardless of data channel media being part of the SDP or not.

......

[TS 24.186, clause 9.2.1]:

The policy related to the UE supporting the IMS data channel can be provided by the network to the UE using e.g., OMA-DM with the management objects specified in 3GPP TS 24.275 [11], ISIM with EFIMSDCI file specified in 3GPP TS 31.103 [30] or USIM with EFIMSDCI file specified in 3GPP TS 31.102 [31]. When the UE is configured as specified in 3GPP TS 24.275 [11], 3GPP TS 31.103 [30] or 3GPP TS 31.102 [31] with configuration for IMS data channel allowed then the UE determines support for IMS data channel according to the configuration. If the UE is configured with both IMS_DC_configuration node and EFIMSDCI  file, then the EFIMSDCI  file shall take precedence.

If the UE is configured with IMS_DC_configuration node specified in 3GPP TS 24.275 [11] or EFIMSDCI  file specified in 3GPP TS 31.103 [30] or 3GPP TS 31.102 [31], and the DC_allowed leaf of the IMS_DC_configuration node or IMS DC Establishment Indication of the EFIMSDCI  file indicates that IMS data channel is allowed, then a UE supporting IMS data channel on sending an unprotected REGISTER request shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type. For the IMS data channel capability indication, the UE shall use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4].

NOTE:	Precedence for EFIMSDCI file configured on both the USIM and ISIM is defined in 3GPP TS 31.102 [31].

On receiving the 200 (OK) response to the REGISTER request, if the 200 (OK) response includes a Feature-Caps header field containing feature-capability indicator "+g.3gpp.datachannel", the UE shall determine that the home network supports the IMS data channel capability as specified in 3GPP TS 23.228 [3].

Reference(s)

3GPP TS 26.114, clause 6.2.10.1 and TS 24.186, clause 9.2.1.

8.17.3   Test purpose

- To verify that UE sends a correctly composed initial REGISTER request including the sip.app-subtype media feature tag with a value "webrtc-datachannel" (as specified in 3GPP TS 26.114 [66]) when UE supporting IMS data channel has an ISIM or USIM inserted is made to to register for IMS; and

- Upon receiving a valid 200 OK response including a Feature-Caps header field with the "+g.3gpp.datachannel" header field parameter as specified in 3GPP TS 24.186 [xx], the UE shall continue to proceed the register procedure.

8.17.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is not registered to IMS services.

SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure

- EPS bearer context activation according annex C.18 for UE with E-UTRA support (TS 34.229-2 A.18/1).

- Optional P-CSCF address discovery using the DHCP procedure according to Annex C.3 for IPv6 or Annex C.4 for IPv4.

- The UE initiates IMS registration request including the sip.app-subtype media feature tag with a value “webrtc-datachannel”. SS waits for the UE to send an initial REGISTER request.

- The SS responds to the initial REGISTER request with a valid 401 Unauthorized response.

- The SS waits for the UE to set up a temporary set of security associations and to send another REGISTER request including the sip.app-subtype media feature tag with a value “webrtc-datachannel”, over those security associations.

- The SS responds to the second REGISTER request with valid 200 OK response including a Feature-Caps header field with header field parameter "+g.3gpp.datachannel", sent over the same temporary set of security associations that the UE used for sending the REGISTER request.

- The SS waits for the UE to send a SUBSCRIBE request over the newly established security associations.

- The SS responds to the SUBSCRIBE request with a valid 200 OK response.

- The SS sends a valid NOTIFY request for the subscribed registration event package.

- The SS waits for the UE to respond to the NOTIFY with a 200 OK response.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  |  | Annex C.17 or C.18. |
| 2 |  |  |  | Optional P-CSCF address discovery using the DHCP procedure according to Annex C.3 for IPv6 or Annex C.4 for IPv4. |
| 3 |  |  | REGISTER | The UE sends a correctly composed initial REGISTER request including the sip.app-subtype media feature tag with a value “webrtc-datachannel”. |
| 4 |  |  | 401 Unauthorized | The SS responds with a valid AKAv1-MD5 authentication challenge and security mechanisms supported by the network. |
| 5 |  |  | REGISTER | The UE completes the security negotiation procedures, sets up a temporary set of SAs and uses those for sending another REGISTER with AKAv1-MD5 credentials and the sip.app-subtype media feature tag with a value “webrtc-datachannel”. |
| 6 |  |  | 200 OK | The SS responds with 200 OK response including a Feature-Caps header field with header field parameter "+g.3gpp.datachannel". |
| - | - |  | EXCEPTION: In parallel to the events described in steps 7-10, the steps specified in C.5 may take place. |  |
| 7 |  |  | SUBSCRIBE | The UE subscribes to its registration event package. |
| 8 |  |  | 200 OK | The SS responds with 200 OK. |
| 9 |  |  | NOTIFY | The SS sends initial NOTIFY for registration event package, containing full registration state information for the registered public user identity in the XML body |
| 10 |  |  | 200 OK | The UE responds with 200 OK. |

Specific Message Contents

REGISTER (Step 3)

Use the default message “REGISTER” in annex A.1.1 with conditions A1.

REGISTER (Step 5)

Use the default message “REGISTER” in annex A.1.1 with conditions A2, A17.

200 OK for REGISTER (Step 6)

Use the default message “200 OK for REGISTER” in annex A.1.3 with condition Azzz.

8.17.5	Test requirements

If the UICC card equipped to the UE contains ISIM, the UE must read the following parameters from ISIM (instead of deriving them from USIM) and use they for the REGISTER requests:

-	the private user identity; and

-	the temporary public user identity; and

-	the home network domain name.

Step 3: SS shall check that, in accordance to 3GPP TS 26.114 [66] clause 6.2.10.1 the UE sends a REGISTER request including the IMS data channel capability indication +sip.app-subtype="webrtc-datachannel".

Step 5: SS shall check that, in accordance to 3GPP TS 26.114 [66] clause 6.2.10.1 the UE sends a REGISTER request including the IMS data channel capability indication +sip.app-subtype="webrtc-datachannel".

Step 6: Upon receiving a valid 200 OK response including a Feature-Caps header field with the "+g.3gpp.datachannel" header field parameter as specified in 3GPP TS 24.186 [xx01], the UE shall continue to proceed the register procedure.

<End of modifed section2>

<Start of modifed section3>

A.1.1	REGISTER

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | REGISTER |  |  |
| Request-URI |  | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or SIP URI formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
|  | A14,A15 | SIP URI formed from home domain name as preconfigured in the UE |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Route |  | not present |  | RFC 3261 [15] |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1,A3, A14,A15 | IP address or FQDN, port (optional) and not checked |  |  |
|  | A2 | IP address or FQDN and, when using UDP, protected server port of the UE |  |  |
| response-port | A1,A3 | rport (when using UDP) |  | RFC 3581 [96] |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec | A1 | any IMPU within the set of IMPUs on ISIM (when using ISIM; NOTE 3) or public user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A2,A15 | same public user identity as in initial REGISTER |  |  |
|  | A3 AND NOT A7 | public user identity derived from IMSI |  |  |
|  | A7 | emergency public user identity (NOTE 4) |  |  |
|  | A14 | IMPU preconfigured in the UE |  |  |
|  | A17 | same public user identity as in initial REGISTER |  |  |
| tag |  | must be present, value not checked |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec | A1 | any IMPU within the set of IMPUs on ISIM (when using ISIM; NOTE 3) or public user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A2,A15 | same public user identity as in initial REGISTER |  |  |
|  | A3 AND NOT A7 | public user identity derived from IMSI |  |  |
|  | A7 | emergency public user identity (NOTE 4) |  |  |
|  | A14 | IMPU preconfigured in the UE |  |  |
|  | A17 | same public user identity as in initial REGISTER |  |  |
| tag |  | not present |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | A1,A3, A14,A15 | SIP URI with IP address or FQDN and indicating either an unprotected port selected by the UE or no port at all |  |  |
|  | A2 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | A7 | The SIP URI shall contain the sos URI parameter |  |  |
| feature-param | A4 AND A34 | +g.3gpp.icsi-ref="(comma-separated list of tag-values)” with comma-separated list of tag-values containing at least tag-value urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2,5) |  |  |
|  | A6 AND NOT A7 AND A34 | +g.3gpp.smsip |  |  |
|  | A10 AND A34 | video |  |  |
|  | A28 AND (A29 OR A30 OR A32) AND A34 | audio |  |  |
|  | A11 AND NOT A7 AND A34 | +g.3gpp.cs2ps-srvcc | Rel-11 |  |
|  | A12 AND NOT A7 AND A34 | +g.3gpp.cs2ps-srvcc-alerting | Rel-11 |  |
|  | A13 AND NOT A16 AND A34 | +g.3gpp.accesstype=”cellular2” | Rel-11 | RFC 3840 [63] |
|  | A13 AND A16 AND A34 | +g.3gpp.accesstype=”wlan1” | Rel-11 | RFC 3840 [63] |
|  | A35 AND A34 | +g.3gpp.ps-data-off=”inactive” | Rel-15 |  |
|  | Ayyy | +sip.app-subtype="webrtc-datachannel" | Rel-18 | RFC 5688 [xx02] |
| c-p-instance | A5 | +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>” where gsma-specifier-defined-substring shall be the IMEI code of the UE, coded as specified in RFC 7254 [122], without optional parameters | Rel-10 | RFC 5627 [61] RFC 7254 [122] |
| expires |  | 600000 (if present) |  |  |
| Expires |  | present if no expires parameter in Contact header |  | RFC 3261 [15] |
| delta-seconds |  | 600000 |  |  |
| Require | A1,A2 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | sec-agree |  |  |
| Proxy-Require | A1,A2 |  |  | RFC 3261 [15] RFC 3329 [21] |
| option-tag |  | sec-agree |  |  |
| Supported |  |  |  | RFC 3261 [15] TS 24.229 [10] |
| option-tag | A5 | gruu |  |  |
|  |  | path |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value | A1,A3 A14 | must be present, value not checked |  |  |
|  | A2,A15 | must be incremented from the previous REGISTER |  |  |
| method |  | REGISTER |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | value not checked |  |  |
| Security-Client | A1,A2 |  |  | RFC 3329 [21] RFC 4835 [124] |
| mechanism-name |  | ipsec-3gpp |  |  |
| algorithm |  | hmac-sha-1-96 |  |  |
| protocol |  | esp (if present) |  |  |
| mode |  | trans (if present) |  |  |
| encrypt-algorithm |  | des-ede3-cbc or aes-cbc or null |  |  |
| spi-c |  | SPI number of the inbound SA at the protected client port |  |  |
| spi-s |  | SPI number of the inbound SA at the protected server port |  |  |
| port-c |  | protected client port |  |  |
| port-s |  | protected server port |  |  |
| Security-Client | A14,A15 | not present |  |  |
| Security-Verify | A2 |  |  | RFC 3329 [21] |
| sec-mechanism | A2 | same value as Security-Server header sent by SS |  |  |
| Security-Verify | A1,A3 A14,A15 | not present |  |  |
| Authorization | A1 | Digest |  | RFC 2617 [16] RFC 3310 [17] |
| username | A1 | private user identity as stored in EFIMPI (when using ISIM) or private user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
| realm | A1 | home domain name as stored in EFDOMAIN (when using ISIM) or home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
| nonce | A1 | set to an empty value |  |  |
| digest-uri | A1 | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
| response | A1 | set to an empty value |  |  |
| Authorization | A14(o) | Header optional |  |  |
| username | A14 | user identity as preconfigured in the UE |  |  |
| realm | A14 | home domain name as preconfigured in the UE |  |  |
| nonce | A14 | set to an empty value |  |  |
| digest-uri | A14 | preconfigured in the UE |  |  |
| response | A14 | set to an empty value |  |  |
| Authorization | A2,A15 | Digest |  | RFC 2617 [16] RFC 3310 [17] |
| username | A2 | private user identity as stored in EFIMPI (when using ISIM) or private user identity derived from IMSI (when no ISIM available on the UICC) |  |  |
|  | A15 | user identity as preconfigured in the UE |  |  |
| realm | A2,A15 | same value as received in the realm directive in the WWW Authenticate header sent by SS |  |  |
| nonce | A2,A15 | same value as in WWW-Authenticate header sent by SS |  |  |
| opaque | A2,A15 | same value as sent by the server in “401 Unauthorized for REGISTER” |  |  |
| digest-uri | A2 | SIP URI formed from home domain name as stored in EFDOMAIN (when using ISIM) or formed from home domain name derived from the IMSI (when no ISIM available on the UICC) |  |  |
|  | A15 | SIP URI formed from home domain name as preconfigured in the UE |  |  |
| qop-value | A2,A15 | auth |  |  |
| cnonce-value | A2,A15 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A2,A15 | counter to indicate how many times UE has sent the same value of nonce within successive REGISTER requests, initial value shall be 1 |  |  |
|  | A17 | value incremented by one for each re-registration request |  | RFC 2617 [16] |
| response | A2 | response calculated by UE |  |  |
|  | A15 | response calculated by UE using password px_DigestPasswordForSIP |  |  |
| algorithm | A2 | AKAv1-MD5 |  |  |
|  | A15 | MD5 |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | A2,A15, A14(o), A16(o) |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A2 AND A31 | access network information for E-UTRAN and, if applicable, the cell ID |  |  |
|  | A14,A15 | access network information for Fixed Broadband with access-type field matching “*DLS*” and a "dsl-location" parameter (value not checked) |  |  |
|  | A16 | access network information, containing any of “IEEE-802.11”, “IEEE-802.11a”, “IEEE-802.11b”, “IEEE-802.11g” or “IEEE-802-11n”, and i-wlan-node-id parameter containing a MAC address according to TS 24.229 [10], 7.2A.4.2. Value of MAC address not to be checked |  |  |
|  | A2 AND A32 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
|  | A2 AND A33 | access network information for UTRAN and, if applicable, the cell ID |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this message and if there is a message-body |  | RFC 3261 [15] |
| value |  | length of request body, if such is present |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | Initial unprotected REGISTER (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | Subsequent REGISTER sent over security associations (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) |
| A3 | REGISTER for the case UE supports GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A4 | UE supports IMS Multimedia Telephony (MTSI) (A.3A/50 3GPP TS 34.229-2 [5]) |
| A5 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]). Mandatory from Rel-10 onwards. |
| A6 | UE supports SM-over-IP receiver (A.3A/62 3GPP TS 34.229-2 [5]) |
| A7 | Initial unprotected or subsequent REGISTER for emergency registration |
| A8 | Void |
| A10 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A11 | UE supports CS to PS SRVCC (A.12/40 3GPP TS 34.229-2 [5]) |
| A12 | UE supports CS to PS SRVCC in alerting state (A.12/41 3GPP TS 34.229-2 [5]) |
| A13 | UE indicates g.3gpp.accesstype media feature tag in REGISTER (A.12/46 3GPP TS 34.229-2 [5]) |
| A14 | Initial REGISTER SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A15 | Subsequent REGISTER SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| A16 | IMS registration over WLAN |
| A17 | UE initiated IMS re-registration or de-registration (A.12/51 3GPP TS 34.229-2 [5]) |
| A18-A27 | Void |
| A28 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A29 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |
| A30 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |
| A31 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A32 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A33 | UE uses UTRAN access (A.18/2 3GPP TS 34.229-2 [5]) |
| A34 | Feature tags in Contact header to be checked. By default this condition is true. |
| A35 | UE supports PS data off (A.12/62 3GPP TS 34.229-2 [5]) |
| Ayyy | UE supports IMS data channel (A.15/yy 3GPP TS 34.229-2 [5]) |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	Public user identity shall be the same for ‘From’ and ‘To’.

NOTE 4:	According to TS 24.229 clause 5.1.1.1A and 5.1.6.2 [10] when the UE is using ISIM the emergency public user identity is the first public user identity in the list stored in the ISIM; when there is no ISIM it is the default public user id if the UE successfully performed IMS registration with the IM CN subsystem before, and the temporary user id (derived from IMSI) in all other cases.

NOTE 5:	URN is the outcome of the  URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

<End of modifed section3>

<Start of modifed section4>

A.1.3	200 OK for REGISTER

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 200 |  |  |
| Reason-Phrase |  | OK |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in REGISTER message |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in REGISTER message |  |  |
| tag |  | common to-tag (register) |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in REGISTER message |  |  |
| tag |  | same value as received in REGISTER message |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in REGISTER message |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in REGISTER message |  |  |
| Contact |  |  |  | RFC 3261 [15] RFC 5627 [61] |
| addr-spec |  | same value as received in REGISTER message |  |  |
| pub-gruu | A1 | Public GRUU as the SIP URI got from the To header of the REGISTER request, together with the gr parameter with an arbitrary value |  |  |
| temp-gruu | A1 | Temporary GRUU with an arbitrary value in the user part and the host part matching with the domain of the To header of the REGISTER and gr parameter without any value |  |  |
|  | A3 | not present |  |  |
| feature-param |  | same value as received in REGISTER message |  |  |
| expires |  | 600000 |  |  |
| P-Associated-URI |  | order of the parameters in this header must be like in the respective rows |  | RFC 7315 [132] |
| addr-spec | A2 | all the IMPUs within the set of IMPUs on ISIM (NOTE 1), additional associated TEL URI (NOTE 2) |  |  |
|  | A3 | emergency public user identity (NOTE 3) |  |  |
|  | A5 | IMPU preconfigured in the UE, additional associated TEL URI (NOTE 2) |  |  |
| Service-Route | A2 |  |  | RFC 3608 [19] |
| addr-spec |  | scscf.3gpp.org |  |  |
| uri-parameter |  | lr |  |  |
| Path |  |  |  | RFC 3327 [20] |
| addr-spec |  | SS P-CSCF address |  |  |
| uri-parameter |  | lr |  |  |
| Feature-Caps |  |  |  | RFC 6809 [125] |
| feature-param | A4 | +g.3gpp.atcf="tel:+1-237-888-9999” | Rel-11 |  |
|  | A4 Azzz | +g.3gpp.cs2ps-srvcc="<sip:sti-sr@atcf.visited2.net>" +g.3gpp.datachannel | Rel-11 Rel-18 | RFC 6809 [125] |
| Feature-Caps | A5 | not present |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | 0 |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |
| A2 | Response for a non-emergency registration (NOTE 4) |
| A3 | Response for an emergency registration (NOTE 4) |
| A4 | Response if the UE provided the +g.3gpp.cs2ps-srvcc and +g.3gpp.cs2ps-srvcc-alerting feature-params in the REGISTER message |
| A5 | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |
| Azzz | UE provided the +sip.app-subtype feature-params in the REGISTER message |

<End of modifed section4>