3GPP TSG-RAN5 Meeting #106	R5-250404r3

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0640 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new test case 6.10 Initial Registration / IMS data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add new test case 6.10 Initial Registration / IMS data channel |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new test case 6.10 Initial Registration / IMS data channel |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 34.229-1 CR 0405 0402 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A6 is new condition defined in section A.1.3 in TS 34.229-1 , CR 0402. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correct the related CR number in coverpage, change statement in Test Purpose(TP) section, correct the release version in requirement section, correct the test procedure in Table 6.10.3.2-1, removed some default message contents and correct the Derivation Path. R2: Remove new condition Ayyy in Table 6.10.3.3-1 and Table 6.10.3.3-3. R3: Add the Source to TSG in coverpage, clarify the index number of condition. |  |  |  |  |  |  |  |  |

<Start of modifed section1>

6.10	      Initial Registration / IMS data channel / 5GS

6.10.1	Test Purpose (TP)

(1)

with { UE supporting IMS data channel has an ISIM or USIM inserted, is registered for 5GS, and has acquired P-CSCF address(es) }

ensure that {

when { UE is made to register for IMS }

then { UE sends a correctly composed initial REGISTER request including the sip.app-subtype media feature tag with a value “webrtc-datachannel” }

}

(2)

with { UE having sent unprotected REGISTER request}

ensure that {

when { UE receiving a valid 401 (Unauthorized) response for the initial REGISTER request sent}

then { UE sends a correctly composed REGISTER request including the sip.app-subtype media feature tag with a value “webrtc-datachannel”}

}

6.10.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-18 requirements.

[TS 26.114, clause 6.2.10.1]:

Support of data channel media is optional for an MTSI client and an MTSI client in terminal. For brevity, an MTSI client supporting data channel is henceforth denoted as a DCMTSI client or DCMTSI client in terminal, respectively.

To indicate support for the procedures in this clause, a DCMTSI client shall when including media feature tags as specified in TS 24.229 [7] include a +sip.app-subtype media feature tag, as specified by RFC 5688 [177], with a value of "webrtc-datachannel" (the application media format used by [172]), regardless of data channel media being part of the SDP or not.

......

[TS 24.186, clause 9.2.1]:

The policy related to the UE supporting the IMS data channel can be provided by the network to the UE using e.g., OMA-DM with the management objects specified in 3GPP TS 24.275 [11], ISIM with EFIMSDCI file specified in 3GPP TS 31.103 [30] or USIM with EFIMSDCI file specified in 3GPP TS 31.102 [31]. When the UE is configured as specified in 3GPP TS 24.275 [11], 3GPP TS 31.103 [30] or 3GPP TS 31.102 [31] with configuration for IMS data channel allowed then the UE determines support for IMS data channel according to the configuration. If the UE is configured with both IMS_DC_configuration node and EFIMSDCI  file, then the EFIMSDCI  file shall take precedence.

If the UE is configured with IMS_DC_configuration node specified in 3GPP TS 24.275 [11] or EFIMSDCI  file specified in 3GPP TS 31.103 [30] or 3GPP TS 31.102 [31], and the DC_allowed leaf of the IMS_DC_configuration node or IMS DC Establishment Indication of the EFIMSDCI  file indicates that IMS data channel is allowed, then a UE supporting IMS data channel on sending an unprotected REGISTER request shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type. For the IMS data channel capability indication, the UE shall use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4].

NOTE:	Precedence for EFIMSDCI file configured on both the USIM and ISIM is defined in 3GPP TS 31.102 [31].

On receiving the 200 (OK) response to the REGISTER request, if the 200 (OK) response includes a Feature-Caps header field containing feature-capability indicator "+g.3gpp.datachannel", the UE shall determine that the home network supports the IMS data channel capability as specified in 3GPP TS 23.228 [3].

6.10.3	Test description

6.10.3.1	Pre-test conditions

System Simulator:

-	SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE.

-	SS is listening to SIP default port 5060 for both UDP and TCP protocols.

-	SS is able to perform IMS AKA authentication for the IMPI, according to 3GPP TS 33.203 [16] clause 6.1.

-	1 NR Cell

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS data channel after switch on.

-	The UE is switched off.

Preamble:

-	None

6.10.3.2	Test procedure sequence

Table 6.10.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE send a correctly composed initial registration request including the sip.app-subtype media feature tag with a value “webrtc-datachannel”? | --> | REGISTER | 1 | P |
| 3 | SS sends 401 Unauthorized. | <-- | 401 Unauthorized | - | - |
| 4 | Check: Does the UE send a subsequent registration request including the sip.app-subtype media feature tag with a value “webrtc-datachannel”? | --> | REGISTER | 2 | P |
| 5 | SS sends a valid 200 OK response including a Feature-Caps header field for REGISTER. | <-- | 200 OK | - | - |
|  | EXCEPTION: In parallel to the events described in steps 6 to 9, the steps specified in Table 6.10.3.2-2 may take place. | - | - | - | - |
| 6 | The UE subscribe to reg-event. | --> | SUBSCRIBE | - | - |
| 7 | SS sends 200 OK for SUBSCRIBE. | <-- | 200 OK | - | - |
| 8 | SS sends NOTIFY for reg-event package, containing full registration state information for the registered public user identity in the XML body. | <-- | NOTIFY | - | - |
| 9 | The UE acknowledge reception of NOTIFY. | --> | 200 OK | - | - |
| NOTE: The associated NR/5GC signalling is specified in TS 38.508-1 [21] generic procedure 4.5.2. |  |  |  |  |  |

Table 6.10.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE sends a PUBLISH request. | --> | PUBLISH |  |  |
| 2 | SS sends a 503 Service Unavailable response | <-- | 503 Service Unavailable |  |  |

6.10.3.3	Specific message contents

Table 6.10.3.3-1: REGISTER (step 2, table 6.10.3.2-1)

Derivation path: TS 34.229-1 [2], Table in subclause A.1.1, Condition A1

Table 6.10.3.3-3: REGISTER (step 4, table 6.10.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.1, Conditions A2, A17

Table 6.10.3.3-4: 200 OK for REGISTER (step 5, table 6.10.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.1.3, Condition A6

<End of modifed section1>