3GPP TSG-RAN5 Meeting #106	R5-250430r1

# Athens, Greece, Feb 17-21, 2025

Title:	Discussion on REFSENS testing for DL CA with single UL

Source:	Huawei, HiSilicon, OPPO

Document for:	Endorsement

# Background

In RAN plenary meeting#106, LS [1] was sent to RAN5, which is about RAN4 agreements on HPUE in DL CA with 1UL CC configured scenario as below table. This paper is to discuss about the necessary changes in RAN5 as per the RAN4 agreements.

- As an exception, UE is allowed to indicate higher power class than default power class for a CA configuration with a single UL CC with DL CA and applicability note for the supported power class is not present in RAN4 specification. In this case, the UE shall be verified with default power class MSD requirements with Tx power being configured with default power class - For UEs already on the market before the HPUE MSD requirements are introduced into RAN4 specification, they are not required to meet the HPUE MSD requirements. - Considering UEs already under development may confront the situation that they were not verified against the HPUE MSD requirements to be defined in the future, it was agreed that a 6 months transition period is applied. The transition period is defined as the moment that the HPUE MSD requirement of a band combination with 1UL CC is added into RAN4 specification to the end of the 6 months period following the introduction. During this transition period, the MSD requirements for UEs shall be verified with the default power class.

# Discussion

## Uplink transmit power configuration in tests

Previously, UE can transmit higher power class than default PC3 on one single uplink carrier only when the HPUE applicability notes present in the second column of the Tables in TS 38.101-1 clause 5.5A.3.1. For example:

Table 5.5A.3.1-1a: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands) NR CA configuration; Uplink CA configuration or single uplink carrier10; NR Band; Channel bandwidth (MHz) (NOTE 3); Bandwidth combination set. CA_n1A-n3A; n38 CA_n1A-n3A; n1; 5, 10, 15, 20; 0. n3; 5, 10, 15, 20, 25, 30. n1; 5, 10, 15, 20, 25, 30, 40, 50; 1. n3; 5, 10, 15, 20, 25, 30, 40. n1; 5, 10, 15, 20; 2. n3; 5, 10, 15, 20, 25, 30, 35, 40. n1; n1 channel bandwidths in Table 5.3.5-1; 4 and 5. n3; n3 channel bandwidths in Table 5.3.5-1. ……. CA_n1A-n41A; n418,9 CA_n1A-n41A8; n1; 5, 10, 15, 20; 0. n41; 10, 15, 20, 40, 50, 60, 80, 90, 100. n1; 5, 10, 15, 20, 25, 30, 40, 50; 1. n41; 10, 15, 20, 30, 40, 50, 60, 80, 90, 100. ……. NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 10: Only single uplink carriers with power class other than PC3 are listed.

- If these notes exist, UE could support the PC2 or PC1.5 MSD requirements for this DL CA with 1UL CC configuration.

- Without these notes, UE should only support default PC3 MSD requirements for this DL CA with 1UL CC configuration.

Observation 1: Previously, in RAN4 the HPUE can be supported only when the applicability notes were added into the band combination tables in TS 38.101-1.

In RAN4#113, CRs [2,3] were agreed which allow UEs to enable higher power class before the corresponding HPUE MSDs are defined. This is a big change to how HPUE is enabled and tested today.

- Currently, when testing the MSD requirements for DL CA with 1UL CC configuration in RAN5 test case 7.3A.1_1 Reference sensitivity power level for 2DL CA exceptions, UE needs to indicate the power class of the single UL carrier in TS 38.508-2 Table A.4.3.2A.4.1-3 based on the TS 38.101-1 Table 5.5A.3.1-1 and select applicable test requirement tables.

- The test requirement of test case 7.3A.1_1 is described as below:

7.3A.1_1.5 Test requirement For inter-band carrier aggregation the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.2.2 with parameters specified in Table 7.3A.1_1.5-1 for PC3 2UL CA configuration and CA configuration with single uplink carrier indicating power class 3 in TS 38.508-2 table A.4.3.2A.4.1-3, and in Table 7.3A.1_1.5-1a for PC2 2UL CA configuration and CA configuration with single uplink carrier indicating power class 2 in TS 38.508-2 table A.4.3.2A.4.1-3. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

And table A.4.3.2A.4.1-3 in TS 38.508-2 is extracted as below:

Table A.4.3.2A.4.1-3: Supported configurations for NR Inter-band CA within FR1 and two bands NR FR1 Inter-band CA configuration / Item (Note 1, 9); Release; Supported; Supported power class for single uplink carrier (Note 12); Supported CA Bandwidth Class(es) in UL (Note 2,5); Supported Bandwidth Combination Set(s) (Note 3); Supported 1Tx-2Tx ULTxSwitch-ing Band Pair (Note 7, 8); Supported 2Tx-2Tx ULTxSwitching Band Pair (Note 7, 8); Supported uplinkTx Switching-DL-Interruption-r16 (Note 10); Supported simultaneousRxTx (Note 11). CA_n1A-n3A; Rel-16. CA_n1A-n5A; Rel-17. Unchanged rows skipped. Note 12: The UE supplier shall indicate the supported single uplink carrier with power class other than PC3, as per TS 38.101-1 [23] Table 5.5A.3.1-1. For this release of specification valid choices are ’-’, ‘nX PC2’, ‘nY PC2’, where both nX and nY are the NR bands. For example, for CA_n1A-n78A, ‘-’ would mean only supports PC3 single uplink carrier, ‘n1 PC2’ would mean supports single carrier PC2 on band n1, ‘n78 PC2’ would mean supports single carrier PC2 on band n78..

As per the new RAN4 agreements, UE could transmit higher power class than PC3 even though the HPUE applicability notes are not present and HPUE MSD requirements are absent.

Taking Power class 2 as an example, when there are no applicable PC2 test requirements for a DL CA with single UL configuration in table 7.3A.1_1.5-1a, there maybe two cases:

| RAN4 has completed the analyzation and confirmed that no PC2 MSD requirements will be defined for this configuration | - The HPUE note in clause 5 already applies for this configuration - PC2 UE should skip the testing of TC 7.3A.1_1 |
| --- | --- |
| RAN4 hasn’t finished the analyzation and the PC2 MSD requirements are not defined yet | - The HPUE note in clause 5 haven’t applied for this configuration - PC2 UE should perform the testing of TC 7.3A.1_1 with PC3 MSD requirements and PC3 UL power class |

To avoid ambiguity about above two cases, p-max limitation should be introduced into test case 7.3A.1_1.

- For the CA configurations without HPUE applicability notes, UE should be verified with default PC3 MSD requirements with Tx power being configured with default PC3. In this case the p-max should be configured as 23.

Observation 2: Enabling HPUE before the HPUE MSDs are defined has impact to RAN5 UE test configurations and requirements to be verified due to different situations.

Proposal 1:	For CA configuration with single uplink carrier testing in 7.3A.1_1, configure P-max = 23 when the HPUE applicability notes do not exist in clause 5 of TS 38.521-1

## Transition period application

According to [4], it can be seen that the purpose of introducing transition period is to handle the tests of UEs which are already on the market or already under development based on the PC3 MSD requirements. RAN4 agreed that for these two kinds of UEs they will only be verified with PC3 MSD requirements. And a 6 months transition period therefore was introduced for the newly introduced HPUE MSD requirements as shown in figure 1 copied from [4].

Figure 1 Transition period for HPUE MSD requirement application [4]

As can be seen, in RAN4 agreements, the transition period is defined as the moment that the HPUE MSD requirement of a band combination with 1UL CC is added into RAN4 specification to the end of the 6 months period following the introduction.

Considering the fact that in RAN4, Release 18 and earlier specification has been frozen over 6 months, and new band combos RF requirements won’t be introduced into frozen release specifications. In this case RAN5 can introduce the HPUE MSD requirements any time as long as there are industry needs and these requirements should be verified once the RAN5 specification is ready.

But in Release 19 and forward, it could happen that the HPUE MSD requirements are defined less than 6 months in RAN4. In this case, before submitting CRs to introduce HPUE MSD requirements into RAN5 specification, the RAN5 contributor should check the RAN4 specification within the recent 6 months (basically the latest two versions).

- If the HPUE MSD have been defined over 6 months, it could be introduced into RAN5 and be verified

- If the HPUE MSD are defined less than 6 months, do not introduce it into RAN5 until the requirement has satisfied the 6 months transition period defined by RAN4

Proposal 2:	For Release 19 and forward, only the HPUE MSD requirements which have been defined over 6 months in TS 38.101-1 could be introduced into TS 38.521-1

# Conclusion

This paper discussed the RAN LS on the HPUE for DL CA with 1UL CC, and has got below observations and proposals.

Observation 1: Previously, in RAN4 the HPUE can be supported only when the applicability notes were added into the band combination tables in TS 38.101-1.

Observation 2: Enabling HPUE before the HPUE MSDs are defined has impact to RAN5 UE test configurations and requirements to be verified due to different situations.

Proposal 1:	For CA configuration with single uplink carrier testing in 7.3A.1_1, configure P-max =23 when the HPUE applicability notes do not exist in clause 5 of TS 38.521-1.

Proposal 2:	For Release 19 and forward, only the HPUE MSD requirements which have been defined over 6 months in TS 38.101-1 could be introduced into TS 38.521-1.

*	Proposal 1 is implemented in CR R5-250431

# Annex

[1] RP‑243308 	LS on HPUE MSD application for DL CA with 1UL CC, RAN->R5/4 OPPO, RAN#106, 2024-12

[2] R4-2420520 	(R17) CR to TS 38.101-1: DL CA HPUE, Qualcomm, RAN4#113 2024-11

[3] R4-2420524 	(R18) CR to TS 38.101-1: DL CA HPUE, Qualcomm, RAN4#113 2024-11

[4] RP-242788 	RAN task on HPUE with 1UL and DL CA, OPPO, Huawei, RAN#106 2024-12