3GPP TSG-RAN5 Meeting #106	 R5-250488r1

Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0546 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test applicability for RRM NES test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability for Netw-Energy test cases is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | The applicability criteria for 3 RRM multiRx test cases has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0, 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-2 CR 0785 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | X1 shall be resolved according to R5-250808 Table A.4.3.2-1 of TS 38.508-2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated the RAN5 dependent CR details on the coversheet |  |  |  |  |  |  |  |  |

## 4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The tested bands selection criteria given in Table 4.0-2 apply. The tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The subtest selection criteria given in Table 4.0-4 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2, Table 4.0-3 and Table 4.0-4 are defined in TS 38.508-2 [8] unless otherwise stated.

----Start of Change 1----

Table 4.0-1: Applicability of conformance test cases conditions

C243 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.1-3/2 AND A.4.3.2B.2.0-1A/2 AND A.4.3.5-1/12 THEN R ELSE N/A

C244 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/1 AND A.4.3.5-1/13 THEN R ELSE N/A

C244a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/1 AND A.4.3.5-1/13 AND A.4.3.2-1/X1 THEN R ELSE N/A

C245 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/119 AND A.4.3.1-7a/1 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C246 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/120 AND A.4.3.1-7a/1 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C247 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/119 AND A.4.3.1-7a/1 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C248 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/120 AND A.4.3.1-7a/1 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C249 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/119 AND (NOT A.4.3.9-1/2 AND A.4.3.1-7a/2) THEN R ELSE N/A

C250 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/120 AND (NOT A.4.3.9-1/2 AND A.4.3.1-7a/2) THEN R ELSE N/A

C251 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/119 AND (NOT A.4.3.9-1/2 AND A.4.3.1-7a/3) THEN R ELSE N/A

C252 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/120 AND (NOT A.4.3.9-1/2 AND A.4.3.1-7a/3) THEN R ELSE N/A

C253 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.6-1/68 AND A.4.3.6-1/72 THEN R ELSE N/A

C254 Void

C255 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.6-1/68 AND A.4.3.6-1/72 AND THEN R ELSE N/A

C256 Void

C257 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-5/1 AND A.4.3.6-1/66 AND A.4.3.6-1/69 THEN R ELSE N/A

C258 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/1 AND A.4.3.6-1/68 AND A.4.3.6-1/72 THEN R ELSE N/A

C259 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND ((A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) OR (A.4.3.2-1/42a OR A.4.3.2-1/43a OR A.4.3.2-1/44a)) AND A.4.3.6-1/73 THEN R ELSE N/A

C260 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND ((A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) OR (A.4.3.2-1/42a OR A.4.3.2-1/43a OR A.4.3.2-1/44a)) AND A.4.3.6-1/74 THEN R ELSE N/A

C261 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.1-7a/1 AND NOT A.4.3.1-7a/2 AND NOT A.4.3.1-7a/3 AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C262 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.1-7a/1 AND NOT A.4.3.1-7a/2 AND NOT A.4.3.1-7a/3 AND A.4.3.2A.1-1/2 THEN R ELSE N/A

C263 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.1-7a/1 AND NOT A.4.3.1-7a/2 AND NOT A.4.3.1-7a/3 AND A.4.3.2A.1-1/3 THEN R ELSE N/A

C264 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/63 THEN R ELSE N/A

C265 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 AND A.4.3.6-1/63 AND A.4.3.6-1/65 THEN R ELSE N/A

C266 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/63 AND A.4.3.6-1/67 THEN R ELSE N/A

C267 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.1-3/2 AND A.4.3.2B.2.0-1A/2 AND A.4.3.2-1/128 THEN R ELSE N/A

C267a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.1-3/2 AND A.4.3.2B.2.0-1A/2 AND A.4.3.2-1/128 AND A.4.3.2-1/X1 THEN R ELSE N/A

C268 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) AND A.4.1-3/2 AND A.4.3.2-1/129 THEN R ELSE N/A

C268a IF A.4.1-1/2 AND A.4.1-2/8 A.4.1-3/2 AND A.4.1-4A/4 AND A.4.3.2-1/129 THEN R ELSE N/A

C268b IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.1-4/6 AND A.4.3.2-1/130THEN R ELSE N/A

C269 IF A.4.1-1/2 AND A.4.1-4/5 AND A.4.1-3/2 AND A.4.3.2B.2.0-1A/2 AND A.4.3.2-1/128 THEN R ELSE N/A

C270 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/1 AND A.4.3.2-1/128 THEN R ELSE N/A

C270a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND (A.4.1-3/1 OR A.4.1-3/2) AND A.4.1-4A/5 AND A.4.3.2A.1-1/1 AND A.4.3.2-1/128 AND A.4.3.2-1/X1 THEN R ELSE N/A

C271 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.1-4A/3 AND A.4.3.2A.1-1/1 AND A.4.3.2-1/128 THEN R ELSE N/A

C271a IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.1-4A/6 AND A.4.3.2A.1-1/1 AND A.4.3.2-1/128 THEN R ELSE N/A

----End of Change 1----

## 4.2	RRM conformance test cases

----Start of Change 2----

Table 4.2-1: Applicability of RRM EN-DC FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Subtest Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 4.3 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 4.3.2 | RRC connection mobility control |  |  |  |  |  |  |
| 4.3.2.2 | Random access |  |  |  |  |  |  |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.3.2.2.1 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.3.2.2.2 has been executed. | 2Rx 4Rx 8Rx | Subtest 2: F001 |
| 4.3.2.2.3 | EN-DC FR1 2-step contention based random access | Rel-16 | C157 | UEs supporting EN-DC FR1 and 2-step RACH | Test execution not necessary if test 6.3.2.2.3 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.3.2.2.4 | EN-DC FR1 2-step non-contention based random access | Rel-16 | C158 | UEs supporting EN-DC FR1 and 2-step RACH | Test execution not necessary if test 6.3.2.2.4 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.4 | Timing |  |  |  |  |  |  |
| 4.4.1 | UE Transmit Timing |  |  |  |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.4.1.1 has been executed. | 2Rx 4Rx 8Rx | Subtest 2: F002 |
| 4.4.2 | UE timer accuracy |  |  |  |  |  |  |
| 4.4.3 | Timing Advance |  |  |  |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.4.3.1 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5 | Signalling characteristics |  |  |  |  |  |  |
| 4.5.1 | Radio link monitoring |  |  |  |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.5.1.1 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C021 | UEs supporting EN-DC FR1 | Test execution not necessary if test 6.5.1.2 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.5.1.3 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle | Test execution not necessary if test 6.5.1.4 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM | Test execution not necessary if test 6.5.1.5 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C038 | UEs supporting EN-DC FR1 and CSI-RS-based RLM | Test execution not necessary if test 6.5.1.6 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if test 6.5.1.7 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C038a | UEs supporting EN-DC FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if test 6.5.1.8 has been executed. | 2Rx 4Rx 8Rx |  |
| 4.5.1.9 | EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | Rel-17 | C021e | UEs supporting EN-DC FR1, long DRX cycle and RLM relaxed measurements |  | 2Rx 4Rx 8Rx |  |
| 4.5.2 | Interruption |  |  |  |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx 8Rx |  |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Rel-15 | C021a | UEs supporting EN-DC FR1 and long DRX cycle |  | 2Rx 4Rx 8Rx |  |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx 8Rx |  |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx 8Rx |  |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx 8Rx |  |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Rel-15 | C068 | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA |  | 2Rx 4Rx 8Rx |  |
| 4.5.2.8 | EN-DC FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC | Rel-16 | C67a | UEs supporting EN-DC FR1 and 2DL CA in NR and NR SRS carrier based switching | Note 1 | 2Rx 4Rx 8Rx |  |
| 4.5.2.9 | EN-DC FR1 interruptions at E-UTRA SRS carrier based switching | Rel-16 | C68a | UEs supporting EN-DC FR1 and 2DL CA in E-UTRA and E-UTRA SRS carrier based switching | Note 1 | 2Rx 4Rx 8Rx |  |
| 4.5.2.10 | EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | Rel-17 | C288 | UEs supporting EN-DC FR1 and activation and deactivation on SCG |  | 2Rx 4Rx 8Rx |  |
| 4.5.3 | Scell activation and deactivation delay |  |  |  |  |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx 8Rx |  |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx 8Rx |  |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C067 | UEs supporting EN-DC FR1 and 2DL CA in NR |  | 2Rx 4Rx 8Rx |  |
| 4.5.3.5 | EN-DC FR1 direct SCell activation at SCell addition of known SCell | Rel-16 | C243 | UEs supporting EN-DC FR1 and 2DL CA in NR and direct SCell activation |  | 2Rx 4Rx 8Rx |  |
| 4.5.3.6 | EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-17 | C267 | UEs supporting EN-DC FR1 and 2DL CA in NR and fast SCell activation |  | 2Rx 4Rx 8Rx |  |
| 4.5.3.7 | EN-DC FR1 fast SCell Activation in non-DRX of known SCell for 640ms SCell measurement cycle | Rel-17 | C267 | UEs supporting EN-DC FR1 and 2DL CA in NR and fast SCell activation |  | 2Rx 4Rx 8Rx |  |
| … |  |  |  |  |  |  |  |
| 4.5.3.12 | EN-DC FR1 Inter-band SSB-less SCell Activation based on A-TRS | Rel18 | C267a | UEs supporting EN-DC FR1 and 2DL CA in NR and fast SCell activation based on A-TRS |  | 2Rx 4Rx 8Rx |  |

Table 4.2-3: Applicability of RRM NR SA FR1 conformance test cases, ref. TS 38.533 [5]

| Clause | TC Title | Release | Applicability |  | Additional Information | Branch | Subtest Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6.1 | RRC_IDLE state mobility |  |  |  |  |  |  |
| 6.1.1 | NR cell re-selection |  |  |  |  |  |  |
| 6.1.1.1 | NR SA FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.3 | NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.4 | NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.5 | NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.6 | NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C093 | UEs supporting 5GS NR SA FR1 and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C052 | UEs supporting 5GS NR SA FR1 and measurement enhancements in HST |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.8 | NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17 | Rel-17 | C052a | UEs supporting 5GS NR SA FR1 and inter-freq measurement enhancements in HST |  | 2Rx 4Rx 8Rx |  |
| 6.1.1.9 | NR SA FR1 cell reselection to intra-frequency NR case for UE operating on a cell with less than 5MHz BW | Rel-18 | C342 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz |  | 2Rx 4Rx 8Rx |  |
| 6.1.2 | NR – E-UTRA cell re-selection |  |  |  |  |  |  |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx 8Rx |  |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx 8Rx |  |
| 6.1.2.3 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion | Rel-16 | C094 | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.2.4 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion | Rel-16 | C094 | UEs supporting 5GS NR SA FR1 and E-UTRA and relaxed RRM measurement |  | 2Rx 4Rx 8Rx |  |
| 6.1.2.5 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16 | Rel-16 | C025b | UEs supporting 5GS NR SA FR1 and E-UTRA and E-UTRA inter-RAT measurement enhancements in HST |  | 2Rx 4Rx 8Rx |  |
| 6.2 | RRC_INACTIVE state mobility |  |  |  |  |  |  |
| 6.2.1 | NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT) | Rel-17 | C001m | UEs supporting 5GS NR SA FR1, CG-SDT and long DRX cycle |  | 2Rx 4Rx 8Rx |  |
| 6.3 | RRC_CONNECTED state mobility |  |  |  |  |  |  |
| 6.3.1 | Handover |  |  |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.3 | NR SA FR1-FR1 handover with unknown target cell | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.6 | NR SA FR1 – UTRAN FDD handover with known target cell | Rel-16 | C096 | UEs supporting 5GS NR SA FR1 and UTRAN FDD |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.7 | NR SA FR1 synchronous DAPS handover | Rel-16 | C101 | UEs supporting 5GS NR SA FR1 and intra-frequency DAPS handover |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.8 | NR SA FR1 asynchronous DAPS handover | Rel-16 | C102 | UEs supporting 5GS NR SA FR1 and intra-frequency async DAPS handover |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.9 | NR SA FR1 Intra-band inter-frequency synchronous DAPS handover | Rel-16 | C107 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.10 | NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover | Rel-16 | C108 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover |  | 2Rx 4Rx 8Rx |  |
| 6.3.1.11 | NR SA FR1 Inter-band inter-frequency synchronous DAPS handover | Rel-16 | C107 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx 8Rx |  |
|  |  |  | C109 | UEs supporting 5GS NR SA FR1 and inter-frequency DAPS handover and supporting different SCSs in source Pcell and inter-frequency target Pcell | For test configuration 3, 6, 7, 8 | 2Rx 4Rx 8Rx |  |
| 6.3.1.12 | NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover | Rel-16 | C108 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover | For test configuration 1, 2, 4, 5, 9 | 2Rx 4Rx 8Rx |  |
|  |  |  | C110 | UEs supporting 5GS NR SA FR1 and inter-frequency async DAPS handover and supporting different SCSs in source Pcell and inter-frequency target Pcell | For test configuration 3, 6, 7, 8 | 2Rx 4Rx 8Rx |  |
| 6.3.1.18 | NR SA FR1 Intra-frequency handover for unknown target cell operating with 12 PRB SSB bandwidth | Rel-18 | C343 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz |  | 2Rx 4Rx 8Rx |  |
| 6.3.2 | RRC connection mobility control |  |  |  |  |  |  |
| 6.3.2.1 | RRC re-establishment |  |  |  |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.3.2.2 | Random access |  |  |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.3.2.2.1 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.3.2.2.2 has been executed. | 2Rx 4Rx 8Rx | Subtest 2: F001 |
| 6.3.2.2.3 | NR SA FR1 2-step contention based random access | Rel-16 | C159 | UEs supporting 5GS NR SA FR1and 2-step RACH | Test execution not necessary if test 4.3.2.2.3 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.3.2.2.4 | NR SA FR1 2-step non-contention based random access | Rel-16 | C159 | UEs supporting 5GS NR SA FR1and 2-step RACH | Test execution not necessary if test 4.3.2.2.4 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.3.2.3 | RRC connection release with redirection |  |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 |  | 2Rx 4Rx 8Rx |  |
| 6.3.2.3.2 | NR SA FR1 - E-UTRA RRC connection release with redirection | Rel-15 | C025 | UEs supporting 5GS NR SA FR1 and E-UTRA |  | 2Rx 4Rx 8Rx |  |
| 6.3.2.4 | LTM PDCCH-order Random Access |  |  |  |  |  |  |
| 6.3.2.4.1 | PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP | Rel-18 | C364 | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s), interruptions caused by PDCCH order |  | 2Rx 4Rx 8Rx | Test 2: F043 Test 3: F044 |
| 6.3.2.4.2 | PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM | Rel-18 | C366 | UEs supporting 5GS NR SA FR1, inter-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, inter-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s), interruptions caused by PDCCH order and Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) |  | 2Rx 4Rx 8Rx | Test 2: F047 |
| 6.3.2.4.3 | PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP | Rel-18 | C365 | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, interruptions caused by PDCCH order |  | 2Rx 4Rx 8Rx | Test 2: F045 Test 3: F046 |
| 6.3.3 | Conditional handover |  |  |  |  |  |  |
| 6.3.3.1 | NR SA FR1 conditional handover | Rel-16 | C105 | UEs supporting 5GS NR SA FR1 and Conditional handover |  | 2Rx 4Rx 8Rx |  |
| 6.3.3.2 | NR SA FR1-FR1 conditional handover | Rel-16 | C105 | UEs supporting 5GS NR SA FR1 and Conditional handover |  | 2Rx 4Rx 8Rx |  |
| 6.3.3.6 | NR SA FR1 Intra-frequency NES triggering conditional handover | Rel-18 |  | UEs supporting 5GS NR SA FR1 and NES-based conditional handover |  | 2Rx 4Rx 8Rx |  |
| 6.3.3.7 | NR SA FR1 Inter-frequency NES-based conditional handover | Rel-18 |  | UEs supporting 5GS NR SA FR1 and NES-based conditional handover |  | 2Rx 4Rx 8Rx |  |
| 6.3.4 | LTM PCell Switch |  |  |  |  |  |  |
| 6.3.4.1 | RACH-based Intra-frequency PCell switch from FR1 to FR1 | Rel-18 | C362 | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | 2Rx 4Rx 8Rx | Test 1B: F037 Test 2A: F038 Test 2B: F039 |
| 6.3.4.2 | RACH based Inter-frequency LTM PCell switch from FR1 to FR1 | Rel-18 | C363 | UEs supporting 5GS NR SA FR1, inter-frequency LTM for MCG with RACH without NR-DC configuration, MAC-CE activated joint LTM TCI states and inter-frequency L1-RSRP measurement and reporting with measurement gaps based on SSB(s) of candidate cell(s) |  | 2Rx 4Rx 8Rx | Test 1B: F040 Test 2A: F041 Test 2B: F042 |
| 6.3.4.3 | RACH-less Intra-frequency PCell switch from FR1 to FR1 | Rel-18 | C357 | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant, TA indication in cell switch command and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) |  | 2Rx 4Rx 8Rx | Test 1B: F031 Test 2A: F032 Test 2B: F033 |
| 6.3.4.4 | RACH-less Intra-frequency PCell switch from FR1 to FR1 without L1-RSRP measurement | Rel-18 | C358 | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated joint LTM TCI states, RACH-less LTM with dynamic grant and TA indication in cell switch command |  | 2Rx 4Rx 8Rx | Test 1B: F034 Test 2A: F035 Test 2B: F036 |
| 6.3.5 | LTM PSCell Switch |  |  |  |  |  |  |
| 6.3.5.1 | NR SA FR1 RACH-based Intra-frequency LTM PSCell switch | Rel-18 | FFS |  | NOTE 1 | 2Rx 4Rx 8Rx |  |
| 6.4 | Timing |  |  |  |  |  |  |
| 6.4.1 | UE transmit timing |  |  |  |  |  |  |
| 6.4.1.1 | NR SA FR1 UE transmit timing accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.4.1.1 has been executed. | 2Rx 4Rx 8Rx | Subtest 2: F002 |
| 6.4.2 | UE timer accuracy |  |  |  |  |  |  |
| 6.4.3 | Timing advance |  |  |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.4.3.1 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5 | Signalling characteristics |  |  |  |  |  |  |
| 6.5.1 | Radio Link Monitoring |  |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.5.1.1 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Rel-15 | C001 | UEs supporting 5GS NR SA FR1 | Test execution not necessary if test 4.5.1.2 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.5.1.3 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Rel-15 | C001b | UEs supporting 5GS NR SA FR1 and long DRX cycle | Test execution not necessary if test 4.5.1.4 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM | Test execution not necessary if test 4.5.1.5 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Rel-15 | C037 | UEs supporting 5GS NR SA FR1 and CSI-RS-based RLM | Test execution not necessary if test 4.5.1.6 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if test 4.5.1.7 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Rel-15 | C037a | UEs supporting 5GS NR SA FR1, CSI-RS-based RLM and long DRX cycle | Test execution not necessary if test 4.5.1.8 has been executed. | 2Rx 4Rx 8Rx |  |
| 6.5.1.9 | SA FR1 radio link monitoring out-of-sync Test for PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | Rel-17 | C037b | UEs supporting 5GS NR SA FR1 and long DRX cycle and CSI-RS-based RLM and RLM relaxation criteria rlm-Relaxation-r17 |  | 2Rx 4Rx 8Rx |  |
| 6.5.1.13 | NR SA FR1 radio Link Monitoring out-of-sync Test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW | Rel-18 | C344 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz and long DRX cycle | NOTE 1 | 2Rx 4Rx 8Rx |  |
| 6.5.1.14 | NR SA FR1 radio Link Monitoring out-of-sync Test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW | Rel-18 | C343 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz | NOTE 1 | 2Rx 4Rx 8Rx |  |
| 6.5.1.16 | NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode | Rel-18 | C344 | UEs supporting 5GS NR SA FR1 and bandwidth of less than 5 MHz and long DRX cycle | NOTE 1 | 2Rx 4Rx 8Rx |  |
| 6.5.2 | Interruption |  |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx 8Rx |  |
| 6.5.2.2 | SA FR1 interruptions at NR SRS carrier based switching | Rel-16 | C031d | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and NR SRS carrier based switching | Note 1 | 2Rx 4Rx 8Rx |  |
| 6.5.3 | Scell activation and deactivation delay |  |  |  |  |  |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | Rel-15 | C031 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.4 | NR SA FR1 direct SCell activation at SCell addition of known SCell | Rel-16 | C244 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.5 | NR SA FR1 direct SCell activation at handover with known SCell | Rel-16 | C244 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.10 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | Rel-17 | C270 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.11 | NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle | Rel-17 | C270 | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation |  | 2Rx 4Rx 8Rx |  |
| … |  |  |  |  |  |  |  |
| 6.5.3.15 | NR SA FR1 TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | Rel-18 | C244a | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and direct SCell activation based on TRS |  | 2Rx 4Rx 8Rx |  |
| 6.5.3.16 | NR SA FR1 Inter-band SSB-less SCell Activation based on A-TRS | Rel-18 | C270a | UEs supporting 5GS NR SA FR1 and CA (2DL CA) and fast SCell activation based on A-TRS |  | 2Rx 4Rx 8Rx |  |

----End of Change 2 ----

----- End of Modified Section ------

<< informal Note for the implementer: /NOK1 is defined in TS 38:508-2 as>>

| X1 | Indicates whether the UE supports SCell without SS/PBCH block for inter-band CA | 38.306 4.2.7.5 | Rel-18 | pc_scellWithoutSSB_InterBandCA_r18 | No |  | FR1 only For supportOfSingleGroup, UE should indicate ‘both’ for two bands or indicate 'referenceBand' for one band and indicate 'scellWithoutSSB' for the other band |
| --- | --- | --- | --- | --- | --- | --- | --- |