3GPP TSG-RAN5 Meeting #106	R5-250533r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4848 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new SL PDCP TC 12.1.10.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TC 12.1.10.1.3 of PC5-only operation / PDCP / data transfer / reordering needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 12.1.10.1.3 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.10.1.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0554 , TS/TR 38.523-3 CR 3621 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: 1.Add the missing verdicts in main behaviour table. 2.Add two Notes at the bottom of the main behaviour table to reduce duplicate content. 3.Update the PIXIT name to “px_NR_SidelinkPDCP_ReorderingTimer” in preamble description. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

12.1.10.1.3	PC5-only operation / PDCP / data transfer / reordering

12.1.10.1.3.1	Test Purpose (TP)

(1)

with { UE has established unicast link with peer UE and has established an unicast SL DRB }

ensure that {

when { A PDCP PDU is received from the lower layer and the COUNT value of its corresponding PDCP SDU is smaller than RX_DELIV }

then { UE discards the PDCP PDU and doesn't submit its corresponding PDCP SDU to upper layer }

}

(2)

with { UE has established unicast link with peer UE and has established an unicast SL DRB }

ensure that {

when { A PDCP PDU is received from the lower layer and the COUNT value of its corresponding PDCP SDU is no less than RX_DELIV and not equal to the COUNT of a PDCP PDU which has been received before }

then { UE stores the corresponding PDCP SDU in reception buffer }

}

(3)

with { UE has established unicast link with peer UE and has established an unicast SL DRB }

ensure that {

when { A PDCP PDU is received from the lower layer and the COUNT value of its corresponding PDCP SDU equals to the COUNT of a PDCP PDU which has been received before}

then { UE discards the PDCP PDU and doesn't submit more than one PDCP SDUs with same COUNT to upper layer }

}

(4)

with { UE has established unicast link with peer UE and has established an unicast SL DRB }

ensure that {

when { A PDCP PDU is received from the lower layer and the COUNT value of its corresponding PDCP SDU equals to RX_DELIV }

then { UE submits PDCP SDU with COUNT = RX_DELIV and all PDCP SDUs with consecutive COUNT values from COUNT = RX_DELIV }

}

(5)

with { UE has established unicast link with peer UE and has established an unicast SL DRB. t-Reordering timer is running }

ensure that {

when { t-Reordering timer expires }

then { UE stops t-Reordering timer, delivers stored PDCP SDUs with COUNT less than RX_REORD and all PDCP SDUs with consecutive COUNT values from the PDCP SDU COUNT = RX_REORD }

}

12.1.10.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.323, subclause 5.2.2 and 5.2.4. Unless otherwise stated these are Rel-16 requirements.

[TS 38.323, subclause 5.2.4]

For sidelink reception of the SLRB, the UE shall follow the procedures in clause 5.2.2 with following modification:

-	perform the header decompression using ROHC as specified in clause 5.7.5, if SDU Type is IP.

[TS 38.323, subclause 5.2.2]

After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall:

-	perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT;

-	if integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the PDCP Data PDU and consider it as not received;

-	if RCVD_COUNT < RX_DELIV; or

-	if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before:

-	discard the PDCP Data PDU;

If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall:

-	store the resulting PDCP SDU in the reception buffer;

-	if RCVD_COUNT >= RX_NEXT:

-	update RX_NEXT to RCVD_COUNT + 1.

-	if outOfOrderDelivery is configured:

-	deliver the resulting PDCP SDU to upper layers after performing header decompression using EHC.

-	if RCVD_COUNT = RX_DELIV:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value > RX_DELIV;

-	if t-Reordering is running, and if RX_DELIV >= RX_REORD:

-	stop and reset t-Reordering.

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

...

When t-Reordering expires, the receiving PDCP entity shall:

-	deliver to upper layers in ascending order of the associated COUNT value after performing header decompression, if not decompressed before:

-	all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD;

-	all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD;

-	update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD;

-	if RX_DELIV < RX_NEXT:

-	update RX_REORD to RX_NEXT;

-	start t-Reordering.

12.1.10.1.3.3	Test description

12.1.10.1.3.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3.

- The UE uses GNSS as the synchronization reference source.

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22. The  PDCP parameter t-Reordering of UE under test is acquired according to px_NR_SidelinkPDCP_ReorderingTimer.

12.1.10.1.3.3.2	Test procedure sequence

Table 12.1.10.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger UE to close UE test loop mode E. (RECEIVE). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 2 | The NR-SS-UE1 sends PDCP Data PDU #0 on SL DRB with PDCP SN = 0. | <-- | PDCP Data PDU #0 | - | - |
| 3 | The NR-SS-UE1 sends PDCP Data PDU #6 on SL DRB with PDCP SN = 0. | <-- | PDCP Data PDU #6 | - | - |
| 4 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. (Note 1) | - | - | - | - |
| 5 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 1? (Note 2) | - | - | 3 | P |
| 6 | The NR-SS-UE1 sends PDCP Data PDU #2 on SL DRB with PDCP SN = 2. The UE starts t-Reordering timer. | <-- | PDCP Data PDU #2 | - | - |
| 7 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets.(t-Reordering is running) (Note 1) | - | - | - | - |
| 8 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 1? (Note 2) | - | -- | 2 | P |
| 9 | Wait until t-Reordering expires. | - | - | - | - |
| 10 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. (Note 1) | - | - | - | - |
| 11 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 2? (Note 2) | - | - | 2,5 | P |
| 12 | The NR-SS-UE1 sends PDCP Data PDU #1 on SL DRB with PDCP SN = 1. | <-- | PDCP Data PDU #1 | - | - |
| 13 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. (Note 1) | - | - | - | - |
| 14 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 2? (Note 2) | - | - | 1 | P |
| 15 | The NR-SS-UE1 sends PDCP Data PDU #4 on SL DRB with PDCP SN = 4. The UE starts t-Reordering timer. | <-- | PDCP Data PDU #4 | - | - |
| 16 | The NR-SS-UE1 sends PDCP Data PDU #3 on SL DRB with PDCP SN = 3.(t-Reordering is running) | <-- | PDCP Data PDU #3 | - | - |
| 17 | NR-SS-UE1 requests the UE to report the counter of successful reception of packets. (Note 1) | - | - | - | - |
| 18 | Check: Does the reported counters of successful reception of SDAP SDU packets equal to 4? (Note 2) | - | - | 4 | P |
| 19 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| Note 1: Requesting the UE to report the counter of successful reception of packets may be performed by MMI or AT command (+CUSPCREQ). Note 2: The UE reporting the counter of successful reception of SDAP SDU packets may be performed by MMI or AT command (+CUSPCREQ). |  |  |  |  |  |

12.1.10.1.3.3.3	Specific message contents

- None.