3GPP TSG-RAN5 Meeting #106	R5-250566r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3702 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to RedCap SS-SINR and SS-RSRQ test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RedCap 1Rx SS-RSRQ and SS-SINR measurement accuracy test case 16.7.2.1 and 16.7.3.1 configures an effective serving cell SINR of <-5 dB which, along with PDCCH aggregation level 4 could require several PHY HARQ and/or RLC ARQ retransmissions for the UE to deliver the measurement report. Increasing the RLC UL maxRetxThreshold from t8 to t32, which is the max allowed as per TS 38.331 will ensure UE’s measurement report is successfully delivered to the TE. This does not alter the RAN4 defined test parameters nor the scope of the test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added message contents exception to test cases 16.7.2.1 and 16.7.3.1 to increase the RLC UL maxRetxThreshold from t8 to t32. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE’s measurement report may not be successfully delivered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.7.2.1, 16.7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: updated the table number for the rlc-config message exceptions |  |  |  |  |  |  |  |  |

<< Start of Changes >>

#### 16.7.2.1	NR SA FR1 SS-RSRQ absolute measurement accuracy for 1 Rx UE

16.7.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands.

16.7.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.7.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.2.1.

16.7.2.1.4	Test description

16.7.2.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.2.1.4.1 with followings exceptions:

-	Table 6.7.2.1.4.1-1 is replaced by Table 16.7.2.1.4.1-1;

-	Table 6.7.2.1.4.1-2 is replaced by Table 16.7.2.1.4.1-2;

Table 16.7.2.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.2.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.7.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.2.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

16.7.2.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.2.1.4.2 with following exceptions:

-	Table 6.7.2.1.5-1 is replaced by Table 16.7.2.1.5-1;

-	Table 6.7.2.1.5-2 is replaced by Table 16.7.2.1.5-2.

16.7.2.1.4.3	Message contents

Same as the message contents given in clause 6.7.2.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.2.1-1 applies to test configuration 16.7.2.1-4.

-	Table H.3.1-3 is used with associated condition SSB.1 RedCap FR1 for test configuration 16.7.2.1-3

Table 16.7.2.1.4.3-1: RLC-Config

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-149 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t32 |  | SRB1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.7.2.1.5	Test requirement

Same as the test requirements given in clause 6.7.2.1.5 with followings exceptions:

-	Table 6.7.2.1.5-1 is replaced by Table 16.7.2.1.5-1;

-	Table 6.7.2.1.5-2 is replaced by Table 16.7.2.1.5-2;

Table 16.7.2.1.5-1: NR cell specific test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  |  |  | Test 2 |  |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 |  |  | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 |  |  |  |  | freq1 |  |  |  | freq1 |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 4 |  | HD-FDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1, 4 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  | Config 1, 4 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 20: NRB,c = 51 |  |  |  |  |  |  |  |  |  |  |  |
| Gap Pattern ID |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1, 4 |  | SR.1.1 FDD |  | - |  |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD |  | - |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | SR2.1 TDD |  |  |  |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1, 4 |  | CR.1.1 FDD |  | - |  |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| Control Channel RMC |  |  | Config 1, 4 |  | CCR.1.1 FDD |  | - |  |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD |  | - |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| TRS Configuration |  |  | Config 1, 4 |  | TRS.1.1 FDD |  | - |  |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD |  | - |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  |  | OP. 1 |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  | Config 1,4 | ms | - | 3 |  |  | - |  | 3 |  | - |  | 3 |  |
|  |  |  | Config 2,3 | s | - | 3 |  |  | - |  | 3 |  | - |  | 3 |  |
| SMTC configuration |  |  | Config 1,4 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |  |  |  |  |  |  |  |  |
| CSI-RS for tracking |  |  | Config 1,4 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2,4 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  | 0 |  |  | 0 |  | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4 |  | Depending on band group | dBm/15kHz | -86.5 |  |  |  |  | -101 |  |  |  | -114+ ΔBG_offset |  |  |
|  | Config 3 |  | Depending on band group |  | -92.6 |  |  |  |  | - |  |  |  | -114+ ΔBG_offset |  |  |
| Note2 | Config 1,2, 4 |  | Depending on band group | dBm/SCS | -86.5 |  |  |  |  | -101 |  |  |  | -114+ ΔBG_offset |  |  |
|  | Config 3 |  | Depending on band group |  | -89.6 |  |  |  |  | - |  |  |  | -111+ ΔBG_offset |  |  |
|  |  |  |  | dB | -1.76 |  |  |  |  | -4.7 |  |  |  | -5.46 |  | -5.46 |
|  |  |  |  | dB | 3 |  | 3 |  |  | -2.9 |  | -2.9 |  | -4 |  | -4 |
| SS-RSRPNote3 | Config 1,2, 4 |  | Depending on band group | dBm/SCS | -83.5 |  | -83.5 |  |  | -103.9 |  | -103.9 |  | -118+ ΔBG_offset |  | -118+ ΔBG_offset |
|  | Config 3 |  | Depending on band Group |  | -85.6 |  | -85.6 |  |  | - |  | - |  | -115+ ΔBG_offset |  | -115+ ΔBG_offset |
| SS-RSRQ Note3 |  |  | Depending on band group | dB | -14.77 |  | -14.77 |  |  | -16.76 |  | -16.76 |  | -17.34+ ΔBG_offset |  | -17.34+ ΔBG_offset |
| IoNote3 |  | Config 1,2, 4 | Depending on band group | dBm/ 9.36MHz | -50 |  |  |  |  | -70 |  |  |  | -83.5+ ΔBG_offset |  |  |
|  |  | Config 3 | Depending on band group | dBm/ 18.36MHz | -50 |  |  |  |  | - |  |  |  | -80.6+ ΔBG_offset |  |  |
| Propagation condition |  |  |  | - | AWGN |  | AWGN |  |  | AWGN |  | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  |  | 1x1 |  | 1x1 |  |  | 1x1 |  | 1x1 |  | 1x1 |  | 1x1 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. Note 7: Subtest 2 is not used when testing with 30kHz SSB SCS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 16.7.2.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x +50 | SS-RSRQ_x +44 | SS-RSRQ_x +42 |
| Highest reported value (Cell 2) | SS-RSRQ_x +64 | SS-RSRQ_x +62 | SS-RSRQ_x +61 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x +47 | SS-RSRQ_x +43 | SS-RSRQ_x +41 |
| Highest reported value (Cell 2) | SS-RSRQ_x +67 | SS-RSRQ_x +63 | SS-RSRQ_x +62 |
| NOTE: SS-RSRQ_x is the reported value of Cell 1. |  |  |  |

<< several sections skipped >>

#### 16.7.3.1	NR SA FR1 SS-SINR absolute measurement accuracy for 1 Rx UE

16.7.3.1.1	Test purpose

To verify that the intra-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands. This test will verify the requirements in TS 38.133[6] clause 10.1A.10.1.1

16.7.3.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards and supporting SS-SINR measurement.

16.7.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.7.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.7.3.1.

16.7.3.1.4	Test description

16.7.3.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.7.3.1.4.1 with followings exceptions:

-	Table 6.7.3.1.4.1-1 is replaced by Table 16.7.3.1.4.1-1;

-	Table 6.7.3.1.4.1-2 is replaced by Table 16.7.3.1.4.1-2;

Table 16.7.3.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.7.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.7.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.7.3.1-3 | NR 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.7.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 16.7.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.7.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.7.3.1.4.2	Test procedure

Same as the test procedure given in clause 6.7.3.1.4.2 with following exceptions:

-	Table 6.7.3.1.5-1 is replaced by Table 16.7.3.1.5-1;

-	Table 6.7.3.1.5-2 is replaced by Table 16.7.3.1.5-2.

16.7.3.1.4.3	Message contents

Same as the message contents given in clause 6.7.3.1.4.3 with following exceptions:

-	Specific message contents exceptions for test configuration 6.7.3.1-1 applies to test configuration 16.7.3.1-4.

Table 16.7.3.1.4.3-1: RLC-Config

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-149 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC SEQUENCE { |  |  |  |
| maxRetxThreshold | t32 |  | SRB1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.7.3.1.5	Test requirement

Same as the test requirements given in clause 6.7.3.1.5 with followings exceptions:

-	Table 6.7.3.1.5-1 is replaced by Table 16.7.3.1.5-1;

-	Table 6.7.3.1.5-2 is replaced by Table 16.7.3.1.5-2;

Table 16.7.3.1.5-1: NR cell specific test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  | freq1 |  | freq1 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
|  | Config 4 |  | HD-FDD |  |  |  |
| TDD configuration | Config 1,4 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  | ms | Not Applicable |  |  |  |
| TRS configuration | Config 1,4 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH RMC | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET RMC | Config 1,4 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC | Config 1,4 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  | Not Applicable |  |  |  |
| SMTC configuration | Config 1,4 |  | SMTC.2 |  |  |  |
|  | Config 2,3 |  | SMTC.1 |  |  |  |
| Time offset with Cell 1 | Config 1,4 | ms | - | 3 | - | 3 |
|  | Config 2,3 | s | - | 3 | - | 3 |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |  |  |
|  | Config 3 |  | SSB.1 RedCap FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2,4 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -93 |  | -116 + ΔBG_offset |  |
| Note2 | Config 1,2,4 | dBm/SCS | -93 |  | -116 + ΔBG_offset |  |
|  | Config 3 |  | -90 |  | -113 + ΔBG_offset |  |
|  |  | dB | 0 | -3.19 | -5.10 | -5.10 |
|  |  | dB | 4.54 | 2.66 | -3.50 | -3.50 |
| SS-RSRPNote3 | Config 1,2,4 | dBm/SCS | -88.46 | -90.34 | -119.50 + ΔBG_offset | -119.50 + ΔBG_offset |
|  | Config 3 |  | -85.46 | -87.34 | -116.50 + ΔBG_offset | -116.50 + ΔBG_offset |
| SS-SINR Note3 |  | dB | 0 | -3.19 | -5.10 | -5.10 |
| IoNote3 | Config 1,2,4 | dBm/ 9.36MHz | -57.5 |  | -85.28 + ΔBG_offset |  |
|  | Config 3 | dBm/ 18.36MHz | -54.57 |  | -82.35 + ΔBG_offset |  |
| Propagation condition |  | - | AWGN |  |  |  |
| Antenna configuration |  | - | 1x1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in TS 38.133 [6] clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

Table 16.7.3.1.5-2: SS-SINR Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SINR_29 | SINR_26 |
| Highest reported value (Cell 2) | SINR_51 | SINR_47 |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SINR_28 | SINR_25 |
| Highest reported value (Cell 2) | SINR_52 | SINR_48 |

<< End of Changes >>