3GPP TSG-RAN5 Meeting #106	R5-250605r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4859 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new SL SDAP TC 12.1.11.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TC 12.1.11.1.1 of PC5-only operation / SDAP / data transfer / QoS flow to SL DRB mapping / transmission side needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 12.1.11.1.1 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.11.1.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0559 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1: 1.Update TP and test descriptions. 2. Hardcode the index of the SL DRBs. 3. Delete PQFI 1 in Table 12.1.11.1.1.3.3-2 and Table 12.1.11.1.1.3.3-3, as it has been created 4. Add IE to disable SL HARQ in Table 12.1.11.1.1.3.3-1, for simplifying the test. 5. Correct some typos. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

12.1.11	PC5-only operation / SDAP

12.1.11.1	PC5-only operation / SDAP / data transfer

12.1.11.1.1	PC5-only operation / SDAP / data transfer / QoS flow to SL DRB mapping / transmission side

12.1.11.1.1.1	Test Purpose (TP)

(1)

with { UE is configured to perform sidelink transmission. UE has established PC5-RRC connection and multiple unicast SL DRBs with peer UE. To each SL DRB there are more than one PC5 QoS flows mapped }

ensure that {

when { UE is indicated by upper layer that data arrives on a QoS flow and UE has stored a mapping rule for this QoS flow }

then { UE builds an SDAP PDU, and maps it to the corresponding SL DRB }

}

(2)

with { UE is configured to perform sidelink transmission. UE has established PC5-RRC connection and multiple unicast SL DRBs with peer UE. To each unicast SL DRB there are more than one PC5 QoS flows mapped and one of them is configured as the default SL DRB }

ensure that {

when { UE is indicated by upper layer that data arrives on a QoS flow and UE has not stored a mapping rule for this QoS flow }

then { UE builds an SDAP PDU, and maps it to the default SL DRB }

}

(3)

with { UE is configured to perform sidelink transmission. UE has established PC5-RRC connection and multiple unicast SL DRBs with peer UE. To each SL DRB there are more than one PC5 QoS flows mapped }

ensure that {

when { RRC configures a new mapping rule for a QoS flow, which is different from the existing mapping rule for this QoS flow }

then { The UE stores the newly configured mapping rule as mapping rule for this QoS flow, transmits an end-marker control PDU on the SL DRB according to the old mapping rule, and uses the new mapping rule for future transmission }

}

(4)

with { UE is configured to perform sidelink transmission. UE has established PC5-RRC connection and multiple unicast SL DRBs with peer UE. To each SL DRB there are more than one PC5 QoS flows mapped }

ensure that {

when { RRC configures a mapping rule for a QoS flow and UE has not stored mapping rule for this QoS flow before }

then { The UE stores the newly configured mapping rule as mapping rule for this QoS flow, transmits an end-marker control PDU on the default SL DRB, and uses the new mapping rule for future transmission }

}

12.1.11.1.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 37.324, clauses 5.2.3 and 5.5.1. Unless otherwise stated these are Rel-16 requirements.

[TS 37.324 clause 5.2.3]

At the reception of an SDAP SDU from upper layer for a PC5 QoS flow, the transmitting SDAP entity shall:

-	if there is no stored PC5 QoS flow to SL-DRB mapping rule for the PC5 QoS flow as specified in the clause 5.5:

-	map the SDAP SDU to the default SL-DRB;

-	else:

-	map the SDAP SDU to the SL-DRB according to the stored PC5 QoS flow to SL-DRB mapping rule;

-	if the SL-DRB to which the SDAP SDU is mapped is configured by RRC (TS 38.331 [3]) with the presence of SDAP header:

-	construct the SL SDAP data PDU as specified in the clause 6.2.2.4;

-	else:

-	construct the SL SDAP data PDU as specified in the clause 6.2.2.1;

-	submit the constructed SL SDAP data PDU to the lower layers.

[TS 37.324 clause 5.5.1]

When RRC (TS 38.331 [3]) configures a PC5 QoS flow to SL-DRB mapping rule for a PC5 QoS flow, the SDAP entity shall:

-	for unicast of NR sidelink communication, if the SDAP entity has already been established and there is no stored SL-DRB mapping rule for the PC5 QoS flow and a default SL-DRB is configured:

-	construct an end-marker control PDU, as specified in the clause 6.2.3, for the PC5 QoS flow;

-	map the end-marker control PDU to the default SL-DRB;

-	submit the end-marker control PDU to the lower layers;

-	for unicast of NR sidelink communication, if the stored PC5 QoS flow to SL-DRB mapping rule is different from the configured PC5 QoS flow to SL-DRB mapping rule for the PC5 QoS flow and the SL-DRB according to the stored PC5 QoS flow to SL-DRB mapping rule is configured by RRC (TS 38.331 [3]) with the presence of SL SDAP header:

-	construct an end-marker control PDU, as specified in the clause 6.2.3, for the PC5 QoS flow;

-	map the end-marker control PDU to the SL-DRB according to the stored PC5 QoS flow to SL-DRB mapping rule;

-	submit the end-marker control PDU to the lower layers;

-	store the configured PC5 QoS flow to SL-DRB mapping rule for the PC5 QoS flow.

12.1.11.1.1.3	Test description

12.1.11.1.1.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.1.11.1.1.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.1.11.1.1.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.1.11.1.1.3.3-1 |  |

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.1.11.1.1.3.2	Test procedure sequence

Table 12.1.11.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | NR-SS-UE1 transmits a DIRECT LINK MODIFICATION REQUEST to add QoS flows | <-- | PC5-S: DIRECT LINK MODIFICATION REQUEST | - | - |
| 2 | UE transmits a DIRECT LINK MODIFICATION ACCEPT message. | --> | PC5-S: DIRECT LINK MODIFICATION ACCEPT | - | - |
| 3 | NR-SS-UE1 sends an RRCReconfigurationSidelink message. | <-- | PC5 RRC: RRCReconfigurationSidelink | - | - |
| 4 | UE sends an RRCReconfigurationCompleteSidelink message. | --> | PC5 RRC: RRCReconfigurationCompleteSidelink | - | - |
| 5 | Trigger UE to close UE test loop mode E. (TRANSMIT). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 6 | Check: Does UE transmit SL SDAP Data PDUs on SL-DRB 2 (with SDAP header PQFI=1, PQFI=2) and SL SDAP Data PDUs on default SL-DRB 1 (with SDAP header PQFI=4, PQFI=5, PQFI=6)? | --> | SL SDAP PDUs | 1,2 | P |
| 7 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 8 | NR-SS-UE1 sends an RRCReconfigurationSidelink message. | <-- | PC5 RRC: RRCReconfigurationSidelink | - | - |
| - | EXCEPTION: In parallel to the event described in step 9 the events specified in Table 12.1.11.1.1.3.2-2 and Table 12.1.11.1.1.3.2-3 shall take place. | - | - | - | - |
| 9 | UE sends an RRCReconfigurationCompleteSidelink message. | --> | PC5 RRC: RRCReconfigurationCompleteSidelink | - | - |
| 10 | Trigger UE to close UE test loop mode E. (TRANSMIT). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 11 | Check: Does UE transmit SL SDAP Data PDUs on SL-DRB 2 (with SDAP header PQFI=1, PQFI=4) and SL SDAP Data PDUs on default SL-DRB 1 (with SDAP header PQFI=2, PQFI=5, PQFI=6)? | --> | SL SDAP PDUs | 3,4 | P |
| 12 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |

Table 12.1.11.1.1.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit End-Marker Control PDU on SL-DRB 2 for PQFI=2? | --> | SDAP Control PDU | 3 | P |

Table 12.1.11.1.1.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit End-Marker Control PDU on default SL-DRB 1 for PQFI=4? | --> | SDAP Control PDU | 4 | P |

12.1.11.1.1.3.3	Specific message contents

Table 12.1.11.1.1.3.3-1: SL-PreconfigurationNR (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |
| sl-RadioBearerPreConfigList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { | 2 entries |  |  |
| SL-RadioBearerConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| slrb-Uu-ConfigIndex-r16 | 1 |  |  |
| sl-SDAP-Config-r16 SEQUENCE { |  |  |  |
| sl-SDAP-Header-r16 | present |  |  |
| sl-DefaultRB-r16 | true |  |  |
| sl-CastType-r16 | unicast |  |  |
| } |  |  |  |
| } |  |  |  |
| SL-RadioBearerConfig-r16[2] SEQUENCE { |  | entry 2 |  |
| slrb-Uu-ConfigIndex-r16 | 2 |  |  |
| sl-SDAP-Config-r16 SEQUENCE { |  |  |  |
| sl-SDAP-Header-r16 | present |  |  |
| sl-DefaultRB-r16 | false |  |  |
| sl-CastType-r16 | unicast |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| sl-RLC-BearerPreConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { | 2 entries |  |  |
| SL-RLC-BearerConfig-r16[1] SEQUENCE { |  | entry 1 |  |
| sl-RLC-BearerConfigIndex-r16 | 1 |  |  |
| sl-ServedRadioBearer-r16 | 1 |  |  |
| sl-MAC-LogicalChannelConfig-r16 SEQUENCE { |  |  |  |
| sl-HARQ-FeedbackEnabled-r16 | disabled | SL HARQ is disabled for simplify |  |
| } |  |  |  |
| } |  |  |  |
| SL-RLC-BearerConfig-r16[2] SEQUENCE { |  | entry 2 |  |
| sl-RLC-BearerConfigIndex-r16 | 2 |  |  |
| sl-ServedRadioBearer-r16 | 2 |  |  |
| sl-MAC-LogicalChannelConfig-r16 SEQUENCE { |  |  |  |
| sl-HARQ-FeedbackEnabled-r16 | disabled | SL HARQ is disabled for simplify |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.11.1.1.3.3-2: DIRECT LINK MODIFICATION REQUEST (Step1, Table 12.1.11.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7D.1-9 with condition Rx |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| PC5 QoS flow description 1 |  |  |  |
| PQFI | '00 0010'B | PQFI 2 |  |
|  | '00 0100'B | PQFI 4 |  |
|  | '00 0101'B | PQFI 5 |  |
|  | '00 0110'B | PQFI 6 |  |

Table 12.1.11.1.1.3.3-3: DIRECT LINK MODIFICATION ACCEPT (Step2, Table 12.1.11.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7D.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| QoS flow descriptions |  |  |  |
| PC5 QoS flow description 1 |  |  |  |
| PQFI | '00 0010'B | PQFI 2 |  |
|  | '00 0100'B | PQFI 4 |  |
|  | '00 0101'B | PQFI 5 |  |
|  | '00 0110'B | PQFI 6 |  |

Table 12.1.11.1.1.3.3-4: RRCReconfigurationSidelink (Step 3, Table 12.1.11.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition SL_DRB and RX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { | 2 entries |  |  |
| SLRB-Config-r16[1] SEQUENCE { |  | entry 1 |  |
| slrb-PC5-ConfigIndex-r16 | 1 |  |  |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  |  |  |
| SL-PQFI-r16 [1] | 5 |  |  |
| SL-PQFI-r16 [2] | 6 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SLRB-Config-r16[2] SEQUENCE { |  | entry 2 |  |
| slrb-PC5-ConfigIndex-r16 | 2 |  |  |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  |  |  |
| SL-PQFI-r16 [1] | 1 |  |  |
| SL-PQFI-r16 [2] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.11.1.1.3.3-5: RRCReconfigurationSidelink (Step 8, Table 12.1.11.1.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.6.1A-3 with condition SL_DRB and RX |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { | 2 entries |  |  |
| SLRB-Config-r16[1] SEQUENCE { |  | entry 1 |  |
| slrb-PC5-ConfigIndex-r16 | 1 |  |  |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  |  |  |
| SL-PQFI-r16 [1] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| SLRB-Config-r16[2] SEQUENCE { |  | entry 2 |  |
| slrb-PC5-ConfigIndex-r16 | 2 |  |  |
| sl-SDAP-ConfigPC5-r16 SEQUENCE { |  |  |  |
| sl-MappedQoS-FlowsToAddList-r16 SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16 { |  |  |  |
| SL-PQFI-r16 [1] | 4 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 12.1.11.1.1.3.3-6: RRCReconfigurationCompleteSidelink (Steps 4 and 9, Table 12.1.11.1.1.3.2-1)

Derivation path: TS 38.508-1 [4], Table 4.6.1A-4 with condition TX