3GPP TSG-RAN5 Meeting #106	R5-250633r1

# Athens, Greece, 17 – 21 Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3204 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RedCap receiver requirements for band n105 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FDD_bands_R18_redcap-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing RedCap receiver requirements for band n105 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added receiver requirements for band n105 in clauses: - 7.3I.2 Reference sensitivity power level for RedCap - 7.3I.3 Reference sensitivity power level for eRedCap |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap receiver requirements for band n105 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3I.2, 7.3I.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: References to Note 1 removed from Table 7.3I.3.4.1-2. |  |  |  |  |  |  |  |  |

-------- Start of changes --------

## 7.3I	Reference sensitivity for (e)RedCap

### 7.3I.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

### 7.3I.2	Reference sensitivity power level for RedCap

7.3I.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3I.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support NR RedCap.

7.3I.2.3	Minimum conformance requirements

For a RedCap UE equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a and Table 7.3.2.3-1b for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 2 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

For a RedCap UE equipped with 1 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2.3-1a and in Table 7.3.2.3-1b shall be modified by the amount given in ΔR1R in Table 7.3I.2.3-1 for the applicable operating bands. The reference sensitivity (REFSENS) requirement specified for a RedCap UE equipped with 1 Rx antenna ports shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2.3-3 and, for FDD bands, with the Tx-Rx separation as defined in clause 5.4.4 for the applicable band and UE channel bandwidth.

Table 7.3I.2.3-1: Single antenna port reference sensitivity allowance ΔR1R

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| TDD band | 5, 10, 15, 20 | 2.5 |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3 |

For a RedCap UE equipped with 2 Rx antenna ports operating in HD-FDD mode, reference sensitivity for 2Rx antenna ports in Table 7.3I.2.3-2 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2.3-4.

Table 7.3I.2.3-2: HD-FDD RedCap UE with 2 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n2 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n3 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
|  | 60 |  | -95.3 | -93.2 | -92.0 |
| n5 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
| n7 | 15 | -98.8 | -95.6 | -93.8 | -92.5 |
|  | 30 |  | -96.0 | -94.0 | -92.7 |
|  | 60 |  | -96.3 | -94.2 | -93.0 |
| n8 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n12 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n13 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n14 | 15 | -97.8 | -94.6 |  |  |
|  | 30 |  | -95.0 |  |  |
| n20 | 15 | -97.8 | -94.6 | -92.8 | -91.5 |
|  | 30 |  | -95.0 | -93.0 | -91.7 |
| n24 | 15 | -100.0 | -96.8 |  |  |
|  | 30 |  | -97.2 |  |  |
|  | 60 |  | -97.5 |  |  |
| n25 | 15 | -97.3 | -94.1 | -92.3 | -91.0 |
|  | 30 |  | -94.5 | -92.5 | -91.2 |
|  | 60 |  | -94.8 | -92.7 | -91.5 |
| n26 | 15 | -98.3 | -95.1 | -93.3 | -92.0 |
|  | 30 |  | -95.5 | -93.5 | -92.2 |
| n28 | 15 | -99.3 | -96.1 | -94.3 | -93.0 |
|  | 30 |  | -96.5 | -94.5 | -93.2 |
| n30 | 15 | -99.5 | -96.3 |  |  |
|  | 30 |  | -96.7 |  |  |
| n65 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n66 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n70 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n71 | 15 | -98.0 | -94.8 | -93.0 | -91.7 |
|  | 30 |  | -95.2 | -93.2 | -91.9 |
| n74 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n85 | 15 | -97.8 | -94.6 | -92.8 |  |
|  | 30 |  | -95.0 | -93.0 |  |
| n91 | 15 | -100.0 |  |  |  |
| n92 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n93 | 15 | -100.0 |  |  |  |
| n94 | 15 | -100.0 | -96.8 | -95.0 | -93.7 |
|  | 30 |  | -97.2 | -95.2 | -93.9 |
| n105 | 15 | -981 | -94.8 | -93 | -91.7 |
|  | 30 |  | -95.2 | -93.2 | -91.9 |
| NOTE 1: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS. |  |  |  |  |  |

For a RedCap UE equipped with 1 Rx antenna ports and operating in HD-FDD mode, reference sensitivity for 1Rx antenna ports in Table 7.3I.2.3-3 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3I.2.3-4.

Table 7.3I.2.3-3: HD-FDD RedCap UE with 1 Rx antenna port reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n2 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n3 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
|  | 60 |  | -92.8 | -90.7 | -89.5 |
| n5 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
| n7 | 15 | -96.3 | -93.1 | -91.3 | -90.0 |
|  | 30 |  | -93.5 | -91.5 | -90.2 |
|  | 60 |  | -93.8 | -91.7 | -90.5 |
| n8 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n12 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n13 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n14 | 15 | -95.3 | -92.1 |  |  |
|  | 30 |  | -92.5 |  |  |
| n20 | 15 | -95.3 | -92.1 | -90.3 | -89.0 |
|  | 30 |  | -92.5 | -90.5 | -89.2 |
| n24 | 15 | -97.5 | -94.3 |  |  |
|  | 30 |  | -94.7 |  |  |
|  | 60 |  | -95.0 |  |  |
| n25 | 15 | -94.8 | -91.6 | -89.8 | -88.5 |
|  | 30 |  | -92.0 | -90.0 | -88.7 |
|  | 60 |  | -92.3 | -90.2 | -89.0 |
| n26 | 15 | -95.8 | -92.6 | -90.8 | -89.5 |
|  | 30 |  | -93.0 | -91.0 | -89.7 |
| n28 | 15 | -96.8 | -93.6 | -91.8 | -90.5 |
|  | 30 |  | -94.0 | -92.0 | -90.7 |
| n30 | 15 | -97.0 | -93.8 |  |  |
|  | 30 |  | -94.2 |  |  |
| n65 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n66 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n70 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n71 | 15 | -95.5 | -92.3 | -90.5 | -89.2 |
|  | 30 |  | -92.7 | -90.7 | -89.4 |
| n74 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
| n85 | 15 | -95.3 | -92.1 | -90.3 |  |
|  | 30 |  | -92.5 | -90.5 |  |
| n91 | 15 | -97.5 |  |  |  |
| n92 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
| n93 | 15 | -97.5 |  |  |  |
| n94 | 15 | -97.5 | -94.3 | -92.5 | -91.2 |
|  | 30 |  | -94.7 | -92.7 | -91.4 |
| n105 | 15 | -95.51 | -92.3 | -90.5 | -89.2 |
|  | 30 |  | -92.7 | -90.7 | -89.4 |
| NOTE 1: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS. |  |  |  |  |  |

Table 7.3I.2.3-4: Uplink configuration for HD-FDD reference sensitivity

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| n1 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n2 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n3 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n5 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n7 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n8 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n12 | 15 | 25 | 50 | 75 |  |
|  | 30 |  | 24 | 36 |  |
| n13 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n14 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n20 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n24 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
|  | 60 |  | 10 |  |  |
| n25 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n26 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n28 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n30 | 15 | 25 | 50 |  |  |
|  | 30 |  | 24 |  |  |
| n65 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n66 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n70 | 15 | 25 | 50 | 75 | NOTE 1 |
|  | 30 |  | 24 | 36 | NOTE 1 |
|  | 60 |  | 10 | 18 | NOTE 1 |
| n71 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n74 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
|  | 60 |  | 10 | 18 | 24 |
| n85 |  | 15 | 25 | 50 | 75 |  |
|  |  | 30 |  | 24 | 36 |  |
| n91 | 15 | 25 |  |  |  |
| n92 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n93 | 15 | 25 |  |  |  |
| n94 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| n105 | 15 | 25 | 50 | 75 | 100 |
|  | 30 |  | 24 | 36 | 50 |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3I.2.

7.3I.2.4	Test description

7.3I.2.4.1	Initial conditions

For RedCap UE with 1Rx or 2 Rx antenna ports, same initial conditions as in 7.3.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

For HD-FDD RedCap UE with 1 Rx or 2 Rx antenna ports, same initial conditions as in 7.3.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

-	The RB allocation for uplink configuration in Table 7.3.2.4.1-1 refers to Table 7.3I.2.4.1-1 for each SCS, channel BW and NR band.

Table 7.3I.2.4.1-1: Uplink configuration for reference sensitivity of HD-FDD RedCap UE, LCRB @ RBstart format

| Operating Band |  | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  |  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n2 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  |  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n3 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  |  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n5 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n7 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  |  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n8 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n12 |  | 15 | 25@0 | 50@2 | 75@4 |  | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 |  |  |
| n13 |  | 15 | 25@0 | 50@0 |  |  | HD-FDD |
|  |  | 30 |  | 24@0 |  |  |  |
| n14 |  | 15 | 25@0 | 50@0 |  |  | HD-FDD |
|  |  | 30 |  | 24@0 |  |  |  |
| n20 |  | 15 | 25@0 | 50@0 | 75@0 | 100@0 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@0 | 50@0 |  |
| n24 |  | 15 | 25@0 | 50@0 |  |  | HD-FDD |
|  |  | 30 |  | 24@0 |  |  |  |
|  |  | 60 |  | 10@0 |  |  |  |
| n25 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  |  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n26 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n28 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n30 |  | 15 | 25@0 | 50@2 |  |  | HD-FDD |
|  |  | 30 |  | 24@0 |  |  |  |
| n65 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  |  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n66 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  |  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n70 |  | 15 | 25@0 | 50@2 | 75@4 | NOTE 1 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | NOTE 1 |  |
|  |  | 60 |  | 10@1 | 18@0 | NOTE 1 |  |
| n71 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n74 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
|  |  | 60 |  | 10@1 | 18@0 | 24@0 |  |
| n85 | 15 | 25@0 | 50@2 | 75@4 |  | HD-FDD |
|  | 30 |  | 24@0 | 36@2 |  |  |
| n91 |  | 15 | 25@0 |  |  |  | HD-FDD |
| n92 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n93 |  | 15 | 25@0 |  |  |  | HD-FDD |
| n94 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| n105 |  | 15 | 25@0 | 50@2 | 75@4 | 100@6 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 50@1 |  |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |  |  |

7.3I.2.4.2	Test procedure

Same test procedure as steps 1~4 of clause 7.3.2.4.2 with the following exceptions of step 3.

-	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3I.2.5-1, 7.3I.2.5-2 and 7.3I.2.5-5 if 2Rx antennas connected or Tables 7.3I.2.5-3, 7.3I.2.5-4 and 7.3I.2.5-6 if 1Rx antennas connected.

7.3I.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

7.3I.2.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.2.5-1 and Table 7.3I.2.5-2 for RedCap UE with 2 Rx antenna port, Tables 7.3I.2.5-3 and Table 7.3I.2.5-4 for RedCap UE with single antenna port, Table 7.3I.2.5-5 for HD-FDD RedCap UE with 2 Rx antenna port, Table 7.3I.2.5-6 for HD-FDD RedCap UE single antenna port, and parameters specified Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3I.2.4.1-1.

Table 7.3I.2.5-1: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n2 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |
| n3 | 15 | -97.0 +TT | -93.8 +TT | -92.0 +TT | -90.8 +TT | FDD |
|  | 30 |  | -94.1 +TT | -92.1 +TT | -91.0 +TT |  |
|  | 60 |  | -94.5 +TT | -92.4 +TT | -91.2 +TT |  |
| n5 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -86.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -88.6 +TT |  |
| n7 | 15 | -98.0 +TT | -94.8 +TT | -93.0 +TT | -91.8 +TT | FDD |
|  | 30 |  | -95.1 +TT | -93.1 +TT | -92.0 +TT |  |
|  | 60 |  | -95.5 +TT | -93.4 +TT | -92.2 +TT |  |
| n8 | 15 | -97.0 +TT | -93.8 +TT | -91.4 +TT | -85.8 +TT | FDD |
|  | 30 |  | -94.1 +TT | -91.7 +TT | -87.2 +TT |  |
| n12 | 15 | -97.0 +TT | -93.8 +TT | -84.0 +TT |  | FDD |
|  | 30 |  | -94.1 +TT | -84.1 +TT |  |  |
| n13 | 15 | -97.0 +TT | -93.8 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n14 | 15 | -97.0 +TT | -93.8 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n20 | 15 | -97.0 +TT | -93.8 +TT | -91.0 +TT | -89.8 +TT | FDD |
|  | 30 |  | -94.1 +TT | -91.1 +TT | -90.0 +TT |  |
| n24 | 15 | -100.0 +TT | -96.8 +TT |  |  | FDD |
|  | 30 |  | -97.1 +TT |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |
| n25 | 15 | -96.5 +TT | -93.3 +TT | -91.5 +TT | -90.3 +TT | FDD |
|  | 30 |  | -93.6 +TT | -91.6 +TT | -90.5 +TT |  |
|  | 60 |  | -94.0 +TT | -91.9 +TT | -90.7 +TT |  |
| n26 | 15 | -97.5 +TT | -94.5 +TT | -92.7 +TT | -87.6 +TT | FDD |
|  | 30 |  | -94.8 +TT | -92.7 +TT | -87.7 +TT |  |
| n28 | 15 | -98.5 +TT | -95.5 +TT | -93.5 +TT | -90.8 +TT | FDD |
|  | 30 |  | -95.6 +TT | -93.6 +TT | -91.0 +TT |  |
| n30 | 15 | -99.0 +TT | -95.8 +TT |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |
| n65 | 15 | -99.5+TT | -96.3+TT | -94.5+TT | -93.3+TT | FDD |
|  | 30 |  | -96.6+TT | -94.6+TT | -93.5+TT |  |
|  | 60 |  | -97.0+TT | -94.9+TT | -93.7+TT |  |
| n66 | 15 | -99.5 +TT | -96.3 +TT | -94.5 +TT | -93.3 +TT | FDD |
|  | 30 |  | -96.6 +TT | -94.6 +TT | -93.5 +TT |  |
|  | 60 |  | -97.0 +TT | -94.9 +TT | -93.7 +TT |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.8 +TT | FDD |
|  | 30 |  | -97.1 +TT | -95.1 +TT | -94.0 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n71 | 15 | -97.2 +TT | -94.0 +TT | -91.6 +TT | -86.0 +TT | FDD |
|  | 30 |  | -94.3 +TT | -91.9 +TT | -87.4 +TT |  |
| n74 | 15 | -99.53 +TT | -96.33 +TT | -94.53 +TT | -89.33 +TT | FDD |
|  | 30 |  | -96.63 +TT | -94.63 +TT | -89.53 +TT |  |
|  | 60 |  | -97.03 +TT | -94.93 +TT | -89.63 +TT |  |
| n85 | 15 | -97.0 +TT | -93.8 +TT | -84.0 +TT |  | FDD |
|  | 30 |  | -94.1 +TT | -84.1 +TT |  |  |
| n105 | 15 | -97.25 +TT | -94.0 +TT | -91.6 +TT | -86.9 +TT | FDD |
|  | 30 |  | -94.3 +TT | -91.9 +TT | -87.9 +TT |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. NOTE 5: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |

Table 7.3I.2.5-2: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11)+TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20, | -97.5 + 10log10(NRB/11)+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n411 | 15 | 5, 10, 15, 20 | -94.8 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -95.5 + 10log10(NRB/11)+TT |  |
| n481 | 15 | 5, 10, 15, 20, | -99 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20, | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20, | -96.5 + 10log10(NRB/11)+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11)+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1+TT |  |
|  | 60 | 10 | -97.5+TT |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 + 10log10(NRB/24)+TT |  |
|  | 60 | 10, 15, 20 | -96.0 + 10log10(NRB/11)+TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(NRB/52) +TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 15, 20 | -96.5 + 10log10(NRB/11)+TT |  |
| n791 | 15 | 10, 20, | -95.8 + 10log10(NRB/52)+TT | TDD |
|  | 30 | 10, 20, | -96.1 + 10log10(NRB/24) +TT |  |
|  | 60 | 10, 20 | -96.5 + 10log10(NRB/11) +TT |  |
| n91 | 15 | 5 | -100+TT | FDD |
| n92 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25)+TT | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24)+TT |  |
| n93 | 15 | 5 | -100+TT | FDD |
| n94 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25)+TT | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24)+TT |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25)+TT | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24)+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |

Table 7.3I.2.5-3: Single antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0+2.5 +TT |  | -96.8 +3 +TT | -95.0 +3 +TT | -93.8 +3 +TT | FDD |  |
|  | 30 |  |  | -97.1 +3 +TT | -95.1 +3 +TT | -94.0 +3 +TT |  |  |
|  | 60 |  |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |  |
| n2 | 15 | -98.0 +2.5+TT |  | -94.8 +3 +TT | -93.0 +3 +TT | -91.8 +3 +TT | FDD |  |
|  | 30 |  |  | -95.1 +3 +TT | -93.1 +3 +TT | -92.0 +3 +TT |  |  |
|  | 60 |  |  | -95.5 +3 +TT | -93.4 +3 +TT | -92.2 +3 +TT |  |  |
| n3 | 15 | -97.0+2.5 +TT |  | -93.8 +3 +TT | -92.0 +3 +TT | -90.8 +3 +TT | FDD |  |
|  | 30 |  |  | -94.1 +3 +TT | -92.1 +3 +TT | -91.0 +3 +TT |  |  |
|  | 60 |  |  | -94.5 +3 +TT | -92.4 +3 +TT | -91.2 +3 +TT |  |  |
| n5 | 15 | -98.0+2.5 +TT |  | -94.8 +3 +TT | -93.0 +3 +TT | -86.8 +3 +TT | FDD |  |
|  | 30 |  |  | -95.1 +3 +TT | -93.1 +3 +TT | -88.6 +3 +TT |  |  |
| n7 | 15 | -98.0+2.5 +TT |  | -94.8 +3 +TT | -93.0 +3 +TT | -91.8 +3 +TT | FDD |  |
|  | 30 |  |  | -95.1 +3 +TT | -93.1 +3 +TT | -92.0 +3 +TT |  |  |
|  | 60 |  |  | -95.5 +3 +TT | -93.4 +3 +TT | -92.2 +3 +TT |  |  |
| n8 | 15 | -97.0+2.5 +TT |  | -93.8 +3 +TT | -91.4 +3 +TT | -85.8 +3 +TT | FDD |  |
|  | 30 |  |  | -94.1 +3 +TT | -91.7 +3 +TT | -87.2 +3 +TT |  |  |
| n12 | 15 | -97.0 +2.5+TT |  | -93.8 +3 +TT | -84.0 +3 +TT |  | FDD |  |
|  | 30 |  |  | -94.1 +3 +TT | -84.1 +3 +TT |  |  |  |
| n13 | 15 | -97.0+2.5 +TT |  | -93.8 +3 +TT |  |  |  |  |
|  | 30 |  |  | -94.1 +3 +TT |  |  |  |  |
| n14 | 15 | -97.0+2.5 +TT |  | -93.8 +3 +TT |  |  | FDD |  |
|  | 30 |  |  | -94.1 +3 +TT |  |  |  |  |
| n20 | 15 | -97.0 +2.5+TT |  | -93.8 +3 +TT | -91.0 +3 +TT | -89.8 +3 +TT | FDD |  |
|  | 30 |  |  | -94.1 +3 +TT | -91.1 +3 +TT | -90.0 +3 +TT |  |  |
| n24 | 15 | -100.0 +2.5+TT |  | -96.8 +3 +TT |  |  | FDD |  |
|  | 30 |  |  | -97.1 +3 +TT |  |  |  |  |
|  | 60 |  |  | -97.5 +3 +TT |  |  |  |  |
| n25 | 15 | -96.5 +2.5+TT |  | -93.3 +3 +TT | -91.5 +3 +TT | -90.3 +3 +TT | FDD |  |
|  | 30 |  |  | -93.6 +3 +TT | -91.6 +3 +TT | -90.5 +3 +TT |  |  |
|  | 60 |  |  | -94.0 +3 +TT | -91.9 +3 +TT | -90.7 +3 +TT |  |  |
| n26 | 15 | -97.5+2.5 +TT |  | -94.5 +3 +TT | -92.7 +3 +TT | -87.6 +3 +TT | FDD |  |
|  | 30 |  |  | -94.8 +3 +TT | -92.7 +3 +TT | -87.7 +3 +TT |  |  |
| n28 | 15 | -98.5 +2.5+TT |  | -95.5 +3 +TT | -93.5 +3 +TT | -90.8 +3 +TT | FDD |  |
|  | 30 |  |  | -95.6 +3 +TT | -93.6 +3 +TT | -91.0 +3 +TT |  |  |
| n30 | 15 | -99.0 +2.5+TT |  | -95.8 +3 +TT |  |  | FDD |  |
|  | 30 |  |  | -96.1 +3 +TT |  |  |  |  |
| n65 | 15 | -99.5 +2.5+TT |  | -96.3 +3+TT | -94.5 +3+TT | -93.3 +3+TT | FDD |  |
|  | 30 |  |  | -96.6 +3 +TT | -94.6 +3 +TT | -93.5 +3 +TT |  |  |
|  | 60 |  |  | -97.0 +3+TT | -94.9 +3+TT | -93.7 +3+TT |  |  |
| n66 | 15 | -99.5 +2.5+TT |  | -96.3 +3+TT | -94.5 +3 +TT | -93.3 +3 +TT | FDD |  |
|  | 30 |  |  | -96.6 +3 +TT | -94.6 +3 +TT | -93.5 +3+TT |  |  |
|  | 60 |  |  | -97.0 +3 +TT | -94.9 +3 +TT | -93.7 +3 +TT |  |  |
| n70 | 15 | -100.0 +2.5 +TT |  | -96.8 +3 +TT | -95.0 +3+TT | -93.8 +3 +TT | FDD |  |
|  | 30 |  |  | -97.1 +3 +TT | -95.1 +3 +TT | -94.0 +3 +TT |  |  |
|  | 60 |  |  | -97.5 +3 +TT | -95.4 +3 +TT | -94.2 +3 +TT |  |  |
| n71 | 15 | -97.2+2.5 +TT |  | -94.0 +3 +TT | -91.6 +3 +TT | -86.0 +3 +TT | FDD |  |
|  | 30 |  |  | -94.3 +3 +TT | -91.9 +3 +TT | -87.4 +3 +TT |  |  |
| n74 | 15 | -99.53 +2.5 +TT |  | -96.33 +3 +TT | -94.53 +3 +TT | -89.33 +3+TT | FDD |  |
|  | 30 |  |  | -96.63 +3 +TT | -94.63 +3 +TT | -89.53 +3 +TT |  |  |
|  | 60 |  |  | -97.03 +3 +TT | -94.93 +3 +TT | -89.63 +3+TT |  |  |
| n85 |  | 15 | -97+2.5 +TT |  | -93.8 +3 +TT | -84.0 +3 +TT |  | FDD |  |
|  |  | 30 |  |  | -94.1 +3 +TT | -84.1 +3 +TT |  |  |  |
| n105 |  | 15 | -97.2+2.5 +TT |  | -94.0 +3 +TT | -91.6 +3 +TT | -86.9 +3 +TT | FDD |  |
|  |  | 30 |  |  | -94.3 +3 +TT | -91.9 +3 +TT | -87.9 +3 +TT |  |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: That the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |  |  |  |

Table 7.3I.2.5-4: Single antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS/Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n411 | 15 | 5, 10, 15, 20 | -94.8 + 10log10(NRB/52) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -95.5 + 10log10(NRB/11) +2.5+TT |  |
| n481 | 15 | 5, 10, 15, 20 | -99 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -96.5 + 10log10(NRB/11) +2.5+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -97.5 + 10log10(NRB/11) +2.5+TT |  |
| n51 | 15 | 5 | -100 +2.5+TT | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5+TT |  |
|  | 60 | 10 | -97.5 +2.5+TT |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 + 10log10(NRB/52) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -96.0 + 10log10(NRB/11) +2.5+TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 + 10log10(NRB/52) +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 15, 20 | -96.5 + 10log10(NRB/11) +2.5+TT |  |
| n791 | 15 | 10, 20, | -95.8 + 10log10(NRB/52) +2.5+TT | TDD |
|  | 30 | 10, 20, | -96.1 + 10log10(NRB/24) +2.5+TT |  |
|  | 60 | 10, 20 | -96.5 + 10log10(NRB/11) +2.5+TT |  |
| n91 | 15 | 5 | -100+2.5+TT | FDD |
| n92 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) +2.5+TT | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
| n93 | 15 | 5 | -100 +2.5+TT | FDD |
| n94 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) +2.5+TT | FDD |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) +2.5+TT | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) +2.5+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. |  |  |  |  |

Table 7.3I.2.5-5: Two antenna port reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n2 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
|  | 60 |  | -96.3 +TT | -94.2 +TT | -93.0 +TT |  |
| n3 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
|  | 60 |  | -95.3 +TT | -93.2 +TT | -92.0 +TT |  |
| n5 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
| n7 | 15 | -98.8 +TT | -95.6 +TT | -93.8 +TT | -92.5 +TT | HD-FDD |
|  | 30 |  | -96.0 +TT | -94.0 +TT | -92.7 +TT |  |
|  | 60 |  | -96.3 +TT | -94.2 +TT | -93.0 +TT |  |
| n8 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
| n12 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT |  |  |
| n13 | 15 | -97.8 +TT | -94.6 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 +TT |  |  |  |
| n14 | 15 | -97.8 +TT | -94.6 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 +TT |  |  |  |
| n20 | 15 | -97.8 +TT | -94.6 +TT | -92.8 +TT | -91.5 +TT | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT | -91.7 +TT |  |
| n24 | 15 | -100.0 +TT | -96.8 +TT |  |  | HD-FDD |
|  | 30 |  | -97.2 +TT |  |  |  |
|  | 60 |  | -97.5 +TT |  |  |  |
| n25 | 15 | -97.3 +TT | -94.1 +TT | -92.3 +TT | -91.0 +TT | HD-FDD |
|  | 30 |  | -94.5 +TT | -92.5 +TT | -91.2 +TT |  |
|  | 60 |  | -94.8 +TT | -92.7 +TT | -91.5 +TT |  |
| n26 | 15 | -98.3 +TT | -95.1 +TT | -93.3 +TT | -92.0 +TT | HD-FDD |
|  | 30 |  | -95.5 +TT | -93.5 +TT | -92.2 +TT |  |
| n28 | 15 | -99.3 +TT | -96.1 +TT | -94.3 +TT | -93.0 +TT | HD-FDD |
|  | 30 |  | -96.5 +TT | -94.5 +TT | -93.2 +TT |  |
| n30 | 15 | -99.5 +TT | -96.3 +TT |  |  | HD-FDD |
|  | 30 |  | -96.7 +TT |  |  |  |
| n65 | 15 | -100.0 +TT | -96.8+TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n66 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n70 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n71 | 15 | -98.0 +TT | -94.8+TT | -93.0 +TT | -91.7 +TT | HD-FDD |
|  | 30 |  | -95.2 +TT | -93.2 +TT | -91.9 +TT |  |
| n74 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
|  | 60 |  | -97.5 +TT | -95.4 +TT | -94.2 +TT |  |
| n85 | 15 | -97.8 +TT | -94.6+TT | -92.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 +TT | -93.0 +TT |  |  |
| n91 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n92 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
| n93 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n94 | 15 | -100.0 +TT | -96.8 +TT | -95.0 +TT | -93.7 +TT | HD-FDD |
|  | 30 |  | -97.2 +TT | -95.2 +TT | -93.9 +TT |  |
| n105 | 15 | -98.03 +TT | -94.8 +TT | -93.0 +TT | -91.7 +TT | HD-FDD |
|  | 30 |  | -95.2 +TT | -93.2 +TT | -91.9 +TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. NOTE 3: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |

Table 7.3I.2.5-6: Single antenna port Reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 |  | -97.5 +TT |  | -94.3 +TT |  | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 +TT |  | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  |  |  | -95.0 +TT |  | -92.9 +TT | -91.7 +TT |  |
| n2 | 15 |  | -96.3 +TT |  | -93.1 +TT |  | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  |  |  | -93.5 +TT |  | -91.5+TT | -90.2+TT |  |
|  | 60 |  |  |  | -93.8 +TT |  | -91.7 +TT | -90.5 +TT |  |
| n3 | 15 |  | -95.3 +TT |  | -92.1 +TT |  | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  |  |  | -92.5 +TT |  | -90.5 +TT | -89.2 +TT |  |
|  | 60 |  |  |  | -92.8 +TT |  | -90.7 +TT | -89.5 +TT |  |
| n5 | 15 |  | -96.3 +TT |  | -93.1 +TT |  | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  |  |  | -93.5 +TT |  | -91.5 +TT | -90.2 +TT |  |
| n7 | 15 |  | -96.3 +TT |  | -93.1 +TT |  | -91.3 +TT | -90.0 +TT | HD-FDD |
|  | 30 |  |  |  | -93.5 +TT |  | -91.5 +TT | -90.2 +TT |  |
|  | 60 |  |  |  | -93.8 +TT |  | -91.7 +TT | -90.5 +TT |  |
| n8 | 15 |  | -95.3 +TT |  | -92.1 +TT |  | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  |  |  | -92.5 +TT |  | -90.5 +TT | -89.2 +TT |  |
| n12 | 15 |  | -95.3 +TT |  | -92.1 +TT |  | -90.3 +TT |  | HD-FDD |
|  | 30 |  |  |  | -92.5 +TT |  | -90.5 +TT |  |  |
| n13 | 15 |  | -95.3 +TT |  | -92.1 +TT |  |  |  | HD-FDD |
|  | 30 |  |  |  | -92.5 +TT |  |  |  |  |
| n14 | 15 |  | -95.3 +TT |  | -92.1 +TT |  |  |  | HD-FDD |
|  | 30 |  |  |  | -92.5 +TT |  |  |  |  |
| n20 | 15 |  | -95.3 +TT |  | -92.1 +TT |  | -90.3 +TT | -89.0 +TT | HD-FDD |
|  | 30 |  |  |  | -92.5 +TT |  | -90.5 +TT | -89.2 +TT |  |
| n24 | 15 |  | -97.5 +TT |  | -94.3 +TT |  |  |  | HD-FDD |
|  | 30 |  |  |  | -94.7 +TT |  |  |  |  |
|  | 60 |  |  |  | -95.0 +TT |  |  |  |  |
| n25 | 15 |  | -94.8 +TT |  | -91.6 +TT |  | -89.8 +TT | -88.5 +TT | HD-FDD |
|  | 30 |  |  |  | -92.0 +TT |  | -90.0 +TT | -88.7 +TT |  |
|  | 60 |  |  |  | -92.3 +TT |  | -90.2 +TT | -89.0 +TT |  |
| n26 | 15 |  | -95.8 +TT |  | -92.6 +TT |  | -90.8 +TT | -89.5 +TT | HD-FDD |
|  | 30 |  |  |  | -93.0 +TT |  | -91.0 +TT | -89.7 +TT |  |
| n28 | 15 |  | -96.8 +TT |  | -93.6 +TT |  | -91.8 +TT | -90.5 +TT | HD-FDD |
|  | 30 |  |  |  | -94.0 +TT |  | -92.0 +TT | -90.7 +TT |  |
| n30 | 15 |  | -97.0 +TT |  | -93.8 +TT |  |  |  | HD-FDD |
|  | 30 |  |  |  | -94.2 +TT |  |  |  |  |
| n65 | 15 |  | -97.5 +TT |  | -94.3 +TT |  | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 +TT |  | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  |  |  | -95.0 +TT |  | -92.9 +TT | -91.7 +TT |  |
| n66 | 15 |  | -97.5 +TT |  | -94.3 +TT |  | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 +TT |  | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  |  |  | -95.0 +TT |  | -92.9 +TT | -91.7 +TT |  |
| n70 | 15 |  | -97.5 +TT |  | -94.3 +TT |  | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 +TT |  | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  |  |  | -95.0 +TT |  | -92.9 +TT | -91.7 +TT |  |
| n71 | 15 |  | -95.5 +TT |  | -92.3 +TT |  | -90.5 +TT | -89.2 +TT | HD-FDD |
|  | 30 |  |  |  | -92.7 +TT |  | -90.7 +TT | -89.4 +TT |  |
| n74 | 15 |  | -97.5 +TT |  | -94.3 +TT |  | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 +TT |  | -92.7 +TT | -91.4 +TT |  |
|  | 60 |  |  |  | -95.0 +TT |  | -92.9 +TT | -91.7+TT |  |
| n85 | 15 | -95.3 +TT |  | -92.1+TT |  | -90.8 +TT |  |  | HD-FDD |
|  | 30 |  |  | -92.5 +TT |  | -90.5 +TT |  |  |  |
| n91 | 15 |  | -97.5 +TT |  |  |  |  |  | HD-FDD |
| n92 | 15 |  | -97.5 +TT |  | -94.3 +TT |  | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 +TT |  | -92.7 +TT | -91.4 +TT |  |
| n93 | 15 |  | -97.5 +TT |  |  |  |  |  | HD-FDD |
| n94 | 15 |  | -97.5 +TT |  | -94.3 +TT |  | -92.5 +TT | -91.2 +TT | HD-FDD |
|  | 30 |  |  |  | -94.7 +TT |  | -92.7 +TT | -91.4 +TT |  |
| n105 | 15 |  | -95.53 +TT |  | -92.3 +TT |  | -90.5 +TT | -89.2 +TT | HD-FDD |
|  | 30 |  |  |  | -92.7 +TT |  | -90.7 +TT | -89.4 +TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.2.5-7. NOTE 3: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |  |  |  |

Table 7.3I.2.5-7: Test Tolerance (TT) for RX sensitivity level for RedCap UE

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

### 7.3I.3	Reference sensitivity power level for eRedCap

7.3I.3.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3I.3.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR eRedCap.

7.3I.3.3	Minimum conformance requirements

For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, the reference sensitivity requirements for 5 MHz channel bandwidth with 15 kHz SCS defined in clause 7.3I.2 apply. These reference sensitivity requirements for 5 MHz channel bandwidth apply also for 10, 15 and 20 MHz channel bandwidths with 15 kHz SCS. In case the reference sensitivity requirements for 5 MHz channel bandwidth are not defined, the reference sensitivity requirements for 10 MHz channel bandwidth apply with the reference sensitivity level reduced by 3.2 dB, and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4  and 25 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 25RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 25 RBs for 15 kHz SCS.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, for 30 kHz SCS, the reference sensitivity requirements defined for 10 MHz channel bandwidth in clause 7.3I.2 apply with reference sensitivity level reduced by 3.0 dB and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4 and 12 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 12RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths with 30 kHz SCS. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 12 RBs for 30 kHz SCS.

NOTE:	It is not necessary to repeat verification when same requirement applies for multiple channel bandwidths.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3I.3.

7.3I.3.4	Test description

7.3I.3.4.1	Initial conditions

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, same initial conditions as in 7.3I.2.4.1 with following exception:

For eRedCap UE not supporting eRedCapNotReducedBB-BW-r18, same initial conditions as in 7.3I.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

-	The downlink RB allocation in test configuration table refers to Table 7.3I.3.4.1-1

-	The uplink RB allocation in test configuration table refers to Table 7.3I.3.4.1-2 for FDD and TDD eRedCap UE, and Table 7.3I.3.4.1-3 for HD-FDD eRedCap UE respectively.

Table 7.3I.3.4.1-1: Downlink Configuration of each RB allocation of eRedCap UE not supporting eRedCapNotReducedBB-BW-r18

| Channel Bandwidth | SCS(kHz) | LCRBmax | Outer RB allocation / Normal RB allocation |
| --- | --- | --- | --- |
| 5MHz | 15 | 25 | 25@0 |
|  | 30 | 11 | 11@0 |
| 10MHz | 15 | 25 | 25@14 |
|  | 30 | 12 | 12@6 |
| 15MHz | 15 | 25 | 25@27 |
|  | 30 | 12 | 12@13 |
| 20MHz | 15 | 25 | 25@41 |
|  | 30 | 12 | 12@20 |

Table 7.3I.3.4.1-2: Uplink configuration for reference sensitivity of FDD or TDD eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, LCRB @ RBstart format

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n2 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 | 10@11 | 12@6 | 12@13 | 12@20 |  |
| n3 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n5 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@221 |  |
| n7 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n8 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@221 |  |
| n12 | 15 | 20@51 | 20@191 | 20@321 |  | FDD |
|  | 30 |  | 10@81 | 10@151 |  |  |
| n13 | 15 | 20@01 | 20@141 |  |  | FDD |
|  | 30 |  | 10@61 |  |  |  |
| n14 | 15 | 20@01 | 20@141 |  |  | FDD |
|  | 30 |  | 10@61 |  |  |  |
| n20 | 15 | 25@0 | 20@141 | 20@27 | 20@41 | FDD |
|  | 30 |  | 10@61 | 10@132 | 10@202 |  |
| n24 | 15 | 25@0 | 25@14 |  |  | FDD |
|  | 30 |  | 12@6 |  |  |  |
| n25 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n26 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n28 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@221 |  |
| n30 | 15 | 20@51 | 20@191 |  |  | FDD |
|  | 30 |  | 10@81 |  |  |  |
| n34 | 15 | 25@0 | 25@14 | 25@27 |  | TDD |
|  | 30 |  | 12@6 | 12@13 |  |  |
| n38 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n39 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n40 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n41 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n48 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n50 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n51 | 15 | 25@0 |  |  |  | TDD |
|  | 30 |  |  |  |  |  |
| n53 | 15 | 25@0 | 25@14 |  |  | TDD |
|  | 30 |  | 12@6 |  |  |  |
| n65 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n66 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n70 | 15 | 25@0 | 25@14 | 25@27 | NOTE 3 | FDD |
|  | 30 |  | 12@6 | 12@13 | NOTE 3 |  |
| n71 | 15 | 25@0 | 25@14 | 20@321 | 20@411 | FDD |
|  | 30 |  | 12@6 | 10@151 | 10@201 |  |
| n74 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@221 |  |
| n77 | 15 |  | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n78 | 15 |  | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n79 | 15 |  | 25@14 |  | 25@41 | TDD |
|  | 30 |  | 12@6 |  | 12@20 |  |
| n85 | 15 | 20@51 | 20@191 | 20@321 |  | FDD |
|  | 30 |  | 10@81 | 10@151 |  |  |
| n91 | 15 | 25@04 | 20@191, 4 |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n92 | 15 | 20@51 | 20@191 | 20@321 | 20@411 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@201 |  |
| n93 | 15 | 25@04 | 25@144 |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n94 | 15 | 25@0 | 25@14 | 20@321 | 20@411 | FDD |
|  | 30 |  | 12@6 | 10@151 | 10@201 |  |
| n100 | 15 | 25@0 |  |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n101 | 15 | 25@0 | 25@14 |  |  | TDD |
|  | 30 |  | 12@6 |  |  |  |
| n105 | 15 | 25@0 | 25@0 | 20@0 | 20@0 | FDD |
|  | 30 |  | 12@0 | 10@0 | 10@0 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For band n20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =8. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. |  |  |  |  |  |  |

Table 7.3I.3.4.1-3: Uplink configuration for reference sensitivity of HD-FDD eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, LCRB @ RBstart format

| Operating Band |  | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n2 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n3 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n5 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n7 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n8 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n12 |  | 15 | 25@0 | 25@14 | 25@27 |  | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 |  |  |
| n13 |  | 15 | 20@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n14 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n20 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n24 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n25 |  | 15 | 25@0 | 50@2 | 75@4 | 25@41 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 12@20 |  |
| n26 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n28 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n30 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n65 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n66 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n70 |  | 15 | 25@0 | 25@14 | 25@27 | NOTE 1 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | NOTE 1 |  |
| n71 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n74 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n85 | 15 | 25@0 | 25@14 | 25@27 |  | HD-FDD |
|  | 30 |  | 12@6 | 12@13 |  |  |
| n91 |  | 15 | 25@0 |  |  |  | HD-FDD |
| n92 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n93 |  | 15 | 25@0 |  |  |  | HD-FDD |
| n94 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n105 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |  |  |

7.3I.3.4.2	Test procedure

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, same test procedure as steps 1~4 of clause 7.3I.2.4.2.

For eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, same test procedure as steps 1~4 of clause 7.3.2.4.2 with the following exceptions on step 3.

-	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3I.3.5-1, 7.3I.3.5-2 and 7.3I.3.5-5 if 2Rx antennas connected or Tables 7.3I.3.5-3, 7.3I.3.5-4 and 7.3I.3.5-6 if 1Rx antennas connected.

7.3I.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.7.3I.3.5	Test requirement

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.2.5-1 and Table 7.3I.2.5-2 for eRedCap UE with 2 Rx antenna port, Tables 7.3I.2.5-3 and Table 7.3I.2.5-4 for eRedCap UE with single antenna port, Table 7.3I.2.5-5 for HD-FDD eRedCap UE with 2 Rx antenna port, Table 7.3I.2.5-6 for HD-FDD eRedCap UE with single antenna port, and parameters specified in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3I.2.4.1-1.

For eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.3.5-1 and Table 7.3I.3.5-2 for eRedCap UE with 2 Rx antenna port, Tables 7.3I.3.5-3 and Table 7.3I.3.5-4 for eRedCap UE with single antenna port, Table 7.3I.3.5-5 for HD-FDD eRedCap UE with 2 Rx antenna port, Table 7.3I.3.5-6 for HD-FDD eRedCap UE with single antenna port, and parameters specified in Table 7.3I.3.4.1-1, Table 7.3I.3.4.1-2 and Table 7.3I.3.4.1-3.

Table 7.3I.3.5-1: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | FDD |
|  | 30 |  | -97.1-3 +TT | -97.1-3 +TT | -97.1-3 +TT |  |
| n2 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n3 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n5 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n7 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n8 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n12 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT |  | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT |  |  |
| n13 | 15 | -97.0 +TT | -97.0 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n14 | 15 | -97.0 +TT | -97.0 +TT |  |  | FDD |
|  | 30 |  | -94.1 -3+TT |  |  |  |
| n20 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n24 | 15 | -100.0 +TT | -100.0 +TT |  |  | FDD |
|  | 30 |  | -97.1 -3+TT |  |  |  |
| n25 | 15 | -96.5 +TT | -96.5 +TT | -96.5 +TT | -96.5 +TT | FDD |
|  | 30 |  | -93.6 -3+TT | -93.6 -3+TT | -93.6 -3+TT |  |
| n26 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | FDD |
|  | 30 |  | -94.8 -3+TT | -94.8 -3+TT | -94.8 -3+TT |  |
| n28 | 15 | -98.5 +TT | -98.5 +TT | -98.5 +TT | -98.5 +TT | FDD |
|  | 30 |  | -95.6 -3 +TT | -95.6 -3 +TT | -95.6 -3 +TT |  |
| n30 | 15 | -99.0 +TT | -99.0 +TT |  |  | FDD |
|  | 30 |  | -96.1 -3+TT |  |  |  |
| n65 | 15 | -99.5+TT | -99.5+TT | -99.5+TT | -99.5+TT | FDD |
|  | 30 |  | -96.6 -3+TT | -96.6 -3+TT | -96.6 -3+TT |  |
| n66 | 15 | -99.5 +TT | -99.5 +TT | -99.5 +TT | -99.5 +TT | FDD |
|  | 30 |  | -96.6 -3+TT | -96.6 -3+TT | -96.6 -3+TT |  |
| n70 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | FDD |
|  | 30 |  | -97.1 -3+TT | -97.1 -3+TT | -97.1 -3+TT |  |
| n71 | 15 | -97.2 +TT | -97.2 +TT | -97.2 +TT | -97.2 +TT | FDD |
|  | 30 |  | -94.3 -3+TT | -94.3 -3+TT | -94.3 -3+TT |  |
| n74 | 15 | -99.53 +TT | -99.53 +TT | -99.53 +TT | -99.53 +TT | FDD |
|  | 30 |  | -96.63 -3+TT | -96.63 -3+TT | -96.63 -3+TT |  |
| n85 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT |  | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT |  |  |
| n105 | 15 | -97.25 +TT | -97.25 +TT | -97.25 +TT | -97.25 +TT | FDD |
|  | 30 |  | -94.3 +TT | -94.3 +TT | -94.3 +TT |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. NOTE 5: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |

Table 7.3I.3.5-2: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 +TT | TDD |
|  | 30 | 10, 15 | -97.1 -3 +TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 -3 +TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n411 | 15 | 5, 10, 15, 20 | -94.8+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 -3+TT |  |
| n481 | 15 | 5, 10, 15, 20, | -99 +TT | TDD |
|  | 30 | 10, 15, 20, | -96.1 -3+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 +TT | TDD |
|  | 30 | 10 | -97.1-3+TT |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 -3 +TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -96.-3 +TT |  |
| n791 | 15 | 10, 20, | -95.8 -3.2+TT | TDD |
|  | 30 | 10, 20, | -96.1 -3 +TT |  |
| n91 | 15 | 5 | -100+TT | FDD |
| n92 | 15 | 5,10,15,20 | -100 +TT | FDD |
|  | 30 | 10,15,20 | -97.1 -3+TT |  |
| n93 | 15 | 5 | -100+TT | FDD |
| n94 | 15 | 5,10,15,20 | -100 +TT | FDD |
|  | 30 | 10,15,20 | -97.1 -3+TT |  |
| n101 | 15 | 5, 10 | -100 +TT | TDD |
|  | 30 | 10 | -97.1 -3+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |

Table 7.3I.3.5-3: Single antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0+2.5 +TT | -100.0+2.5 +TT | -100.0+2.5 +TT | -100.0+2.5 +TT | FDD |
|  | 30 |  | -97.1 +TT | -97.1 +TT | -97.1 +TT |  |
| n2 | 15 | -98.0 +2.5+TT | -98.0 +2.5+TT | -98.0 +2.5+TT | -98.0 +2.5+TT | FDD |
|  | 30 |  | -95.1 +TT | -95.1 +TT | -95.1 +TT |  |
| n3 | 15 | -97.0+2.5 +TT | -97.0+2.5 +TT | -97.0+2.5 +TT | -97.0+2.5 +TT | FDD |
|  | 30 |  | -94.1 +TT | -94.1 +TT | -94.1 +TT |  |
| n5 | 15 | -98.0+2.5 +TT | -98.0+2.5 +TT | -98.0+2.5 +TT | -98.0+2.5 +TT | FDD |
|  | 30 |  | -95.1 +TT | -95.1 +TT | -95.1 +TT |  |
| n7 | 15 | -98.0+2.5 +TT | -98.0+2.5 +TT | -98.0+2.5 +TT | -98.0+2.5 +TT | FDD |
|  | 30 |  | -95.1 +TT | -95.1 +TT | -95.1 +TT |  |
| n8 | 15 | -97.0+2.5 +TT | -97.0+2.5 +TT | -97.0+2.5 +TT | -97.0+2.5 +TT | FDD |
|  | 30 |  | -94.1 +TT | -94.1 +TT | -94.1 +TT |  |
| n12 | 15 | -97.0 +2.5+TT | -97.0 +2.5+TT | -97.0 +2.5+TT |  | FDD |
|  | 30 |  | -94.1 +TT | -94.1 +TT |  |  |
| n13 | 15 | -97.0+2.5 +TT | -97.0+2.5 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n14 | 15 | -97.0+2.5 +TT | -97.0+2.5 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n20 | 15 | -97.0 +2.5+TT | -97.0 +2.5+TT | -97.0 +2.5+TT | -97.0 +2.5+TT | FDD |
|  | 30 |  | -94.1 +TT | -94.1 +TT | -94.1 +TT |  |
| n24 | 15 | -100.0 +2.5+TT | -100.0 +2.5+TT |  |  | FDD |
|  | 30 |  | -97.1 +TT |  |  |  |
| n25 | 15 | -96.5 +2.5+TT | -96.5 +2.5+TT | -96.5 +2.5+TT | -96.5 +2.5+TT | FDD |
|  | 30 |  | -93.6 +TT | -93.6 +TT | -93.6 +TT |  |
| n26 | 15 | -97.5+2.5 +TT | -97.5+2.5 +TT | -97.5+2.5 +TT | -97.5+2.5 +TT | FDD |
|  | 30 |  | -94.8 +TT | -94.8 +TT | -94.8 +TT |  |
| n28 | 15 | -98.5 +2.5+TT | -98.5 +2.5+TT | -98.5 +2.5+TT | -98.5 +2.5+TT | FDD |
|  | 30 |  | -95.6 +TT | -95.6 +TT | -95.6 +TT |  |
| n30 | 15 | -99.0 +2.5+TT | -99.0 +2.5+TT |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |
| n65 | 15 | -99.5 +2.5+TT | -99.5 +2.5+TT | -99.5 +2.5+TT | -99.5 +2.5+TT | FDD |
|  | 30 |  | -96.6 +TT | -96.6 +TT | -96.6 +TT |  |
| n66 | 15 | -99.5 +2.5+TT | -99.5 +2.5+TT | -99.5 +2.5+TT | -99.5 +2.5+TT | FDD |
|  | 30 |  | -96.6 +TT | -96.6 +TT | -96.6 +TT |  |
| n70 | 15 | -100.0 +2.5 +TT | -100.0 +2.5 +TT | -100.0 +2.5 +TT | -100.0 +2.5 +TT | FDD |
|  | 30 |  | -97.1 +TT | -97.1 +TT | -97.1 +TT |  |
| n71 | 15 | -97.2+2.5 +TT | -97.2+2.5 +TT | -97.2+2.5 +TT | -97.2+2.5 +TT | FDD |
|  | 30 |  | -94.3 +TT | -94.3 +TT | -94.3 +TT |  |
| n74 | 15 | -99.53 +2.5 +TT | -99.53 +2.5 +TT | -99.53 +2.5 +TT | -99.53 +2.5 +TT | FDD |
|  | 30 |  | -96.63 +TT | -96.63 +TT | -96.63 +TT |  |
| n85 |  | 15 | -97+2.5 +TT | -97+2.5 +TT | -97+2.5 +TT |  | FDD |
|  |  | 30 |  | -94.1 +TT | -94.1 +TT |  |  |
| n105 |  | 15 | -97.25 +2.5 +TT | -97.25 +2.5 +TT | -97.25 +2.5 +TT | -97.25 +2.5 +TT | FDD |
|  |  | 30 |  | -94.3 +2.5 +TT | -94.3 +2.5 +TT | -94.3 +2.5 +TT |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: That the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. NOTE 5: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |

Table 7.3I.3.5-4: Single antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS/Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15 | -97.1 +2.5 -3+TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n411 | 15 | 5, 10, 15, 20 | -94.8 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 +2.5 -3+TT |  |
| n481 | 15 | 5, 10, 15, 20 | -99 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1+2.5 -3+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n51 | 15 | 5 | -100 +2.5+TT | TDD |
| n53 | 15 | 5, 10 | -100 +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5 -3+TT |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 +2.5 -3.2 +TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 +2.5 -3 +TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 +2.5 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 +2.5 -3 +TT |  |
| n791 | 15 | 10, 20, | -95.8 +2.5+TT | TDD |
|  | 30 | 10, 20, | -96.1 +2.5 -3+TT |  |
| n91 | 15 | 5 | -100+2.5+TT | FDD |
| n92 | 15 | 5,10,15,20 | -100 +2.5+TT | FDD |
|  | 30 | 10,15,20 | -97.1 +2.5 -3+TT |  |
| n93 | 15 | 5 | -100 +2.5+TT | FDD |
| n94 | 15 | 5,10,15,20 | -100 +2.5+TT | FDD |
|  | 30 | 10,15,20 | -97.1 +2.5 -3+TT |  |
| n101 | 15 | 5, 10 | -100 +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5 -3+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |

Table 7.3I.3.5-5: Two antenna port reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n2 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n3 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n5 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n7 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n8 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n12 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT |  |  |
| n13 | 15 | -97.8 +TT | -97.8 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT |  |  |  |
| n14 | 15 | -97.8 +TT | -97.8 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT |  |  |  |
| n20 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n24 | 15 | -100.0 +TT | -100.0 +TT |  |  | HD-FDD |
|  | 30 |  | -97.2 -3+TT |  |  |  |
| n25 | 15 | -97.3 +TT | -97.3 +TT | -97.3 +TT | -97.3 +TT | HD-FDD |
|  | 30 |  | -94.5 -3+TT | -94.5 -3+TT | -94.5 -3+TT |  |
| n26 | 15 | -98.3 +TT | -98.3 +TT | -98.3 +TT | -98.3 +TT | HD-FDD |
|  | 30 |  | -95.5 -3+TT | -95.5 -3+TT | -95.5 -3+TT |  |
| n28 | 15 | -99.3 +TT | -99.3 +TT | -99.3 +TT | -99.3 +TT | HD-FDD |
|  | 30 |  | -96.5 -3+TT | -96.5 -3+TT | -96.5 -3+TT |  |
| n30 | 15 | -99.5 +TT | -99.5 +TT |  |  | HD-FDD |
|  | 30 |  | -96.7 -3+TT |  |  |  |
| n65 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n66 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n70 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n71 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | HD-FDD |
|  | 30 |  | -95.2 -3+TT | -95.2 -3+TT | -95.2 -3+TT |  |
| n74 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n85 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT |  |  |
| n91 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n92 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n93 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n94 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n105 |  | -98.03 +TT | -98.03 +TT | -98.03 +TT | -98.03 +TT | HD-FDD |
|  |  |  | -95.2 -3+TT | -95.2 -3+TT | -95.2 -3+TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. NOTE 3: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |

Table 7.3I.3.5-6: Single antenna port Reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n2 | 15 | -96.3 +TT | -96.3 +TT | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  | -93.5 -3+TT | -93.5 -3+TT | -93.5 -3+TT |  |
| n3 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT | -92.5 -3+TT |  |
| n5 | 15 | -96.3 +TT | -96.3 +TT | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  | -93.5 -3+TT | -93.5 -3+TT | -93.5 -3+TT |  |
| n7 | 15 | -96.3 +TT | -96.3 +TT | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  | -93.5 -3+TT | -93.5 -3+TT | -93.5 -3+TT |  |
| n8 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT | -92.5 -3+TT |  |
| n12 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT |  | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT |  |  |
| n13 | 15 | -95.3 +TT | -95.3 +TT |  |  | HD-FDD |
|  | 30 |  | -92.5 -3+TT |  |  |  |
| n14 | 15 | -95.3 +TT | -95.3 +TT |  |  | HD-FDD |
|  | 30 |  | -92.5 -3+TT |  |  |  |
| n20 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT | -92.5 -3+TT |  |
| n24 | 15 | -97.5 +TT | -97.5 +TT |  |  | HD-FDD |
|  | 30 |  | -94.7 -3+TT |  |  |  |
| n25 | 15 | -94.8 +TT | -94.8 +TT | -94.8 +TT | -94.8 +TT | HD-FDD |
|  | 30 |  | -92.0 -3+TT | -92.0 -3+TT | -92.0 -3+TT |  |
| n26 | 15 | -95.8 +TT | -95.8 +TT | -95.8 +TT | -95.8 +TT | HD-FDD |
|  | 30 |  | -93.0 -3+TT | -93.0 -3+TT | -93.0 -3+TT |  |
| n28 | 15 | -96.8 +TT | -96.8 +TT | -96.8 +TT | -96.8 +TT | HD-FDD |
|  | 30 |  | -94.0 -3+TT | -94.0 -3+TT | -94.0 -3+TT |  |
| n30 | 15 | -97.0 +TT | -97.0 +TT |  |  | HD-FDD |
|  | 30 |  | -94.2 -3+TT |  |  |  |
| n65 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n66 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n70 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n71 | 15 | -95.5 +TT | -95.5 +TT | -95.5 +TT | -95.5 +TT | HD-FDD |
|  | 30 |  | -92.7 -3+TT | -92.7 -3+TT | -92.7 -3+TT |  |
| n74 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n85 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT |  | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT |  |  |
| n91 | 15 | -97.5 +TT |  |  |  | HD-FDD |
| n92 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n93 | 15 | -97.5 +TT |  |  |  | HD-FDD |
| n94 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n105 |  | -95.53 +TT | -95.53 +TT | -95.53 +TT | -95.53 +TT | HD-FDD |
|  |  |  | -92.7 -3+TT | -92.7 -3+TT | -92.7 -3+TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. NOTE 3: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |

Table 7.3I.3.5-7: Test Tolerance (TT) for RX sensitivity level for eRedCap UE

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

-------- End of changes --------