3GPP TSG-RAN5 Meeting #106	R5-250744r1

# Athens, Greece, Feb 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1891 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases 6.5H.2.3 OOBE for inter-band EN-DC with UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test cases 6.5H.2.3 out of band emissions for inter-band EN-DC with UL MIMO are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the new test cases 6.5H.2.3 out of band emissions for inter-band EN-DC with UL MIMO. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5H.2.3(new), 6.5H.2.3.0(new), 6.5H.2.3.1(new), 6.5H.2.3.2(new), 6.5H.2.3.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The 1st revision: - Adding the title for 6.5H.2. - Adding “Void” to 6.5H.2.1 and 6.5H.2.2. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5H	Output RF spectrum emissions for DC with UL MIMO

6.5H.1

6.5H.2	Out-of-band emissions for DC with UL MIMO

6.5H.2.1	Void

6.5H.2.2	Void

6.5H.2.3	Out-of-band emissions for inter-band EN-DC with UL MIMO within FR1

6.5H.2.3.0	Minimum conformance requirements

Unless otherewise stated, the OOBE requirements specified in clause 6.6.2.1 of TS 36.101, sub- clause 6.6.2 of TS 36.101 and clause 6.5D.2 of TS 38.101-1 apply for each component carrier.

The normative reference for this requirement is TS 38.101-3 [4] subclause 6.5H.2.3.

6.5H.2.3.1	Spectrum emission mask for inter-band EN-DC with UL MIMO within FR1

6.5H.2.3.1.1	Test purpose

Same test purpose as in clause 6.5D.2.2 in TS 38.521-1 [8] for the NR carrier.

6.5H.2.3.1.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band EN-DC with UL MIMO.

6.5H.2.3.1.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5H.2.3.0.

6.5H.2.3.1.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.5D.2.2 Spectrum emission mask for UL MIMO in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.5D.2.2 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.5D.2.2.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.5D.2.2.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.5D.2.2.4.2 in TS 38.521-1 [8].

6.5H.2.3.1.5	Test requirements

Same test requirement as 6.5D.2.2 for the band supporting UL MIMO.

6.5H.2.3.2	Additional spectrum emission mask for inter-band EN-DC with UL MIMO within FR1

6.5H.2.3.2.1	Test purpose

Same test purpose as in clause 6.5D.2.3 in TS 38.521-1 [8] for the NR carrier.

6.5H.2.3.2.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band EN-DC with UL MIMO.

6.5H.2.3.2.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5H.2.3.0.

6.5H.2.3.2.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.5D.2.3 Additional spectrum emission mask for UL MIMO in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.5D.2.3 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.5D.2.3.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.5D.2.3.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.5D.2.3.4.2 in TS 38.521-1 [8].

6.5H.2.3.2.5	Test requirements

Same test requirement as 6.5D.2.3 for the band supporting UL MIMO.

6.5H.2.3.3	NR ACLR for inter-band EN-DC with UL MIMO within FR1

6.5H.2.3.3.1	Test purpose

Same test purpose as in clause 6.5D.2.4 in TS 38.521-1 [8] for the NR carrier.

6.5H.2.3.3.2	Test applicability

This test case applies to all types of NR UE release 17 and forward that support inter-band EN-DC with UL MIMO.

6.5H.2.3.3.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.101-3 [4] clause 6.5H.2.3.0.

6.5H.2.3.3.4	Test description

If Standalone FR1 can be tested, this test is covered by 6.5D.2.4 NR ACLR for UL MIMO in TS 38.521-1 [8].

If Standalone FR1 cannot be tested (due to test case not being complete or if the UE does not support the band in SA mode), LTE anchor agnostic below is applied.

Same test description as in clause 6.5D.2.4 in TS 38.521-1 [8] for the NR carrier with the following exceptions:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1.

For Initial conditions as in clause 6.5D.2.4.4.1 in TS 38.521-1 [8], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for the E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3 with E-UTRA channel bandwidth and test frequencies defined in Table 4.6-1.

3.1.	Downlink E-UTRA signals are initially set up according to TS 36.521-1 [10] Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.1.	The E-UTRA UL Reference Measurement channels are set according to Table 4.6-1.

Step 6 of Initial conditions as in clause 6.5D.2.4.4.1 in TS 38.521-1 [8] is replaced by the following two steps:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

Same test procedure as in clause 6.5D.2.4.4.2 in TS 38.521-1 [8].

6.5H.2.3.3.5	Test requirements

Same test requirement as 6.5D.2.4 for the band supporting UL MIMO.

<End of Changes>